3GPP TSG-RAN2 Meeting #129bis	R2- 2502849

# Wuhan, China, Apr 7th – 11st, 2025

Agenda item:		8.7.4.2 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on DSR enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to specify the DSR enhancements with multiple pairs of remaining time and buffer size for an LCG.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This document continues discussion on further details of DSR enhancement for XR.

2.	Discussion

Clarification on non-delay critical data ahead of delay-critical data

In RAN2#128 meeting [2], it is agreed to support including the non-delay critical data ahead of delay-critical data in buffer size calculation of Rel-19 DSR, with a capability indication:

The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

However, during the post-email discussion on PDCP running CR [3], there are different interpretations on the above agreement, and the following open issue is remained for further discussion:

- Whether the RAN2 agreement “non-delay critical data ahead of delay critical data” applies non-delay-reporting data ahead of delay-reporting data.

In detail, there are different views on which data can be included as a delay-reporting data volume in Rel-19 DSR, if the data is buffered ahead of the delay-reporting data associated with its delay-reporting threshold:

- Interpretation 1: For all delay-reporting threshold, non-delay-reporting data ahead of delay-reporting data can be reported, based on UE capability indication.

- Interpretation 2: Only non-delay-critical data ahead of delay-critical data can be reported, based on UE capability indication. In other words, for reporting threshold larger than the triggering threshold, the non-delay reporting data ahead of delay-reporting data associated with the reporting threshold is not calculated as delay-reporting data volume.

For example, if PDCP SDU is buffered as Figure 1 and pdu-SetDiscard is not configured, the delay-reporting data volume calculation for each interpretation will be as follows, if other data (i.e., which is not identified as delay-reporting data) ahead of delay-reporting data is reported as delay-reporting data:

Figure 1.

- For Interpretation 1:

  - Delay reporting data volume associated with the 1st reporting threshold (= 10ms): 250 bytes

    - Delay reporting PDCP SDU: SDU2

    - Other data (i.e., PDCP SDU ahead of delay-reporting data): SDU1

  - Delay reporting data volume associated with the 2nd reporting threshold (= 20ms): 700 bytes

    - Delay reporting PDCP SDU: SDU3, SDU4

    - Other data (i.e., PDCP SDU ahead of delay-reporting data): none

  - Delay reporting data volume associated with the 3rd reporting threshold (= 30ms): 3300 bytes

    - Delay reporting PDCP SDU: SDU6, SDU7, SDU9

    - Other data (i.e., PDCP SDU ahead of delay-reporting data): SDU5, SDU8

- For Interpretation 2:

  - Delay reporting data volume associated with the 1st reporting threshold (= 10ms): 250 bytes

    - Delay reporting PDCP SDU (= delay-critical data): SDU2

    - Other data (i.e., PDCP SDU ahead of delay-critical data): SDU1

  - Delay reporting data volume associated with the 2nd reporting threshold (= 20ms): 700 bytes

    - Delay reporting PDCP SDU: SDU3, SDU4

  - Delay reporting data volume associated with the 3rd reporting threshold (= 30ms): 1650 bytes

    - Delay reporting PDCP SDU: SDU6, SDU7, SDU9

For Interpretation 2, it defines different principle to calculate delay-reporting data volume for each reporting threshold, based on the relationship between the reporting threshold and the triggering threshold, i.e., whether the reporting threshold is less than or equal to the triggering threshold or not. However, there is no critical reason to include the non-delay reporting data ahead of the delay-reporting data only for delay-critical data. On the other hand, from PDCP point of view, applying different principle for delay-reporting data volume calculation for each reporting threshold would be complicated, so it is preferred to apply the unified principle for data volume calculation for all reporting thresholds based on the configuration or capability indication. In addition, for data volume calculation of Rel-19 DSR, there is no need to calculate delay-critical data volume, i.e., only the delay-reporting data volume associated with each reporting threshold would be calculated. Therefore, it is not reasonable to mix the delay-critical data for delay-reporting data volume calculation in Rel-19 DSR.

In our understanding, the intention of the above agreement is to report the exact buffer size to transmit all data whose remaining time till PDCP discard expiry is less than reporting time threshold for Rel-19 DSR MAC CE. In this sense, the RAN2#128 agreement should be applied for the delay-reporting data associated with each reporting time threshold (i.e., Interpretation 1), rather than delay-critical data which is determined by triggering threshold.

Therefore, to avoid the confusion on the existing delay-critical data definition, the intention of the above agreement should be interpreted as “the UE may also support including other data (i.e., which is not identified as a delay-reporting data) ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication”

Proposal 1. Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication”

How to determine non-delay-reporting data ahead of delay-reporting data

During the post e-mail discussion on PDCP running CR [3], there were different views on how to identify the non-delay-reporting data ahead of delay-reporting data, and following open issue is remained:

- Whether to use COUNT to describe the non-delay-critical data ahead of delay-critical data.

When the UE considers non-delay-critical data ahead of delay reporting data for Rel-19 DSR, the delay reporting data volume should further consider all the buffered data ahead of last buffered delay-reporting data associated with the reporting threshold, in addition to the delay-reporting data associated with this reporting threshold. However, from the specification point of view, it is hard to describe the queue status, given that the order of buffered data is determined based on the UE implementation, unless the COUNT value is assigned to the UE in the PDCP layer. In other words, unless the COUNT value is assigned to the PDCP SDU, the UE may determine the processing order of buffered data by UE implementation, regardless of its actual queue status.

Given that the transmitting PDCP entity allocates COUNT value when the PDCP entity receives PDCP SDU from the upper layer, it can be assumed that COUNT value is allocated for PDCP SDUs in ascending order, i.e., the order of buffered data can be identified based on the COUNT value.

In our understanding, one of the easiest ways to identify “non-delay-reporting data ahead of last buffered delay-reporting data associated with the reporting threshold” is to use the COUNT value, which is assigned by the ascending order in the PDCP layer. Therefore, in order to report all the buffered data ahead of last buffered delay-reporting data associated with i:th reporting threshold, delay-reporting data volume should further include PDCP SDUs associated with a COUNT value smaller than the largest COUNT value of the delay-reporting PDCP SDU.

Proposal 2. Use COUNT value to determine non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation.

Capability indication to include non-delay reporting data ahead of delay reporting data

In RAN2#128 meeting [2], it is agreed to indicate the UE capability to the network on whether the UE supports inclusion of other data (i.e., which is not identified as a delay-reporting data) ahead of delay-reporting data for buffer size calculation of Rel-19 DSR. Since the intention of the agreement is to have a common understanding between the network and the UE on delay-reporting data volume calculation, it would be straightforward to define the UE behaviour on Rel-19 DSR based on the network configuration, including whether to include other data ahead of delay-reporting data for delay-reporting data volume calculation. Here, the network may configure whether to include other data ahead of delay-reporting data, after the network receives the UE capability on whether the UE supports the corresponding functionality.

Proposal 3. In the Rel-19 DSR configuration, the network configures whether to include other data ahead of delay-reporting data for delay-reporting data volume calculation, based on UE capability signalling.

Handling of PDCP/RLC control PDU and PDCP/RLC data to be retransmitted for delay-reporting data volume

In Rel-18 XR, it was discussed whether the PDCP control PDUs and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs should be considered as delay-critical PDCP data volume. Based on the discussion in Rel-18 XR, it was agreed to include the PDCP control PDUs as delay-critical PDCP data volume, since PDCP control PDUs and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs are expected to be prioritised over other delay-critical PDCP data PDUs.

In RAN2#129 meeting, it was further discussed on how to report the PDCP control PDUs and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs in Rel-19 DSR [4], and the following Options are identified for further discussion

Choose one of the followings. - Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting. - Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold. - Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

In detail, there are different views on whether the PDCP control PDUs and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs should be reported with the shortest “configured” threshold or shortest “reported” threshold.

If the PDCP control PDU and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs is reported with the shortest “reported” threshold, it could avoid unnecessary overhead, when there is no delay-reporting data associated with the shortest configured threshold.

However, given that the LCG may include multiple logical channels and the delay-reporting data volume calculation is performed in PDCP, it could cause additional UE complexity to consider inter-PDCP entity dependencies. For example, assume that LCG1 includes LCH1 and LCH2, and PDCP entity 1 is associated with LCH1 and PDCP entity 2 is associated with LCH2. If the reporting thresholds are configured as [10ms, 20ms, 30ms], then, for PDCP control PDU generated in PDCP entity 1, the delay status of data buffered in PDCP entity 2 should be checked in order to determine which reporting threshold should be considered for data volume calculation for PDCP control PDU (as depicted in Figure 2), considering that data volume calculation for each reporting threshold is performed in PDCP layer.

Figure 2.

If more than two PDCP entities are associated with an LCG, the UE complexity to consider the delay status of data in multiple PDCP entities would be further increased.

Therefore, in order to avoid any inter-PDCP entity dependencies and to simplify the UE operation for delay-reporting data volume calculation in Rel-19 DSR, the PDCP control PDUs and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs should be reported with the shortest “configured” threshold.

Proposal 4. The PDCP entity should consider followings for delay-reporting PDCP data volume calculation associated with the first configured reporting threshold:

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

Similarly, in RLC layer, followings should be considered as delay-reporting RLC data volume associated with the shortest “configured” reporting threshold as well, in order to avoid any inter-DRB dependency when more than one LCHs are associated with one LCG:

- RLC data PDUs that are pending for retransmission (RLC AM).

- Estimated size of STATUS PDU.

Proposal 5. The RLC entity should consider followings for delay-reporting RLC data volume calculation associated with the first configured reporting threshold:

- RLC data PDUs that are pending for retransmission (RLC AM).

- Estimated size of STATUS PDU.

Meanwhile, if proposal 4 and proposal 5 are agreed, it could be ambiguous how to set the remaining time field, if there is no other data associated with the first reporting threshold, given that PDCP/RLC control PDU and retransmitted data are not associated with discard timer. In our view, for this specific case, the remaining time field in Rel-19 DSR MAC CE can be set to 0, which can explicitly indicate that the data volume associated with the shortest reporting time threshold only includes the PDCP/RLC control PDU and retransmitted data.

Proposal 6. If the delay-reporting data volume associated with shortest reporting threshold only includes the PDCP/RLC control PDU and retransmitted data, the remaining time field is set to 0.

Rel-19 DSR MAC CE vs. Rel-18 DSR MAC CE

In RAN2#129 meeting [5], it was agreed that the truncated version DSR MAC CE or the fallback procedure from R19 DSR MAC CE to R18 DSR MAC CE is not supported for the limited size of PUSCH resource. However, it was remained as FFS on whether R19 DSR MAC CE should be always used if at least one LCG is configured with multiple thresholds.

FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size.

On the other hand, in the details of Rel-19 DSR MAC CE, it is agreed to include each portion of remaining time only if the amount of buffered data is larger than 0, in order to reduce the overhead of Rel-19 DSR MAC CE. In other words, it is possible that only one portion can be reported for each LCG in Rel-19 DSR MAC CE, if there is delay-reporting data for only one reporting threshold for each LCG. Then, there would be no harm to use Rel-19 DSR MAC CE in any case, if the reporting threshold(s) are configured for at least for one LCG.

Therefore, for simplicity, it would be better to not mix the usage of Rel-18 DSR MAC CE and Rel-19 DSR MAC CE, i.e., always use Rel-19 DSR MAC CE when the UE is configured with reporting threshold for at least one LCG.

Proposal 7. When the UE is configured with reporting thresholds for at least one LCG configured with, the UE always transmits Rel-19 DSR MAC CE.

Delay Status Information for the case of BS = 0

In RAN2#127bis meeting [6], it is agreed that Rel-19 DSR includes the buffer size and the shortest remaining time associated with the reporting threshold, only if the corresponding Buffer size is larger than zero.

- For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion.

However, in the running MAC CR, it is implemented such that the delay status information associated with the reporting threshold should be reported consecutively in ascending order and may not be included in the Rel-19 DSR MAC CE, i.e., based on UE implementation, if the corresponding Buffer size is zero.

In the Multiple Entry DSR MAC CE, as illustrated in Figure 6.1.3.72-2, the delay status information associated with a reporting threshold, which includes the BT, the EXT, the Remaining Time and the Buffer Size fields, shall be reported in two consecutive octets. If an LCG is configured with more than one reporting threshold, the delay status information associated with different reporting thresholds in the LCG should be reported consecutively in ascending order based on the values of the reporting thresholds. The delay status information associated with a reporting threshold may not be reported if the total amount of UL data associated with it is zero, according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively. The delay status information for different LCGs should be included in the Multiple Entry DSR MAC CE in ascending order based on the LCGi.

In the specification, “consecutively” means that there should be no missing element between two elements. Thus, our interpretation of “reported consecutively in ascending order” is that if an LCG is configured with more than one reporting threshold, the delay status information of the LCG should be reported regardless of Buffer Size. In other words, the LCGi shall be always set to 1, if the LCG is configured with more than one threshold. However, the next sentence says that if a Buffer Size of a certain reporting threshold is zero, the delay status information for the reporting threshold may be omitted.

Then, we have a question what if all reporting thresholds of the LCG have zero Buffer Size. If LCGi is set to 1, there should be at least one delay status information of reporting threshold. However, the UE can omit delay status information for all reporting thresholds of the LCG, in which case the network cannot decode the DSR MAC CE.

Therefore, if all reporting thresholds of the LCG have zero Buffer Size, the UE shall set the LCGi to 0. This means that “consecutively” should be removed.

Proposal 8. Remove “consecutively” from the text “If an LCG is configured with more than one reporting threshold, the delay status information associated with different reporting thresholds in the LCG should be reported consecutively in ascending order based on the values of the reporting thresholds”.

3.	Conclusion

In this contribution, it is discussed for the DSR enhancement for XR. This document includes following proposals.

Proposal 1. Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication”

Proposal 2. Use COUNT value to determine non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation.

Proposal 3. In the Rel-19 DSR configuration, the network configures whether to include other data ahead of delay-reporting data for delay-reporting data volume calculation, based on UE capability signalling.

Proposal 4. The PDCP entity should consider followings for delay-reporting PDCP data volume calculation associated with the first configured reporting threshold:

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

Proposal 5. The RLC entity should consider followings for delay-reporting RLC data volume calculation associated with the first configured reporting threshold:

- RLC data PDUs that are pending for retransmission (RLC AM).

- Estimated size of STATUS PDU.

Proposal 6. If the delay-reporting data volume associated with shortest reporting threshold only includes the PDCP/RLC control PDU and retransmitted data, the remaining time field is set to 0.

Proposal 7. When the UE is configured with reporting thresholds for at least one LCG configured with, the UE always transmits Rel-19 DSR MAC CE.

Proposal 8. Remove “consecutively” from the text “If an LCG is configured with more than one reporting threshold, the delay status information associated with different reporting thresholds in the LCG should be reported consecutively in ascending order based on the values of the reporting thresholds”.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA, 18 - 22 November, 2024

[3] R2-2501951 Discussion of [POST129][511][XR] PDCP running CR (LGE)

[4] R2-2501348 Report of [AT129][504][XR] Data volume calculation for DSR

[5] Report of 3GPP TSG RAN WG2 meeting #129, Athens, Greece, 17-21 February, 2025

[6] Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024