3GPP TSG-RAN WG2 #129bis	R2-2502853

Wuhan, China, 7th – 11th Apr. 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on RLC enhancements

Document for:        	Discussion and decision

Agenda Item:         	8.7.5		RLC enhancements

- Introduction

In RAN #107, revised WID on Rel-19 XR enhancements was approved [1], and the following objective was specified regarding RLC enhancements.

- Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

And the following agreements related to RLC enhancements were reached in RAN2#129 meeting.

Agreements on RLC enhancements Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

In this contribution, we provide the discussion on RLC enhancements.

- Discussion

Timely RLC retransmission

In RAN2#128 meeting, it was agreed that autonomous retransmission and/or polling would be triggered when the remaining time of the RLC SDU falls below a specified threshold. However, there is FFS on whether the remaining time is determined based on the existing discardTimer or a new timer at RLC layer. In our opinion, it would be preferable to reuse the existing discardTimer for simplicity. If the new timer for timely RLC retransmission is introduced, the UE will need to maintain many additional timers, which may complicate the UE operation. Furthermore, the new timer may not accurately reflect the actual delay status of each SDU (i.e., the remaining time of discardTimer associated with the SDU). For example, if the new timer starts too early, it may trigger retransmission and/or polling for data that is not delay-critical. Conversely, if it starts too late, it may trigger retransmission and/or polling for data that has already been discarded. From these perspectives, we propose that autonomous retransmission and/or polling would be triggered based on the remaining time of the existing discardTimer.

Proposal 1:	Autonomous retransmission and/or polling would be triggered based on the remaining time of the existing discardTimer.

For autonomous retransmission, it is necessary to consider avoiding unnecessary or redundant retransmissions to mitigate the capacity impact. For example, there is no need to retransmit an RLC SDU that has received a discard indication from the upper layer or an RLC SDU that has been positively acknowledged by a status report. Furthermore, we need to consider the case where a negative acknowledgement for an RLC SDU has already been received when its remaining time falls below the threshold. In this case, retransmitting the same SDU twice would result in resource waste. Therefore, it would be better to rely on the legacy retransmission due to the NACK rather than additionally triggering autonomous retransmission. Another case to discuss is whether to trigger retransmission for an RLC SDU that has not previously been transmitted. This may occur if the initial transmission is delayed and remains pending for a long period. In this case as well, the UE can wait for the initial transmission to be performed without triggering autonomous retransmission.

Proposal 2:	To mitigate the capacity impact, UE can ensure that autonomous retransmission is not triggered for the following data.

- RLC SDU that has received a discard indication from the upper layer

- RLC SDU that has been positively acknowledged by a status report

- RLC SDU pending for retransmission (e.g., due to a negative acknowledgement)

- RLC SDU pending for initial transmission

Avoid unnecessary RLC retransmissions

RAN2 agreed that a new RLC timer (i.e., t-RxDiscard) at the Rx side would be introduced to determine obsolete RLC SDUs, and t-RxDiscard would start per RLC entity based on the detection of an SN gap. When t-RxDiscard expires, the receiving side of an RLC entity can abandon the obsolete RLC SDUs and can update RX_Next to advance the receiving window. One issue would be whether to trigger a status report based on the expiry of t-RxDiscard. The status report can indicate positive acknowledgements for the obsolete RLC SDUs to the transmitting side. This allows the transmitting window to be advanced as well, addressing the window stalling issue. Thus, it would be beneficial to introduce a new triggering condition for the status report based on t-RxDiscard to advance the transmitting window as soon as possible.

Proposal 3:	RAN2 to introduce a new triggering condition for a status report based on the expiry of a new RLC timer (i.e., t-RxDiscard) to advance the transmitting window as soon as possible.

When the status report is triggered, the RLC entity constructs a STATUS PDU for RLC SDUs with SN which is determined based on RX_Next and RX_Highest_Status. RX_Highest_Status holds the highest possible value of the SN which can be indicated by ACK_SN. However, after the expiry of t-RxDiscard, RX_Next may be updated to a value larger than RX_Highest_Status. In such a scenario, it may not be clear which SN to set ACK_SN to in the STATUS PDU. Therefore, the status report should be triggered after both RX_Next and RX_Highest_Status are updated based on the expiry of t-RxDiscard.

Observation 1:	An RLC entity constructs a STATUS PDU for RLC SDUs with SN which is determined based on RX_Next and RX_Highest_Status when a status report is triggered.

Observation 2:	RX_Next may be updated to a value larger than RX_Highest_Status after the expiry of t-RxDiscard. In such a scenario, it may not be clear which SN to set ACK_SN to in a STATUS PDU.

Proposal 4:	If a new triggering condition based on the expiry of t-RxDiscard is introduced, a status report should be triggered after RX_Next and RX_Highest_Status are updated.

We also need to consider the case where an RLC SDU that triggered t-Reassembly become obsolete (i.e., RX_Next_Status_Trigger falls outside of receiving window) after t-RxDiscard has expired. If this t-Reassembly expires, a status report for the obsolete SDU would be triggered. However, if t-RxDiscard-based trigger is supported, this status report would be redundant. Therefore, it would be preferable to stop t-Reassembly which is still running for the obsolete RLC SDU. Furthermore, in the current CR [2], if there is still an SN gap even after t-RxDiscard has expired and RX_Next has been updated, t-RxDiscard will be started again. In this case, it would be necessary to start t-Reassembly at the same time. The reason is that if t-Reassembly is not started, an RLC SDU may become obsolete without requesting retransmission via the status report triggered by t-Reassembly. From these reasons, the following additional changes can be considered.

R2-2502162, Running RLC CR for XR [2] 5.2.3.2.x Actions when t-RxDiscard expires When t-RxDiscard expires, the receiving side of an AM RLC entity shall: - discard the AMD PDU(s) in the reception buffer with SN < RX_Next_Discard_Trigger, if any; - update RX_Next to the SN of the first RLC SDU with SN >= RX_Next_Discard_Trigger for which not all bytes have been received; - if t-Reassembly is running; - if RX_Next_Status_Trigger = RX_Next; or - if RX_Next_Status_Trigger = RX_Next + 1 and there is no missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU; or - if RX_Next_Status_Trigger falls outside of the receiving window and RX_Next_Status_Trigger is not equal to RX_Next + AM_Window_Size: - stop and reset t-Reassembly. - if RX_Next_Highest > RX_Next +1; or - if RX_Next_Highest = RX_Next + 1 and there is at least one missing byte segment of the SDU associated with SN = RX_Next before the last byte of all received segments of this SDU: - start t-RxDiscard; - set RX_Next_Discard_Trigger to RX_Next_Highest. - if t-Reassembly is not running (includes the case t-Reassembly is stopped due to actions above); - start t-Reassembly; - set RX_Next_Status_Trigger to RX_Next_Highest.

Observation 3:	If t-RxDiscard-based status report trigger is supported, a status report triggered by t-Reassembly associated with an obsolete RLC SDU would be redundant.

Proposal 5:	An RLC entity can stop t-Reassembly which is still running for an obsolete RLC SDU.

Proposal 6:	If there is still an SN gap even after t-RxDiscard has expired and RX_Next has been updated, an RLC entity can start t-Reassembly as well as t-RxDiscard.

- Conclusion

In this contribution, we provided the following observations and proposals.

Timely RLC retransmission

Proposal 1:	Autonomous retransmission and/or polling would be triggered based on the remaining time of the existing discardTimer.

Proposal 2:	To mitigate the capacity impact, UE can ensure that autonomous retransmission is not triggered for the following data.

- RLC SDU that has received a discard indication from the upper layer

- RLC SDU that has been positively acknowledged by a status report

- RLC SDU pending for retransmission (e.g., due to a negative acknowledgement)

- RLC SDU pending for initial transmission

Avoid unnecessary RLC retransmissions

Proposal 3:	RAN2 to introduce a new triggering condition for a status report based on the expiry of a new RLC timer (i.e., t-RxDiscard) to advance the transmitting window as soon as possible.

Observation 1:	An RLC entity constructs a STATUS PDU for RLC SDUs with SN which is determined based on RX_Next and RX_Highest_Status when a status report is triggered.

Observation 2:	RX_Next may be updated to a value larger than RX_Highest_Status after the expiry of t-RxDiscard. In such a scenario, it may not be clear which SN to set ACK_SN to in a STATUS PDU.

Proposal 4:	If a new triggering condition based on the expiry of t-RxDiscard is introduced, a status report should be triggered after RX_Next and RX_Highest_Status are updated.

Observation 3:	If t-RxDiscard-based status report trigger is supported, a status report triggered by t-Reassembly associated with an obsolete RLC SDU would be redundant.

Proposal 5:	An RLC entity can stop t-Reassembly which is still running for an obsolete RLC SDU.

Proposal 6:	If there is still an SN gap even after t-RxDiscard has expired and RX_Next has been updated, an RLC entity can start t-Reassembly as well as t-RxDiscard.

- References

[1]	RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3

[2]	R2-2502162, RLC running CR for XR, vivo