3GPP TSG-RAN WG2 Meeting #129bis	                                 R2-2502867

Wuhan, China, Apr. 7th – 11th, 2025

Source:        ZTE Corporation

Title:           Consideration on the UEIBM and Other Issues

Agenda item:   8.12.3

Document for:  Discussion and Decision

- Introduction

In the last meeting, a working assumption was made for the UEIBM topic:

Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision.

Furthermore, a paper on the UL skipping was also presented [2] and RRC issues were discussed in the [Post129][208][ MIMO_Ph5] Issues and proposals on RRC impact (Ericsson).

In this paper, we’d like to share our views on these topics.

- Discussion

In this chapter, we first discuss the UEIBM impact on MAC layer, then discuss its impact on the UL skipping, at last we discuss the other ASN1 coding related issues.

2.1 UEIBM Impact on MAC Layer

For the UEIBM event evaluation impact on MAC layer, the below issues can be first discussed:

- Issue 1: Related Parameters configured by upper layer;

- Issue 2: When to start the UEIBM event evaluation timer and the granularity of the UEIBM event evaluation timer;

- Issue 3: When and how to indicate the lower layer to trigger the UEIBM;

- Issue 4: When to stop to UEIBM evaluation timer and reset the counting;

- Issue 5: Actions upon UEIBM evaluation timer expiry.

In the next paragraph, we’d like to discuss these issues one by one.

Issue 1: Related Parameters configured by upper layer

For the UEIBM event evaluation, according to the below agreement, obviously, the event detection timer length and event instance count would be needed for the UEIBM event evaluation.

If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

Proposal 1: RRC configures the following parameters the UEIBM procedure:

- eventDetectionTimer: timer for the UEIBM event evaluation;

- eventInstanceCount: instance count for the UEIBM event evaluation;

Issue 2: When to start the UEIBM event evaluation timer and the granularity of the UEIBM event evaluation timer

According to the RAN1 discussion, there are 4 options as below on when to start the window:

- Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured. - Option-2: The measurement window is from T_Instance – T_window to T_Instance, where T_Instance is an evaluation occasion of event instance, and T_proc is RRC configured. - The UEI beam report in the second PUSCH is based on the most recent measurement for new/current beam RS(s). - Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW. - Option-4: If an Event-2 instance for a new beam is obtained at the time t, UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: T_window is the agreed time window parameter for measurement.

These options can be divided into 2 types:

- Type 1: The start point of the window is configured, e.g. option 1 and option 3

- Type 2: The start point of the window is determined based on the first instance, e.g. option 2 and option 4.

From RAN2 perspective, the first type belongs to the fixed window mechanism, while the second one belongs to the sliding window. Furthermore according to the RAN2	discussion on the other measurement windows, the sliding window can give more flexibility and thus improve the performance better. Thus we give our second proposal as below.

Proposal 2: Use the sliding window for the UEIBM event evaluation, i.e. start the timer upon receiving the instance from the lower layer.

The second issue is about the granularity of the UEIBM event evaluation timer, there are also two types based on the RAN1’s options on table:

- Type 1: Per Event, e.g. the option 1/3, which means that all of the beams are evaluated in the same window.

- Type 2: Per beams per Event, e.g. the option 2 (maybe) and the option 4, which means each beam would have it’s own window.

Considering that RAN1 has agreed to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2, the Type 2 can provide more accurate results, and thus the Type 2 is more preferred.

RAN1#120 Agreement On UE-initiated/event-driven beam reporting, Option-1 is supported as an extension on L1-RSRP report format depending on Event-2. - Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2. - The presence of this field is enabled by RRC with subjective to UE capability. - Note: The presence of this field is only for the case that N > 1 and the time window and M are configured - Above is NOT applicable for event 1.

Take the Fig 1 as an example, if only one window was used for an event, the UE may only indicate the beam 1 that satisfies the event trigger condition.

Fig 1: UEIBM Event Evaluation

Proposal 3: The UEIBM event evaluation timer shall be managed per beam per Event.

Issue 3: When and how to indicate the lower layer to trigger the UEIBM

According to the RAN1’s agreement, once the instances achieve the maximum value M, the UE initiated beam report occurs. Thus a UEIBM_COUNTER should be introduced per beam per event, and once this counter is greater than or equal to a configurable number M, the MAC layer shall inform the lower layer to trigger a UEIBM beam reporting procedure.

Regarding the triggering event determination for Event 2: If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam -Above feature is subject to UE capability. -Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs -FFS: Whether the above is captured in RAN1 or RAN2 specification.

Proposal 4: A UEIBM_COUNTER should be introduced per beam per event, and once this counter is greater than or equal to a maximum number M, the MAC layer shall inform the lower layer to trigger a UEIBM beam reporting procedure.

Issue 4: When to stop to UEIBM evaluation timer and reset the counting

If the above proposal 1/2/3/4 were agreed, the timer should be stopped once the UEIBM_COUNTER is greater than or equal to the maximum number M. Besides, if the network reconfigure the MAC layer related parameters, e.g.

-	eventDetectionTimer: timer for the UEIBM event evaluation;

-	eventInstanceCount: instance count for the UEIBM event evaluation;

The timer should also be stopped and the counter should be reset. Furthermore, RAN1 has also agreed to reset the counting at least for the case that “indicated TCI state is updated”, which means that some indication maybe received from RAN1 to stop the evaluation procedure.

Agreement Regarding triggering event determination for Event 2, at least Candidate #2 is supported for resetting the counting. - Candidate#1: RS reconfiguration/update or MAC-CE signaling (if supported) for new beam is received; - FFS: whether to reset the counting for all new beams. - FFS: whether to maintain the counting whose new beam is NOT updated. - Candidate#2: [The measured current beam based on] indicated TCI state is updated; - In such case, the UE need to reset the counting for all new beams. - Candidate#3: UEI beam report is transmitted; - FFS: Only reset the counting of new beams fulfilling triggering condition and reported by the UEI beam report - Candidate#4: NW response (e.g., DCI in step-2 of Mode-A) is detected. - Candidate#5: The time window expires - Candidate#6: The threshold for event evaluation is re-configured by RRC signaling - (FFS) Candidate#7: The RRC parameter(s) associated with the CSI report configuration for UEI beam report is reconfigured. - FFS: RRC parameter(s) - FFS: Other candidates Note: Whether this proposal is captured in RAN1 or RAN2 is a separate discussion point.

Proposal 5: Stop the UEIBM evaluation timer and reset the counting upon:

- The UEIBM_COUNTER is greater than or equal to the maximum number M;

- Network reconfigures the MAC layer related parameters, e.g. eventDetectionTimer and eventInstanceCount;

- Receiving an indication from RAN1 to stop the evaluation procedure.

Issue 5: Actions upon UEIBM evaluation timer expiry

If the timer has expired, the only action was to reset the corresponding UEIBM_COUNTER.

Proposal 6: Reset the corresponding UEIBM_COUNTER upon the UEIBM evaluation timer expiry.

A TP as in the Annex was also provided based on the above proposals.

Proposal 7: Take the TP in the annex as baseline for the UEIBM procedure.

Considering that these proposals would also have impact on the RAN1 progress, a LS to RAN1 is suggested to include these proposals.

Proposal 8: Send an LS to RAN1 to include the above proposals.

2.2 Impact on the UL Skipping

In the last meeting, a paper on the UL skipping was discussed [2]. According to the current specification, for the aperiodic CSI requested for this PUSCH transmission, the UL grant can’t be skipped because that without MAC PDU, there may be coding error at the PHY layer. Semi-persistent CSI reporting on PUSCH is generated by the PHY layer and this procedure is invisible to the MAC layer, so it does not involve UL skipping mechanism

TS 38.321 clause 5.4.3.1.3 The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

Furthermore, according to the RAN1 agreements, for the Mode-B, the PUSCH can not carry UL-SCH but can carry any other UCI.

Agreement (R1-118bis) On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least Option-3 is supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. - Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI. FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report

For the mode A/B, if it’s similar to the legacy aperiodic CSI, some modification to the above spec would be needed to above specification on the UL skipping, e.g. to include UEIBM case. On this issue, it mainly depends on RAN1, thus it’s better to send an LS to confirm with RAN1.

Proposal 9: On the UL skipping issue, send LS to ask RAN1 whether the current UEIBR is similar to the legacy aperiodic CSI reporting or is transparent to the MAC layer.

2.3 RRC Parameters Configuration

In this paragraph, we’d like to share our views on the remaining issues in the [Post129][208][ MIMO_Ph5] Issues and proposals on RRC impact (Ericsson), i.e.

Proposal 1 FFS whether UEI BM parameters should be moved to CSI-MeasConfig. Proposal 4 Discuss which option to use for the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank: Option 2: (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2; Option 3: (N1, N2) is signaled as a separate parameter, and CBSR is optionally signaled as a variable BIT STRING.

First issue is about the whether UEI BM parameters should be moved to CSI-MeasConfig, on this issue, we can wait for RAN1’s further progress, on the other hands, we can also include this issue in the LS to RAN1 to confirm their understanding for that RAN1 has started their running CR with the “CSI-ReportConfig”.

Proposal 10: Send an LS to ask RAN1 whether it would have RAN1 impact if change the CSI-ReportConfig to the CSI-MeasConfig.

The second issue is about the the signaling of typeI-CBSR and typeII-CBSR and typeI-softScalingRank:

- Option 2: (N1, N2) can be signaled as a separate parameter, and CBSR can be signaled as a CHOICE of (X1, X2) and a CHOICE of N1N2; - Option 3: (N1, N2) is signaled as a separate parameter, and CBSR is optionally signaled as a variable BIT STRING.

The Asn.1 coding of the option 3 is as below:

n1-n2-r19		ENUMERATED {eight-three, six-four, sixteen-two, eight-four, sixteen-four, eight-eight},

typeI-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two, four-four}	OPTIONAL    -- Need R

typeII-x1-x2-r19		ENUMERATED {one-one, two-one, two-two, four-one, four-two}	OPTIONAL    -- Need R

typeI-codebookSubsetRestriction-r19		BIT STRING (SIZE (24, 32, 48, 64, 96, 128, 192, 256, 384,   512, 1024))							 OPTIONAL    -- Need R

typeII-codebookSubsetRestriction-r19		BIT STRING (SIZE (4, 6, 8, 12, 16, 24, 32, 64))		OPTIONAL    -- Need R

For the option 3, according to the ASN.1 rule, the correct format for the BIT STRING with these discrete sizes should be

typeI-codebookSubsetRestriction-r19	BIT STRING (SIZE (24|32|48|64|96|128|192|256|384|512|1024))

However, this type has not been used before, it would depends on whether the current ASN.1 compiler can support this format encoding and decoding.

Proposal 11: RAN2 to confirm whether the current ASN.1 compiler can support BIT STRING with discrete size values, e.g. BIT STRING (SIZE (a|b|c)).

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: RRC configures the following parameters the UEIBM procedure:

- eventDetectionTimer: timer for the UEIBM event evaluation;

- eventInstanceCount: instance count for the UEIBM event evaluation;

Proposal 2: Use the sliding window for the UEIBM event evaluation, i.e. start the timer upon receiving the instance from the lower layer.

Proposal 3: The UEIBM event evaluation timer shall be managed per beam per Event.

Proposal 4: A UEIBM_COUNTER should be introduced per beam per event, and once this counter is greater than or equal to a maximum number M, the MAC layer shall inform the lower layer to trigger a UEIBM beam reporting procedure.

Proposal 5: Stop the UEIBM evaluation timer and reset the counting upon:

- The UEIBM_COUNTER is greater than or equal to the maximum number M;

- Network reconfigures the MAC layer related parameters, e.g. eventDetectionTimer and eventInstanceCount;

- Receiving an indication from RAN1 to stop the evaluation procedure.

Proposal 6: Reset the corresponding UEIBM_COUNTER upon the UEIBM evaluation timer expiry.

Proposal 7: Take the TP in the annex as baseline for the UEIBM procedure.

Proposal 8: Send an LS to RAN1 to include the above proposals.

Proposal 9: On the UL skipping issue, send LS to ask RAN1 whether the current UEIBR is similar to the legacy aperiodic CSI reporting or is transparent to the MAC layer.

Proposal 10: Send an LS to ask RAN1 whether it would have RAN1 impact if change the CSI-ReportConfig to the CSI-MeasConfig.

Proposal 11: RAN2 to confirm whether the current ASN.1 compiler can support BIT STRING with discrete size values, e.g. BIT STRING (SIZE (a|b|c)).

- Reference

- R1-2502744 Offline summary for Rel-19 MIMO UEIBM (9.2.1) pre-RAN1#120b  ZTE

- R2-2501176 Enhancements for UE-initiated/event-driven beam management Huawei, HiSilicon

- Annex

5.xy	UE Initiated/Event-driven Beam Management

The MAC entity may be configured by RRC with a UE Initiated/Event-driven Beam Management (UEIBM) event evaluation procedure which is used for indicating the lower layer whether the UEIBM event(s) condition is satisfied. The UEIBM event is detected by counting event instance indication from the lower layers to the MAC entity.

RRC configures the following parameters in the [CSI-ReportUE-IBM-r19] for the UEIBM procedure:

-	eventDetectionTimer: timer for the UEIBM event evaluation;

-	eventInstanceCount: instance count for the UEIBM event evaluation;

The following UE variables are used for the UEIBM event evaluation procedure:

-	UEIBM_COUNTER: (per beam of each Event): counter for UEIBM instance indication which is initially set to 0.

The MAC entity shall for each beam of each event configured for the UEIBM event evaluation:

1>	if UEIBM instance indication has been received from lower layers:

2>	start the eventDetectionTimer if it is not running;

2>	increment UEIBM_COUNTER by 1;

2>	if UEIBM_COUNTER >= eventInstanceCount:

3>	inform the lower layer to trigger a UEIBM beam reporting procedure.

3> stop the eventDetectionTimer if it is running;

- if the eventDetectionTimer expires;

2> set UEIBM_COUNTER for each beam to 0.

1>	if [the configuration for this event] is reconfigured by upper layers: or

1>	if [a stop indication for this event] has been received from lower layers

2> stop the eventDetectionTimer if it is running;

2>	set UEIBM_COUNTER for each beam to 0.