3GPP TSG-RAN WG2 Meeting #129b	R2-2502870

Wuhan, China, 7th – 11th Apr, 2025

Agenda Item:	8.8.2

Source: 	TCL

Title:  	Discussion on DL coverage enhancements

Document for:	Discussion and Decision

- Introduction

In the past meetings, RAN2 discussion on DL coverage enhancements is focused on SMTC enhancements, DTX/DRX, and cell barring.

In this contribution, we will provide further analysis on the remaining issue of cell barring.

- Discussion on Store & Forward satellite operation

Below are the agreements related to the cell baring in the previous RAN2 meetings.

RAN2#128: 1. If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.) RAN2#127bis: 1. If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) 2. If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) RAN2#127: 1. If there is a need to bar pre-Rel19 NTN UEs from accessing a cell operating with DL coverage enhancement (e.g. because of extreme SSB periodicity) the existing NTN bar bit can be used. FFS about the behaviour for Rel-19 UEs not supporting DL coverage enhancement when the existing NTN bar bit is set.

Based on existing agreements, the existing NTN bar bit is used to control access for legacy UEs and Rel-19 UEs not supporting DL-CE, while a new barring mechanism in SIB1 is introduced to selectively bar UEs supporting DL-CE.

At the RAN2#129 meeting, companies proposed a two-tier barring mechanism, which involves introducing a new field specifically for Rel-19 UEs supporting DL-CE. This mechanism works as follows:

- If the new field is present:

- The new field controls access for Rel-19 UEs supporting DL-CE.

- The existing NTN bar bit continues to control access for legacy UEs and Rel-19 UEs not supporting DL-CE.

- If the new field is absent:

- The existing NTN bar bit controls access for all UE types (including legacy UEs, Rel-19 UEs not supporting DL-CE, and Rel-19 UEs supporting DL-CE).

We basically support this approach given the following reasons.

1. Forward Flexibility: The new field allows independent control for Rel-19 DL-CE UEs, enabling future enhancements without disrupting legacy behavior.

2. Signaling Efficiency: No additional bits are required in SIB1and the new field is optional and conditionally interpreted.  Based on the existing agreement anyway it is necessary to introduce a new field for Rel-19 UEs supporting DL-CE, this two-tier mechanism achieves separate control over the new Rel-19 NTN DL-CE feature without introducing additional bits in SIB1.

3. Backward Compatibility: Legacy UEs and non-DL-CE Rel-19 UEs remain governed by the existing NTN bar bit.

- Support the introduction of a new field for Rel-19 UEs supporting DL-CE, implementing a two-tier barring mechanism.

- If the new field is present:

- Use the new field to control Rel-19 UEs supporting DL-CE.

- Use the existing NTN bar bit to control legacy UEs and Rel-19 UEs not supporting DL-CE.

- If the new field is absent:

- Use the existing NTN bar bit to control all UE types.

3. Conclusion

Based on the discussion above, we made the following proposal:

- Support the introduction of a new field for Rel-19 UEs supporting DL-CE, implementing a two-tier barring mechanism.

- If the new field is present:

- Use the new field to control Rel-19 UEs supporting DL-CE.

- Use the existing NTN bar bit to control legacy UEs and Rel-19 UEs not supporting DL-CE.

- If the new field is absent:

- Use the existing NTN bar bit to control all UE types.

4. Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek