3GPP TSG-RAN WG2 Meeting #129bis	R2-2502879

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.3.2.2

Source:	Nokia

Title:	Evaluation results for RRM measurement prediction generalization

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#128 the following agreements on the generalization performance were made:

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases, - Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B. - Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B. - Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B. 2 Companies can choose which case they compare with and should report it with simulation results. 3 Generalization issues on RRM measurement prediction are prioritized.

In RAN2#129 the following agreements were made for further study of model generalization:

- Agreement 1: Two sets of parameters (ISD, BS antenna height, BS Tx power) are used for the generalization across cell configurations study. - Agreement 2: FR1 is the primary focus, companies can also submit results for FR2 (however, each set of generalization results covers either FR1 or FR2). - Agreement 3: Agree on the two sets of configurations as in tables 2 and 3 (for FR1 and FR2). Table 2: generalization parameters for FR1, Table 3: generalization parameters for FR2

In associated email discussion R2-2500559, the agreed parameters are given in tables 2 and 3 for FR1 and FR2, respectively.

Table 1-1: generalization parameters for FR1

| Parameter | Configuration #A | Configuration #B |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

Table 1-2: generalization parameters for FR2

| Parameter | Configuration #A | Configuration #B |
| --- | --- | --- |
| Deployment scenario | UMa | UMi |
| ISD | 200m | 500m |
| BS antenna height | 10m | 25m |
| BS Tx power | 40dBm | 44dBm |

In the sequel we present our evaluation results for model generalization across different configurations.

# 2	Discussion

## 2.1	Configuration #A and #B

To evaluate the generalization capabilities of RRM measurement prediction in temporal case B, we conducted a study using various generalization configurations, as detailed in Table 2 [1] for FR1 according to the agreement. To ensure a fair comparison, the inference data was kept consistent across the baseline and generalization configurations (baseline scenario, GC#1, and GC#2). The simulation assumptions are outlined in Table 2.1-1.

Table 2.1-1: Simulation parameters for the 7-site scenario based on TR38.843 (FR1)

| Reported simulation assumptions | UE trajectory option | Option 3 |  |  |
| --- | --- | --- | --- | --- |
|  | UE trajectory boundary processing option | Option 3 |  |  |
|  | UE speed | 30 km/h |  |  |
|  | SoftLOS | Yes |  |  |
|  | Spatial consistency option | Option B |  |  |
|  | Number of TX beams | 8 |  |  |
|  | Number of RX beams | 4 |  |  |
|  | Measurement reduction rate | 50%, 66%, 80%, 90% |  |  |
|  | Sub-use-case | Case 2 (cell to cell) |  |  |
|  | Observation window (OW) | 240 ms |  |  |
|  | Prediction window (PW) | 40 ms |  |  |
| Data Size (Number of Samples) | Training | Baseline | GC#1 | GC#2 |
|  |  | 1776845 | 6454616 | 8231466 |
|  | Testing | 760872 |  |  |
| AI/ML model input/output | Model input | L3 filtered serving cell RSRP measurements |  |  |
|  | Model output | L3 filtered serving cell RSRP measurements |  |  |
| AI/ML model description | Model type | Linear regression, Random Forest, Gradient Boosting, |  |  |
|  | Model complexity in number of parameters | Linear regression, Random Forest, Gradient Boosting: N/A |  |  |

The evaluation results are then presented in the sequel for different models and MRRTs. In this section, the results are generated from data collected at a UE speed of 30 km/h. For additional speed categories, such as 60 km/h, please refer to the appendix for a summarized overview.

Figure 2.1-1: Average MAE comparisons for different generalization configurations, baseline configuration A (UMi), linear regression

Figure 2.1-2: Average MAE comparisons for different generalization configurations, baseline configuration B (UMa), linear regression

Figure 2.1-3: Average MAE comparisons for different generalization configurations, baseline configuration A (UMi), gradient boosting

Figure 2.1-4 Average MAE comparisons for different generalization configurations, baseline configuration B (UMa), gradient boosting

Figure 2.1-5 Average MAE comparisons for different generalization configurations, baseline configuration A (UMi), random forest

Figure 2.1-6 Average MAE comparisons for different generalization configurations, baseline configuration B (UMa), random forest

The generalization configuration is assessed across various deployment scenarios, specifically UMa versus UMi, with detailed parameters provided in Table 2.1-1. The above simulation results analysis on generalization study for RRM measurement prediction temporal domain case B led to the following observations and proposals.

Observation 1: GC# 1 results in the poorest performance for both UMa and UMi.

Observation 2: GC# 2 with mixed training data from both deployment scenarios (UMa and Umi) can achieve performance comparable to the baseline configuration.

Observation 3: Generalization performance for UMa and UMi shows little variation among different configurations.

## 2.2	Radio Environment Generalization

### 2.2.1 Simulation Scenario

In this simulation study, we evaluate the generalization capabilities of a measurement event prediction model. Namely, we evaluate the ML model’s ability to generalize to a different localized radio environment, by training the model on UEs moving through a certain trajectory and testing it on UEs moving through a different trajectory. We set up a 7-site FR2 simulation scenario based on TR38.843 according to Table 2.2.1-1.

The UEs in this simulation can move through one of the three trajectories shown in Figure 2.2.1-1. Each arrow represents a street of width 12 meters. The UEs begin their trajectory at a random point along the width of the street at its beginning and move along the street till they reach its end. Moreover, the large-scale parameters of the propagation model are held constant across the simulation drops, while the small-scale parameters are randomized across the drops (see section 3.2 of our previous contribution in [2] for further discussion).

Table 2.2.1-1: Simulation assumptions for measurement event prediction

| Reported simulation assumptions | UE trajectory option | See above |
| --- | --- | --- |
|  | UE trajectory boundary processing option | See above |
|  | UE speed | 90 km/h |
|  | SoftLOS | Yes |
|  | Spatial consistency option | Option B |
|  | Number of TX beams | 8 |
|  | Number of RX beams | 4 |
|  | Measurement reduction rate | N/A |
|  | Sub-use-case | Case 2 (cell to cell) |
|  | Observation window (OW) | 2000 ms |
|  | Prediction window (PW) | 320 ms |
| Data Size (Number of Samples) | Training | 4K |
|  | Testing | 2K |
| AI/ML model input/output | Model input | L3 filtered RSRP measurements |
|  | Model output | A3 event prediction |
| AI/ML model description | Model type | Random Forest |

Figure 2.2.1-1: The three UE trajectories used for evaluating generalization capabilities

### 2.2.2 Evaluation Results

The direct measurement event prediction use-case is then evaluated in the same way as in our previous contribution in [3] (section 3.2), but now the ML model’s training and testing sets will be split according to the street on which the UEs move. The model takes as an input a time series of RSRP measurements of both the serving and target cells at the point of A3 event entry and predicts whether the A3 event will remain fulfilled for the duration of the TTT or not. The results are shown in Table 2.2.2-1, where the F1 score on the test set is shown for different combinations of training and testing streets. Table 2.2.2-2 shows the corresponding ROC AUC score.

Table 2.2.2-1: F1 score for different combinations of training and testing streets

|  |  | Testing Street |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 |
| Training Street | 1 | 0.733 | 0.573 | 0.528 |
|  | 2 | 0.522 | 0.795 | 0.475 |
|  | 3 | 0.561 | 0.510 | 0.767 |
|  | 1, 2 | 0.717 | 0.778 | 0.493 |
|  | 1, 3 | 0.713 | 0.536 | 0.760 |
|  | 2, 3 | 0.538 | 0.779 | 0.746 |
|  | All | 0.708 | 0.775 | 0.743 |

Table 2.2.2-2: ROC AUC score for different combinations of training and testing streets

|  |  | Testing Street |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 |
| Training Street | 1 | 0.801 | 0.566 | 0.509 |
|  | 2 | 0.542 | 0.885 | 0.497 |
|  | 3 | 0.550 | 0.543 | 0.844 |
|  | 1, 2 | 0.773 | 0.875 | 0.504 |
|  | 1, 3 | 0.776 | 0.554 | 0.837 |
|  | 2, 3 | 0.546 | 0.873 | 0.825 |
|  | All | 0.767 | 0.869 | 0.818 |

We can see that, while the model performs well if it is trained and tested on the same trajectory, the performance degrades significantly when the training and testing trajectories are different. Nevertheless, when it is trained on all trajectories and tested on each one of them individually, the performance is comparable to training and testing on the same trajectory. This highlights the importance of training set diversity for ML model generalization.

Observation 4: For measurement event prediction, the ML model generalizes poorly if it is tested on a trajectory that is not included in the training set.

Observation 5: Including all UE trajectories in the training set enhances generalization performance.

# 3	Conclusion

Always echo in the conclusion all proposals made above.

This document has made the following observations:

Observation 1: GC# 1 results in the poorest performance for both UMa and UMi.

Observation 2: GC# 2 with mixed training data from both deployment scenarios (UMa and Umi) can achieve performance comparable to the baseline configuration.

Observation 3: Generalization performance for UMa and UMi shows little variation among different configurations.

Observation 4: For measurement event prediction, the ML model generalizes poorly if it is tested on a trajectory that is not included in the training set.

Observation 5: Including all UE trajectories in the training set enhances generalization performance.

# 4	References

[1] R2-2500559 Email discussion summary for [Post128][018][AI Mob] generalization, Apple, 3GPP RAN2#129, Athens, Greece, February 17 – 21, 2025.

[2] R2-2500948 Generalization of ML mobility use-cases, Nokia, Nokia Shanghai Bell, 3GPP RAN2#129, Athens, Greece, February 17 – 21, 2025.

[3] R2-2500947 On the measurement event prediction, Nokia, Nokia Shanghai Bell, 3GPP RAN2#129, Athens, Greece, February 17 – 21, 2025.

# 5	Appendix

## 5.1	Additional simulation results for temporal case B

Additional simulation results for RRM measurement prediction in temporal domain case B, based on the data set for a UE speed of 60 km/h, are presented below.

Figure 5.1-1: Average MAE comparisons for different generalization configurations, baseline configuration A (UMi), linear regression

Figure 5.1-2: Average MAE comparisons for different generalization configurations, baseline configuration B (UMa), linear regression

Figure 5.1-3: Average MAE comparisons for different generalization configurations, baseline configuration A (UMi), gradient boosting

Figure 5.1-4 Average MAE comparisons for different generalization configurations, baseline configuration B (UMa), gradient boosting

Figure 5.1-5 Average MAE comparisons for different generalization configurations, baseline configuration A (UMi), random forest

Figure 5.1-6 Average MAE comparisons for different generalization configurations, baseline configuration B (UMa), random forest