3GPP TSG-RAN WG2 Meeting #129bis	R2-2502881

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.3.4

Source:	Nokia

Title:	Considerations on LCM for AIML Mobility

WID/SID:	FS_NR_AIML_Mob - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#129, the following were agreed:

Agreements on priority - RLF prediction will not be studied in Rel-19 - Further RRM prediction simulations are not expected. For next specification general impact study phase we will consider spatial, temporal, and frenquency domain cell level prediction. FFS if L3 beam level needs to be considered. For now, temporal and frequency domain prediction will be considered with higher priority for detailed study (if needed). - Continue with generalization across cell configurations for RRM prediction - Continue with measurement events

In the sequel, we present our views on the framework considerations for AIML Mobility. We also present some initial considerations on LCM aspects for RRM measurement prediction use-case.

# 2	ML Management in Mobility

## 2.1	Framework considerations

As agreed, and noted in the TR 38.744 draft [1], this SI will reuse the common framework of LCM captured in TR 38.843 [2] and the agreement concluded under WID NR_AIML_air-Core in principle. In this study item, the mobility-related aspects of LCM are studied. TR 38.843 lists the following aspects to be studied in LCM:

- Data collection - Model training - Functionality/model identification - Model delivery/transfer - Model inference operation - Functionality/model selection, activation, deactivation, switching, and fallback operation. - Functionality/model monitoring - Model update - UE capability

The 38.843 ML LCM scope can be taken as a starting point. Considering the limited time that remains in the current SI, we think that some prioritization of the topics that would be discussed is needed. The SID [3] for AIML Mobility clearly captures that 2-sided models are not included, and the focus should be either on NW- or UE-side deployments.

Observation 1: The SID for AIML Mobility suggests that the focus should be either on NW- or UE-side deployments.

Based on the need to provide specification support for both UE- and NW-side models, it seems natural that RAN2 prioritizes the discussion on LCM components that would enable progress for both types of deployments.

Proposal 1: RAN2 to prioritize discussion on LCM components identified in TR 38.843 that are needed to enable either NW- or UE-side deployments.

To facilitate progress in the discussion, we suggest focusing on the aspects that have highest RAN2 impact, and at the same time are essential to developing AIML-enabled features for mobility. From this viewpoint, we think that the starting point of the discussions should be the configurability of the AIML-enabled features, which is enabled through the functional framework developed in the context of Rel. 18-19 AIML Phy. Yet another key enabler for designing AIML-based features is data collection.

Observation 2: Studying both the functional and data collection framework are essential for the use-cases being considered in Rel. 19 AIML Mobility.

Proposal 2: As starting point, study the enhancements needed for the functional and data collection frameworks.

## 2.1.1 Use-case aspects

The following use-cases have been discussed in AIML Mobility

- RRM Measurement prediction (both NW- and UE-sided model)

- Measurement event prediction (UE-sided model)

- HOF/RLF prediction (UE-sided model)

When considering mobility, arguably the most important use-case is measurement prediction, for which both NW- and UE-sided models are being considered. Moreover, for the other two use-cases, both direct and indirect approaches are studied. In case of indirect prediction, the common understanding in RAN2 is that it comprises of the following steps: i) Measurement prediction; ii) post-processing to derive the desired event. This is also captured in Figures 4.3.1 and 4.4.1 in TR. 38.744 [1].

Observation 3: For all the use-cases being considered in the Rel. 19 AIML Mobility SI, some aspect of RRM measurement prediction would be present.

Proposal 3: Prioritize discussions on RRM measurement prediction use-case.

## 2.1.2 Deployment aspects

For the different use-cases being considered in the ongoing AIML Mobility SI, both NW- and UE-sided models are in scope. Specifically, for RRM measurement prediction both deployment options are considered. On the other hand, for measurement event and RLF/HOF prediction use-cases, only UE-side deployments are proposed in the SID [3]. Considering the progress in the Rel. 18-19 use-cases studied in AIML PHY, it is clear that there are topics concerning UE-side deployments that are not yet at a mature stage. For instance, the LCM and data collection frameworks for UE-side deployments are still not complete and for certain aspects (e.g., UE-side data collection) companies still have different views. Moreover, in the SID [3], we have the following

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

Observation 4: Refining the details for functionality control and data collection for UE-side deployments still requires more discussion in the context of AIML Phy use-cases.

On the other hand, discussions for the framework enabling NW-side models are at a more mature phase.

Proposal 4: RAN2 to start with identifying the ML mobility use case LCM requirements for each deployment option and continue with potential solutions following progress in the AI/ML Phy use case LCM work.

## 2.1.3 Scenario/configuration -specific deployments

TR 38.843 remarks that scenario/configuration specific (including site-specific configuration/channel conditions) models may provide performance benefits in some studied use cases. Mobility use cases can have a strong site-, area-specific context, which in some use cases can lead to limited generalization to different scenarios. Figure 2.1-1 is showing an example of site-specific conditions that may be, implicitly or explicitly, learned by mobility ML models:

- Radio propagation map, influenced by, for example:

  - Network deployment and configuration

  - Radio environment, LOS/NLOS, shadowing

- Different mobility profiles

  - For example, cars, pedestrians, HST, drones

  - Speed, trajectories (e.g. streets)

Figure 2.1.2-1 Site-specific conditions that may be useful in mobility ML use cases

On the other hand, management of site-specific ML models can introduce significant complexity in the LCM and scalability due to potentially higher number of ML models that need to be managed. Especially in UE-side ML, where model deployment and switching may pose additional challenges.

Proposal 5: RAN2 to discuss, if scenario/configuration specific ML management for mobility use cases is studied.

## 2.1.4 Data collection aspects

Data is an important enabler for both NW- or UE-side deployments. Considering the ongoing Rel. 18-19 AIML Phy SI/WI discussions, at least the following agreements for NW-side data collection for the BM use-case would be relevant for the AIML Mobility SI.

RAN2 agreement@125bis: For the NW-side data collection related to beam management use cases, RAN2 to consider gNB-centric and OAM-centric approaches RAN2 agreement@126: Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

Considering the ongoing study on AIML Mobility, for RRM prediction the following cases are described in TR 38.744 [1]:

Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Considering the sub-use cases identified for RRM prediction, either the L1 or L3 measurement and reporting frameworks would be of interest. For these two frameworks, the termination points of the protocols are either the DU or the CU, which were also studied for the Rel. 18-19 AIML Phy use-cases. Thus, we see no reason to consider different frameworks for NW-sided model.

Observation 5: Considering the sub-use cases identified for RRM prediction, the termination points of the algorithms are either the CU or the DU.

Proposal 6: Consider immediate MDT and the measurement reporting framework as the starting point for data collection for NW-side deployments. Study additional data elements and methods needed for data collection at the NW-side.

Data collection for UE-side deployments was also thoroughly studied in the context of Rel. 18-19 AIML Phy use-cases. Different options were considered, which are also captured in TR 38.843 [2] Section 7.2.1.3.2:

1. UE collects and directly transfers training data to the Over-The-Top (OTT) server; 1a) OTT (TRansparent) 1b) OTT (non-TRansparent) 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the needed data to the OTT server. RAN2 did not study or analyse these proposals and did not agree to requirements or recommendations.

Despite the ongoing efforts in concluding on the topic, RAN2 has not yet agreed on the WF for this topic. Thus, we suggest postponing discussions on UE-side data collection until sufficient maturity is achieved on the matter in the context of the use-cases considered in AIML Phy.

Proposal 7: Postpone discussions on UE-side data collection until there is more progress in the Rel. 18-19 AIML Phy use-cases.

## 2.2	Signalling aspects

In the sequel, we present our considerations on signalling for RRM and measurement event prediction use-cases.

## 2.2.1 RRM prediction

So far, we have considered both Case A and Case B RRM measurement predictions and have observed that the choice of observation window (OW) and prediction window (PW) lengths significantly impacts the performance of the ML model. In case of time domain Case B, the OW and PW depend on the MRRT and the measurement skipping pattern. In time domain Case A, the required PW depends on the use case, where the predicted measurements are used. For example, in indirect measurement event prediction the PW may be aligned with the measurement event Time-to-Trigger (TTT) configuration. The OW length may be an implementation decision within specified constraints, but it can have an impact at least on the data collection and on the testability of the features (RAN4).

Observation 6: The OW and PW lengths may need to be aligned with different use cases.

In earlier contributions where RRM prediction was evaluated, we have observed that larger PW may result in a higher MAE in temporal domain RRM measurement prediction. Lower UE speeds generally result in better intra-frequency RRM prediction performance. The optimal OW length may depend on factors like the expected speed range of the UE, frequency range etc. This indicates that the same OW and PW configuration may not be suitable in different radio environments.

Observation 7: The same OW and PW configuration may not be suitable in different radio environments and mobility scenarios.

Proposal 8: RAN2 to discuss the configuration aspects for OW and PW for temporal RRM measurement prediction.

Considering legacy mobility procedures, the UE reports measurements based on a configuration provided by the network. This configuration includes detailed measurement and reporting configs that instruct the UE on when and how to report its measurements. These reports are critical to enabling the network to make timely and accurate handover decisions. Moving toward Release 19 and AI/ML-enabled use cases like RRM measurement prediction, similar to the legacy behaviour, it is expected that prediction configurations would also be provided by the network. This would enable the network to configure, which predictive insights it receives and when, facilitating better decision-making in fast-changing scenarios.

Observation 8: In legacy mobility procedures, the network provides configuration for UE to perform measurements and reporting. Similar approach can also be followed for reporting RRM measurement predictions.

Similar to the legacy framework, the UE might be configured to report predictions only under specific, non-periodic conditions or events, allowing for reporting of predictions when they are truly needed by the NW. This non-periodic approach could lower reporting overhead while still providing critical predictive data to the network when necessary.

Proposal 9: NW configures the UE to make and report measurement predictions. RAN2 to discuss the prediction reporting mechanisms.

## 2.2.2 Measurement event prediction

The prediction may be implemented by at least two implementation options:

- Indirect: First predicting future measurements for the serving cell and the target candidate cell(s) with the RRM measurement prediction time domain case A ML model.

- Direct: Predict the measurement event directly with a classification ML model using similar input features as the RRM measurement prediction time domain case A.

Figure 2.1.1-1 Indirect and Direct Measurement Event Prediction

As shown in Figure 2.1.1-1, these two options can be seen mainly as implementation choices. Related questions to consider in the specification study may include:

- What can be configured in the inference phase and what is part of the capability information

  - For example, the configuration of the predicted measurement event

  - Prediction trigger and reporting conditions

- What is reported based on a predicted measurement event, for example

  - Occurrence of a predicted measurement event in the configured PW

  - Measurements

  - Predicted measurements

  - More specific predicted time of measurement event condition fulfilment in the PW

  - Confidence/probability of the prediction

Any specification impact between the choices to the UE Capability Information, RRC configuration and reporting based on the predictions can be considered during the WI phase.

Observation 9: Indirect and direct measurement event prediction can be seen largely as implementation choices.

# 3	Conclusion

This document has made the following observations:

Observation 1: The SID for AIML Mobility suggests that the focus should be either on NW- or UE-side deployments.

Observation 2: Studying both the functional and data collection framework are essential for the use-cases being considered in Rel. 19 AIML Mobility.

Observation 3: For all the use-cases being considered in the Rel. 19 AIML Mobility SI, some aspect of RRM measurement prediction would be present.

Observation 4: Refining the details for functionality control and data collection for UE-side deployments still requires more discussion in the context of AIML Phy use-cases.

Observation 5: Considering the sub-use cases identified for RRM prediction, the termination points of the algorithms are either the CU or the DU.

Observation 6: The OW and PW lengths may need to be aligned with different use cases.

Observation 7: The same OW and PW configuration may not be suitable in different radio environments and mobility scenarios.

Observation 8: In legacy mobility procedures, the network provides configuration for UE to perform measurements and reporting. Similar approach can also be followed for reporting RRM measurement predictions.

Observation 9: Indirect and direct measurement event prediction can be seen largely as implementation choices.

And proposed the following:

Proposal 1: RAN2 to prioritize discussion on LCM components identified in TR 38.843 that are needed to enable either NW- or UE-side deployments.

Proposal 2: As starting point, study the enhancements needed for the functional and data collection frameworks.

Proposal 3: Prioritize discussions on RRM measurement prediction use-case.

Proposal 4: RAN2 to start with identifying the ML mobility use case LCM requirements and continue with potential solutions following progress in the AI/ML Phy use case LCM work.

Proposal 5: RAN2 to discuss, if scenario/configuration specific ML management for mobility use cases is studied.

Proposal 6: Consider immediate MDT and the measurement reporting framework as the starting point for data collection for NW-side deployments. Study additional data elements and methods needed for data collection at the NW-side.

Proposal 7: Postpone discussions on UE-side data collection until there is more progress in the Rel. 18-19 AIML Phy use-cases.

Proposal 8: RAN2 to discuss the configuration aspects for OW and PW for temporal RRM measurement prediction.

Proposal 9: NW configures the UE to make and report measurement predictions. RAN2 to discuss the prediction reporting mechanisms.

# 4	References

[1] TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR (Release 19)

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

[3] RP-234055 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR, TSG RAN Meeting 102, Dec. 2023, Edinburgh, Great Britain.