3GPP TSG-RAN WG2 Meeting #129bis	R2- 2502882

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	LP-WUS in RRC_CONNECTED

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

RAN2 discussed LP-WUS for connected with the following agreemetns:

For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used.

RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer.

RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

In this contribution, we discuss the remaining issues including FFS and considering on-going discussion in RAN1.

# 2	Discussion

## 2.1	Short DRX cycle

In RAN2#129bis, RAN2 briefly discussed whether the short DRX cycle can be used with LP-WUS.

### 2.1.1	Option 1-1

For Rel-16 DCP, RAN1 has discussed short DRX cycle and DCP, making the working assumption in RAN1#98bis that:

Agreements: - The PDCCH-based power saving signal/channel can be configured for long DRX for a UE - (Working assumption) If there is also short DRX cycle configured for the UE, the PDCCH-based power saving signal/channel is not applicable for the short DRX cycle

Our understanding is that short DRX cycle can still be configured with DCP, but DCP is applicable only when the UE is using the long DRX cycle.

Observation 1: For Rel-16 DCP, RAN1 assumes that DCP can be configured with short DRX cycle, but DCP monitoring is applied only when the UE is in the long DRX cycle.

In this context, RAN2 also considered whether the short DRX cycle could be used with DCP [RAN2#107bis, R2-1914301], and concluded that from RAN2 perspective it is desirable to support WUS for both short and long DRX cycle with configurability [R2-1914060]. RAN2 tried to finalize the issue in RAN2#109bis-e [R2-2003379] but just re-confirmed that DCP can be supported with short DRX cycle, provided that the network configures the short DRX cycle properly, i.e., only when a valid configuration exist.

Observation 2: For Rel-16 DCP, RAN2 agreed that it is even desirable to use both short and long DRX cycle with DCP.

Turning to Rel-19 LP-WUS, no critical issues have been identified with using short DRX cycle. In fact, during the study phase, the short DRX cycle demonstrated good power saving gains [TR 38.869]. In addition, short DRX cycle is still configurable by the network, meaning that the network would not configure short DRX if not needed. Therefore, from RAN2 perspective, there is no reason to get rid of short DRX cycle.

Observation 3: From RAN2 perspective, no critical issue has been identified using the short DRX cycle with LP-WUS option 1-1 and option 1-2.

In RAN1#120, for option 1-1, RAN1 has decided to introduce an offset 2, which is a time offset prior to a slot of On-duration and configured by RRC:

Agreement For LP-WUS MOs in connected mode for Option 1-1, support Approach 1: - LP-WUS MOs, including periodicity and time offset1, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - Time offset2 indicates a time prior to a slot where the drx-onDurationTimer would start - UE monitors the first Z LP-WUS MO(s) at or after Time offset2. - FFS: How to determine the value of Z

Unlike offset 1, which is configured independently from C-DRX, offset 2 is linked to On-Duration, and its value will be determined by RAN1. Therefore, the final decision also needs to be made in RAN1. Thus, we suggest sending an LS to RAN1 asking what the intended behaviour if LP-WUS is configured with short DRX cycle, i.e., like in DCP, short DRX cycle can be configured but LP-WUS is not applicable when the UE is in use of short DRX cycle.

Proposal 1: For option 1-1, it is up to network configuring short DRX cycle with LP-WUS. Send an LS to RAN1 asking if LP-WUS is applicable to short DRX cycle as well as long DRX cycle for option 1-1.

### 2.1.2	Option 1-2

For option 1-2, it is generally understood that On-Duration is not used for PDCCH monitoring because the UE immediately resumes LP-WUS monitoring immediately once active time by drx-InactivityTimer, drx-RetransmissionTimerUL/DL, and the new timer (drx-lpwusInactivityTimer) ends. Accordingly, RAN2 agreed in RAN2#128 that the drx-onDurationTimer is not started with Option 1-2 LP-WUS.

One question remains regarding the use of short/long DRX cycles in option 1-2. According to TS 38.321, the On-Duration is not solely for PDCCH monitoring but is also used for CSI reporting with csi-Mask.

RAN2 agreed in 128bis that:

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

More specifically, RAN1 agreed in RAN1#119 that:

Agreement For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required.

Our view is that the discussions in RAN1 and RAN2 did not preclude the possibility of reporting CSI during the On-Duration (without on-durationTimer running) and use of short DRX cycle. However, it would be advisable to check whether any technical concerns exist regarding the use of short DRX cycle for CSI reporting.

Proposal 2: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. Send an LS to RAN1 asking if there are any technical concerns regarding configuring of short DRX cycle in option 1-2.

## 2.2	Start of the new timer

In RAN2, it has been agreed that the new timer triggered by LP-WUS is introduced, intending to start monitoring of PDCCH when the UE is woke up by LP-WUS.

In RAN1, time window containing multiple MOs is under discussion, which might impact the start time of the new timer (drx-lpwusInactivityTimer). For instance, drx-lpwusInactivityTimer could start either at the end of the time window, i.e., at the last MO, or immediately upon detection of the LP-WUS indication (without waiting until the end of time window).

In our understanding, the network can select any MO, even multiple MOs, within the time window. In this case, it would be safe to start the new timer at the end of time window to make sure that the active time is aligned between the UE and the network.

Proposal 3: In option 1-2, if the time window contains multiple MOs, the UE starts the new timer (tentatively named drx-lpwusInactivityTimer) at the last MOs of the window.

## 2.3	SR and LP-WUS

For both option 1-1 and 1-2, it is assumed that the UE does not monitor LP-WUS during C-DRX active time. This raises the question of whether the UE can automatically transition to active time by uplink traffic, such as when SR is pending, or the UE should continue waiting for LP-WUS from the network.

When SR is pending, the network may not yet be aware that the UE has entered C-DRX active time. As a result, the network might send LP-WUS for downlink scheduling, which would be missed if the UE has stopped monitoring of LP-WUS. Only after the gNB receives the SR, will it stop sending LP-WUS and schedule the UE via PDCCH.

Although downlink latency is generally considered non-critical, the potential failure of the initial SR transmission may motivate to maintain LP-WUS monitoring until the gNB successfully receives the SR and responds – the UE starts monitoring of PDCCH based on the new timer started by LP-WUS received SR is pending. It would imply also that the SR is responded by LP-WUS instead of PDCCH.

Alternatively, the UE could monitor both LP-WUS and PDCCH while the SR is pending; however, the feasibility of this may depend on a decision by RAN1.

In our view, the simplest solution is to begin PDCCH monitoring when SR is pending, as is done in legacy, unless there is good reason to keep the UE’s C-DRX active time synchronized with the gNB, thereby allowing the gNB to continue scheduling of downlink data while uplink traffic is pending.

Proposal 4: RAN2 discuss whether the UE should continue monitoring LP-WUS while an SR is pending or switch to monitoring PDCCH by stopping LP-WUS monitoring.

## 2.4	CG/SPS

It has not yet been discussed whether the MR keeps the SPS and CG activated or deactivated while the UE monitors the LP-WUS.

If SPS and CG are kept activated, transmission or reception of MAC PDU on SPS/CG will start the corresponding timers, i.e., drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL, which might then keep the MR awake. To save more power, it might be preferred to deactivate SPS and CG. As it is possible to deactivate SPS and CG by PDCCH and activation/deactivation MAC CE, respectively, it seems not necessary to define additional deactivation mechanism such as the UE by itself deactivate the SPS or CG when starting LP-WUS monitoring.

Proposal 5: In option 1-2, when the UE starts LP-WUS monitoring, SPS and CG are kept as it is unless activated or deactivated by the network explicitly.

If SPS and CG are deactivated while monitoring LP-WUS, the gNB needs to send the LP-WUS to wake up the MR first and then activates the SPS/CG. This would introduce additional latency because the UE waits for receiving the activation command after starting the PDCCH monitoring. Moreover, the gNB may intend to rely on SPS/CG scheduling instead of dynamic scheduling in the MR, in which cases further PDCCH monitoring after SPS/CG activation would be needlessly consuming power.

Thus, RAN2 needs to discuss how to reduce unnecessary PDCCH monitoring when using SPS/CG. For instance, we can consider that the LP-WUS indicates SPS/CG activation.

Proposal 6: For option 1-2, RAN2 to discuss the LP-WUS impact activation of SPS and CG Type 2.

## 2.3	Option 1-3

Option 1-1 and 1-2 are being actively discussed in RAN1 and RAN2, and the detailed procedures are gradually taking shape. However, option 1-3 has not yet been discussed.

Option 1-3 is understood that the UE monitors LP-WUS inside C-DRX active time. As concluded from the study [TR 38.869], the power saving gain is expected by reducing unnecessary PDCCH monitoring within C-DRX active time. However, there may be limited use of this option 1-3 considering the length of On duration [R2-2404380].

In our view, we already have some mechanism to end or reduce unnecessary PDCCH monitoring during active time, e.g., sending DRX command MAC CE or PDCCH skipping. Thus, it would be desirable to focus more on first two options, i.e., option 1-1 and option 1-2.

Proposal 7: Option 1-3 is excluded in RAN2.

# 3	Conclusion

In this paper, we discuss the RRC_CONNECTED for LP-WUS operation:

Proposal 1: For option 1-1, it is up to network configuring short DRX cycle with LP-WUS. Send an LS to RAN1 asking if LP-WUS is applicable to short DRX cycle as well as long DRX cycle for option 1-1.

Proposal 2: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. Send an LS to RAN1 asking if there are any technical concerns regarding configuring of short DRX cycle in option 1-2.

Proposal 3: In option 1-2, if the time window contains multiple MOs, the UE starts the new timer (tentatively named drx-lpwusInactivityTimer) at the last MOs of the window.

Proposal 4: RAN2 discuss whether the UE should continue monitoring LP-WUS while an SR is pending or switch to monitoring PDCCH by stopping LP-WUS monitoring.

Proposal 5: In option 1-2, when the UE starts LP-WUS monitoring, SPS and CG are kept as it is unless activated or deactivated by the network explicitly.

Proposal 6: For option 1-2, RAN2 to discuss the LP-WUS impact activation of SPS and CG Type 2.

Proposal 7: Option 1-3 is excluded in RAN2.

# 4	Agreements

Agreements from RAN2#126

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

Agreements from RAN2#127

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Agreements from RAN2#127bis

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

Agreements from RAN2#128:

The drx-onDurationTimer is not started with Option 1-2 LP-WUS. For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

Agreements from RAN2#129:

For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer.

Agreements from RAN1#119:

Agreement LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication. - Note: LP-WUS indication does not impact SCell dormancy behavior. Agreement For LP-WUS CONNECTED mode operation, the following case is not supported: - Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell. Agreement - For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer. - Approach 2: - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle. - For LP-WUS MOs in connected mode for Option 1-2, following is considered further - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset. - Approach 2: - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters. - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start. Agreement For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask the feasibility of the above potential solution. Conclusion For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask whether RAN2 has same understanding or not. Agreement For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required.

Agreements from RAN1#120:

Agreement For RRC connected, for LP-WUS SCS: - Alt 1: LP-WUS SCS is same as the active DL BWP R1-2501519 Summary #3 of discussions on LP-WUS and LP-SS design Moderator (vivo) Working Assumption Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs: - Codepoint based - The maximum number of codepoints checked per MO by a UE is up to 8 - Depending on UE capability, a UE may support less than 8 Agreement For RRC connected, LP-WUS frequency resource can be outside of active DL BWP but has to be within the same carrier as the active DL BWP - Basic capability is LP-WUS, if present, frequency resource within active DL BWP. LP-WUS frequency resource outside of active DL BWP is subject to separate UE capability - No RAN1 optimization specific to the case where LP-WUS frequency resource is outside of active DL BWP

Agreement

For RRC connected, for LP-WUS SCS:

- Alt 1: LP-WUS SCS is same as the active DL BWP

R1-2501519	Summary #3 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

Working Assumption

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs:

- Codepoint based

  - The maximum number of codepoints checked per MO by a UE is up to 8

    - Depending on UE capability, a UE may support less than 8

Agreement

For RRC connected, LP-WUS frequency resource can be outside of active DL BWP but has to be within the same carrier as the active DL BWP

- Basic capability is LP-WUS, if present, frequency resource within active DL BWP. LP-WUS frequency resource outside of active DL BWP is subject to separate UE capability

- No RAN1 optimization specific to the case where LP-WUS frequency resource is outside of active DL BWP