3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502916

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SIB1 for Idle/Inactive mode UEs

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

The release 19 work item on network energy saving (RP-242354) includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we will discuss the issues related to the on-demand SIB1 for idle/inactive mode UEs. And in our companion contribution, we will discuss the issues on-demand SSB SCell operation and adaptation of common signal/channel transmissions in [1] and [2], respectively.

The following agreements were made:

Agreements on OD-SIB1: RAN2#129

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this paper we discuss further details of the OD-SIB1 for case 2 agreed in the RAN#105 plenary meeting.

# 2	Baseline operation for case 2 – For information only

- UE obtains the WUS configuration from cell A,

  - PCI+Frequency (at least) used to associate configuration to a cell.

  - Cell A may provide configuration for multiple cells.

  - Multiple Cell A(s) may provide WUS configuration for same cell.

  - At least configuration is provided by cell A SIB(s). in addition to cell A SIB (s), RRCRelease may provide WUS configuration. This makes sense to us as there is likely scenarios that cell starts to use OD-SIB1 and releases UES to IDLE/INACTIVE – It seems quite good idea not to mandate reselection to Cell A and then back to NES Cell

- Once UE (re)selects to cell for which UE has WUS configuration and NW indicates it is utilizing OD-SIB1 procedure - UE may request OD-SIB1. Legacy UE should bar this kind of cell but OD-SIB1 supporting UE should not.

  - Request is assumed to be preamble transmission

    - We don’t combine this with direct request for moving to RRC_CONNECTED i.e. UE will first acquire SIB1 and after that resume/request to be moved to RRC_CONNECTED

  - RAN2 assumes UE will receive RAR

  - Assuming NW detects OD-SIB1 request it will provide SIB1 to UE. It has not been discussed in RAN2 whether SIB1 provision is:

    - via OD-SIB1 specific transmission AND/OR

    - NW resumes regular SIB1 transmission

- If SIB1 reception is successful

  - UE may camp on the cell will apply regular operations – paging reception, SI change modification monitoring etc…

  - It was left for further consideration how UE determines validity of SIB1

  - RAN2 also agreed that UE needs reacquire SIB1 in case of SIB change notification (legacy behaviour).

- If not successful

  - As baseline upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. Without modification to specification if UE has not been able to acquire SIB1 it will bar the cell for up to 300 seconds

As we can see RAN2 has established baseline operation for case 2 but of course there are still various aspects that need to be handled.

# 3	Switching between legacy SIB1 transmission and on-demand SIB1 transmission

First, we provide a summary about related agreements and discussion for this aspect. These agreements are used as starting point and assumptions for our proposals and observations for this aspect.

RAN2 agreed about the following:

RAN2 #127bis agreements:

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

RAN2 #127bis agreements:

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

RAN1#118bis agreed about the following:

For further work of on-demand SIB1 in idle/inactive mode, it is assumed that:

-	SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster

-	UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

From the above agreements and RAN1 discussion in FL summary R1-241068, switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission should be supported in release 19 for OD-SIB1 operation. In addition, how the UE check, detect or being informed about the broadcast status for OD-SIB1 needs to be discussed.

Observation 1: Switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission is a valid issue to be considered by RAN2.

For NES SIB1 transmission mode switch, the following points are discussed in this contribution:

- UE expectation to the value of K_SSB in NES, if SIB1 is currently being broadcasted or provided on-demand.

- How UE can check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- MIB TTI is 80ms. This means that MIB payload does not change every 80ms. However, with repetition MIB can be transmitted every 20ms.

For point 1) and 2) we summarize in the state diagram below, K_SSB value with respect to NES cell SIB1 transmission mode based on RAN1 and RAN2 agreements.

Observation 2: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing:

- K_SSB < 24 for FR1 and K_SSB < 12 for FR2, is broadcasting SIB1 as legacy and NES UE will acquire SIB1 as in legacy.

- K_SSB > 23 for FR1 and K_SSB > 11 for FR2, is providing SIB1 on-demand. UE needs to monitor SIB1 to check if it is being broadcasted within a limited window before requesting SIB1 of that cell. If UE does not detect a SIB1, UE to request OD-SIB1 for that cell.

For 3), 38.212 specifies that:

“Data arrives to the coding unit in the form of a maximum of one transport block every 80ms.”

In 38.331, it specifies that SSB periodicity is at least 80 ms, with repetition within 80 ms. Basically from 38.331 and 38.212, UE is expecting to no change in PBCH payload. Based on this, many UEs apply combining for SSB detection/decoding, mainly cell-edge UEs.

Observation 3: Legacy and NES UEs assume MIB TTI is 80ms.

To avoid impact to legacy UE, NW should ensure MIB TTI of 80ms is respected. As K_SSB is a part of MIB, hence NW cannot switch between SIB1 broadcasting state or SIB1 provided on demand with a periodicity less than 80ms. This implies that when NES cell is operating with OD-SIB1 and K_SSB > 23 for FR1 and K_SSB > 11 for FR2, UE assumes that the same value of K_SSB within 80ms.

To avoid impact to legacy UEs and to guarantee MIB TTI of 80ms (8 radio frames), NW shall consider only specific timing for the switching from broadcasting SIB1 to on-demand SIB1. RAN2 can discuss to define this specific timing as function of SFN, e.g. SFN mod 8 = 0. Hence, Rel. 19 NES UE with SIB1 request configuration intended to transmit a request for OD-SIB1, should check at least every 80ms the SIB1 transmission status based on K_SSB carried by the MIB.

Based on above discussion, it is clear to avoid any misalignment between NES UE and NW, if NES UE has SIB1 request configuration of a cell, NES UE needs to acquire/re-acquire MIB.

Proposal 1: For switching of SIB1 transmission mode, RAN2 to consider that NES UE which has SIB1 request configuration of a cell, NES UE is expected to update MIB until SIB1 acquisition is completed

# 5	RAR response handling

RAN2 shall discuss the issue whether or not the common PRACH preamble is used for all UEs requesting on-demand SIB1 operation, and this could allow a RAR response to be interpreted/decoded by all UEs. Practically, when a UE intends to transmit the UL WUS for requesting SIB1, it can actually monitor if any RAR response is sent by network, and to check if on-demand SIB1 is already requested by other UEs. RAN1 agreed that dedicated ROs, time/frequency resources can be used for requesting OD-SIB1. To allow other UEs intended to request OD-SIB1 to monitor the RAR response and OD-SIB1, a common preamble index per SSB should be used.

Observation 4: For SIB1 requesting by assuming a common preamble used by all UEs, it allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS.

Observation 5: Configuration of a common preamble may be required in order to enable UEs monitoring on-demand SIB1 without transmitting UL WUS.

Based on TS38.321, a MAC PDU subheader with RAPID only is used for acknowledgment for on-demand SI request. For OD-SIB1, same clause can be used. No need for designing a new RAR response as the WID recommend low specification impact.

Proposal 3: RAN2 to align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

# 6	RRC connected mode UEs

In RAN#106 following was endorsed in RP-243297:

RAN2 #129 agreed about the following for RRC connected UEs:

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

The chair notes of last meeting indicate the following:

Proposal 5: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, rely on the NW implementation to deliver SIB1 as baseline.

[Vivo]: NW cannot know when the UE wants to request OSI, then proposal 5 cannot work in the case. [Ericsson]: Want to have more time to think of two options.

- We will make conclusion on P5 next meeting.

For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting, one needs to enable possibility for UE to acquire SIB1.In order to allow Rel-19 NES UE to acquire SIB1, RAN2 can consider these two alternatives:

- Alternative 1: UE is in RRC_CONNECTED and reachable by network as then NW can by implementation update SIB1 (if needed) via existing RRC ASN.1 procedures, namely dedicated dedicatedSIB1-Delivery.

- Alternative 2:  UE in RRC_CONNECTED can obtain the SIBX from NW by the NW dedicated RRC for SIBX delivery. And then UE can use OD-SIB1 request procedure to acquire SIB1

From NW overhead point of view alternative 2 seems more appropriate as well as power saving point of view as one can avoid NW “guessing” when UE needs updated SIB1, which could easily results in over provisioning of SIB1 to UEs.  Thus if not considered unfeasible for UE implementations we would strongly recommend going forward with alternative 2;

Proposal 4: In case of NW operation in OD-SIB1 mode, UE in RRC_CONNECTED can obtain the SIBX from NW by the NW dedicated RRC for SIBX delivery. And then UE can use OD-SIB1 request procedure to acquire SIB1

For the case of radio link failure or release to IDLE/INACTIVE state with RRCRelease one needs to consider how UE can acquire SIB1 in a cell using OD-SIB1 procedure. We have following options:

- Alternative 1: UE re-establish the connection using stored SIB1, if still valid, (SIB1 that has been delivered by dedicated RRC for SIB1 delivery). This would only work for the case cell selection occurs to same cell as UE was in RRC_CONNECTED

- Alternative 2: UE re-establish the connection by triggering the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

    - The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

    - The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

As noted alternative 1 is too limited in usefulness as most likely cell selection occurs to different cell – thus UE should be able to utilize SIBX  in  case of RLF/release to IDLE/INACTIVE. Note also, in the two alternatives, the SIB1 or SIBX are delivered to UE in RRC_CONNECTED whatever RLF occurs or not. Which mean, in case UE is not facing an RLF re-establishment the SIB1 and SIBX delivery are useless. For alternative 2, it allows UE to request SIB1 from the cell that UE was connected and from neighbor cells with OD-SIB1 activated. However, alternative 1, only allows UE to request SIB1 from the cell that UE was connected.

For alternative 2, in RAN2 we have agreed to provide information for OD-SIB1 operation in system information, namely SIBX. UE in RRC_CONNECTED in RAN2 discussion has been seen able to obtain such a SIBX as well. One reason for not agreeing adding the information in RRCRelease was that UE will have this from SIBX in RRC_CONNECTED as well.

Observation 6: RAN2 understanding is that UE can acquire SIBX in RRC_CONNECTED (being one reason not to include this information in RRCRelease message).

Now question is that is the understanding that UE can utilize this SIBX in the RRC_CONNECTED when RLF occurs. In this scenario UE performs cell selection and it may select cell that uses OD-SIB1 procedure. In order to reestablish connection in a cell using OD-SIB1 operation one should allow/require UE to receive OD-SIB1 in RRC_CONNECTED.

It should be noted that also UE performs cell selection in case of RRCRelease and RAN2 implicitly already agreed that in this scenario UE can use SIBX (And agreed not to include WUS configuration in RRCRelease).  Thus it would seem logical to capture this also for RLF and for RRCRelease explicitly in the specification.

So technically what in stage-3 one would need is:

    - SIBX is/may be acquired in RRC_CONNECTED by UE

    - UE can use SIBX information in case of RLF and RRCRelease and it selects to OD-SIB1 utilizing cell for which nformationincluded in the acquired SIBX)

If SIBX cannot be received by RRC_CONNECTED mode ( we don’t see why that would be the case) we would need to reconsider decision of having  the WUS configuration in the RRCRelease to at least allow then UE to utilize SIBX in case of RRCRelease to IDLE/INACTIVE camp on cell utilizing OD-SIB1.

Proposal 5: If SIBX reception is not possible in RRC_CONNECTED then we should revisit decision on not having WUS configuration in RRCRelease.

We should also ensure in the specification that the SIBX is usable after cell selection::

Proposal 6: Agree to add in stage-3 CR UE to be allowed to use SIBX in case of cell selection (e.g. due to RLF/RRCRelease)  in order to reestabslish connection and/or camp in a cell using OD-SIB1 feature.

# 7	UL WUS configuration contents

WUS configuration contents shall be designed considering that the size of WUS configuration is not large to avoid high resource utilization for transmitting WUS configuration and thus reducing NES gain.

The starting point of WUS configuration contents are the RAN1 proposed parameters. However, RAN2 to decide if any other parameters needed.

Regarding barring of specific UEs, for RedCap UEs, NTN UEs, Rel 18 Cell DTX UEs, they can be barred by SIB1 based on release 17 or release 18. Thus, to avoid requesting SIB1 by these UE while OD-SIB1 cell is barring these UEs, it is important to indicate the barring status for these UEs using WUS configuration.

Regarding CellAccessRelatedInfo in SIB1, it is used to indicate cell access related information for the cell. It is important to indicate to UE by WUS configuration about the CellAccessRelatedInfo to avoid that UE requesting SIB1 and then determines from SIB1, that it is not allowed to access the cell. RAN2 to discuss that CellAccessRelatedInfo to be provisioned to UE using WUS configuration.

Proposal 7: Cell access related information should be included in the the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access.

# 8	Conclusions:

Observation 1: Switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission is a valid issue to be considered by RAN2.

Observation 2: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing:

Observation 3: Legacy and NES UEs assume MIB TTI is 80ms.

Proposal 1: For switching of SIB1 transmission mode, RAN2 to consider that NES UE which has SIB1 request configuration of a cell, NES UE is expected to update MIB until SIB1 acquisition is completed

Observation 4: For SIB1 requesting by assuming a common preamble used by all UEs, it allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS.

Observation 5: Configuration of a common preamble may be required in order to enable UEs monitoring on-demand SIB1 without transmitting UL WUS.

Proposal 3: RAN2 to align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

Proposal: In case of NW operation in OD-SIB1 mode, UE in RRC_CONNECTED can obtain the SIBX from NW by the NW dedicated RRC for SIBX delivery. And then UE can use OD-SIB1 request procedure to acquire SIB1

Observation : RAN2 understanding is that UE can acquire SIBX in RRC_CONNECTED (being one reason not to include this information in RRCRelease message).

Proposal 6: If SIBX reception is not possible in RRC_CONNECTED then we should revisit decision on not having WUS configuration in RRCRelease.

Proposal 7: Agree to add in stage-3 CR UE to be allowed to use SIBX in case of cell selection (e.g. due to RLF/RRCRelease)  in order to reestabslish connection and/or camp in a cell using OD-SIB1 feature.

Proposal 8: Cell access related information should be included in the the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access.