3GPP TSG-RAN WG2 Meeting #129bis	R2-2502932

# Wuhan, China, 7th – 11th	April 2025

Agenda Item:	8.7.4.2

Source:	Ericsson

Title:	DSR enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN#105 the XR WID [1] for Rel-19 was updated with the following objective on the scheduling enhancements related to DSR:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

In RAN2#129 the enhanced DSR solution was further clarified on the MAC CE format and configuration details. Several FFS remains such as how to handle control and retransmission data or if new DSR will always be used when multiple thresholds are configured. This paper will discuss the remaining aspects.

# 2 Discussion on DSR enhancements

## 2.1 New Rel19 multi threshold DSR format

It was agreed in previous meetings that there can be multiple reporting thresholds configured for each LCG. For different traffic requirements and scheduler implementations there may be varying usefulness in the granularity of the reported information. It is therefore up to network decision how many thresholds that are used for each LCG. It has also been agreed that not all configured thresholds will be reported every time and that an extension bit in the DSR format will be used to convey if further thresholds for the LCG is present in the report. Through this solution only the thresholds that contains data need to be reported. How this works is shown in Figure 1. For each LCG this solution enables to start at any threshold, skip any threshold and report for any number of thresholds.

Figure 1: Mapping of data from the LCGs, with different remaining times and buffer sizes, to a new DSR format using multiple thresholds and a n-bit indicating if a further threshold is present for the current LCG. In the example for LCG 0 threshold D and B is reported. For LCG 1 only threshold C is reported.

## 2.2 Relation between triggering and reporting thresholds in DSR

It was also agreed that network can configure the triggering and reporting thresholds without constraints and that triggering threshold is not used as reporting threshold. This means that a DSR can be triggered but there is no data in any reporting threshold. This may be an unwanted behaviour, and then network simply does not make such configuration. However, it may also be wanted behaviour by the network, so that a trigger threshold is configured at a higher value than the reporting range (which is limited by the reporting threshold range, e.g. 1 to 64). In this case the DSR can report the value that is outside the reporting threshold range. This may be useful since the reporting range is quite limited with a max value of 64ms. The limitation is based on the available number of bits to use for the reporting values.

A trigger threshold can be configured with larger value than the range of the reporting thresholds.

When a DSR is triggered but no data resides inside the reporting threshold range the multi threshold DSR can indicate the data volume above the reporting threshold range.

## 2.3 Remaining time range of DSR

To increase the usefulness of the DSR for different types of traffic and applications it would be beneficial to increase the time reporting range. It can be assumed that the granularity can be less accurate the higher the remaining time is, thus one solution would be to configure new tables with varying granularity across the range.

RAN2 to consider solutions to increase the remaining time range for DSR.

## 2.4 Reporting of data volume in the Rel19 DSR

There is a remaining FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted. It has been argued that this type of data will likely be prioritized and thus should be reported in the lowest threshold. It has also been questioned what will happen if there is no delay reporting data present. A simple solution would be to always report the control and retransmission data in the lowest threshold and if there is no reporting data present use a reserved value, e.g. 1, to report this data. Using this low value (which very unlikely will be used by any delay reporting data) can provide an indication to network that there are only control and retransmission data in this threshold, and thus that the reported low remaining time value does not indicate such urgency.

PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted is to be reported in the lowest remaining time threshold configured. If no delay reporting data is present in this threshold use a reserved value for indicating the remaining time, e.g. 1ms.

## 2.5 Relation between multi-threshold and single threshold DSR

RAN2 has agreed that there will be no fallback to legacy DSR (single threshold DSR) or truncated DSR in case of limited PUSCH grant size. It was acknowledged that reporting with varying granularity will create problems with ambiguity for the network scheduler. There was discussion if multi threshold DSR should always be used as long as there were multiple thresholds configured for one LCG. This is a reasonable solution to avoid any ambiguity for the network scheduler. The only possible relaxation that can be allowed would be when the network can deduce what the granularity of the report is even when receiving a single threshold DSR. This can only be the case if there is data in a single threshold when there are multiple thresholds configured. If the rule is that the single threshold DSR is only reported for a multi threshold LCG in this case, the network will know when receiving the report that data is only inside the threshold belonging to the reported remaining time. In this way the single threshold DSR can be used for reporting also for multi threshold DSR without creating any ambiguity for the network scheduler.

If an LCG has multiple reporting thresholds configured a single threshold DSR can only be used for reporting when this creates no ambiguity for the network scheduler.

New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds and has data in more than one threshold.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	If an LCG has multiple reporting thresholds configured a single threshold DSR can only be used for reporting when this creates no ambiguity for the network scheduler.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A trigger threshold can be configured with larger value than the range of the reporting thresholds.

Proposal 2	When a DSR is triggered but no data resides inside the reporting threshold range the multi threshold DSR can indicate the data volume above the reporting threshold range.

Proposal 3	RAN2 to consider solutions to increase the remaining time range for DSR.

Proposal 4	PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted is to be reported in the lowest remaining time threshold configured. If no delay reporting data is present in this threshold use a reserved value for indicating the remaining time, e.g. 1ms.

Proposal 5	New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds and has data in more than one threshold.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024