3GPP TSG-RAN WG2 Meeting #129bis	R2-2502934

# Wuhan, China, 7th – 11th	April 2025

Agenda Item:	8.7.4.1

Source:	Ericsson

Title:	LCP enhancements

Document for:	Discussion, Decision

# 1	Introduction

At RAN#105 the XR WID [1] for Rel-19 was updated with the following objective on the scheduling enhancements related to LCP:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

In this contribution we continue the discussion on remaining details of the enhanced LCP solution. Furthermore, we provide a TP for how the enhanced LCP mechanism can be modelled.

# 2	Discussion on LCP enhancements

For the LCP enhancement it has been agreed to introduce an independent per-LCH remaining time threshold for applying adjusted priority. The adjusted priority will be applied for at least the first round of LCP as long as there is LCH priority-adjusted data available for transmission in the LCH. It has also been agreed that no additional PBR values are needed. However there is a FFS if that based on UE capability all the priority adjusted data can be transmitted before moving to the next LCH during the LCP procedure (i.e. transmit the priority adjusted data when Bj is negative). To analyse the usefulness of such solution it is important to look at some traffic scenarios where the solution can have an impact. Such analysis is provided in the following sections.

## 2.1 Scenarios to utilize priority-adjusting LCP

The priority-adjusting LCP enhancement will enable the LCP to support the prioritizations between multiple LCH in complex multi flow traffic scenarios. The most basic scenario is a two-traffic flow case, where one flow consists of latency critical XR traffic while the other flow consists of traditional eMBB traffic. Even in such a relatively simple traffic setup there are a multitude of possible solutions how to configure a priority-adjusting LCP procedure to best support the traffic. The following scenarios compare the solution using legacy Bj behaviour (i.e. stop selecting data from a LCH when reaching Bj = 0) or continue to select priority adjusted data when Bj < 0.

### 2.1.1 Scenario 1: Adjust XR flow priority with legacy Bj behaviour

In the first scenario the XR flow has a high priority setting and the eMBB flow has a low priority setting while the PBR settings are being balanced between the flows, i.e. the XR flow will be able to use most of the transmission resources but the eMBB flow will still get to transmit some data to not get starved. Eventually there may be instances where data from the XR traffic has not been served fast enough and data becomes more time critical, i.e. remaining time of the data falls below the time threshold and becomes LCH priority adjusted data. As can be seen in the Figure 1 only adjusting the priority will in this case not have any impact on the transmissions. The relative priority between the flows will remain the same and the XR flow will still be prioritized.

Figure 1: XR flow has high priority, eMBB flow has low priority and XR flow has balanced PBR setting to allow eMBB flow to also transmit some data and not be starved. During priority adjustment period only the priority is changed which doesn’t have any impact on the transmissions.

### 2.1.2 Scenario 2: Adjust XR flow priority with Bj negative behaviour

In the second scenario the XR flow is again configured with higher priority than the eMBB flow and the PBR settings are the same as in Scenario 1. However at this time the LCP procedure will apply the behaviour to transmit all the priority adjusted data during the priority adjusted period. The impact of such solution can be seen in Figure 2. In this case, despite the relative priorities between the logical channels being the same the LCH priority adjusted data in the XR flow will be transmitted faster (i.e. prioritized to be delivered).

Figure 2: XR flow has high priority, eMBB flow has low priority and XR flow has balanced PBR setting to allow eMBB flow to also transmit some data and not be starved. During priority adjustment the LCP procedure transmit all the priority adjusted data before moving to the next LCH (referred to as the “Bj negative” behaviour).

### 2.1.3 Conclusion

When analysing LCP procedure in a basic multi flow traffic scenario it is evident that only adjusting the priority is not a complete solution to handle time critical data.

- Simply adjusting the priority is not a complete solution for handling time critical data in the LCP procedure.

Being able to adjust how much data that is transmitted from a LCH is essential to limit the delays in these traffic scenarios. Based on this we propose the following.

RAN2 to agree on the FFS that all LCH priority adjusted data can be transmitted from a LCH, despite Bj being negative, before moving to the next LCH in the LCP procedure.

The behaviour in Proposal 1 should be controlled by network configuration, as the wanted behaviour depends on the traffic served and LCP parameters configured.

## 2.2 Priority-adjusting LCP enhancement in specification

To introduce the solution in Proposal 1 there is a need to apply some changes to the LCP procedure in MAC specification. To keep track of the transmission of the LCH priority-adjusted data a new counter, Rj, can be applied. This counter can be configured so that only when all of the LCH priority-adjusted data is transmitted from the LCH the resource allocation moves on to another lower priority LCH. This solution assures that the time critical data is being included in the transmission before any other low important data. An example of how this new counter Rj is tracking the data in the LCH is shown in Figure 6.

Figure 6: Examples of how the counter Rj can be calculated for two different LCHs.

We thus propose for the priority-adjusting LCP enhancements:

A priority-adjusting LCP solution uses a new counter variable to keep track of the scheduling of the LCH priority-adjusted data.

The described priority-adjusting LCP solution is simple to implement and understand in the specification. It ensures a fair LCP procedure while at the same time enabling that the LCH priority-adjusted data is transmitted before other data when possible. A text proposal implementing the priority-adjusting LCP solution using the new counter Rj and overriding the PBR can be found in Appendix 1.

Implement the TP provided in Appendix 1.

Note that the text proposal is written such that the priority-adjusting LCP behaviour is controlled by the additional RRC parameters.

## 2.3 Relation to Intra-UE prioritization

At RAN2#129 the following was agreed concerning Intra-UE prioritization and SR priority:

There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority.

Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact

FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination

As a starting point for this discussion, it should be noted that the priority of a LCID is changed during the logical channel prioritization. According to the MAC specifications, this procedure is executed whenever a new transmission is performed. Triggering of a BSR and transmission of the SR does not result in a Logical Channel Prioritization and, therefore, the priority of the LCID will be the “legacy” value when a SR is triggered.

Ultimately, changing the priority of the logical channel had the intent to emulate a simple delay-aware scheduler, allowing the UE to choose data which remaining latency was below a threshold. The purpose was never to impact other procedures and processes at the UE. Thus, UL grant priority determination and SR priority determination should simply use the legacy value.

Intra-UE prioritization shall not use the additional LCP priority for SR priority determination.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	Simply adjusting the priority is not a complete solution for handling time critical data in the LCP procedure.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree on the FFS that all LCH priority adjusted data can be transmitted from a LCH, despite Bj being negative, before moving to the next LCH in the LCP procedure.

Proposal 2	The behaviour in Proposal 1 should be controlled by network configuration, as the wanted behaviour depends on the traffic served and LCP parameters configured.

Proposal 3	A priority-adjusting LCP solution uses a new counter variable to keep track of the scheduling of the LCH priority-adjusted data.

Proposal 4	Implement the TP provided in Appendix 1.

Proposal 5	Intra-UE prioritization shall not use the additional LCP priority for SR priority determination.

# References

RP-241771 Revised WID: XR (eXtended Reality) for NR Phase 3, September 2024

# Appendix 1

Changes to specification TS 38.321, additions highlighted in yellow:

### 5.4.3	Multiplexing and assembly

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

-	timeCriticalPriority an alternative priority value.

-	timeCriticalTimeThreshold which set the remainingTimeThreshold (RETT).

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

-	if timeCriticalPriority is configured, Rj which is maintained for each logical channel j.

The MAC entity shall initialize Bj and Rj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

1>	if timeCriticalPriority is configured:

2>	if the remaining time until discard for any MAC SDU in logical channel j is less than or equal to RETT:

3>	set Rj to the size in bytes for MAC SDUs with remaining time less or equal to RETT.

3>	adjust priority of logical channel j to timeCriticalPriority

2>	if the remaining time until discard for all MAC SDU in logical channel j is greater than RETT:

3>	set Rj to zero.

3>	adjust priority of logical channel j to priority.

NOTE:	The exact moment(s) when the UE updates Bj and Rj between LCP procedures is up to UE implementation, as long as Bj and Rj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 or Rj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2> decrement Rj by the total size of MAC SDUs with remaining time less or equal to RETT served to logical channel j above;

2>	if Rj ≤ 0:

3>	set Rj to zero.

3>	adjust priority of logical channel j to priority.

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.