3GPP TSG-RAN WG2 Meeting #129bis	R2-2502945

Wuhan, China, 7th – 11th April 2025

Agenda Item:	8.6.4

Source:	Huawei, HiSilicon

Title:	Intra-CU conditional LTM

Document for: Discussion and Decision

# Introduction

At RAN #107, the WID for Rel-19 NR mobility enhancements Phase 4 was updated. According to the updated WID [1], conditional LTM (C-LTM) should be specified for the intra-CU case in Rel-19, with limitations defined for the scenario where the UE is in a non-DC case.

RP-250339, Revised Work Item: NR mobility enhancements Phase 4 - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

In RAN2#127bis, #128 and #129 meetings, RAN2 has already made some agreements. In this contribution, we discuss the CG-based RACH-less procedures for intra-CU C-LTM, TA delivery and maintenance, and the co-existence between network triggered LTM and C-LTM.

# 2	Discussion

## 2.1	CG-based RACH-less cell switch

RAN2#129 meeting Agreements on C-LTM: 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

According to the agreements made during the previous meeting for L1-based C-LTM, the beam selection for the RACH-less CG LTM cell switch is based on whether the beam satisfies the C-LTM execution condition. If only a single beam meets the C-LTM execution condition, the UE should perform the CG-based RACH-less LTM cell switch using this beam to access the target cell. If multiple beams satisfy the C-LTM execution condition, the beam selection is left to the UE implementation.

In Rel-18, the CG for RACH-less configuration can indicate a subset of SSBs, so it is possible that not all candidate beams are configured with CG resources. If the UE would select a beam that is not associated with CG resources, only a RACH-based cell switch could be performed, which would result in increased mobility latency compared to the RACH-less cell switch. Consequently, the UE should prioritize beams that have associated CG occasions when making the beam selection and if none of the beams that fulfil the execution condition have an associated CG occasion, the UE should perform RACH-based LTM cell switch.

Proposal 1a: If the L1-based C-LTM execution condition is met, to select a beam, the UE prioritizes beams that have associated CG occasions.

Proposal 1b: If none of the beams that meet the L1-based C-LTM execution condition have an associated CG occasion, the UE performs RACH-based LTM cell switch.

For L3-based C-LTM execution, RAN2 did not decide how to perform beam selection. As discussed above, some SSB many not have any associated CG occasion, so the UE could also give priority to the beams that have associated CG occasions. However, for L3-based C-LTM execution, there is no way to judge whether a beam is suitable or not to be used for RACH-less LTM cell switch, as the L3-based condition is normally not applies on individual beams, and may not be suitable for this purpose.

In the legacy RACH-based LTM cell switch procedure, the UE selects an SSB with an SS-RSRP above the rsrp-ThresholdSSB among the candidate SSBs, where the parameter rsrp-ThresholdSSB is configured during the LTM preparation procedure.

TS 38.321-i40 clause 5.1.2 1> else if contention-free Random Access Resources have not been explicitly provided by an LTM Cell Switch Command MAC CE, the Random Access procedure was not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3, contention-free Random Access Resources associated with SSBs have been explicitly provided in rach-ConfigDedicated and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available: 2> select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs; 2> set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB. …… 1> else (i.e. for the contention-based Random Access preamble selection): 2> if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available: 3> select an SSB with SS-RSRP above rsrp-ThresholdSSB. 2> else: 3> select any SSB.

A similar threshold could be used to determine whether a beam is suitable for RACH-less LTM cell switch. If the candidate cell satisfies the L3 execution condition, the UE should select a target beam that exceeds the threshold. If none of the beams above the threshold is associated with a CG occasion, the UE should perform RACH-based LTM cell switch.

Proposal 2a: If the L3 execution condition is met, the UE can only select beams above a threshold (FFS whether to reuse an existing threshold or define a new one).

Proposal 2b: If the L3 execution condition is met, the UE prioritizes beams that have an associated CG occasion.

Proposal 2c: If the L3 execution condition is met but no beam above the threshold has an associated CG occasion, the UE performs RACH-based LTM cell switch.

If the target cell uses joint TCI mode, for a particular CG occasion, the target cell knows which UL beam to use to receive the UE UL message and also knows which DL beam to use to schedule the UE afterwards. If the target cell uses separate TCI mode, the network may not know the beam to use for DL transmissions since RAN2 agreed that the CG occasions are associated with UL TCI states when separate TCI mode is used. However, it would be possible for the UE to include in the first UL transmission an indication of a good DL beam, so that the network can further schedule the UE. There is a risk that this first UL transmission is not successfully decoded by the network and then the network cannot schedule a retransmission, but the network could configure automatic repetition in the CG so that the UE autonomously performs retransmissions if the network does not provide a further UL grant.

Proposal 3: For CG-based RACH-less cell switch, if separate TCI mode is used, the UE indicates a suitable DL TCI state in the initial UL transmission (details FFS).

## 2.2	TA delivery and maintenance

### 2.2.1	TA Command MAC CE

RAN2#128 meeting Agreements on C-LTM: 9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

RAN2#129 meeting Agreements on MAC issues: 4. The CLTM TAT is per candidate configuration.

Editor’s NOTE: Whether the new MAC CE “LTM Candidate Timing Advance Command MAC CE” from the source cell indicates the LTM candidate cell or TAG is FFS.

For PDCCH-ordered early RACH, RAN2 has agreed that it is the source cell that sends the Timing Advance to the UE via a new MAC CE. RAN2 should further discuss how to transmit the TA value in this MAC CE for C-LTM early TA acquisition, specifically whether it should be transmitted per candidate cell or per TA group. According to the agreement, the C-LTM TA timer is configured per candidate cell. Although using a TA per TA group may save several bits compared to transmitting the TA value per candidate cell in the MAC CE, this approach would not align with the C-LTM TA timer. The C-LTM TA timer is used to verify the validity of the TA value, therefore, the TA value should also be transmitted per candidate cell to keep alignment with the C-LTM TA timer.

Proposal 4: The TA value in the LTM Candidate Timing Advance Command MAC CE from the source cell should be specified for each LTM candidate cell.

### 2.2.2	TAT associated with PTAG

RAN2#129 meeting CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell. Rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session: - Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18; - Option 2 (keep the offline agreement): discuss the options below in the MAC running CR o Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR; o Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running. o Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer. Will be revisited next meeting.

RAN2 discussed how to initiate the TAT associated with PTAG upon the execution of the C-LTM cell switch to the target cell, and how the running C-LTM TAT functions during this process. We should now consider which option is most applicable for the Rel-19 C-LTM cell switch.

Option 1 is similar to the Rel-18 mechanism, where the TAT associated with PTAG is initiated when the LTM cell switch procedure begins. Upon receiving the TA command from the network, the UE starts the C-LTM TAT. During the LTM cell switch procedure, the UE disregards the remaining time of the C-LTM TAT, stops it, and begins the TA timer associated with the PTAG. While this approach is simple, it needs to stop the CLTM TAT and starting the TAT associated with PTAG, which also uses the same TA value as before, without any change in the TA value itself.

Option 2.1 clarifies that the running C-LTM TAT acts as the timeAlignmentTimer associated with PTAG, while Option 2.3 suggests defining that the UE can perform UL transmission when the TAT for the target cell is running. We prefer the option-2.3 since discussion how the TAT associated with PTAG functions after the LTM cell switch should be considered as first.

Option 2.3 differs from the legacy Rel-18 mechanism, as the TAT associated with PTAG is started upon the LTM cell switch procedure using the remaining TA time from the C-LTM timer. Since there is no change in the TA value during the LTM cell switch, there is no need for the UE to ignore the remaining TA time. Therefore, the remaining time of the running TAT should be aligned with the PTAG after the cell switch. As a result, Option 2.3 should be supported.

Proposal 5: Upon C-LTM cell switch execution, the UE stops the C-LTM timer and starts the PTAG using the remaining time from the C-LTM timer (Option 2.3).

### 2.2.3	TA maintenance for other candidate cell(s)

RAN2#129 meeting Agreements on MAC issues: 5. FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

According to the agreement made during the offline discussions, RAN2 should further address whether the received C-LTM TAs for other candidate cells should be released or remain valid at the UE side upon the execution of the C-LTM cell switch. If the UE releases the stored TA values after the cell switch, the network needs to transmit the TA command MAC CE again for the subsequent LTM, which requires additional downlink transmission resources. If the UE retains these TA values, there is no need for the network to transmit them again, which is a simpler solution. In this case, the corresponding TA timer should also be maintained, in order to verify the validity of the TA values.

Proposal 6: Upon LTM cell switch execution, the received TA(s) for other candidate cell(s) remain valid until the associated timer expires.

Editor’s NOTE: How to handle ltm-Candidate-TimeAlignmentTimers when CLTM cell switch and MAC reset for other cases needs further discussion.

We should also consider how to handle the C-LTM TAT of the candidate cells when a handover occurs. At the handover, the UE stops all TATs and considers them as expired. However, since the TAT associated with PTAG is maintained after the cell switch procedure, as mentioned above, the C-LTM TAT should not be stopped during a LTM handover.

Proposal 7: A handover, the received TA(s) for LTM candidate cell(s) remain valid until the associated timer expires (the validity timers for TAs of LTM candidate cells is not be stopped).

Maintaining the C-LTM TAT after the cell switch requires that the TA remaining time for these candidate cells to be known by the target cell. Otherwise, the target cell needs to initiate the PDCCH order-triggered early RACH procedure to restart the C-LTM TAT.

Proposal 8: The target cell of LTM cell switch should know the remaining validity time of other candidate cells.

### 2.2.4	C-LTM TA re-acquisition

If the validity timer for the C-LTM TA of an LTM candidate cell is about to expire while the UE still might perform a cell switch to the candidate cell, the source may want to send a PDCCH order to renew the C-LTM TA so that RACH-less LTM cell switch is still possible.

Proposal 9: The source DU can maintain the C-LTM TAT before the timer expires and re-start the early UL synchronization procedure prior to expiration.

## 2.3	Co-existence between network triggered LTM and C-LTM

Conditional LTM shares a lot of characteristics with Rel-18 LTM: L1 measurement reporting, early RA, early TCI state activation, LTM cell switch procedure, using candidate and possibly reference configuration. Apart from the reception of a MAC CE following early RA and the use of execution conditions instead of a MAC CE, the UE behaviour is the same and then the configuration of a candidate LTM cell should be exactly the same plus the execution conditions, as is already the case in the draft RRC CR.

For all the procedures that the UE performs before LTM cell switch, whether an LTM candidate is for conditional LTM or for Rel-18 LTM does make any difference, so it is possible for LTM and C-LTM to be configured simultaneously.

Proposal 10: The UE can be simultaneously configured with Rel-18 LTM candidate configurations and with C-LTM candidate configurations.

As discussed above, when an LTM candidate includes execution condition, apart from the measurements for the execution condition and the reception of a TA value, all the UE behaviours associated with this LTM candidate are the same. Therefore, there would be no additional specification complexity for the UE to execute LTM cell switch to a C-LTM candidate if the network sends an LTM cell switch command MAC CE to the UE indicating this candidate. There can be cases where such a scenario is useful e.g., for load balancing while the candidate cell is good enough according to L1 or L3 measurement reports but the execution condition is not triggered.

Proposal 11: Upon reception of an LTM cell switch command MAC CE, the UE performs LTM cell switch to the indicated candidate, regardless whether it is configured for Rel-18 LTM or for conditional LTM.

# 3	Conclusion

This contribution makes the following proposals:

CG-based RACH-less cell switch

Proposal 1a: If the L1-based C-LTM execution condition is met, to select a beam, the UE prioritizes beams that have associated CG occasions.

Proposal 1b: If none of the beams that meet the L1-based C-LTM execution condition have an associated CG occasion, the UE performs RACH-based LTM cell switch.

Proposal 2a: If the L3 execution condition is met, the UE can only select beams above a threshold (FFS whether to reuse an existing threshold or define a new one).

Proposal 2b: If the L3 execution condition is met, the UE prioritizes beams that have an associated CG occasion.

Proposal 2c: If the L3 execution condition is met but no beam above the threshold has an associated CG occasion, the UE performs RACH-based LTM cell switch.

Proposal 3: For CG-based RACH-less cell switch, if separate TCI mode is used, the UE indicates a suitable DL TCI state in the initial UL transmission (details FFS).

TA delivery and maintenance

Proposal 4: The TA value in the LTM Candidate Timing Advance Command MAC CE from the source cell should be specified for each LTM candidate cell.

Proposal 5: Upon C-LTM cell switch execution, the UE stops the C-LTM timer and starts the PTAG using the remaining time from the C-LTM timer (Option 2.3).

Proposal 6: Upon LTM cell switch execution, the received TA(s) for other candidate cell(s) remain valid until the associated timer expires.

Proposal 7: A handover, the received TA(s) for LTM candidate cell(s) remain valid until the associated timer expires (the validity timers for TAs of LTM candidate cells is not be stopped).

Proposal 8: The target cell of LTM cell switch should know the remaining validity time of other candidate cells.

Proposal 9: The source DU can maintain the C-LTM TAT before the timer expires and re-start the early UL synchronization procedure prior to expiration.

Co-existence between LTM and C-LTM

Proposal 10: The UE can be simultaneously configured with Rel-18 LTM candidate configurations and with C-LTM candidate configurations.

Proposal 11: Upon reception of an LTM cell switch command MAC CE, the UE performs LTM cell switch to the indicated candidate, regardless whether it is configured for Rel-18 LTM or for conditional LTM.

# 4	Reference

- RP-250339, Revised Work Item: NR mobility enhancements Phase 4 Apple Inc, China Telecom.