3GPP TSG-RAN WG2 Meeting #129bis		                               R2-2502972

Wuhan, China, April. 7th – 11th, 2024

Agenda Item:		8.7.5

Source:			MediaTek Inc.

Title:				Remaining issues for XR RLC AM enhancement

Document:			Discussion and decision.

### 1	Introduction

This document discusses RLC AM enhancement according to the fourth objective in R19 XR WID [1].

-	Specify the following user plane enhancements [RAN2]:

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

### 2	Discussion

In current RLC running CR [2], there are still some Editor’s Note need to be addressed. The first one is whether we need to distinguish retransmission due to NACK in RLC status report and autonomous retransmission. Most companies think there is no need to distinguish them. We also agree with that. Then, this leads to another question, whether we need another separate clause to capture autonomous retransmission behaviors? Or merge autonomous retransmission into clause 5.3.2. We think the concept of autonomous retransmission is a special case of ARQ, therefore, it is appropriate to capture autonomous retransmission behaviors into clause 5.3.2. For example: RETX_COUNT should be incremented due to autonomous retransmission.

Proposal 1: Capture autonomous retransmission behaviors into RLC spec clause 5.3.2.

Proposal 2: RETX_COUNT should be increased by 1 due to autonomous retransmission.

In RAN2 #129 meeting, we had one agreement ”Only a single autonomous retransmission will be triggered per RLC SDU.” In order to handle this agreement and distinguish the behavior from current state variable (i.e., RETX_COUNT), we need to introduce a new state variable to capture autonomous retransmission (E.g., AUTO_RETX_COUNT). For example, AUTO_RETX_COUNT is initially set to 0, and increase by 1 when autonomous retransmission is triggered. And check when AUTO_RETX_COUNT equal to 1 or not when retransmission is triggered.

|  | Normal retransmission | Autonomous retransmission |
| --- | --- | --- |
| RETX_COUNT | Increment by 1 | Increment by 1 |
| New state variable (E.g., AUTO_RETX_COUNT) | N.A. | Increment by 1 |

Proposal 3: Introduce a new state variable (E.g., AUTO_RETX_COUNT) to capture only a single autonomous retransmission is allowed per RLC SDU.

### 3	Conclusion

This document promulgated the following proposals:

Proposal 1: Capture autonomous retransmission behaviors into RLC spec clause 5.3.2.

Proposal 2: RETX_COUNT should be increased by 1 due to autonomous retransmission.

Proposal 3: Introduce a new state variable (E.g., AUTO_RETX_COUNT) to capture only a single autonomous retransmission is allowed per RLC SDU.

### 4	Reference

[1]	RP-250107, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur).

[2]       R2-2502162, RLC running CR for XR, vivo.