3GPP TSG RAN WG2 Meeting #129bis            	                      R2-2502974

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.10.1

Source:	Xiaomi

Title:	Discussion on Rel-19 SONMDT open issues

Document for:	Discussion and Decision

Introduction

In the post RAN2 #129 meeting [Post129][601][SONMDT] Running NR RRC CR (Ericsson) [1], the RRC Running CR captured previous RAN2 agreements pertains to Rel-19 SON/MDT WI, and some issues are left which need to be brought for online decision.

In this contribution, we discuss some RRC left issues and summarize them for discussion, e.g., UE capability of supporting RLF reports/SHR for LTM cell switch, last handover type, explicit or implicit indicator to indicate whether a neighbour cell is an LTM candidate cell. Additionally, some companies have different understandings of previous agreements, e.g., the definition of "timeBetweenFulfillment", whether the agreement of SDT from RAN2#126bis meeting is in conjunction with the 1st agreement from RAN2#127-bis meeting.

Discussion

Summary of open issues

The below figure denotes the open issues related to [Post129][601][SONMDT] Running NR RRC CR (Ericsson) [1].

Table 1: stage-3 open issue list for Rel-19 SON/MDT

| Topic | Open issues | Remark |
| --- | --- | --- |
| (1) UE capability | a) FFS of the UE capability of supporting RLF-Report for LTM cell switch? b) FFS of UE capability of supporting SHR for LTM cell switch? c) Whether to check UE capability of reporting CHO with candidate SCG information in RRC specs? | Until RAN2#129 meeting, discussion on the UE capability of Rel-19 SON/MDT had not been initiated. For a) and b), the answer will determine whether RRC CR needs to capture “if the UE supports successful handover report for LTM(SHR)” in each related sentence. One example is listed as below: 4> if the UE supports successful handover report for LTM and the cell was an MCG LTM candidate target cell contained in the current UE configuration: 5> set the ltm-Candidate to true in measResultNR; For c), some companies commented on the highlighted sentence that the additional information for CHO with candidate SCG(s) should be included only when it is configured and the UE has capability to report CHO with candidate SCG information. 1> if the SS/PBCH block-based measurement quantities are available: Omitted… 2> set the rsIndexResults in measResultLastServPSCell to include all the available measurement quantities of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPScell, ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure; |
| (2) CHO with candidate SCGs | a) FFS how to avoid duplication of measurements? b) FFS how to capture the following agreement: “Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any.” c) FFS lastHO-Type for CHO with Candidate SCG. d) FFS whether to log the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and handover execution in SHR when only one condition (CHO or CPAC) is fulfilled. e) Whether to add “firstFulfilledConfig”, “timeBetweenFulfillment”, and “timeBetweenLastFulfillmentAndEvent” directly in SCGFailureInformation or include this information in choWithCandidateSCGInfoList IE. f) Whether to put the cell id and measurement results together in SHR (similar as sourceCellInfo/ targetCellInfo structure in SHR)? | For a): For CHO with candidate SCGs, RAN2#127 meeting agreed as follows: UE includes following information in RLF report: Measurement results of PCells and PSCells. RAN2#128 meeting agreed as follows: Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this. Companies have concerns on duplication PCells and PScells measurements logging in the RLF report (SCGFailureInformation message), the example is listed as below: 1> if the UE has both condExecutionCond and condExecutionCondPSCell available in the MCG VarConditionalReconfig at the moment of the SCG failure, for each ChoWithCandidateSCGInfo in choWithCandidateSCGInfoList: Omitted… 2> include the available measurement quantities (SS/PBCH block or CSI-RS) in the MeasResults; 2> set the measResultNeighCellList in measResultFreqList to include the best measured cells, other than candidate PCells, and candidate PSCells if the UE was configured with condExecutionCond and condExecutionCondPScell ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows; For b) RAN2#129 agreed as below: Enhance SCGFailureInformation for CHO with candidate SCGs to include the information for each CHO, i.e., first fulfilled event and time duration between two events fulfilled, if any. For c) Some companies propose that existing lastHO-Type of cho already covers CHO with Candidate SCG(s), as they all fit into “reconfigurationWithSync was concerning a conditional handover, while some companies argued that for a scenario of failure shortly after the CHO with candidate SCG, there might not be additional information to differentiate the last handover was CHO, or CHO with candidate SCG if lastHO-Type of cho with candidate SCGs is not introduced. For d) No discussion and agreement are made for this. For e) In both RLF-Report and SuccessHO-Report, the below structure is captured. choCandidateCellList-r17 ChoCandidateCellList-r17 OPTIONAL For f) sourceCellInfo/ targetCellInfo structure captured in SHR in current TS 38.331 are listed as below: SuccessHO-Report-r17 ::= SEQUENCE { sourceCellInfo-r17 SEQUENCE { sourcePCellId-r17 CGI-Info-Logging-r16, sourceCellMeas-r17 MeasResultSuccessHONR-r17 OPTIONAL, rlf-InSourceDAPS-r17 ENUMERATED {true} OPTIONAL }, targetCellInfo-r17 SEQUENCE { targetPCellId-r17 CGI-Info-Logging-r16, targetCellMeas-r17 MeasResultSuccessHONR-r17 OPTIONAL }, |
| (3) LTM | a) For RACH-less LTM cell switch in MCG, the LTM cell switch execution is considered as successfully completed when successfully sending RRCReconfigurationComplete message, thus UE cannot determine the successHO-InfoAvailable in this RRCReconfigurationComplete message. FFS whether and how to fix this. b) FFS on whether to explicitly introduce ltmCandidate (similar to choCandidate) in SHR. | For a) RAN2#127-bis meeting agreed to include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation. UE logs available L1 measurement results for the serving cell, the target cell and other LTM candidate cells when a successful LTM cell switch triggers SHR. For b): RAN2#127 meeting agreed that we aim to log some info to deduce the ltmCandidate (similar to choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. RAN3 has not discussed this issue for SHR, while making decisions already on the RLF reports: RAN3#127 made the agreement of “Network based solution is used for LTM candidate cell list., which means to add a list indicating which LTM candidates the UE had at RLF. |
| (4)SDT | a) How to capture RSRP and Data volume in case of SDT failure in RA-report? b) For the definition of timeSinceSdtExecution IE, FFS for the case beyond this value and the understanding from UE and network side, as for value beyond this range NW cannot recognize the right SDT configuration. | For a), RAN2#127bis made agreements: 1) For failed SDT case, UE includes the DL RSRP and UL data volume at the time of SDT evaluation in SON report. For successful SDT procedure, the UE does not log. RAN2#128 made agreements: 1) The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report For b) RAN2#129 made the below agreements: Introduce a new field in RA-Report to indicate the elapsed time since the execution of RA-SDT. Value in seconds. The maximum value is 172800 seconds. Some examples of how to handle the case beyond the maximum time value are listed as below: timeSCGFailure This field is used to indicate the time elapsed since the last execution of RRCReconfiguration with reconfigurationWithSync for the SCG until the SCG failure. Actual value = field value * 100ms. The maximum value 1023 means 102.3s or longer. timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment or resume) failure. Value in seconds. The maximum value 172800 means 172800s or longer. elapsedTimeSCG-Failure This field is used to indicate the time elapsed between the SCG failure and the MCG failure. The maximum value 1023 means 1023ms or longer. |

For capability-related issues, the decision to postpone capability checks depends on whether RLF-Report and SHR reporting for LTM cell switch is optional or not. If these capabilities are optional, RRC specifications must include conditional clauses like "if the UE supports successful handover report for LTM(SHR)" in all relevant procedures to ensure backward compatibility. Conversely, if they are mandatory, such conditional wording would be redundant and complicate the specs. A similar dependency applies to checking UE capability for reporting CHO with candidate SCG information. Thus, it is proposed to postpone capability checks in the RRC running CR until Rel-19 SON/MDT capability discussions are initiated.

Proposal 1: RAN2 to postpone Rel-19 SON/MDT capability checks in the RRC running CR until the corresponding capability discussion is triggered.

For CHO with candidate SCGs issue a), the current stage-3 running CR captures PCell/PSCell measurement results in MeasResults IE of RLF reports (SCGFailureInformation), while excluding them from measResultNeighCellList to avoid duplication. However, some companies believe the existing specs already log these measurements.

Observation 1: Two options exist for capturing PCell/PSCell measurements in RLF reports (or SCGFailureInformation):

Option 1: Reuse current stage-3 wording, including PCell/PSCell measurements in neighbour measurements.

Option 2: Include PCell/PSCell measurements in MeasResults IE and exclude them from neighbour measurements.

While Option 2 aims to prevent measurement duplication by segregating PCell/PSCell data into a new MeasResults IE, it introduces unnecessary complexity without functional improvements. Option 1 preserves specs stability and simplicity by reusing existing wording, which can ensure better compatibility with previous specs versions.

Proposal 2: For CHO with candidate SCGs, RAN2 to reuse the current stage-3 description, i.e.  include PCell and PSCell measurement results in neighbor measurements within RLF reports (or SCGFailureInformation).

For CHO with candidate SCGs issue b), logging time duration between events in SCGFailureInformation is only meaningful when both CHO and CPAC conditions are fulfilled. If only one condition is met, such timing cannot be determined.

Proposal 3: For CHO with candidate SCGs, RAN2 to clarify that the time duration between two fulfilled events in SCGFailureInformation shall only be included when both CHO and CPAC conditions are satisfied.

For CHO with candidate SCGs issue c), some companies propose that existing lastHO-Type of cho already covers CHO with Candidate SCG(s), as they all fit into “reconfigurationWithSync was concerning a conditional handover. However, for a scenario of failure shortly after the CHO with candidate SCG, there might not be additional information to differentiate the last handover was CHO, or CHO with candidate SCG if lastHO-Type of cho with candidate SCGs is not introduced. This lack of clarity can lead to misinterpretation by the network, making it hard to accurately diagnose problems. So, it is better to define a new explicit lastHO-Type for CHO with candidate SCGs to avoid network misinterpretation.

Proposal 4: For CHO with candidate SCGs, RAN2 to explicitly define a new lastHO-Type for CHO with candidate SCGs.

For CHO with candidate SCGs issue d), it’s clear that successfully handover execution does not occur if only one of condExecutionCond or condExecutionCondPSCell is fulfilled, making it unnecessary to log elapsed time between condition fulfillment in such cases as the handover never proceeded.

Proposal 5: For CHO with candidate SCGs, logging of elapsed time between fulfilling the last triggering event and handover execution in SHR is not required when only one condition (CHO or CPAC) is fulfilled.

For CHO with candidate SCGs issue e), including firstFulfilledConfig, timeBetweenFulfillment, and timeBetweenLastFulfillmentAndEvent in choWithCandidateSCGInfoList IE aligns with existing RLF/SHR structures and ASN.1 modularity, which avoids structural duplication, and maintains backward compatibility, making it preferable to adding them directly to SCGFailureInformation.

Proposal 6: For CHO with candidate SCGs, RAN2 to agree to include “firstFulfilledConfig”, “timeBetweenFulfillment”, and “timeBetweenLastFulfillmentAndEvent” in the choWithCandidateSCGInfoList IE within the SCGFailureInformation message.

For CHO with candidate SCGs issue f), for the approach of separating cell ID and measurement, its key advantage lies in enabling independent extension or modification for either the cell ID or measurement segment, thus offering higher flexibility. In contrast, combining cell ID and measurement results in SHR (similar to the sourceCellInfo / targetCellInfo structure), yet its specific benefits remain unclear. Therefore, further discussion is needed.

Proposal 7: For CHO with candidate SCGs, RAN2 to discuss consolidating cell ID and measurement results in SHR (similar to the sourceCellInfo/targetCellInfo structure)

For LTM related issue a), In the MCG RACH-less LTM cell switch scenario, sending the RRCReconfigurationComplete message marks successful switch execution, but the UE cannot include successHO-InfoAvailable (indicating SHR availability) in this message. If unaddressed, the gNB lacks clarity on whether the UE has prepared SHR, thus disrupts the SHR retrieving procedures. Companies also proposed to reuse TS 38.321/TS 38.300 wording (LTM cell switch is denoted as success when the UE confirms the network received its first UL data) for successful LTM execution in stage-3 specifications. This approach seems workable. However, translating it into the wording of TS 38.331 remains a subject for further discussion.

Proposal 8: RAN2 should discuss standardized wording for successful LTM execution in stage-3 specifications (e.g., "UE confirms network received first UL data" or "successful transmission of RRCReconfigurationComplete").

For LTM related issue b), adopting an explicit ltmCandidate indication in SHR would simplify implementation by directly flagging LTM candidates, mirroring the choCandidate approach. However, this deviates from RAN3’s RLF decision to use a network-based implicit solution (adding a ltmCandidateList in RLF reports), creating inconsistency across RLF reports and SHR. Conversely, reusing RAN3’s network-based method for SHR may introduce some complexity. This issue needs RAN3’s decision and feedback.

Proposal 9: RAN2 to send an LS to RAN3 to determine whether to implicitly introduce ltmCandidate in SHR, i.e. add a list indicating which LTM candidates the UE had in the SHR.

For SDT issue a), there are two agreements related to DL RSRP and UL data volume logging in RA-report, which are associated with different cases. For failed SDT case, which means UE initiated SDT procedure successfully but failed to execute SDT. For the case that UE failed to initiate of RA-SDT, which means UE initiate RA procedure not the RA-SDT procedure. RAN2 needs to clarify these two cases are separate to avoid misunderstanding in RRC specs, e.g. SDT UL data volume is included when the RA report entry is included because of SDT initiation failure or if the SDT procedure failed.

Proposal 10: For SDT, RAN2 to clarify that there are two scenarios that UE needs to log DL RSRP and UL data volume in the RA-report:

Case 1: UE initiated SDT but failed execution.

Case 2: UE failed to initiate RA-SDT and instead initiated RACH.

For SDT issue b), as refer to the timeSCGFailure, timeSinceFailure, and elapsedTimeSCG-Failure IE, the maximum value (e.g., xx seconds) in those IEs implies that values exceeding this threshold are treated as "xx seconds or longer.", which can easily fix the issue that the timeSinceSdtExecution value is beyond its value range.

Proposal 11: For SDT, the maximum value of 172800 in timeSinceSdtExecution IE shall indicate 172800 seconds or longer.

Conclusion

In this contribution, we propose:

Observation 1: Two options exist for capturing PCell/PSCell measurements in RLF reports (or SCGFailureInformation):

Option 1: Reuse current stage-3 wording, including PCell/PSCell measurements in neighbour measurements.

Option 2: Include PCell/PSCell measurements in MeasResults IE and exclude them from neighbour measurements.

Proposal 1: RAN2 to postpone Rel-19 SON/MDT capability checks in the RRC running CR until the corresponding capability discussion is triggered.

Proposal 2: For CHO with candidate SCGs, RAN2 to reuse the current stage-3 description, i.e.  include PCell and PSCell measurement results in neighbor measurements within RLF reports (or SCGFailureInformation).

Proposal 3: For CHO with candidate SCGs, RAN2 to clarify that the time duration between two fulfilled events in SCGFailureInformation shall only be included when both CHO and CPAC conditions are satisfied.

Proposal 4: For CHO with candidate SCGs, RAN2 to explicitly define a new lastHO-Type for CHO with candidate SCGs.

Proposal 5: For CHO with candidate SCGs, logging of elapsed time between fulfilling the last triggering event and handover execution in SHR is not required when only one condition (CHO or CPAC) is fulfilled.

Proposal 6: For CHO with candidate SCGs, RAN2 to agree to include “firstFulfilledConfig”, “timeBetweenFulfillment”, and “timeBetweenLastFulfillmentAndEvent” in the choWithCandidateSCGInfoList IE within the SCGFailureInformation message.

Proposal 7: For CHO with candidate SCGs, RAN2 to discuss consolidating cell ID and measurement results in SHR (similar to the sourceCellInfo/targetCellInfo structure)

Proposal 8: RAN2 should discuss standardized wording for successful LTM execution in stage-3 specifications (e.g., "UE confirms network received first UL data" or "successful transmission of RRCReconfigurationComplete").

Proposal 9: RAN2 to send an LS to RAN3 to determine whether to implicitly introduce ltmCandidate in SHR, i.e. add a list indicating which LTM candidates the UE had in the SHR.

Proposal 10: For SDT, RAN2 to clarify that there are two scenarios that UE needs to log DL RSRP and UL data volume in the RA-report:

Case 1: UE initiated SDT but failed execution.

Case 2: UE failed to initiate RA-SDT and instead initiated RACH.

Proposal 11: For SDT, the maximum value of 172800 in timeSinceSdtExecution IE shall indicate 172800 seconds or longer.

Reference

[1] R2-2501354 [Post129][601][SONMDT] Running NR RRC CR (Ericsson)