3GPP TSG-RAN WG2 #129bis	R2-2502977

Wuhan, CN, 7 – 11 Apr. 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on the LP-WUS capability issue within non-homogeneous deployment

Document for:	Discussion and decision

Agenda Item:	8.4.2

- Introduction

In the previous meeting RAN2#129 sent the Reply LS to SA2 (R2-2501388)[1] on LP-WUS subgrouping

where

- On the issue of UE radio capability for paging information, RAN2 had the following agreement:

- Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container).

Whether there is issue for Release 15 gNB and Release 16 gNB and for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the coming RAN2 meetings based on company contributions.

In this contribution we would like to discuss about the issue above whether there is no issue for Release 15 and 16 gNB.

- Discussion

From the operator’s perspective, there may be the case where Release 15/16 gNBs remain within a Release 18 network. If the Rel-19 UE an initial registration to the Release 15/16 gNB with LP-WUS capability and subsequently the UE camps on a Release 19 gNB that is LP-WUS capable, the UE can transition to the low power state, but the Rel-19 gNB might not be aware of the UE's LP-WUS capability from the Rel-15/16 gNB. Rel-19 gNB sends PAGING without WUS indication but the UE waiting for WUS indication then the termination call can be failed.

It is possible to prevent such scenarios through the operator’s policy by ensuring that all gNB(e.g. at least within the Tracking Area) are updated to Release 17 or later, in case the operator provide LP-WUS service. But from the spec perspective, NW should control the UE to activate LP-WUS depending on the UE capability with dedicated signal

Proposal 1: gNB can be implemented to control the UE to activate LP-WUS depending on the UE capability with dedicated signal

- Summary and proposal

Proposal 1: gNB control the UE to activate LP-WUS depending on the UE capability with dedicated signal

- References

[1]	R2-2501388 Reply LS on LP-WUS subgrouping.