3GPP TSG-RAN WG2 Meeting #129bis	R2-2502981
Wuhan, China, April 7th – 11th, 2025

Agenda Item:	9.1

Source: 		Vice Chairman (Samsung)

Title: 			Report from session on R18 SL, R18/19 MOB and R19 NES

Document for:	Approval

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

## NBC Changes for Rel-18 corrections

R2-2503021	Correction on the maximum number of SSB rsources for L1 maeasurement without gaps in LTM	vivo, Ericsson, Xiaomi	CR	Rel-18	38.331	18.5.1	5302	2	F	NR_Mob_enh2-Core

R2-2503020	Corrections on eEMR/IMR and LTM L2 Reset/UE-based TA measurement	Samsung, Ericsson	CR	Rel-18	38.331	18.5.1	5334	1	F	NR_Mob_enh2-Core

R2-2503008	Handling of the default SRB configuration upon LTM execution	CATT	CR	Rel-18	38.331	18.5.1	5285	1	F	NR_Mob_enh2-Core

R2-2503009	Handling of PUCCH resources for L1 LTM reports at TAT expiry	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5332	2	F	NR_Mob_enh2-Core	R2-2502942

## Approved outgoing LSs

R2-2503019	LS on on-demand SSB	LSout	To: RAN4	Rel-19	Netw_Energy_NR_enh-Core

R2-2503012	LS on RAN2 agreements for security key handling in inter-CU LTM	LSout	To: RAN3	Rel-19	NR_Mob_Ph4-Core

## List and Status of Offline/Email Discussions

POST Email discussion:

[POST129b][111][NES] (Ericsson)

Scope: Update RRC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list.

Deadline: Long email discussion

[POST129b][112][NES] (IDC)

Scope: Update MAC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (MAC running CR can keep editor’s notes for readability).

Intended outcome: Updated MAC running CR and essential MAC open issue list.

Deadline: Long email discussion

[POST129b][113][NES] (Apple)

Scope: Update 38.304 running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (38.304 running CR can keep editor’s notes for readability).

Intended outcome: Updated 38.304 running CR and essential 38.304 open issue list.

Deadline: Long email discussion

[POST129b][114][NES] (Huawei)

Scope: Update 38.300 running CR based on RAN2#129bis progress.

Intended outcome: Updated 38.300 running CR.

Deadline: Long email discussion

[POST129b][115][MOB] (Ericsson)

Scope: Update RRC running CR for inter-CU LTM and C-LTM based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list for inter-CU LTM and C-LTM.

Deadline: Long email discussion

[POST129b][116][MOB] (Huawei)

Scope: Update RRC running CR for L1 event-driven MR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list for L1 event-driven MR.

Deadline: Long email discussion

[POST129b][117][MOB] (Vivo)

Scope: Update MAC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (MAC running CR can keep editor’s notes for readability).

Intended outcome: Updated MAC running CR and essential MAC open issue list.

Deadline: Long email discussion

[POST129b][118][MOB] (CATT)

Scope: Update UE capability running CRs (including 38.331 and 38.306) based on RAN2#129bis progress.

Intended outcome: Updated UE capability running CRs (including 38.331 and 38.306).

Deadline: Long email discussion

[POST129b][119][MOB] (Apple)

Scope: Update 38.300 running CR based on RAN2#129bis progress.

Intended outcome: Updated 38.300 running CR.

Deadline: Long email discussion

[POST129b][120][MOB] (China Telecom)

Scope: Update 37.340 running CR based on RAN2#129bis progress.

Intended outcome: Updated 37.340 running CR.

Deadline: Long email discussion

[POST129b][121][NES] (ZTE)

Scope: Discuss initial discussion points for UE capabilities.

Intended outcome: Discussion summary.

Deadline: Long email discussion

[POST129b][122][NES] (OPPO)

Scope: Discuss and make conclusions on proposal 1 in R2-2501817.

Intended outcome: Discussion summary.

Deadline: Long email discussion.

AT Email discussion:

[AT129b][101][SL] (LG)

Scope: Discuss multiple candidate options for SL resource (re)selection in MCSt, identify any real blocking issue for candidate options, and attempt to find a unified solution (if possible) that can be applicable to all MCSt cases.

Intended outcome: Discussion summary in R2-2503014.

Deadline: Comeback in CB session => Completed.

[AT129b][102][MOB] (Ericsson)

Scope: Discuss the identified RRC FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502535 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503001.

Deadline: Comeback in CB session => Completed.

[AT129b][103][MOB] (Huawei)

Scope: Discuss the identified RRC FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502096 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503002.

Deadline: Comeback in CB session => Completed.

[AT129b][104][MOB] (Vivo)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502158 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503003.

Deadline: Comeback in CB session => Completed.

[AT129b][105][NES] (Ericsson)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502150 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503004.

Deadline: Comeback in CB session => Completed.

[AT129b][106][NES] (Apple)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502129 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503005.

Deadline: Comeback in CB session => Completed.

[AT129b][107][NES] (IDC)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502584 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503006.

Deadline: Comeback in CB session => Completed.

[AT129b][108][MOB] (CATT)

Scope: Discuss proposals in R2-2501900 and check if proposals can be endorsed/agreed. If not, identify what companies’ concerns are.

Intended outcome: Discussion summary in R2-2503011.

Deadline: Comeback in CB session => Completed.

[AT129b][109][MOB] (ZTE)

Scope: Prepare LS to RAN3 to inform RAN2 progress that is related to security aspect.

Intended outcome: LS to RAN3 in R2-2503012.

Deadline: Comeback in CB => Completed.

[AT129b][110][MOB] (Ericsson)

Scope: Clearly clarify how option2 works and check if there is a real technical blocking issue. If it works w/o real technical blocking issue, then we can go to option2. If there is a real technical blocking issue for option2, we’ll go to option1.

Intended outcome: Discussion summary in R2-2503013.

Deadline: Comeback in CB session => Completed (it was included in [AT129b][102]).

## 7.2	Further NR mobility enhancements

(NR_Mob_enh2-Core; leading WG: RAN2; REL-18; WID:RP-233970)

Time budget: 0 TU)

Minor and editorial issues should be coordinated with the CR rapporteur. Note RRC CR and MAC CR rapporteurs’ summary and suggestion (based on the submitted contributions) may be provided.

Tdoc Limitation: 1 tdocs.

R2-2501730	Reply LS on L3 measurement based LTM support over F1 (R3-250879; contact: Vodafone)	RAN3	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2	Cc:RAN4, RAN

Noted.

R2-2501748	Reply LS on L3 measurement based LTM support over F1 (RP-250790; contact: Vodafone)	RAN	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN3	Cc:RAN2, RAN4

Noted.

R2-2501736	Reply LS on the fast RRC processing for LTM (R4-2420218; contact: Ericsson)	RAN4	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2

Noted.

R2-2501737	LS on UE capability for L1-RSRP measurement in LTM (R4-2502619; contact: vivo)	RAN4	LS in	Rel-18	NR_Mob_enh2-Core	To:RAN2

Noted.

R2-2502170	Correction on UE capability for L1-RSRP measurement in LTM	vivo, Ericsson, Xiaomi	CR	Rel-18	38.306	18.5.0	1249	-	F	NR_Mob_enh2-Core

[Huawei]: Think “Both LTM candidate cell(s) and serving cells for SSB-based L1-RSRP measurement are counted” is not needed since the previous text already covers it. Also, the second text is not clear, it would be better to enhance the wordings. [Apple]: We should also consider that we have used “neighboring cells” instead of candidate cells.

Wordings can be further discussed offline.

Comeback in CB session (revised CR in R2-2503016 if needed).

R2-2503016	Correction on UE capability for L1-RSRP measurement in LTM	vivo, Ericsson, Xiaomi	CR	Rel-18	38.306	18.5.0	1249	1	F	NR_Mob_enh2-Core

Agreed in principle.

R2-2502171	Correction on the maximum number of SSB rsources for L1 maeasurement without gaps in LTM	vivo, Ericsson, Xiaomi	CR	Rel-18	38.331	18.5.1	5302	-	F	NR_Mob_enh2-Core

[Huawei]: Not sure whether it’s good to make dummy this level IE that is proposed by Vivo. [Apple]: The linkage with CA parameter IE is not clear. [Qualcomm]: Can we agree with general principle, NBC change is acceptable only when there is no other option.

IE level for dummy and the linkage with CA parameter IE can be further discussed offline.

NBC change is acceptable only when there is no other option.

Comeback in CB session (revised CR in R2-2503017 if needed).

R2-2503017	Correction on the maximum number of SSB rsources for L1 maeasurement without gaps in LTM	vivo, Ericsson, Xiaomi	CR	Rel-18	38.331	18.5.1	5302	1	F	NR_Mob_enh2-Core

[MediaTek]: Wonder if it is NBC change. [Vivo, Ericsson]: Understand it is BC change. Note UE behaviour is not changed. [Huawei]: Understand although it is BC change in ASN.1 point of view, it is still NBC change in the functional point of view.

Cover page needs to indicate functional NBC change.

With the change above, CR in R2-2503021 is agreed in principle.

R2-2502966	Discussion on eEMR/IMR and LTM L2 Reset/UE-based TA  measurement	Samsung Electronics Czech	discussion	Rel-18	38.331	NR_Mob_enh2-Core

Proposal 1: RAN2 to adopt the TP provided in Annex 1.

RAN2 to adopt the TP provided in Annex 1.

[Ericsson]: Think that the text proposal achieves the same as the current text, or at least fail to understand what the real difference is. [Nokia]: What is the problem with the current spec? [Samsung]: The current text does not cover when the timer was not configured. Proposed text is not new, just same as what is specified for inactive. [ZTE]: Agree with Samsung.

Proposal 2: RAN2 to adopt the TP provided in Annex 2.

RAN2 to adopt the TP provided in Annex 2.

[Ericsson]: Indeed seems this case was overlooked in the procedural text and needs to be added.

Proposal 3: RAN2 to adopt the TP provided in Annex 3.

RAN2 to adopt the TP provided in Annex 3.

[Ericsson]: The proposed text is related to SIB11 and for this case UE will do something according to what is support. The existing text seems to be fine.

Proposal 4: RAN2 to discuss whether to remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored, after discarding the VarMeasIdleReport.

Remove the measIdleValidityDuration in VarEnhMeasIdleConfig, if stored, after discarding the VarMeasIdleReport.

[Samsung]: UE deletes all other related variables only except measIdleValidityDuration. Note this is configured only by RRC release or SIB. [Qualcomm]: Ok with this change.

Proposal 5: RAN2 to agree to change the serving cell IDs under VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID as “ID associated with the serving cell”.

Change the serving cell IDs under VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID as “ID associated with the serving cell”.

[Ericsson]: Is a small clarification but is good to have. Otherwise, a reader may think that is the serving cell ID, which is not.

CR in R2-2503007 will be provided in CB session.

R2-2503007	Corrections on eEMR/IMR and LTM L2 Reset/UE-based TA measurement	Samsung, Ericsson	CR	Rel-18	38.331	18.5.1	5334	-	F	NR_Mob_enh2-Core

Cover page needs to indicate functional NBC change.

With the change above, CR in R2-2503020 is agreed in principle.

R2-2502533	Capability coordination for LTM	Ericsson	CR	Rel-18	38.331	18.5.1	5305	-	F	NR_Mob_enh2-Core

[Ericsson]: We may need more parameters based on offline discussion (e.g. number of candidate per TCI state configuration). [ZTE]: Understand if UE capability is defined per band, then out of the current bands, the NW will assume the lowest number to be configured.

Offline discussion (including P3 and P4 in R2-2502553).

Comeback in CB session (revised CR in R2-2503018 if needed).

R2-2503018	Capability coordination for LTM	Ericsson	CR	Rel-18	38.331	18.5.1	5305	1	F	NR_Mob_enh2-Core

Agreed in principle.

R2-2501904	Handling of the default SRB configuration upon LTM execution	CATT	CR	Rel-18	38.331	18.5.1	5285	-	F	NR_Mob_enh2-Core

[Ericsson]: It is true that if the LTM is at the SCG the UE will apply the default SRB1 and SRB2 configuration, but this may be correct in case of split SRB. Also, applying the default is not harmful as the network does not really need to provide new SRB1 and SRB2 configurations in this case. So, what we have now it works, even if this implies that for SRB1 and SRB2 the default configuration is always re-applied…but maybe this is a safe approach. [Qualcomm]: Think technically correct, but we can survive without this change. [Ericsson]: We agreed that MCG LTM and SCG LTM are not executed at the same time, once LTM happens either MCG or SCG, the other note will know what happened for SRB (whether it is changed to default configuration or not). [Apple]: RAN2 has never considered like MSG control plane or SCG control plane up to now. [CATT]: To Ericsson, it requires additional coordination between nodes to have same understanding on SRBs, which needs more specification efforts. [Samsung]: Support the proposal. Technically correct and it will not bring any further network coordination. [Huawei, MediaTek]: Same view as Samsung. [MediaTek]: It is functional NBC change so it should be indicated by cover page.

Proposed change is agreed.

Cover page should indicate functional NBC change.

CR will be revised in R2-2503008.

Comeback in CB session.

R2-2503008	Handling of the default SRB configuration upon LTM execution	CATT	CR	Rel-18	38.331	18.5.1	5285	1	F	NR_Mob_enh2-Core

Agreed in principle.

R2-2501976	Clarification on LTM cell switch failure recovery	Google Korea LLC	discussion	Rel-18	38.331

Proposal: 	RAN2 is requested to discuss the two options and decide option 1 or option 2 is adopted.

1)	Modify Section 5.3.5.8.3: Add a NOTE clarifying that during T304 expiry in an LTM cell switch, the UE only reverts back to the configurations and variables retained as per section 5.3.5.18.6.

2)	Modify Section 5.3.5.18.6: Add a NOTE clarifying that the UE retains all current dedicated and common radio configurations for reverting back to the UE configuration upon T304 expiry as per section 5.3.5.8.3.

Noted.

[Ericsson]: This is exactly the same case when there is a reconfiguration with sync with full configuration. With full configuration UE delete all the dedicated and common configuration, but then if the reconfiguration with sync fails, the UE needs to revert back to the previous configuration. In this case, there is nothing really new and we should not pursue this change. [Nokia]: Agree with Ericsson. [Google]: UE behaviour for release is specified under LTM cell switch execution. [Ericsson]: Note it is not LTM specific issue since the same text is there for L3 HO (with full configuration). [Apple]: Share the view with Ericsson. Do not see a real problem.

R2-2502169	Discussion on indication from MAC to RRC layer in RACH-based LTM	vivo	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: Remove the indication from MAC to RRC layer for the completion of RACH-based LTM.

Proposal 2: If P1 is agreeable, RAN2 to adopt the TP provided in Annex1.

Not pursued.

[Ericsson]: It seems that there is no issue in the procedural text point by vivo in RRC. The indentation 1> are all “or” and this means that the UE will only execute one of those. Also, even if the indication from lower layer is repeated in one indentation 2> we still think there is no issue and this does not mean the indication needs to be repeated twice. [Huawei]: Nothing is broken with this indication.

R2-2502553	Remaining Rel-18 LTM Corrections	Nokia	discussion	Rel-18	NR_Mob_enh2-Core

Proposal 1: Update the first sentence to "Indicates whether the UE supports early TA acquisition based on PDCCH ordered CFRA procedure before the cell switch command MAC CE”. Adopt the TP in Annex A for TS 38.306.

Noted.

[Ericsson]: We understand the limitation comes from WGs which are not RAN2 and for this reason we cannot simply modify the meaning of the capability as we want (description which we believe is actually correct). [MediaTek]: It is RAN1/4 issue, and to perform early UL synchronization, the UE needs to perform measurements on SSB and selection of SSB. Understand the maximum number of this IE considered this aspect.

Proposal 2: RAN2 to clarify the understanding on the timeframe associated with UE “maintaining” the UE-measured TA and to reflect this in 38.306. The following options can be considered as a starting point (other alternatives are not precluded):

1)	UE measures and maintains the TA value starting from the moment when it is configured to perform UE-based TA measurement.

2)	UE measures and maintains the TA value starting from the moment when it receives the CSC

Noted.

[Ericsson]: The UE-based TA is a UE-only feature and the intention is that the UE calculate itself the TA (if indicated by the network). When the UE calculate the TA and how to maintain it is up to the UE implementation and RAN4 did not specify any requirement for this feature.

Proposal 3: In addition to the number of LTM cells and L1 measurement configuration, MN informs SN about the number of cells or/and TCI states for which early DL synchronization procedures can be performed. Adopt the TP in Annex B in TS 38.331.

Proposal 4: MN informs SN about the number of cells which early UL synchronization procedures can be performed. Adopt the TP in Annex B in TS 38.331.

[Session chair]: P3 and P4 can be further discussed as part of R2-2502533 offline.

R2-2502600	Rel-18 LTM issues	Ofinno, LLC	discussion	Rel-18	NR_Mob_enh2-Core

Proposal: To avoid the mismatch of UE-based TA measurements between the UE and the network, RAN2 agreed one of the following alternatives:

1.	A UE determines to configure the lower layer for UE based TA measurements for each LTM candidate cell if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID but not include the ltm-CandidateToAddModList

2.	the ltm-CandidateToAddModList is mandatory present if the received LTM-Config includes ltm-ServingCellUE-MeasuredTA-ID

Not pursued.

[Ericsson]: The field description of ltm-UE-MeasuredTA-ID already handle the case described in the contribution: “the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID” [Nokia]: NW will handle all cases that should be done correctly. [Apple]: Do not prefer changing the UE behaviour. Prefer NW handle it.

R2-2502942	Handling of PUCCH resources for L1 LTM reports at TAT expiry	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5332	-	F	NR_Mob_enh2-Core

R2-2502971	Handling of PUCCH resources for L1 LTM reports at TAT expiry	Huawei, HiSilicon	CR	Rel-18	38.331	18.5.1	5332	1	F	NR_Mob_enh2-Core	R2-2502942

[Ericsson]: We are wondering if this is really correct. For the CSI-ReportConfig of the serving cell this makes sense since the TAT is maintained towards the serving cell. However, LTM-CSI-ReportConfig provide resource for cells which are not “serving cell”, since these are candidate cells. So in principle the UE may still continue to use those PUCCH resources. [Qualcomm, Xiaomi, CATT, Nokia]: Agree with the proposal. [Ericsson]: It is functionally NBC change.

Cover page will indicate functionally NBC change.

CR with the change above in R2-2503009 is in principle agreed.

R2-2502943	MAC CR rapporteur summary	Huawei, HiSilicon	discussion	Rel-18	NR_Mob_enh2-Core

R2-2502979	Summary of RRC proposals for FeMob	Ericsson	discussion	Rel-18	NR_Mob_enh2-Core

Summaries were included as Ericsson comments in the session minutes.

R2-2501932	Correction on LTM UE Capability based on the RAN4 LS	MediaTek Inc.	discussion	Rel-18	NR_Mob_enh2-Core

The proposed change was already treated above.

## 7.6	NR Sidelink evolution

(NR_SL_enh2-Core; leading WG: RAN1; REL-18; WID: RP-230077)

Time budget: 0 TU

Tdoc Limitation: 1 tdocs

R2-2501735	LS on AdditionalSpectrumEmission in NR SL Pre-configuration (R4-2418075; contact: LGE)	RAN4	LS in	Rel-18	NR_SL_enh2-Core	To:RAN2	Cc:RAN1

Noted.

R2-2502061	Discussion on resource re-selection in case of LBT failure on MCSt	LG Electronics Inc.	discussion	Rel-18	38.321	NR_SL_enh2-Core

Proposal 1. RAN2 agrees that the RAN2 agreement (“Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure.”) for single TB MCSt based on multi-slot candidate resources can be applied to multi-TB MCSt (i.e., Single slot candidate resources based multi-TBs MCSt, Multi-slot candidate resources based multi-TBs MCSt) as well.

Proposal 2. Adopt the corresponding TP captured in Appendix.

[OPPO]: Understand the resource (re)selection should be triggered in a HARQ process point of view. Resource (re)selection is triggered if, from a HARQ process point of view, consecutive resources that were assigned have LBT failure indications. [Ericsson]: Prefer resource (re)selection is triggered for other MCSt cases just as non-MCSt case. [Session chair]: There would be various options to handle each MCSt case. Is there real technical blocking issue for any direction or is it a matter of taste? [Qualcomm]: One thing we should consider is related to the HARQ buffer refresh. Also, it is related to the efficiency of resource (re)selection. If HARQ retransmission is sent successfully later (within MCSt), the resource (re)selection does not need to be triggered earlier.

[Session chair]: Examples of MCSt TB patterns:

- Pattern#1: TB1-TB1-TB1-TB1

- Pattern#2: TB1-TB1-TB2-TB2

- Pattern#3: TB1-TB2-TB1-TB2

- Pattern#4: TB1-TB2-TB3-TB4

Question#1: To determine whether retransmission is needed or not:

- Option#A: When an LBT failure indication is received for any resource within a MCSt, which is assigned to a given TB.

- Option#B: When LBT failure indication(s) is/are received for all consecutive resource within a MCSt, which is assigned to a given TB.

- Option#C: When LBT failure indication(s) is/are received for all resources within a MCSt, which are assigned to a given TB.

Question#2: To determine when resource (re)selection for retransmission is triggered.

- Option#W: At each resource for a given TB within a MCSt

- Option#X: At the end of contiguous resource for a given TB within a MCSt

- Option#Y: At the end of resource for a given TB within a MCSt

- Option#Z: At the end of resource within a MCSt

[AT129b][101][SL] (LG)

Scope: Discuss multiple candidate options for SL resource (re)selection in MCSt, identify any real blocking issue for candidate options, and attempt to find a unified solution (if possible) that can be applicable to all MCSt cases.

Intended outcome: Discussion summary in R2-2503014.

Deadline: Comeback in CB session => Completed.

R2-2503014	Summary of [AT129b][101][V2X/SL] Discuss multiple candidate options for SL resource (re)selection in MCSt (LG)	LG Electronics Inc.	discussion	Rel-18	38.321	NR_SL_enh2-Core

[Session chair]: Based on that there is no technical blocking issue for any direction, would like to suggest A+W considering MCSt using single slot candidate resource is done as best effort manner.

Option (A + W) for MCSt using single slot candidate resources is agreed.

- To determine whether retransmission is needed or not:

- Option A: When an LBT failure indication is received for any resource within a MCSt, which is assigned to a given TB.

- To determine when resource (re)selection for retransmission is triggered:

- Option W: At each resource for a given TB within a MCSt.

We will see CR next meeting (LG).

R2-2502935	Correction on sidelink RRC specification	ZTE Corporation, Sanechips, Ericsson	CR	Rel-18	38.331	18.5.1	5330	-	F	NR_SL_enh2-Core

[Qualcomm]: For change 2, LTM was a parallel WI in Rel-18, and there was no explicit agreement on co-existence between SL and LTM. Prefer not adding this change. [Vivo]: Although we understand intention of change, also agree with Qualcomm. [Ericsson]: Ok with change 2. [NEC, Samsung]: Have similar view as Qualcomm.

[OPPO]: For change 3, although we agree with intention, think adding “for the NR sidelink unicast, groupcast and broadcast communication transmission” is not needed since TX profile is only applied to groupcast/broadcast. [Huawei, Qualcomm]: Agree with OPPO. [Xiaomi]: Agree with the change 3 for clearer understanding.

Change 2 and change 3 will be removed.

CR with the change above in R2-2503010 is in principle agreed.

R2-2502062	MAC correction on Release-18 Sidelink evolution	LG Electronics Inc.	CR	Rel-18	38.321	18.5.0	2057	-	F	NR_SL_enh2-Core

Agreed in principle.

R2-2502185	Correction on MCSt Restriction for Resource Selection	Apple	CR	Rel-18	38.321	18.5.0	2060	-	F	NR_SL_enh2-Core

Agreed in principle.

[OPPO]: With this change, it seems MCSt is not the reason for this note, it sounds MCSt is the result of this note. Propose to add “For SL unlicensed band operation” instead. [LG]: There are other notes and normative texts that are similar to this note. Do we apply “For SL unlicensed band operation” to only note? [Xiaomi]: For all NOTEs, it is related to multi-consecutive slots transmission. Then with this change, some note is related to multi-consecutive slots transmission and some note is related to “SL unlicensed band operation”. [OPPO]: Ok with the originally proposed text.

R2-2502322	Discussion on resource re-selection from SL LBT Failure indication	Ericsson, Apple, Qualcomm Incorporated, vivo	discussion	Rel-18	NR_SL_enh2-Core

R2-2502305	Correction to MAC on IUC	Ericsson, Apple, ZTE Corporation, Sanechips	CR	Rel-17	38.321	17.12.0	2061	-	F	NR_SL_enh-Core

R2-2502306	Correction to MAC on IUC	Ericsson, Apple, ZTE Corporation, Sanechips	CR	Rel-18	38.321	18.5.0	2062	-	A	NR_SL_enh-Core

Noted.

[Huawei]: Understand the second change is not correct since there is some conditions in 38.214.  Also understand the first change is not essential correction, which does not give any useful information. [OPPO]: Agree with Huawei on the second change.

## 8.5	Network Energy Saving Enh.

(Netw_Energy_NR_enh-Core; leading WG: RAN1; REL-19; WID: RP-242354)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.5.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST129][101], [102], [103], [104], summary of identified open issues that need online discussion and rapporteur’s suggestions if needed).

R2-2501722	LS on time location of on-demand SSB for Scell (R1-2501633; contact: LGE)	RAN1	LS in	Rel-19	Netw_Energy_NR_enh	To:RAN4	Cc:RAN2

Noted.

R2-2501984	Introduction of Network Energy Savings Enhancements running CR	Huawei, HiSilicon	draftCR	Rel-19	38.300	18.5.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2502173	Running RRC CR for enhancements for network energy efficiency 	Ericsson	draftCR	Rel-19	38.331	18.5.1	Netw_Energy_NR_enh-Core

Endorsed.

R2-2502150	Summary of Comments to 38.331 CR for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

[AT129b][105][NES] (Ericsson)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502150 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503004.

Deadline: Comeback in CB session => Completed.

R2-2503004	[AT129bis ][105] [NES] FFSs to 38.331 CR for NES (Ericsson)	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Define features, not devices. That is, definition of OD-SIB1, OD-SSB, paging adaptation etc in 38.300 and in other specifications refer to 38.300:

- Avoid NES, Cell A, UL-WUS

- “UE supporting OD-SIB1” where/if needed, and to replace with the UE capability name

- UL-WUS is replaced with SIB1 request in e.g. field descriptions

- ul-WUS-Config renamed as od-sib1-Config

Current RRC only has this for idle inactive in 5.2.2.5, add what is needed for connected mode. SIBxx is not defined as essential SI and that EN is deleted.

Keep FFS “FFS how to capture the UE first should acquire a valid SIB1 (e.g. via SIB1 request) for camping on an OD-SIB1 NES cell.” And “FFS how does UE check is SIB1 is already provided.” for now but change “whether to capture”.

Keep the FFS for normal uplink for now.

“acquire the requested SIB1 message as defined in FFS, immediately;” changed to “acquire the requested SIB1 message as defined in FFS, immediately;”

Add in 331 a branch where UE receives notification from MAC that number to max RACH attempts were reached and UE barrs the cell according to 304.

R2-2502323	Running 38.304 CR for network energy saving	Apple (Rapporteur)	draftCR	Rel-19	38.304	18.4.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2502129	Summary report of [POST129][104][NES] 38.304 running CR and stage 3 open issues	Apple (Rapporteur)	discussion	Rel-19	Netw_Energy_NR_enh-Core

[AT129b][106][NES] (Apple)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502129 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503005.

Deadline: Comeback in CB session => Completed.

R2-2503005	Summary of [AT129b][106][NES] (Apple)	Apple (Rapporteur)	discussion	Rel-19	Netw_Energy_NR_enh-Core

Following agreement on “barring behavior clean-up”, capture the UE barring behavior of OD-SIB1 in both TS 38.331 and TS 38.304. Check offline whether we can rely on existing legacy UE barring specification in 38.304 (e.g. the case of RACH failure).

(modified) On the unbarring behavior of OD-SIB1 UE, capture it as normative text in TS 38.304. Way-forward Proposal (on offline check of Proposal 1): For UE barring the OD-SIB1 cell in case of RACH failure, capture in TS 38.304, e.g.

- If the cell is to be treated as if the cell status is "barred" due to the OD-SIB1 request number reaching preambleTransMax;

2> UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

[Session chair]: Suggest “shall” to be changed to “may” to be aligned with MIB and SIB1 acquisition failure. [Vivo]: Understand we discussed some related topic in the past and it was decided as shall. [Session chair]: Let’s have “may” now. If problem, we can reconsider it later.

R2-2502587	Running 38.321 CR for network energy saving	InterDigital	draftCR	Rel-19	38.321	18.5.0	B	Netw_Energy_NR_enh-Core

Endorsed.

R2-2502584	Summary of Comments to 38.321 CR for NES	InterDigital	discussion	Rel-19

[AT129b][107][NES] (IDC)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502584 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503006.

Deadline: Comeback in CB session => Completed.

R2-2503006	[AT129bis][107][NES] Open issues for 38.321 CR (InterDigital)	InterDigital	discussion	Rel-19

Capture the following understanding in meeting notes: RA on an SpCell can include a cell on which the UE is requesting OD-SIB1. No specification changes needed.

Reuse legacy SSB selection procedure in MAC spec for RA initiated for SIB1 request, i.e. the UE selects any SSB if no SSB is above the configured RSRP threshold. This can be revisited if RAN1 agrees otherwise.

R2-2501956	Running 38.306 CR for NES UE capability	ZTE Corporation, Sanechips	draftCR	Rel-19	38.306	18.5.0	Netw_Energy_NR_enh-Core

Endorsed.

[POST129b][111][NES] (Ericsson)

Scope: Update RRC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list.

Deadline: Long email discussion

[POST129b][112][NES] (IDC)

Scope: Update MAC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (MAC running CR can keep editor’s notes for readability).

Intended outcome: Updated MAC running CR and essential MAC open issue list.

Deadline: Long email discussion

[POST129b][113][NES] (Apple)

Scope: Update 38.304 running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (38.304 running CR can keep editor’s notes for readability).

Intended outcome: Updated 38.304 running CR and essential 38.304 open issue list.

Deadline: Long email discussion

[POST129b][114][NES] (Huawei)

Scope: Update 38.300 running CR based on RAN2#129bis progress.

Intended outcome: Updated 38.300 running CR.

Deadline: Long email discussion

[POST129b][121][NES] (ZTE)

Scope: Discuss initial discussion points for UE capabilities.

Intended outcome: Discussion summary.

Deadline: Long email discussion

### 8.5.2	On-demand SSB SCell operation

Remaining open issues, including details of OD-SSB MAC CE (with the consideration of RAN1 progress), L3 RRM measurements, stage-3 identified open issues if needed, etc.

1. OD-SSB Activation/deactivation MAC CE Format

R2-2502130	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1 (modified): For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

[Huawei]: Our proposal is to include only SCells supporting OD-SSB. [OPPO, Nokia, ZTE, IDC]: Prefer simple option (Apple P1). [IDC]: Can we also agree with A/D bit, “1” means activation, “0” means deactivation? [Apple]: Yes, for explicit deactivation case.

For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

Proposal 2 (modified): For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

Proposal 3 (modified): OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

R2-2502840	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: For OD-SSB MAC CE, RAN2 to consider a variable size MAC CE, in which a bitmap is used for SCells supporting OD-SSB in increasing order, and a configuration index is indicated for each SCell supporting OD-SSB.

Proposal 3: Use an eLCID for the OD-SSB transmission MAC CE.

2. Need of separate MAC CE for implicit deactivation

R2-2502130	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4a: For one Rel-19 UE, the Network configures whether to use either implicit deactivation or explicit deactivation (but not both) of OD-SSB transmission in all its SCells (i.e. per UE rather than per SCell config).

Proposal 4b: If Proposal 4a can be agreed, RAN2 confirm that it is not necessary to introduce two separate MAC-CEs for implicit deactivation and explicit deactivation respectively.

[Vivo, Nokia]: Do not think we need separate MAC CE for implicit deactivation. [Nokia]: Even without Apple’s proposal 4a, we still can have common MAC CE to handle both implicit deactivation and explicit deactivation. For example, the UE can start a timer whenever it receives activation indication for the corresponding SCell. [Session chair]: Without proposal 4a, there may be some issue. Let’s see the following Xiaomi’s contribution, observation#1. Understand w/ proposal 4a, if implicit deactivation is configured for the UE, network will set “0” for the SCell that already activated its OD-SSB and the UE will simply ignore it (since “0” deactivation is only applicable to explicit deactivation) and continue the ongoing counting.

R2-2501805	Discussion on on-demand SSB	Xiaomi	discussion

Proposal 4: For OD-SSB activation and deactivation, RAN2 to discuss and further down select among the following options:

·	Alternative1: separate MAC CEs for OD-SSB activation and deactivation.

·	Alternative2: single MAC CE for OD-SSB activation and deactivation with two bitmaps

·	Alternative3: single MAC CE for OD-SSB activation and deactivation with one bitmap (similar as legacy SCell activation/deactivation MAC CE)

- Alternative3.1: NW ensures either implicit deactivation or explicit deactivation is used for all OD-SSB SCells

- Alternative3.2: NW updates the associated number N for the SCells of which OD-SSB is “re-activated”

Offline discussion among interested companies: Whether common MAC CE format can handle both implicit deactivation and explicit deactivation. If not, do we need a separate MAC CE format for implicit deactivation or do we need any additional information field in the MAC CE (OPPO).

Will comeback in CB session.

CB session:

RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

[Ericsson]: Wonder if we need to send LS to RAN1, e.g. ask the number of OD-SSB configurations, do we really need periodicity change, etc. [LG]: RAN2 can make decision (do not see the need to send an LS). [Apple]: Share the view with Ericsson. For example, RAN1 only defined periodicity change up to now. [IDC]: RAN1 is working on them and RAN1 understands configuration index/id is enough. [Nokia]: RAN2 can make decision on the number of OD-SSB configurations.

3. L3 RRM: MO configuration

R2-2502130	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8 (modified): When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

R2-2502148	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: RAN2 to agree to use the existing serving cell measurement object to support the serving cell measurements of OA-SSB and OD-SSB when these are within the same BWP.

Working assumption:

When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB.

Send LS to inform RAN2 working assumption to RAN4, and to check if ok to them.

Comeback in CB session (Apple, LS in R2-2503015)

FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

[Ericsson]: Clarifies RAN1 decided both A-SSB and OD-SSB are located in the same BWP, that’s why we changed our position. [Nokia]: The requirement that one MO per carrier has been there from Rel-15. Afraid to change this principle based on Ericsson proposal. For Apple proposal, if multiple OD-SSBs are configured for a given SCell, how to handle it? [Apple]: Do not consider multiple OD-SSB for a given SCell. [OPPO]: Agree with Apple proposal. For multiple OD-SSB configurations, we can handle it by MO index in the corresponding configuration. [ZTE]: Multiple OD-SSB per SCell is possible, but not sure whether it is possible with the different frequencies. [Huawei]: Agree with Apple proposal. However, it will be good to remove “RedCap” since the detailed signalling can be different. [Ericsson]: Should RAN4 decide it? [Nokia]: Understand it is RAN2 job. Ok with the proposal, but wants to add “FFS whether/where multiple OD-SSBs are on the different frequencies for a given SCell.” [Apple, LG]: For Nokia’s FFS proposal, it is RAN1 scope. We do not need to create this explicit FFS in RAN2. If RAN1 introduces it, we can discuss it otherwise not needed. [Ericsson]: If we go Apple proposal, it may bring some issues when OD-SSB is activated, e.g. how to combine measurements, how to apply filtering from different MO, etc. Good to check with RAN4.

R2-2503015	LS on on-demand SSB	LSout	To: RAN4	Rel-19	Netw_Energy_NR_enh-Core

Add “A-SSB” to “always-on SSB” and “OD-SSB” to “on-demand SSB”.

Change “RAN4” to “RAN4/1” in Note.

Add RAN1 into Cc.

Will remove the sentence “That is, when A-SSB and OD-SSBs have different center frequencies, except the servingCellMO to indicate MO of A-SSB, RAN2 to introduce a new servingCellMOs in ServingCellConfig to indicate MOs of configured OD-SSBs.”

Change “frequency” to “frequencies” in the note.

With the changes above, LS is approved in R2-2503019.

[OPPO, Huawei]: Multiple OD-SSBs are not agreed. Would like to focus on single case now. [Ericsson]: Without adding FFS on multiple OD-SSBs, RAN4 may consider additional MO will handle all multiple OD-SSBs.

4. L3 RRM: Measurement on neighbouring cells

R2-2502384	Remaining issues of on-demand SSB SCell operation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: No specification impact is needed for the RRM measurement on intra-frequency neighbour cells when the OD-SSB of SCell is activated.

Proposal 5: RAN2 assumes the L3 measurement of neighbour cell OD-SSB is based on legacy SMTC.

5. Need of MAC-CE triggering MR for OD-SSB

R2-2501886	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 10: R2 not pursue MAC-CE triggered MR for OD-SSB.

R2-2502148	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: RAN2 to support MAC CE based trigger of serving cell OD-SSB measurement report.

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

R2-2501805	Discussion on on-demand SSB	Xiaomi	discussion

R2-2501833	Discussion on on-demand SSB	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501867	On-demand SSB SCell Operation	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501886	Discussion on On-Demand SSB	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501901	Consideration on on-demand SSB SCell operation	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501957	Remaining issues of on demand SSB Scell operation	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502130	Remaining issues on on-demand SSB for Scell	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502136	Discussion on OD-SSB	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502148	Discussion on on-demand SSB for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502219	NES SCell handling for OD-SSB	ETRI	discussion

R2-2502225	Remaining issues on on-demand SSB SCell operation	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502320	Further discussion on On-demand SSB for SCell	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502371	Issues on the procedure of on-demand SSB SCell operation	Lenovo	discussion	Rel-19

R2-2502384	Remaining issues of on-demand SSB SCell operation	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502487	On-demand SSB Scell operation discussion	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502540	Discussion on On-demand SSB SCell Operation	Qualcomm Incorporated	discussion

R2-2502582	On demand SSB transmission for SCell	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502724	Discussion on On-demand SSB for Scell	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502742	On-demand SSB SCell operation	Panasonic	discussion	Rel-19

R2-2502780	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502840	Discussion on on-demand SSB SCell operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502847	Discussion on on-demand SSB transmission	LG Electronics France	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502864	Discussion on On-demand SSB	KDDI Corporation (TTC)	discussion

R2-2502907	Discussion on on-demand SSB SCell operation	Sharp	discussion

R2-2502915	On demand SSB handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502002	Discussion on on-demand SSB SCell operation	Sharp	discussion

=> Withdrawn

### 8.5.3	On-demand SIB1

Remaining open issues, including the unsettled issue for RRC connected UEs, details of OD-SIB1 request procedure, stage-3 identified open issues if needed, etc.

1. Handling RRC connected UE

R2-2502131	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: For Rel-19 NES UE in RRC_CONNECTED, if searchSpaceSIB1 is configured and SIB1 is not broadcasting:

•	Rely on the NW implementation (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1) as baseline.

•	If majority prefer enhancement, allow the UE to send request for SIB1 via dedicated RRC message DedicatedSIBRequest.

NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

[ZTE]: Ok with relying on NW implementation. Understand OD-SIB1 request is only applicable to RLF case according to RAN guideline. Also think searchSpaceSIB1 is configured and sib1 is not broadcasting is not often. [Huawei]: With relying on NW implementation, how does NW expect the UE behaviour? [Session chair]: Understand relying on NW implementation means whenever broadcast status is changed, NW sends SIB1 in dedicated RRC. [Nokia]: Do not understand what NW difficulties that Apple assume if OD-SIB1 request procedure is applied (like idle/inactive UE). [Samsung]: With the proposal, it would bring burden to NW and it may not be desirable for NES point of view. For dedicatedSIBRequest, we can just add one code-point. [CATT]: Understand the first option works in NES cell assuming there are not many connected UEs. [Samsung]: Heard some technical concerns for the first option, however for the second option, no technical concern. [LG, ZTE]: With option2, some companies still have concerns that it is not aligned with RAN discussion and guideline. [Nokia]: Agree with ZTE, we can first agree with option1 then if proponent companies want other option that may not be aligned with RAN conclusion, they need to bring it to RAN.

2. Whether legacy UE can camp in NES Cell if it is broadcasting SIB1

R2-2501866	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1.

When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

[Ericsson]: Isn’t it dependent on Kssb configuration, which is up to NW implementation. [ZTE]: We can confirm proposal 5 but do not see any specification impact. [CATT]: Agree with Ericsson and there is no specification impact.

3. UL WUS in RRC release

R2-2502580	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

[Nokia]: If SIBX that was acquired during RRC connected state can be used for OD-SIB1 REQ in RLF, we are ok with the proposal. [Samsung]: Do not see any difficult to maintain SIBX that was required for RRC connected state and use it in RLF. Agree with the proposal and Nokia’s comment.

4. Understanding of RAR format

R2-2502916	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: RAN2 to align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

R2-2502385	Discussion on OD-SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: RAN2 to discuss the following options for a Rel-19 NES UE to correctly decode the RAR MAC PDU containing the MAC subPDU for the OD-SIB1 request upon cell reselection to a NES cell:

-	Option 1: The UL WUS configuration includes the IE totalNumberOfRA-Preambles;

-	Option 2: For the MAC subPDU with RAPID not used for the OD-SIB1 request, R19 NES UE considers the size of the MAC subPDU with RAPID < ra-PreambleStartIndex for the OD-SIB1 request is 8 bytes and the size of the MAC subPDU with other RAPID is 1 byte.

-	Option 3: The MAC subPDU for the OD-SIB1 request is placed at the second place after the MAC subPDU with BI only.

The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

[Apple]: Option1 is the easiest option and all other options need a new UE behaviour.

5. Backoff in RAR

R2-2502226	Remaining issues on on-demand SIB1 procedure	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: No need to change the preamble backoff handling in the RAR MAC PDU during SIB1 request procedure.

Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

[Samsung, Ericsson, Nokia]: Do not see a need to use backoff considering SIB1 is essential SIB.

6. Paging interruption

R2-2502131	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: On paging interruption issue during OD-SIB1 acquisition, RAN2 discuss the following 2 solutions:

•	Alt-1: leave it to UE implementation. RAN2 send LS to RAN4 to check whether the paging interruption requirement during cell reselection specified in TS 38.133 needs change.

•	Alt-2: include PCCH-Config of NES cell in UL WUS configuration provided by Cell A, so that Rel-19 UEs can monitor paging from NES cell during OD-SIB1 acquisition.

R2-2502229	Remaining issues on OD-SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: If the RAR window or the OD-SIB1 window is overlapped with the paging occasion, the UE monitors the serving cell during the overlapping period.

[Nokia]: Do not see a need of LS. Interested companies can submit the corresponding contribution directly in RAN4. [ZTE]: Agree with alternative 1 and we can reword that RAN2 understand it’s up to RAN4 for any update of paging interruption requirements. [Vivo, OPPO]: Understand RAN4 already started this discussion, so LS is not needed. [Lenovo]: Agree with alternative 1.

Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

7. Condition to bar the cell due to SIB1 acquisition failure

R2-2502104	Remaining issues on the support for OD-SIB1	Google	discussion	Rel-19	Netw_Energy_NR_enh-Core	R2-2500463

Proposal 4 (modified): If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

[Nokia, Xiaomi]: Think we can leave it to UE implementation. [LG, Samsung]: Support the proposal. [Samsung]: If the UE fails to acquire SIB1 within SIB1 monitoring window, can the UE perform RACH procedure again to acquire OD-SIB1? [ZTE, OPPO]: Want to have a consistent UE behaviour. Since we already agreed to bar the cell when RACH for OD-SIB1 fails, it should ok with the proposal. [Nokia]: Think 300s restriction is too harsh in the case (including RACH failure for OD-SIB1 acquisition). Prefer up to 300s (not as fixed 300s). [Ericsson]: For barring, some case is specified as “shall” and some other case is specified as “may”. We may need to consider consistent behaviour on this wording as well. [Session chair]: For 300s restriction and wording difference (may and shall), suggest interested companies have offline, so we can check in CB session.

CB session: 
It was covered by Apple’s earlier offline discussion.

8. Area validity

R2-2501958	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: Define and provide valid area for WUS configuration.

Proposal 5: The valid area for WUS configuration can be a list of cell identities or a new system information area id referring to multiple cells.

WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

Area id will be used as legacy.

[Huawei]: We need to discuss area for WUS and list of cells for WUS in separate. If we introduce new area for SIBx or each WUS, it will bring additional bits for the new area id. [Ericsson]: Agree with Huawei. Note area and area id concept is the legacy mechanism. [Nokia]: Wonder if same WUS configuration used for multiple cells is realistic. [Huawei]: Understand similar discussion happened in RAN1, and it was not forbidden.

9. Access related information in UL WUS

R2-2502541	Discussion on On-demand SIB1	Qualcomm Incorporated	discussion

Proposal 3: RAN2 discusses the indication of feature-specific cell barring for a NES cell with on-demand SIB1.

R2-2502916	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 7: Cell access related information should be included in the the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE does not have access.

[Ericsson, ZTE, NTTDCM]: Prefer not to have this optimization. WUS configuration can be more updated often than access related information in SIB1. [Huawei, Apple, Google, Vivo, Xiaomi, Samsung]: Support the proposal. [Huawei]: Assume no big additional signaling overhead to include access related information. [Lenovo]: Additional information is not small including PLMN id, TAC, and other access related information. [Nokia]: This information will be defined as optional, so if NW does not want it, it can be omitted (NW choice). [Vivo]: Wonder if we need all information. [Ericsson]: If we put this information, it will make a list of cells associated with WUS configuration more difficult. [CMCC]: RAN1 already decided not to include barring information into WUS. [Samsung]: Barring is RAN2 scope. [Huawei]: We can only consider feature-specific cell barring in WUS since it needs only a few bits. [Nokia]: It does not make sense. Should be common to all access related information.

Offline among interested companies. Comeback in CB session (Nokia).

[Comeback session]:

[Nokia]: Understand companies are not strongly against making this information optional. However there are more details we need to consider for determination. [Lenovo]: NPN, slice, blabla.. so many IEs are related to access related information. Understand it is not so simple. [Nokia]: Understand slice does not impact on access determination. [Lenovo, Ericsson]: Have different view than Nokia.

Revisit it next meeting.

10. Others

R2-2502131	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: Trigger condition of OD-SIB1 acquisition (i.e. reusing cell reselection criteria) is different from legacy. So, RAN2 capture it in both TS 38.331 and TS 38.304.

We do not need a separate new triggering condition of OD-SIB1 acquisition.

R2-2502226	Remaining issues on on-demand SIB1 procedure	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4:	When timer T311 is running, the cell selection criterion S is evaluated by the existing cell reselection parameters provided from Cell A.

When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

Agreements on OD-SIB1

- NW ensures that the RRC connected UE has the latest SIB1 (e.g. dedicated RRC message to deliver SIB1 or not configure searchSpaceSIB1), as baseline. UE understands that the stored SIB1 is the latest SIB1.

- When the cell supporting on demand SIB1 is broadcasting SIB1 (e.g. upon SIB1 request), legacy UE can camp on the cell if the legacy UE is able to acquire the broadcasted SIB1. No specification impact is foreseen.

- Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

- SIBX that was acquired during RRC connected state can be used for OD-SIB1 request in RLF.

- Align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

- The UL WUS configuration includes the IE totalNumberOfRA-Preambles.

- Backoff is not applied to OD-SIB1 request when backoff is included in RAR.

- Leave this issue (how to handle paging interruption during OD-SIB1 acquisition) to RAN4.

- If UE has not received the PDCCH scheduling SIB1 upon the expiry of the SIB1 monitoring window, UE may consider the cell as being barred.

- WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

- Area id will be used as legacy.

- We do not need a separate new triggering condition of OD-SIB1 acquisition.

- When timer T311 is running, SIB1 acquisition triggering condition is same as legacy. No additional spec impact is foreseen.

R2-2501795	Discussion on on-demand SIB1	Xiaomi	discussion	Rel-19

R2-2501834	Discussion on on-demand SIB1	HONOR	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501864	Remaining issues for On-demand SIB1 request and UE behaviour	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501866	On-demand SIB1	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501902	Consideration on on-demand SIB1	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501958	Remaining issues of on demand SIB1	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501982	Discussion on remaining issues of on-demand SIB1 operation for NES	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502104	Remaining issues on the support for OD-SIB1	Google	discussion	Rel-19	Netw_Energy_NR_enh-Core	R2-2500463

R2-2502131	Remaining issues on on-demand SIB1	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502134	Discussion on on-demand SIB1 for NES	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502149	Discussion on on-demand SIB1 for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502226	Remaining issues on on-demand SIB1 procedure	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502229	Remaining issues on OD-SIB1	LG Electronics Inc.	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502261	Discussion on Ondemand-SIB1	KDDI Corporation (TTC)	discussion	Rel-19

R2-2502385	Discussion on OD-SIB1 for RRC IDLE and INACTIVE UE	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502400	Discussion on on-demand SIB1 for handling RLF	ITRI	discussion	Netw_Energy_NR_enh-Core

R2-2502488	Remaining details for on-demand SIB1	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502541	Discussion on On-demand SIB1	Qualcomm Incorporated	discussion

R2-2502577	SIB1 acquisition for Rel-19 NES UE in RRC_CONNECTED state	Fainity Innovation	discussion

R2-2502580	On-demand SIB1 request and reception	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502596	Remaining OD SIB1 issues	Lenovo	discussion

R2-2502725	Discussion on On-demand SIB1	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502761	Consideration on on-demand SIB1	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502781	Discussion on on-demand SIB1	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502876	Discussion on remaining issues of on-demand SIB1	SHARP Corporation	discussion	Rel-19

R2-2502900	Discussion on On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-19

R2-2502916	On demand SIB1 handling	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

### 8.5.4	Adaptation of common signal/channel transmissions

Remaining open issues, including RAN2 spec impacts and discussion with consideration of RAN1 progress, stage-3 identified open issues if needed, etc.

1. Paging adaptation: NES UE’s PEI monitoring

R2-2501959	Remaining issues of common signal/channel transmissions	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2a: For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

Proposal 2b: For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

R2-2502917	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 15: A single PEI with DCI format 2_7 can address both legacy POs and additional release 19 POs using a mixture of multiplexing and separate signalling.

Proposal 16: A separate PEI configuration can be used for release 19 UEs to define how the release 19 POs are mapped to the same DCI format 2_7 used by legacy UEs for PEI.

R2-2502841	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: (related to 38.331 FFS) A new firstPDCCH-MonitoringOccasionOfPO-r19 is added, value range is extended to “(0..143359)” to accommodate SCS=480kHz and N=T/32. RAN2 to down-select from the following options:

•	Option 1) R19 NES UEs use firstPMO-r19 for all POs in the PF

•	Option 2) R19 NES UEs use legacy PMO for POs that overlap with legacy POs, use firstPMO-r19 for additional POs not overlapping with legacy Pos

[LG]: NES UE should determine based on Rel-19 configuration. Huawei proposal sounds very complicated. [Apple, OPPO, Samsung, Vivo]: Support ZTE proposal. Nokia’s proposal will impact RAN1 and we do not prefer it now. RAN1 almost finalizes WI completion. [Huawei]: Do not think our proposal is controversial to RAN2 previous agreements. [Vivo]: Agree with ZTE’s first proposal. Ok with the second proposal (after some discussion).

For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

2. Paging adaptation: RRC connected UE

R2-2501868	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED

[Nokia]: What is specification impact? We can leave it to UE implementation (e.g. RRC connected UE can monitor either R18 or R19 PO). [OPPO, Xiaomi]: Agree with Nokia. [Ericsson, Lenovo]: Agree with proposal. [Samsung]: Would like to have time to explain the proposal to the companies.

Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

3. Paging adaptation: UE capability

R2-2502386	Discussion on adaptation of common signal transmissions	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 2: No need to define a common capability for all Rel-19 NES features.

4. Additional RO configuration

R2-2501796	Discussion on common signal adaptation	Xiaomi	discussion	Rel-19

Proposal 3: The additional PRACH configuration is configured in SIB1.

R2-2502841	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 4: New RACH-ConfigCommon is added in BWP-ConfigCommon for R19 NES UEs.

R2-2501868	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 3: Introduce prach-ConfigurationIndex and msg1-FrequencyStart for additional rach resouces in RACH-ConfigGeneric IE.

5. RA-RNTI Calculation

R2-2502132	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 8: RAN2 discuss RA-RNTI ambiguity issue, and consider the following alternatives:

•	Alt 1: Introduce a fixed k_offset for the additional PRACH resources, i.e. RA-RNTI = 1 + s_id + 14 x t_id + 14 x 80 x f_id + 14 x 80 x8 x ul_carrier_id + 14 x 80x 8 x 2 x k_offset (k_offset>1)

•	Alt 2: f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs.

6. RACH Adaptation for CFRA

R2-2501817	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 1: R2 confirms time-domain RACH adaptation is supported for CFRA initiated by normal PDCCH order, but not for other CFRA cases. Send LS to R1 if any concern.

[Nokia]: Any technical reason not to apply PDCCH order based CFRA RACH adaptation to other use case? [OPPO]: For example, BFR, RACH configuration is presented as mandatory configuration. And if needed, NW can put additional RACH RO into this configuration. [Samsung, Apple]: Agree with OPPO proposal. And note RAN1 decided that additional RO is only applicable to initial BWP. [Huawei]: Understand RAN1 introduced PDCCH order can be applicable to CBRA also. [Apple]: Understand there is no restriction that PDCCH order is applied to CBRA. [Ericsson]: Not sure if new mechanism for additional RO cannot be used for other cases. [Nokia]: Understand the additional RO activation/deactivation still can be applicable e.g. into BFR case. [OPPO]: Understand without additional RO activation/deactivation, if network wants, the network just includes additional RO into BFR configuration, then the UE uses it unless it is reconfigured or released. [Nokia]: Can we try to agree no additional mechanism is needed for BFR case in order to use additional RO. [Samsung]: For any kind of HO mechanisms, RACH generic is optional and if network wants to use additional RACH RO, network can configure the additional RACH RO to RACH generic configuration. Dynamic activation/deactivation is based on the serving cell’s short message. It seems clear no need of dynamic activation/deactivation to the non-serving cell(s). [Spreadtrum]: Wonder how it works if multiple initial BWPs are configured. [OPPO]: Propose to have post email discussion on this issue.

[POST129b][122][NES] (OPPO)

Scope: Discuss and make conclusions on proposal 1 in R2-2501817.

Intended outcome: Discussion summary.

Deadline: Long email discussion.

7. Handling of RRC Connected UE

R2-2501868	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: if adaptation for additional PRACH resources is supported in active UL BWP, network ensures that paging search space is configured in active DL BWP.

Option 2: Introduce other mechanism (such RRC Reconfiguration message or MAC CE or group DCI) for adaptation for additional PRACH resources in RRC_CONNECTED.

Proposal 6: RAN2 to discuss and agree one of the following for RRC_CONNECTED

Option 1: For DCI-based adaptation indication for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every defaultPagingCycle.

Option 2: For DCI-based adaptation for additional PRACH resources in RRC_CONNECTED, UE monitors the indication in any paging occasion at least once every modification period.

8. Need of MAC CE based signalling

R2-2502782	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.	discussion	Rel-19

Proposal 1: Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

R2-2501865	PRACH, paging and SSB adaptation for NES	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 5: RAN2 should adopt a unified MAC CE design for signaling SSB adaptation and OD-SSB to optimize efficiency and streamline standardization.

[Session chair]: How many companies object to introduce MAC-CE based signalling solution in addition to DCI? NTTDCM, CATT, Fujitsu, LG, Apple, Xiaomi, IDC, Huawei, Samsung, Ericsson, KDDI. [Session chair]: Considering we already have one mechanism based on DCI, suggest to agree with NTTDCM proposal.

Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

9. Signalling for SMTC adaptation

R2-2501903	Adaptation of Common signal channel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

Proposal 6: For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

Proposal 7: Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

Agreements on common channel adaptation

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

R2-2501796	Discussion on common signal adaptation	Xiaomi	discussion	Rel-19

R2-2501817	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501865	PRACH, paging and SSB adaptation for NES	NEC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501868	Adaptation of common signal channel transmissions	Samsung	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501903	Adaptation of Common signal channel transmissions	CATT	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2501959	Remaining issues of common signal/channel transmissions	ZTE Corporation, Sanechips	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502031	Adaptation of common signal or channel	Fujitsu	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502132	Remaining issues on common signal transmission adaptation	Apple	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502372	Paging and PRACH adaptation	Lenovo	discussion	Rel-19

R2-2502386	Discussion on adaptation of common signal transmissions	vivo	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502489	RAN2 impacts on SSB and RACH adaptations	Sony	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502542	Discussion on Adaptation of Common Signal/Channel Transmissions	Qualcomm Incorporated	discussion

R2-2502581	Time domain adaptation of common signalling and channels	InterDigital	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502726	Discussion on Common signalling adaptation	CMCC	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502766	Adaptation of common signal/channel transmissions for NES	Ericsson	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502782	Discussion on adaptation of common signal and channel	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502841	Discussion on adaptation of common signals/channels transmissions	Huawei, HiSilicon	discussion	Rel-19	Netw_Energy_NR_enh-Core

R2-2502917	Adaptation of common signals	Nokia, Nokia Shanghai Bell	discussion	Rel-19	Netw_Energy_NR_enh-Core

## 8.6	Mobility Enhancement Ph4

(NR_Mob_Ph4-Core; leading WG: RAN2; REL-19; WID: RP-242356)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.6.1	Organizational

Incoming LS, WI rapporteur inputs, CR rapporteur inputs (including post email discussion [POST129][106], [107], [108], [109], [110], [111], summary of identified open issues that need online discussion and rapporteur’s suggestions if needed).

R2-2501709	LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells (R1- 2501500; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2	Cc:RAN3

Noted.

[Qualcomm]: Not sure if semi-persistent CSI-RS is also applied to event-triggered MR. [Apple]: It is about semi-persistent CSI-RS activation/deactivation, RAN2 does not need to discuss how to use it. Understand there is no restriction to NW. [Samsung, Huawei]: Understand it can be used for both periodic and event-triggered MR. [ZTE]: Wonder if SP CSI-RS can be used for early DL/UL synchronization. [Apple]: Yes, up to NW.

R2-2501715	LS on L1filtering for LTM event triggered reporting (R1-2501577; contact: Fujitsu)	RAN1	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2

Noted.

[Ericsson, Nokia]: Wonder if we need any MAC-based filtering. [Apple]: RAN1 clearly clarified that L1 filtering is up to UE implementation. [Vivo]: Agree with Apple. We can capture it in stage 2 spec, and we do not need any kind of L2 filtering. [LG]: Agree with Ericsson and Nokia. [ZTE]: We should not discuss this in RAN2 since it was already discussed and concluded in RAN1 and it is more complicated to RAN2. Also consider there will be minimum RAN4 performance requirements. [Apple]: Running CR already capture this RAN1 LS correctly.

No L1/L2 filtering issue will be discussed in RAN2.

R2-2501756	Reply LS on security handling for inter-CU LTM in non-DC cases (S3-251124; contact: CATT)	SA3	LS in	Rel-19	NR_Mob_Ph4-Core	To:RAN2, RAN3

Noted.

[OPPO]: Wonder if it is also applicable to DC case. Do need to ask it to SA3? [Apple]: No. Once we ask it to SA3, it will take more than 2-3 meeting cycles, which is not acceptable. For NCC, understand it is related to MCG only.

R2-2502198	Observations on some topics related to Rel-19 Mobility Enh WI	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 to design MAC CE based NCC transfer and can evaluate the option of indexing the NCC (instead of sending the actual NCC value), but limit the discussion scope to only this, to ensure that the solution is simple.

Proposal 2: Inform RAN3 of this so that RAN3 discussions would be scoped better and practical.

Proposal 3: For Rel-19 WIs which are enhancements (for LTM, NES, MIMO) RAN2 to discuss the inter-working of Rel-19 LTM with not only Rel-19 but also with earlier versions of WIs, if the previous release WI inter-working is agreed to be not prohibited.

Proposal 4: RAN2 to agree on inter-operability of WIs with Rel-19 LTM using the below driving guidelines:

1.	If the inter-working works without any changes or only depends on NW ensuring consistent configuration to the UE and;

2.	If the inter-working does NOT need any standardized addition of interfaces or parameters to the existing interfaces arcoss NW nodes and between UE and the NW.

Proposal 5: When discussing inter-working of Rel-19 LTM with other WIs, RAN2 should evaluate inter-CU LTM inter-working seperately from CLTM inter-working.

Noted.

[Ericsson]: For inter-working, we should not start this discussion until the whole Rel-19 LTM becomes mature enough. [ZTE]: At least Rel-18 MIMO need to be discussed this meeting.

R2-2502197	Report of [POST129][106][Mob] Stage-2 CR 38.300 (Apple)	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: RAN2 to discuss and conclude that the new sequence diagram from draftCR R2-2502196 replaces the Figure 9.2.3.5.2-1 from existing TS38.300.

Proposal 2: Endorse the draft CR from R2-2502196.

[Apple]: Propose to have more time for offline discussion. [Nokia]: Prefer to add a figure for Rel-19 LTM in addition to the existing Rel-18 LTM.

Comeback in CB session.

R2-2502196	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.5.0	B	NR_Mob_Ph4-Core

[Session chair]: Two updated figures 9.2.3.5.2.-1. [Apple]: It is editorial error.

Comeback in CB session.

R2-2503154	Introduction of NR mobility enhancements Phase 4 in TS 38.300	Apple	draftCR	Rel-19	38.300	18.5.0	B	NR_Mob_Ph4-Core

Endorsed.

R2-2502234	Introduction of NR mobility enhancements Phase 4 in TS 37.340	China Telecom	draftCR	Rel-19	37.340	18.5.0	B	NR_Mob_Ph4-Core

Endorsed.

R2-2502534	Running RRC CR for inter-CU and conditional LTM	Ericsson	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

Endorsed.

R2-2502535	List of FFSs for LTM and CLTM	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

[AT129b][102][MOB] (Ericsson)

Scope: Discuss the identified RRC FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502535 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503001.

Deadline: Comeback in CB session => Completed.

R2-2503001	Summary of offlines [AT129b][102][MOB] and [AT129b][110][MOB]	Ericsson	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: A list of sk-counters is linked to a Rel-19 ID configured by the SN.

Proposal 2 (modified): At RLF and reconfiguration with sync failure:

a.	If RLF:

i.	If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

b.	if reconfiguration failure (inter-CU LTM)

i.	If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

Proposal 3: How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

Proposal 4: The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

Proposal 5: Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

A list of sk-counters is linked to a Rel-19 ID configured by the SN.

At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

[Fujitsu]: Wonder if the UE revert old configuration back in proposal 2.b? [Ericsson]: We need to further define the exact UE behaviours. [Fujitsu]: If the UE reverts old configuration back, proposal does not make sense. [Session chair]: Seems it is not clear on that, with this uncertainty, can we still agree with proposals? [Ericsson]: Confirmed yes.

R2-2502095	Summary of [POST128][108][MOB] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal1: Discuss whether ltm-CSI-RS-ReportConfigToAddModList should only be configured on SpCell.

Will be discussed as part of offline discussion [103].

Proposal2: Add additional candidate values {20ms, 40ms, 60ms} for report interval for event-triggered periodic report for LTM.

[MediaTek, Ericsson]: Prefer to restrict up to two additional values otherwise we need to extend the whole IE.

We will aim the additional values up to two values.

Which values will be further discussed as part of offline discussion [103].

R2-2502096	Draft running CR for event-triggered meas report for RRC	Huawei, HiSilicon	draftCR	Rel-19	38.331	18.5.1	B	NR_Mob_Ph4-Core

Endorsed.

[AT129b][103][MOB] (Huawei)

Scope: Discuss the identified RRC FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502096 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503002.

Deadline: Comeback in CB session => Completed.

R2-2503002	Summary of [AT129b][103][MOB] RRC running CR (Huawei)	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal2: Select two values of 20ms and 60ms for the additional values

Select two values of 20ms and 60ms for the additional values.

R2-2502158	Running MAC CR for Mob Ph4	vivo (Rapporteur)	draftCR	Rel-19	38.321	18.5.0	B	NR_Mob_Ph4-Core

Noted.

[Ericsson]: Think it is still premature to be endorsed.

[AT129b][104][MOB] (Vivo)

Scope: Discuss the identified FFSs (offline discussion rapporteur can pick up which FFSs) in R2-2502158 (offline discussion rapporteur can pick up company’s proposal if needed) and make conclusions as much as possible. Note discussion should not be overlapped with the identified issues that are already included in the agenda.

Intended outcome: Discussion summary in R2-2503003.

Deadline: Comeback in CB session => Completed.

R2-2503003	Discussion report on [AT129bis][104][MOB] MAC open issues (vivo)	Vivo	discussion	Rel-19	NR_Mob_Ph4-Core

	Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase.

	For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

	(modified) When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

	When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed.

	The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE.

	Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA.

	FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

[Ericsson]: For truncated MR MAC CE, the reported beam order will be changed? [Apple]: For truncated MR MAC CE, the event triggered beam should be included first, then if more beams are included, differential measured results should be based on the first one. [Vivo]: First here means with the first priority (it does not propose exact order in all cases).

Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase.

For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed.

The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE.

Same as Rel-18: When the C-LTM execution is triggered, if the TA of the target cell acquired by early RACH is valid, UE applies the TA acquired by early RACH, otherwise if UE has successfully measured the TA of the target cell, UE applies the measured TA.

FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

R2-2501900	Report of [POST129][111][MOB] (CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1a: Rel-18 LTM capabilities are the baseline for Rel-19 inter-CU LTM.

[Huawei]: What does the proposal 1a mean? [Ericsson, CATT]: Intention was that for Rel-19 LTM operation, if we can reuse Rel-18 LTM capability, no additional new capability is needed for Rel-19.

Proposal 3: Define a per-band capability for conditional LTM(e.g. cltm-MCG-r19) . It indicates whether the UE supports conditional LTM on the band where the UE is in non-DC. This includes supports execution condition, subsequent LTM. UE supports this capability should support ltm-MCG-IntraFreq-r18 on the same band. FFS whether UE should set the value consistently for all FDD-FR1 band, all TDD-FR1 bands and so on respectively.

Proposal 5: Define a per-band capability for L3 execution condition, e.g. cltm-ExecutionConditionL3-r19 is defined to indicate whether the UE supports L3 execution condition for conditional LTM and whether the UE supports 2 trigger events for same execution condition. UE supports this capability should support capability for conditional LTM(e.g. cltm-MCG-r19) on the same band.

Proposal 6: A UE that supports conditional LTM(e.g. cltm-MCG-r19) should indicate the support for at least one of cltm-ExecutionConditionL3-r19 or cltm-ExecutionConditionL1-r19.

Proposal 7: When a UE indicates support for both conditional LTM(e.g. cltm-MCG-r19)  and ltm-RACH-LessCG-r18, it implies that the UE has the capacity to support RACH-less conditional LTM with a configured grant. Whether/how to update the field description of ltm-RACH-LessCG-r18 can be addressed in the running CR review.

Proposal 8a: Reuse the R18 capability (i.e., ue-TA-Measurement-r18) to indicate whether UE supports UE-based TA measurement for C-LTM. There is no need to define a separate capability for this purpose.

Proposal 9: Define a per-band capability, such as “cltm-TA-Indication-r19”. This capability serves to indicate whether the UE can support TA indication within the early TA MAC CE for conditional LTM.

Proposal 10: Reuse the R18 capability (i.e., ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18) to indicate whether the UE supports MAC-CE activated joint/separate LTM TCI states for CLTM.

[AT129b][108][MOB] (CATT)

Scope: Discuss proposals in R2-2501900 and check if proposals can be endorsed/agreed. If not, identify what companies’ concerns are.

Intended outcome: Discussion summary in R2-2503011.

Deadline: Comeback in CB session => Completed.

R2-2503011	Report of [POST129][111][MOB] (CATT)	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

No need to define a separate capability for the reference configuration for inter-CU LTM.

Define per-UE capabilities for security key change of MCG LTM (e.g. ltm-KeyUpdate-MCG-r19) and security key change of SCG LTM (e.g. ltm-KeyUpdate-SCG-r19). UE supports these capabilities should also support ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18 respectively. No new UE capability on inter-CU MCG LTM with SN unchanged and inter-CU MCG LTM SN with SCG addition.

Define a per-band capability for L1 execution condition, e.g. cltm-ExecutionConditionL1-r19 is defined to indicate whether the UE supports L1 execution condition for CLTM and subsequent CLTM.

Define a per-band capability for L3 execution condition, e.g. cltm-ExecutionConditionL3-r19 is defined to indicate whether the UE supports L3 execution condition for CLTM and subsequent CLTM and whether the UE supports 2 trigger events for same execution condition.

A UE that supports conditional LTM should indicate the support for at least one of cltm-ExecutionConditionL3-r19 or cltm-ExecutionConditionL1-r19.

When a UE indicates support for both conditional LTM and ltm-RACH-LessCG-r18, it implies that the UE supports RACH-less conditional LTM with a configured grant. Whether/how to update the field description of ltm-RACH-LessCG-r18 can be addressed in the running CR review.

Rely on the R18 capability (i.e., ue-TA-Measurement-r18) to indicate whether UE supports UE-based TA measurement for C-LTM. There is no need to define a separate capability for this purpose.

Define a new per UE capability for UE support of early TA MAC CE reception for CLTM and also the max number of maintaining TA values. The value range is (1~8).

Rely on the R18 capability (i.e., ltm-MAC-CE-JointTCI-r18 and ltm-MAC-CE-SeparateTCI-r18) to indicate whether the UE supports MAC-CE activated joint/separate LTM TCI states for CLTM.

R2-2502256	Draft 306 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.306	18.5.0	NR_Mob_Ph4-Core

R2-2502257	Draft 331 running CR for UE capability for Mob Ph4	CATT	draftCR	Rel-19	38.331	18.5.1	NR_Mob_Ph4-Core

Noted.

[POST129b][115][MOB] (Ericsson)

Scope: Update RRC running CR for inter-CU LTM and C-LTM based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list for inter-CU LTM and C-LTM.

Deadline: Long email discussion

[POST129b][116][MOB] (Huawei)

Scope: Update RRC running CR for L1 event-driven MR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (RRC running CR can keep editor’s notes for readability).

Intended outcome: Updated RRC running CR and essential RRC open issue list for L1 event-driven MR.

Deadline: Long email discussion

[POST129b][117][MOB] (Vivo)

Scope: Update MAC running CR based on RAN2#129bis progress and maintain essential open issue list in a separate contribution (MAC running CR can keep editor’s notes for readability).

Intended outcome: Updated MAC running CR and essential MAC open issue list.

Deadline: Long email discussion

[POST129b][118][MOB] (CATT)

Scope: Update UE capability running CRs (including 38.331 and 38.306) based on RAN2#129bis progress.

Intended outcome: Updated UE capability running CRs (including 38.331 and 38.306).

Deadline: Long email discussion

[POST129b][119][MOB] (Apple)

Scope: Update 38.300 running CR based on RAN2#129bis progress.

Intended outcome: Updated 38.300 running CR.

Deadline: Long email discussion

[POST129b][120][MOB] (China Telecom)

Scope: Update 37.340 running CR based on RAN2#129bis progress.

Intended outcome: Updated 37.340 running CR.

Deadline: Long email discussion

[Apple]: For detailed stage 3 issues, please contact CR rapporteurs first.

For stage 3 essential issues, please contact CR rapporteurs first.

### 8.6.2	Inter-CU LTM

Essential remaining open issues, including security key change handling based on S3-251124, details of fast recovery, FFS on R19 LTM coexistence with MIMO 2TA/mTRP, stage-3 identified open issues if needed, etc.

1. Signaling for NCC delivery

R2-2502340	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: For security key update in inter-CU LTM, RAN2 agree to include NCC value in the LTM cell switch command MAC CE.

R2-2502045	On remaining issues for Inter-CU LTM	Nokia	discussion

Proposal 1: RAN2 to support delivery of master key update related parameters via RRC message prior to LTM switch.

[Session chair]: Note that security related concern is SA3 scope and SA3 already responded it. Let’s check companies’ views. Rakuten, LG, ZTE, Lenovo, Apple, NTTDCM, Sharp, Samsung, Vivo, NEC, Xiaomi, MediaTek, OPPO, Ofinno, China Telecom, CATT, Ericsson, Fujitsu, CMCC, IDC, Denso, TCL (22) support proposal 1 (ZTE). Qualcomm, Nokia (2) support proposal 1 (Nokia). [Qualcomm]: If we go with proposal 1 (ZTE), it will be good to clarify to include actual NCC value (i.e. no need of further optimization, e.g. index for NCC, etc.) [NTTDCM]: Concern that putting plain NCC to CSC MAC CE.

For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

2. NCC and Rel-19 ltm-NoSecurityChangeID

R2-2501897	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: The NW ensures that NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

R2-2502509	Discussion on remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: RAN2 to confirm that NCC info can be provided to UE via LTM Command MAC CE when the source and target cell(s) have the same Rel-19 ltm-NoSecurityChangeID (intra-CU LTM).

R2-2502444	Discussion on security for inter-CU LTM	Ericsson	discussion	NR_Mob_Ph4-Core

Proposal 2: The existing LTM cell switch MAC CE is baseline but enhanced by including the NCC value and an indication on whether the UE should perform security key change or not (this indication override the Rel-19 security IDs).

[Rakuten]: Support Apple proposal. We also have intra-CU CU-UP change. [Apple]: The concerned scenario is 1) UE switched to inter-CU target cell, 2) new target cell receives a new NCC from AMF, 3) there was no time for the new target cell provides the NCC to the UE (e.g. UE is moving so fast), 4) So NCC needs to be provided by CSC MAC CE. [Huawei]: Do not think Apple’s concerned scenario is really realistic in the field. [Lenovo]: Wonder if inclusion of NCC really needs to be associated with R19 ID. If included, the UE will check and perform either horizontal key change or vertical key change. [Ericsson]: We may first agree that NCC is included in CSC MAC CE when R19 set id is different, and then we can use the legacy mechanism to change security for intra-cell. [Samsung, Nokia, Qualcomm]: Agree with Ericsson.

NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

[Lenovo]: Wonder if we need to send an LS to inform RAN2 progress? [Apple]: Support sending LS to RAN3 to inform the related RAN2 progress.

[AT129b][109][MOB] (ZTE)

Scope: Prepare LS to RAN3 to inform RAN2 progress that is related to security aspect.

Intended outcome: LS to RAN3 in R2-2503012.

Deadline: Comeback in CB => Completed.

R2-2503012	LS on RAN2 agreements for security key handling in inter-CU LTM	LSout	To: RAN3	Rel-19	NR_Mob_Ph4-Core

Approved.

3. SN key regeneration in inter-CU MCG LTM

R2-2502159	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 18: RAN2 to discuss and choose one of the two options for the security key handling of inter-CU MCG LTM with DC:

-	Option 1: Default sk-Counter = 0 is used by UE for generating the SN key when security key update is performed in MCG.

-	Option 2: Network configures sk-Counter = 0 in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

[Nokia]: Prefer option2 and think we do not need to configure sk-Counter for all candidates, it should be sufficient to configure sk-Counter for each Rel-19 ID. [LG]: Understand “when a new AS root key, KNG-RAN, in the associated 5G AS security context is established” does not include horizontal or vertical key change due to HO/LTM cell switch. [Ericsson]: Do not think it should be set to fixed value, i.e. “0” in the security point of view. Prefer network-configuration based option. [Apple, CATT]: Setting sk-Counter to “0” comes from SA3 spec. [LG]: Note Kng-ran is different than Kng*. Kng-ran is used when the new security key should be created by AMF due to e.g. key mismatch. [CATT, OPPO]: Also prefer option 2. Network can configure the appropriate value whatever is. We don’t need to discuss whether sk-Counter 0 is correct or not. [Samsung]: Agree with Apple and CATT. [Ericsson]: Note security key change can happen for intra-CU case. [ZTE, Ericsson]: Current running CR is aligned with the agreement below. If we go with Nokia’s proposal, it needs further discussion to change the current running CR. Not sure if it is really needed.

4. Fast recovery

R2-2502285	Discussion on inter-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2 (modified): RAN2 to not support fast LTM recovery if the cell selected during RRC re-establishment procedure is one of the LTM candidate cell(s) that has different Rel-19 ID.

Proposal 3 (modified): RAN2 to not support fast LTM recovery after LTM failure that has different Rel-19 ID.

[Lenovo]: Support proposal 2 and proposal 3. Note in Rel-18, after CHO/HO failure, LTM recovery is not applied. [Ericsson]: Rel-19 ID does not distinguish whether intra-LTM or inter-LTM at least in the specification point of view (though it can be specified dependent on Rel-19 ID, the UE behaviour is defined when the key change is required or when LTM recovery is applied).

R2-2502509	Discussion on remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: RAN2 to confirm that UE can perform fast LTM recovery to all candidate cells (from same or different CU as source cell), regardless intra-CU or inter-CU LTM failure, as long as NCC info is provided in LTM Command MAC CE.

Proposal 5: For fast LTM recovery, UE performs AS key derivation to the cell for recovery using NCC info provided in LTM Command MAC CE.

Proposal 6: When NCC is not provided (as in Rel-18) in LTM Command MAC CE, UE can only perform fast LTM recovery to intra-CU candidate cells with the same Rel-19 ltm-NoSecurityChangeID as source cell.

[Apple]: Think proposals are feasible, but it may be too optimistic. One compromise would be that we will allow LTM recovery to the candidates that belong to the same Rel-19 ID as the target cell. [Huawei]: Good to support the proposal and there would be another alternative, even though the candidate does not get the latest key, it can fetch from the source cell. [ZTE]: Understand if the selected candidate fetches the context from the source, it is not fast recovery. [Rakuten]: As Ericsson mentioned, Rel-19 id does not distinguish between intra-CU LTM and inter-CU LTM. [CATT]: Think common understanding is that, if Rel-19 ID is different, the UE needs to reestablish PDCP and change the security key (we do not need to know whether it is intra- or inter-). Think we do not need a complicated solution for recovery since it is anyway a kind of optimization. Ok with not supporting LTM recovery in inter-CU LTM failure. [Nokia]: Prefer supporting original Apple proposal.

R2-2502852	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1-6 (modified): For inter-CU LTM cell switch failure (due to expiry of T304), RAN2 is kindly asked to discuss whether to support the fast failure recovery if the selected LTM candidate cell has same Rel-19 ID w.r.t. the target cell (Case 2).

Proposal 1-4: After inter-CU/intra-CU LTM cell switch failure (due to the expiry of T304), RAN2 is kindly asked to agree that the UE can perform the fast recovery if the selected LTM candidate cell is the intra-CU cell w.r.t. to the source cell (Case 1 & 4).

Proposal 1-5: After intra-CU LTM cell switch failure (due to the expiry of T304), RAN2 is kindly asked to agree that the UE performs RRC reestablishment procedure (not fast recovery) if the selected LTM candidate cell is the inter-CU candidate cell w.r.t. to the source cell (Case 3) .

Proposal 1-7: For RLF after the LTM cell switch, RAN2 is kindly asked to agree that the UE can perform the fast recovery if the selected LTM candidate cell is the intra-CU cell w.r.t. the target cell (Case 1 & 2).

[Fujitsu]: Understand one NCC is only used for single candidate. [Apple]: In NW, there is only one NCC (either used NCC or just new NCC). Understand NCC can be used for multiple candidates belonging to the same Rel-19 ID. [MediaTek]: Understand in inter-CU LTM, PDCP is reestablished and all variables will be released. Like what we discussed in Rel-18, then no LTM recovery can be applied. [Ericsson]: Ok with Rel-19 ID based LTM recovery. [Apple]: Note LTM recovery here is different compared to Rel-18 LTM. In Rel-18 LTM recovery, no PDCP reestablishment -> UE revert back to the source configuration -> the UE uses old key. In Rel-19 LTM recovery, PDCP reestablishment -> UE revert back to the source configuration -> apply the latest NCC to derive a new key for the newly selected candidate cell.

[Session chair]: Let’s summarize:

- Option1: No LTM recovery is applied for inter-CU LTM (once NCC is included in CSC MAC CE)

- Option2: LTM recovery can be applied if the selected candidate cell belongs to the same Rel-19 with the target cell.

- Option3: LTM recovery can be applied for all candidates.

[Session chair]: Ok to exclude option3? Seems only few companies support option3 now.

Option3 (LTM recovery can be applied for all candidates) is not considered.

Offline discussion to clearly understand how option2 (LTM recovery can be applied if the selected candidate cell belongs to the same Rel-19 with the target cell) works and check if there is a real technical blocking issue. If it works w/o real technical blocking issue, then we can go to option2. If there is a real technical blocking issue for option2, we’ll go to option1 (No LTM recovery is applied for inter-CU LTM).

[AT129b][110][MOB] (Ericsson)

Scope: Clearly clarify how option2 works and check if there is a real technical blocking issue. If it works w/o real technical blocking issue, then we can go to option2. If there is a real technical blocking issue for option2, we’ll go to option1.

Intended outcome: Discussion summary in R2-2503013.

Deadline: Comeback in CB session => Completed (it was included in [AT129b][102]).

5. UE-based TA measurement

R2-2502159	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7 (modified): From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

[Lenovo]: Wonder if we need to send an LS to RAN3 to inform this agreement. They may need to take it into account for NW interface. [ZTE]: Understand RAN3 impacts, but we may need to make further decision before sending an LS to RAN3 (e.g. who will allocate ID). [Ericsson]: Understand RAN3 already discussed it, so there is no need to send an LS. And think one option would be source will decide it.

6. Co-existence with Rel-18 MIMO 2 TA

R2-2502727	Discussion on Inter-CU LTM	CMCC	discussion	Rel-. 19	NR_Mob_Ph4-Core

Proposal 4: MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

Proposal 6: If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA

MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

7. Co-existence with Rel-18 NES

R2-2502944	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6: An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

Agreements on inter-CU LTM

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

- An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

R2-2501835	Further discussion on inter-CU LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501887	Discussion on open issues for inter-CU LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501897	Discussion on Inter-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501929	Security discussion and other remaining issues of inter-CU LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501977	Discussion on inter-CU LTM	Google	discussion	Rel-19

R2-2501995	Remaining issues of inter-CU LTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502017	Remaining Issues for Inter-CU LTM	Qualcomm Incorporated	discussion

R2-2502045	On remaining issues for Inter-CU LTM	Nokia	discussion

R2-2502066	Discussion on inter-CU LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502120	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502121	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502122	Discussion on Inter-CU LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502135	Discussion on Inter-CU LTM security and fast recovery	InterDigital, Europe, Ltd.	discussion	Rel-19

R2-2502159	Discussion on inter-CU LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502235	Discussion on inter-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502285	Discussion on inter-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502299	Discussion on inter-CU LTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502340	Discussion on inter-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502359	Remaining issues for Inter-CU LTM	Lenovo	discussion	Rel-19

R2-2502381	Discussion on issues for supporting inter-CU LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502423	Further discussion remaining issues of inter-CU LTM cell switch	Transsion Holdings	discussion	Rel-19

R2-2502444	Discussion on security for inter-CU LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2502481	Discussion on inter-CU LTM	NTT DOCOMO, INC.	discussion	Rel-19

R2-2502490	LTM Resource consumption for the target cells	Sony	discussion	Rel-19	NR_Mob_Ph4

R2-2502509	Discussion on remaining issues in Inter-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502601	Discussion on open issues for inter-CU LTM	Ofinno, LLC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502683	Discussion on security handling for inter-CU LTM	KT Corp.	discussion

R2-2502727	Discussion on Inter-CU LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502852	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502944	Inter-CU LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502965	Discussion on inter-CU LTM	DENSO CORPORATION	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502774	Further Considerations to Support Inter-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

=> Withdrawn

### 8.6.3	L1 event triggered measurement reporting

Essential remaining open issues, including MAC CE introduction based on R1-2501500, details of MR MAC CE, stage-3 identified open issues if needed, etc.

1. Activation/deactivation of SP CSI-RS for LTM

R2-2502433	Discussion on L1 event triggered measurement reporting	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 1: Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

Proposal 2 (modified): The new MAC CE includes:

- A/D indication

- Target configuration id or {SP CSI-RS resource set id and/or candidate id}

- TCI state id

The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

R2-2502799	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: RAN2 decide How to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM) between following options.

•	Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch

•	Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch

•	Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch

[LG, Nokia, OPPO]: Think option1 is sufficient. [CATT, MediaTek, Apple, IDC, Honor]: Think option2 is more reasonable. [ZTE]: We need to consider RAN3 impact. For option2, source should indicate to the target regarding the activated SP CSI-RS information. [Samsung]: Even with option1, the target may need to indicate the deactivation to all corresponding candidates. Probably similar impact on RAN3. In the functional point of view, option2 will be helpful and make decision this meeting will also help RAN3. [Honor]: With option1, it will also bring additional signalling overhead since it needs to send activation again for the same SP CSI-RS resources.

Will be revisited next meeting.

2. Restriction of evaluated RSs via candidate cell indication

R2-2502950	L1 event triggered measurement report 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

Proposal 13: Network can optionally configure a candidate cell via RRC within report configuration of event LTM. If the candidate cell is configured, UE only evaluates RSs associated with the candidate cell among RSs associated with the event.

R2-2501898	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 4: RAN2 confirms the previous agreement that "For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible". No additional enhancements are needed.

[ZTE, Nokia, Ericsson]: Support the LG proposal. Question is how the source cell can configure measurement RS without coordination with candidate cells. [CATT, OPPO, Samsung, Apple]: Support CATT proposal. Coordination is RAN3 scope. [Samsung]: Consider it is a kind of optimization and seems somewhat overlapped with possible Rel-20 scope. [ZTE]: There is clear difference compared to possible Rel-20 scopes. [Apple]: It may bring the additional works to RAN3 if we go to LG proposal. [ZTE]: Have different understanding. [Session chair]: Understand in Rel-18, measurement RS will be generated in the source, and it will be forwarded to all candidates. Then candidate will configure reporting configuration accordingly. What makes Rel-19 now different? [ZTE]: In Rel-19, difference is to have event triggering related parameters in reporting configuration, which makes candidate configure appropriately more difficult. [Session chair]: Maybe good to have offline among interested companies to understand the issue.

Comeback in CB session with some offline among interested companies (ZTE)

CB Session:

[ZTE] Propose the following WF, based on the offline:

FFS The beam specific offset Obn and Obs is configured as per-beam or per-report.

Proposed WF: In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

3. MR MAC CE: Indication of beams(s) that meets the event condition

R2-2502445	Discussion on event-based L1 measurements	Ericsson	discussion	NR_Mob_Ph4-Core

Proposal 2	RAN2 to consider the following options to indicate the report-on-leave and event-triggered beams in the MR MAC CE:

a)	A bitmap to indicate whether the reported beam is an event-triggered or a non-triggered beam and whether the reported beam fulfills the entering condition(s) or the leaving condition(s) for the given LTM event.

b)	A single-bit per beam to indicate whether the beam is a triggering beam and an additional single-bit indication per reported beam to distinguish the event-triggered and non-triggered beams for the reported beam.

R2-2502552	Further Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500332

Proposal 8: RAN2 agrees that MR MAC CE has a single field indicating that the report is for report on leave.

Proposal 9: If the MR MAC CE is for report on leave it contains only RSRIs that are reported as they have met the event leaving condition.

R2-2501898	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2a: In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event).

[LG]: Prefer Ericsson option b). When leaving condition is met, the NW needs to know other beam information also, which means we need an indication per beam level. [ZTE, Xiaomi, Apple]: We should allow to report both beam(s) that entered the condition and beam(s) that left the condition. [ZTE]: Understand we need four code points, indicating beam(s) that newly entered the condition, beam(s) that left the condition, beam(s) that maintained as the beam(s) that already entered the condition and reported, and beam(s) that additional beam(s) that doesn’t met any of entering and leaving conditions. [Apple]: Understand Ericsson 2b) is supported by majority companies. [Nokia]: Although not popular, with our proposal, it would be beneficial in the signalling overhead point of view. [Xiaomi]: With differential RSRP, we can already have two spare bits per beam. Do not think signalling overhead is something critical here. [Nokia]: Do not understand why we really need explicit four code points. Now, we can agree with two indications per beam, which indicates beam(s) that entered the condition and beam(s) that left the condition. [Huawei]: Nokia proposal seems ok since it is aligned with how legacy L3 MR works.

In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event).

Introduce two bits per beam.

Comeback in CB session with further offline w.r.t. how to define two bits. (LG)

CB session:

[LG]: Propose the following WF, based on the offline:

1. RAN2 considers the following beams types subject to reporting (not other types):

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

- Type1 beam is now in beamsTriggeredList since the beam has been just included in beamsTriggeredList.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

- Type2 beam is NOT in beamsTriggeredList since the beam has been just excluded from beamsTriggeredList.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before)

- Type3 beam is still in beamsTriggeredList.

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Type4 beam is NOT in beamsTriggeredList.

2. All companies agreed to choose either one of option1 or option3.

option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting

3. Majority of the companies are fine to go for option1, since it is straightforward.

4. A few companies think option3 is sufficient, since distinction between type1 and type3 from network side is not essential.

[Nokia]: Prefer not to make decision now. [Apple]: At least want to make a working assumption with option1 and if problem is really justified with option1, we can revert it back.

RAN2 will consider only the following options:

Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

4. Truncated MR MAC CE

R2-2502799	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6: UE considers the LTM MR procedure is completed and cancel the LTM MR procedure after UE send the truncated event triggered L1 measurement report MAC CE for the reported ID.

R2-2502094	Discussion on event-triggered meas report	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 13: If only part of the MR for the corresponding ReportConfigId is transmitted in the truncated MAC CE, the triggering status of the corresponding event should not be cancelled. The triggering state should only be cancelled once the complete MR for the corresponding ReportConfigId has been transmitted.

R2-2502950	L1 event triggered measurement report 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

Proposal 12: Network configures UE via RRC with whether UE should still consider triggered L1 MAC CE as pending or cancel it after transmitting truncated L1 MAC CE.

[Lenovo]: Exact format of MR MAC CE is not decided yet. It will be difficult to make progress on this topic now. [CATT]: It should be ok to discuss general principle. Agree with Huawei’s proposal. [Vivo]: Prefer Samsung proposal. [Apple]: Understand Samsung proposal is aligned with BFR, support Samsung proposal. Truncated MR MAC CE will be generated after reception of grant (if not enough), so there would be no remaining part of MR MAC CE because MR MAC CE was not generated. [Nokia]: Also prefer Samsung proposal. Remaining measurement reporting information may be changed when the following additional grant is received. [ZTE]: Do not understand how Samsung proposal works. If multiple beams met the condition, which beam will be included? [Apple]: Probably Huawei’s proposal and Samsung’s proposal wouldn’t be much different. For single beam that met a condition, anyway there would be no additional MR once truncated MR was sent. For multiple beams that met a condition simultaneously, once the truncated MR MAC CE was sent, if following additional grant is received, the UE still can trigger another MR procedure for the remained event-triggered beam(s). [Apple]: Huawei confirms it. [Nokia]: Think still not clear whether initially triggered MR procedure is cancelled or not. [Huawei]: If cancelled, does it mean the UE needs to perform new event triggering condition. [Session chair]: Understand MR procedure is triggered by event (not by remained event-triggered beam(s) that were not reported by the truncated MR MAC CE). So, MR needs to be either cancelled or suspended. Cancelled and trigger another new MR procedure by the remained event-triggered beam(s) that were not included in the truncated MR MAC CE, sounds new.

Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

R2-2502552	Further Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500332

Proposal 10: The UE uses truncated MR MAC CE when it is not able to report all the beams that triggered the report, up to the configured number of beams.

Not agreed.

[LG]: Prefer simple approach. If the UE cannot report whole MR MAC CE, then the UE uses truncated MR MAC CE. [Lenovo]: Prefer Nokia’s proposal. The additional results up to N is optional for NW side. [CATT]: LG’s comment is quite aligned with RAN2 previous agreement.

5. Co-existence with mTRP

R2-2502360	Event based L1 measurement report	Lenovo	discussion	Rel-19

Proposal 2: mTRP at serving cell is not supported for event based L1 measurement report in Rel-19.

R2-2501898	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 6a: Support the coexistence of LTM event triggered reporting and mTRP operation at serving cells.

Proposal 6b: When LTM event-triggered reporting and mTRP operation coexist in the serving cells, the network configures which of the indicated TCI states of the serving cell is to be used for LTM event-triggered measurements evaluation.

[Samsung, Apple, Xiaomi]: Agree with the proposal. [Samsung]: Would like to remove “at serving cell” in the proposal. [Apple]: Note it was discussed for MIMO and no consensus was made in RAN1. [ZTE, Vivo, Huawei, Nokia, OPPO, NTTDCM. Ericsson, Rakuten]: Prefer supporting mTRP for event-triggered MR in Rel-19. [Apple]: As WI rapporteur, propose to complete all essential functions first, then we can further consider if mTRP can be easily introduced if time allows. [Nokia]: Understand there are two main questions to support mTRP. Q1) How to define the serving beam? Q2) What should be reported in MR MAC CE? Consider they are not so complicated. [Ericsson]: Although we want to support mTRP, we don’t need any discussion for optimization. It may be achieved w/o any specification impact (or w/ little change). [Apple]: As compromise, would like to suggest to see if it can be supported easily once MAC running CR becomes mature (assuming in August meeting). [Session chair]: Would like to suggest proponent companies propose one simple option based on offline discussion among the companies.

Will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

- In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to).

- In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam.

- RAN2 will consider only the following options:

- Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting.

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

- For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

R2-2501836	Discussion on measurement event evaluation and report	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501879	Remaining issues of L1 event triggered measurement reporting	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501888	Open issues for event triggered  L1 measurement reporting	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501898	L1 event triggered measurement reporting	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501930	Remaining issues on event triggered L1 MR	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502019	L1 event-triggered measurement reporting for LTM	Qualcomm Incorporated	discussion

R2-2502032	Discussions on measurement related enhancements for LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502094	Discussion on event-triggered meas report	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502133	Discussion on L1 event triggered measurement reporting	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502160	Discussion on measurement event evaluation and reporting	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502236	Discussion on L1 event triggered measurement reporting	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502311	Discussion on L1 event triggered  measurement	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502321	Details of MAC CE for event-triggered L1 measurement report	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502341	Discussion on L1 event triggered measurement reporting	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502360	Event based L1 measurement report	Lenovo	discussion	Rel-19

R2-2502382	Discussion on issues for supporting L1 event triggered measurement reporting	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502424	Discussion on L1 event triggered measurement reporting	Transsion Holdings	discussion	Rel-19

R2-2502433	Discussion on L1 event triggered measurement reporting	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502445	Discussion on event-based L1 measurements	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2502552	Further Details on L1 Measurement Reporting Enhancements for Rel-19 LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500332

R2-2502602	Discussion on L1 event triggered measurement report	Ofinno, LLC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502606	LTM event-triggered measurement reporting 	Fraunhofer HHI, Fraunhofer IIS	discussion

R2-2502721	Discussion on L1 event triggered measurement reporting	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502799	Remaining Issues of L1 Event Triggered Measurement Report	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502823	Discussion on MR MAC CE for LTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core	R2-2500417

R2-2502831	Discussion on L1 event triggered measurement reporting for LTM	KDDI Corporation	discussion	Rel-19

R2-2502950	L1 event triggered measurement report 	LG Electronics Inc.	discussion	NR_Mob_Ph4-Core

### 8.6.4	Conditional intra-CU LTM

Essential remaining open issues, including details of early TAT handling, stage-3 identified open issues if needed, etc.

1. TAT handling for candidate cells (except the target cell that becomes the new serving cell)

R2-2502018	Conditional intra-CU LTM	Qualcomm Incorporated	discussion

Proposal 5: (modified) Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell:

•	Maintain the TA value.

•	Keep the early TA timer running.

Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

2. TAT handling for the target cell that becomes the new serving cell

R2-2501899	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2a: CLTM TAT for target cell is stopped upon CLTM cell switch execution to the target cell. Then UE starts the timeAlignmentTimer associated with PTAG as in Rel-18.

R2-2502945	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: Upon C-LTM cell switch execution, the UE stops the C-LTM timer and starts the PTAG using the remaining time from the C-LTM timer (Option 2.3).

R2-2502800	Remaining Issues of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, RAN2 clarify the running TAT to act as the timeAlignmentTimer associated with PTAG.

R2-2501931	Further discussion on Conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1: When UE perform CLTM cell switch to the target cell, the timeAlignmentTimer associated with PTAG is started. The remaining time of the CLTM TAT timer for target cell is used for the pTAG TAT if RACH-less CLTM is performed. Otherwise (RACH-based), the TAT of pTAG start from a pre-configured TAT value as legacy.

Proposal 2: When the UE performs CLTM RACH-less cell switch to the target cell, the CLTM TAT timer of that cell is maintained until it expires. When the timer expires, the UE does not perform any special behavior.

[Xiaomi]: Support CATT proposal. [LG]: Support Huawei proposal. CATT proposal is not aligned with legacy principle. To CATT proposal, TAT is started with old TA value that was received in early UL sync procedure. [ASUSTek, OPPO, NEC]: Support Huawei proposal. [CATT]: In Rel-18 LTM, when the UE receives CSC MAC CE, the UE starts TAT according to target configuration. [Samsung]: Agree with LG. [Session chair]: Agree with LG’s comment. We can decide between Huawei and Samsung proposals. [Ericsson, Apple]: Support Samsung proposal. [MediaTek]: Ask if PTAG timer does not run at all then? [Samsung]: Clarifies that if the UE receives TA from the new source, PTAG timer should run. [Ericsson]: It would be up to UE implementation. [ZTE]: Understand partially Samsung and Huawei’s proposals are correct, we can combine each part.

C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

3. Selection of target cell if multiple cells met C-LTM execution condition, based on L3 measurement

R2-2502067	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: For L3 based CLTM, the CLTM execution is triggered when at least one candidate cell fulfills the CLTM condition. When multiple candidate cells satisfy the L3 CLTM condition, it is up to UE implementation to select a candidate cell and perform CLTM.

R2-2502342	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 2: In case more than one CLTM candidate cells fulfill the condition, the CLTM candidate cell that UE has valid TA is prioritized.

Proposal 3: In case more than one CLTM candidate cells fulfill the condition and UE has valid TA (or UE does not have valid TA for any of them), the CLTM candidate cell which associated with L3 execution condition is prioritized.

Proposal 4: In case more than one CLTM candidate cells fulfill the condition with the same type (L1 event or L3 event), and UE has valid TA (or UE does not have valid TA for any of them), it is up to UE to decide which LTM candidate cell will be selected for CLTM execution.

4. Selection of the target beam in L3-based RACH-less C-LTM

R2-2502438	Discussion on conditional LTM	Spreadtrum, UNISOC	discussion	Rel-19

Proposal 2: For L3 based C-LTM, beam selection for RACH-less LTM with CG is based on the configured RSRP threshold, e.g. similar to cg-RRC-RSRP-ThresholdSSB.

Proposal 3: If multiple candidate beams satisfy the RSRP threshold, it is up to UE implementation to select a beam.

R2-2501931	Further discussion on Conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 7:  For L3 based C-LTM, it is up to UE implementation to select the target beam.

5.1 Co-existence with MIMO 2 TA

R2-2502342	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5: RAN2 confirm CLTM with MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

Proposal 6: For CLTM with MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

[Samsung]: Is TAG id one bit index or full id? [ZTE]: 1bit. [Apple]: In addition to P5 and P6, we need to define when RACH-less C-LTM is performed and when RACH-based CLTM is performed based on the selected beam. [CATT]: Do not see real complication to support 2TA. There will be mapping between TCI-state and TAG id. If the beam meets C-LTM condition and it does not belong to TAG id where the UE has the corresponding TA, the UE performs RACH-based C-LTM. [Apple]: Not to prefer to make decision now. Want to see co-existence later.

RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

R2-2502312	Discussion on conditional Intra-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 10: Do not support the coexistence of CLTM and MIMO 2TA.

5.2 Co-existence with Other mobility schemes

R2-2501889	Discussion on conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 8: CLTM and L3 HO coexistence is supported.

Proposal 9: CLTM and DAPS coexistence is not supported.

Proposal 10: Network is allowed to trigger LTM cell switch execution for a candidate cell which is associated with CLTM execution condition.

R2-2502945	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 10: The UE can be simultaneously configured with Rel-18 LTM candidate configurations and with C-LTM candidate configurations.

Proposal 11: Upon reception of an LTM cell switch command MAC CE, the UE performs LTM cell switch to the indicated candidate, regardless whether it is configured for Rel-18 LTM or for conditional LTM.

R2-2502878	On conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

Proposal 10: Follow similar principle as Rel. 18 LTM, CLTM and CHO can be configured independently for different candidate cells.

Proposal 11: RAN2 to discuss if co-existence of CHO and CLTM for the same candidate target cell is supported.

R2-2502067	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 1c: The coexistence of LTM and CHO is not supported in Rel-19 (which revises the associated agreement in RAN2 #127). And the coexistence of CLTM and CHO is not supported in Rel-19.

5.3 Co-existence with (e)RedCap and CovEnh

R2-2501996	Remaining issues of CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

Proposal 5. Support coexistence between Rel-19 CLTM and (e)RedCap.

Proposal 6. Support coexistence between Rel-19 CLTM and CovEnh.

Agreements on C-LTM

- Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

- C-LTM TAT for target cell is not stopped upon C-LTM cell switch execution to the target cell if it is running, and starts the PTAG using the remaining time from the C-LTM timer.

- RAN2 confirm CLTM with Rel-18 MIMO 2TA is supported, introducing TAG ID field in LTM candidate TAC MAC CE.

- For CLTM with Rel-18 MIMO 2TA, two TA timer length (i.e. ltm-TimeAlignmentTimer) can be configured via RRC.

R2-2501826	Discussion on Conditional Intra CU LTM	Lekha Wireless Solutions	discussion	Rel-19

R2-2501837	Discussion on conditional LTM	HONOR	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501889	Discussion on conditional LTM	OPPO	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501899	Discussion on Conditional Intra-CU LTM	CATT	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501931	Further discussion on Conditional LTM	MediaTek Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2501996	Remaining issues of CLTM	LG Electronics Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502018	Conditional intra-CU LTM	Qualcomm Incorporated	discussion

R2-2502033	Discussion on conditional Intra-CU LTM	Fujitsu	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502067	Discussion on conditional LTM	Xiaomi	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502118	Conditional LTM.	Interdigital, Inc.	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502119	Remaining issues of Conditional LTM	Rakuten Mobile, Inc	discussion	Rel-19

R2-2502161	Discussion on conditional LTM	vivo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502237	Discussion on conditional intra-CU LTM	China Telecom	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502287	Remaining open issues for CLTM	NEC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502289	Discussion on Conditional intra-CU LTM remaining issues	Baicells Technologies Co. Ltd	discussion

R2-2502312	Discussion on conditional Intra-CU LTM	Apple	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502342	Discussion on conditional intra-CU LTM	ZTE Corporation, Sanechips	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502383	Discussion on issues for supporting conditional LTM	Sharp	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502396	CLTM Scenarios and remaining points	Lenovo	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502399	Discussion on coexistence of conditional LTM and LTM	ITRI	discussion	NR_Mob_Ph4-Core

R2-2502425	Discussion on Early TA handling	Transsion Holdings	discussion	Rel-19

R2-2502438	Discussion on conditional LTM	Spreadtrum, UNISOC	discussion	Rel-19

R2-2502446	Discussion on Conditional LTM	Ericsson	discussion	NR_Mob_Ph4-Core

R2-2502603	Discussion on open issues for conditoinal LTM	Ofinno, LLC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502728	Discussion on Conditional LTM	CMCC	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502748	Remaining issues on conditional LTM	ETRI	discussion	Rel-19

R2-2502756	Discussion on Conditional intra-CU LTM	ITL	discussion	Rel-19

R2-2502800	Remaining Issues of Conditional Intra-CU LTM	Samsung	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502824	Discussion on early TAT handling for CLTM	ASUSTeK	discussion	Rel-19	NR_Mob_Ph4-Core

R2-2502846	Considerations on CLTM TAT handling 	Kyocera 	discussion	Rel-19

R2-2502878	On conditional LTM	Nokia	discussion	Rel-19	NR_Mob_Ph4

R2-2502945	Intra-CU conditional LTM	Huawei, HiSilicon	discussion	Rel-19	NR_Mob_Ph4-Core