3GPP TSG-RAN WG2 Meeting #129bis	R2-2503009

# Wuhan, China, 7 - 11 April 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5332 | rev | 2 | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of PUCCH resources for L1 LTM reports at TAT expiry |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 5.3.12, upon an indication from lower layers to release PUCCH, the UE releases PUCCH-CSI-Resources configured in CSI-ReportConfig but does not release the PUCCH-CSI-Resources in LTM-CSI-ReportConfig, which are used for LTM L1 measurements. This clause is executed when, in TS 38.321, the TAT of the SpCell or of an SCell expires i.e., when uplink timing alignment is no more maintained. There is no statement to stop performing the measurements and the reports, so the UE may continue transmissing LTM L1 measurements on PUCCH. If this happens while the uplink timing is not correct, these transmissions will not be succesfully received by the network and may interfere with uplink transmission of other UEs, causing failures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Upon an indication from lower layers to release PUCCH resources, the PUCCH resources for reports configure with LTM-CSI-ReportConfig are also released. Impact Analysis Impacted 5G architecture options: NR, NR-DC Impacted functionality: LTM Inter-operability: If the network is implemented according to the CR and UE is not, the network may receive L1 LTM reports on PUCCH with wrong timing, so the network fails to decode them and may fail to decode uplink transmissions from other UEs. Then, after RA and TA command establishes the timing alignment again, the UE may receive L1 LTM reports on unexpected PUCCH resources. If the UE is implemented according to the CR and the network is not, when uplink timing alignment is resumed following RA and TA command, the network may not configure PUCCH resources for LTM L1 measurement reports, so that the UE does not report LTM L1 measurements and LTM does not work. The CR is mandatory to implement for UEs and networks supporting L1 measurement for LTM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may perform PUCCH transmissions with wrong timing, so that they are not decoded by the network and cause interference with uplink transmissions from other UEs, resulting in unpredictable failures. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.12	UE actions upon PUCCH/SRS release request

Upon receiving a PUCCH release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release PUCCH-CSI-Resources configured in CSI-ReportConfig;

1>	release PUCCH-CSI-Resources configured in LTM-CSI-ReportConfig;

1>	release SchedulingRequestResourceConfig instances configured in PUCCH-Config.

Upon receiving an SRS release request from lower layers, for all bandwidth parts of an indicated serving cell the UE shall:

1>	release SRS-Resource instances configured in SRS-Config;

1>	release SRS-PosResourceSetLinkedForAggBW, if configured;

1>	release ue-TxTEG-RequestUL-TDOA-Config, if configured.

Upon receiving a positioning SRS configuration for RRC_INACTIVE release request from lower layers, the UE shall:

1>	release the configured srs-PosRRC-Inactive;

1>	release SRS-PosResourceSetLinkedForAggBW, if configured.