3GPP TSG-RAN WG2 Meeting #129bis	R2-2503031

Wuhan, China, 7th - 11th April 2025

Agenda item:		8.4.2

Source:				CATT

Title:				[AT129bis][201][LPWUS] Proposals for UE ID based sub-grouping (CATT)

Document for:	Discussion and Decision

- Introduction

This document is to collect the comments for the following offline discussion on remaining details for the formula of UE ID based sub-grouping and potential issue for emergency PDU session:

- [AT129bis][201][LPWUS] Proposals for UE ID based sub-grouping (CATT)

Scope: Discuss remaining details for the formula of UE ID based sub-grouping, and also cover potential issue for emergency PDU session

Intended outcome: Summary with proposals in R2-2503031.

Deadline: Before Thursday CB

- Discussion

  - UE_ID related issues

For UE-ID based subgrouping for LP-WUS, the following formula was agreed in RAN2#129 meeting.

- Agreements: For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively.

In the 38.304 Running CR for LP-WUS, there is an FFS for UE-ID in UE-ID based LP-WUS subgrouping as follows:

UE_ID: 5G-S-TMSI mod X, X is FFS

The size of 5G-S-TMSI is 48bit. The UE-ID value range for UE-ID based LP-WUS subgrouping depends on the maximum number of POs, the maximum number of UE-ID based PEI sub-groups and the maximum number of UE-ID based LP-WUS subgroups. And we already agreed to support LP-WUS with eDRX. According to company contributions, the possible options for UE_ID include:

- Option 1: 5G-S-TMSI mod X, where X is depended on if eDRX is applied and/or the number of subgroupNumForUEID for PEI is applied.

- Option 1-1 (32 is used):

If eDRX is not applied, and the number of subgroupsNumForUEID for PEI is applied, X=262144 (256*4*8*32);

If eDRX is not applied, and the number of subgroupsNumForUEID for PEI is not applied, X=32768 (256*4*32);

If eDRX is applied, and the number of subgroupsNumForUEID for PEI is applied, X=1048576(1024*4*8*32);

If eDRX is applied, and the number of subgroupsNumForUEID for PEI is not applied, X=131072(1024*4*32).

- Option 1-2 (31 is used):

If eDRX is not applied, and the number of subgroupsNumForUEID for PEI is applied, X=253952 (256*4*8*31);

If eDRX is not applied, and the number of subgroupsNumForUEID for PEI is not applied, X=31744 (256*4*31);

If eDRX is applied, and the number of subgroupsNumForUEID for PEI is applied, X=1015808 (1024*4*8*31);

If eDRX is applied, and the number of subgroupsNumForUEID for PEI is not applied, X=126976 (1024*4*31).

- Option 2: 5G-S-TMSI mod X, where X is depended on if eDRX is applied without considering  if the number of subgroupNumForUEID for PEI is applied or not.

If eDRX is not applied, X=262144 (256*4*8*32);

If eDRX is applied, X= 1048576 (1024*4*8*32).

- Option 3: 5G-S-TMSI mod X, where X is 1015808, i.e., the largest UE ID range in all LP_WUS cases can be used for all LP-WUS monitoring cases.

- Option 4: 18 bits of the Hashed UE Identity Index Value IE is used, since hashed 5G-S-TMSI can achieve a more “random” UE_ID.

Companies are invited to provide their preference on following questions.

Q1: Whether hashed 5G-S-TMSI or 5G-S-TMSI is used to calculate UE_ID?

[Ericsson, DCM]: Hashed 5G-S-TMSI is more random than 5G-S-TMSI. Provide better power saving gain.

[Huawei, OPPO, Lenovo, Xiaomi]: Support 5G-S-TMSI, which is the used in PEI. Not sure how random for Hased ID.

[NEC, Lenovo, Ericsson]: postpone to the next meeting, due to RAN3 progress. And provide more analysis.

[Ericsson]: Fine to use more bits of the UE_ID for LP-WUS subgrouping (instead of using more bits of the hashed UE_ID).

Q2: If hashed 5G-S-TMSI instead of 5G-S-TMSI is used, how many bits (most significant) are needed for the Hashed UE Identity Index Value IE for UE-ID based LP-WUS subgrouping?

[Rapp]: No need to be discussed.

Q3: When determining the UE_ID, whether DRX and eDRX cases are needed to be distinguished?

Q4: When determining the UE_ID, whether the cases that PEI is applied or not are needed to be distinguished?

Q5: When determining the UE_ID, regarding to the maximum number of UE-ID based LP-WUS subgroups, which value should be used, i.e., 31 or 32?

Q6: Do companies agree to send LS to RAN3 (CC SA2/CT1) to ask their concern on the agreed formula of UE_ID based subgrouping for LP-WUS?

Conclusion: Companies agree to use extend the UE_ID. We can have the following proposals and further discuss during the CB session on the details on UE_ID determination.

Proposal 1: Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X, X is FFS.

Proposal 2: When determining the value of X, regarding to the maximum number of UE-ID based LP-WUS subgroups, suggest RAN2 to discuss which value should be used, i.e., 31 or 32?

Proposal 3: When determining the value of X, suggest RAN2 to discuss the following Options:

- Option 1: Only distinguish DRX and eDRX cases, similar as PEI;

- Option 2: Besides DRX and eDRX cases, also distinguish PEI is applied or not;

- Option 3: No any distinction case.

Proposal 4: Send LS to RAN3 (CC SA2/SA3/CT1) to ask their concern on the agreed formula of UE_ID based subgrouping for LP-WUS.

  - Np

According to the above agreements, as following Option 1, of UE-ID based subgrouping formula for LP-WUS, for a PEI capable UE in a PEI supporting cell, Np (i.e. UE-ID based subgroup number for PEI) is taken in account, which aims to distribute UEs in the same PEI subgroup into different LP-WUS subgroup, so that the UE can use LP-WUS together with PEI to further reduce paging false alarm rate. Otherwise (i.e., PEI is not supported by the UE or by the cell), Np is 1. During online discussion, some companies propose to revise the previous agreement as following Option 2. Rapporteur thinks Option 2 has a problem that network doesn’t know whether UE monitor PEI or not, so that the formula of UE_ID based sub-grouping cannot be aligned between network and UE.

- Option 1: Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1.

- Option 2: Np is the number of subgroupNumForUEID for PEI, if configured and UE supports could use PEI in this cell; otherwise, Np is 1.

Companies are invited to provide their views.

[OPPO]: how UE know the last cell? In case of UE move to another cell, UE doesn’t use PEI.

[Xiaomi, Lenovo, Huawei, TEJAS, Ericsson, vivo]: What’s problem of current formula? See no problem/issue/issue. It’s up to UE whether use PEI.

Conclusion: Most companies think the current agreement is fine and prefer to keep. But there is one typo on the IE, i.e., subgroupNumForUEID, which should be subgroupsNumForUEID.

Proposal 5: Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS:

- Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1,

  - Determination on the type of LP-WUS subgrouping

At the beginning of LP-WUS WI discussion, it was agreed:

=> The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

But there is no clear agreement to confirm how the UE to determine the type of subgrouping for LP-WUS, CN assigned subgrouping for LP-WUS or UE_ID based subgrouping for LP-WUS.

For PEI, the following description is captured in TS 38.304.

7.3.0 General //skip UE's subgroup can be either assigned by CN as specified in clause 7.3.1 or formed based on UE_ID as specified in clause 7.3.2: - If subgroupsNumForUEID is absent in subgroupConfig, the subgroup ID based on CN assigned subgrouping as specified in clause 7.3.1, if available for the UE, is used in the cell. - If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and subgroupsNumForUEID has the same value as subgroupsNumPerPO, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.3.2 is used in the cell. - If both subgroupsNumPerPO and subgroupsNumForUEID are configured, and subgroupsNumForUEID < subgroupsNumPerPO: - The subgroup ID based on CN assigned subgrouping as specified in clause 7.3.1, if available for the UE, is used in the cell; - Otherwise, the subgroup ID based on UE_ID based subgrouping as specified in clause 7.3.2 is used in the cell. If a UE has no CN assigned subgroup ID or does not support CN assigned subgrouping, and there is no configuration for subgroupsNumForUEID, the UE monitors the associated PO according to clause 7.1. //skip 7.3.2 UE_ID based subgrouping //skip If the UE is not configured with a CN assigned subgroup ID, or if the UE configured with a CN assigned subgroup ID is in a cell supporting only UE_ID based subgrouping, the subgroup ID of the UE is determined by the formula below: //skip

Some companies propose to confirm the principle determining CN assigned subgrouping or UE_ID based subgrouping for PEI for a UE is reused for LP-WUS subgrouping.

Proposal: Confirm the principle that determines CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping.

Companies views: All companies agree the above proposal.

Proposal 6: Confirm the principle that determines CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping.

  - Emergency PDU session related issue

How to handle UE-ID based subgrouping during an emergency PDU session is FFS in RAN2#128:

=>FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

The possible options proposed by companies are listed as following:

- Option 1: When the UE has an emergency PDU session LP-WUS is not used.

- Option 2: RAN2 does not specify specific LP-WUS handling when the UE has an emergency PDU session.

- Option 3: UE can determine whether to disable UE-ID based subgrouping based on the wake-up delay it supports when it has an emergency PDU session.

- Option 4: RAN2 waits for RAN1’s conclusion on the increased delay caused by LP-WUS to decide whether RAN2 need to specify that UE having an emergency PDU session will not monitor LP-WUS.

- Option 5: RAN2 is not suggested to optimize the case for UE-ID based subgrouping when the UE has an emergency PDU session unless requested by SA2/CT1.

Rapporteur thinks this issue may relate with the SA2 LS R2-2501752, which will be discussed in NR Rel-17. And RAN3 will also discuss this issue during this week. Suggest to wait and discuss further in the CB session if time is allow.

- Conclusion

Based on the offline discussion, the following are proposed:

Proposal 1: Use 5G-S-TMSI to determine the UE_ID in the formula of UE_ID based subgrouping for LP-WUS, i.e., UE_ID=5G-S-TMSI mod X, X is FFS.

Proposal 2: When determining the value of X, regarding to the maximum number of UE-ID based LP-WUS subgroups, suggest RAN2 to discuss which value should be used, i.e., 31 or 32?

Proposal 3: When determining the value of X, suggest RAN2 to discuss the following Options:

- Option 1: Only distinguish DRX and eDRX cases, similar as PEI;

- Option 2: Besides DRX and eDRX cases, also distinguish PEI is applied or not;

- Option 3: No any distinction case.

Proposal 4: Send LS to RAN3 (CC SA2/SA3/CT1) to ask their concern on the agreed formula of UE_ID based subgrouping for LP-WUS.

Proposal 5: Correct the typo as following for the previous agreed formula of UE_ID based subgrouping for LP-WUS:

- Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1.

Proposal 6: Confirm the principle that determines CN assigned subgrouping or UE_ID based subgrouping for PEI is reused for LP-WUS subgrouping.

- References

- R2-2501831	LP-WUS in RRC_IDLE/INACTIVE	HONOR	discussion	Rel-19	NR_LPWUS-Core

- R2-2501893	Discussion on LP-WUS in RRC_IDLE INACTIVE 	NEC	discussion	Rel-19	NR_LPWUS-Core

- R2-2501960	Procedure and configuration of LP-WUS for IDLE and INACTIVE mode	ZTE Corporation, Sanechips	discussion	Rel-19	NR_LPWUS-Core

- R2-2501997	Procedure and Configuration of LP-WUS in RRC Idle Inactive Mode	Samsung	discussion	Rel-19

- R2-2502005	Remaining issues on LP-WUS paging monitoring	Xiaomi Communications	discussion

- R2-2502014	IDLE/INACTIVE mode procedures for supporting LP-WUS	Tejas Network Limited	discussion	Rel-19

- R2-2502097	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Huawei, HiSilicon	discussion	Rel-19

- R2-2502143	LP-WUS in RRC_IDLE/INACTIVE	CATT	discussion	Rel-19	NR_LPWUS-Core

- R2-2502155	Discussion on LP-WUS WUR in RRC_IDLE INACTIVE	vivo	discussion	Rel-19	NR_LPWUS-Core

- R2-2502212	LP-WUS in IDLE and INACTIVE	Nokia	discussion	Rel-19	NR_LPWUS-Core

- R2-2502227	Remaining issues on LP-WUS in RRC IDLE or INACTIVE	LG Electronics Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2502308	Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE	Apple	discussion	Rel-19	NR_LPWUS-Core

- R2-2502324	Discussion on LP-WUS procedure and configuration	OPPO	discussion	Rel-19	NR_LPWUS-Core

- R2-2502447	LP-WUS operation in IDLE/Inactive state	Qualcomm Incorporated	discussion	NR_LPWUS-Core

- R2-2502486	RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode	Sony	discussion	Rel-19	NR_LPWUS-Core

- R2-2502597	Procedure and Configuration of LP-WUS in RRC IDLE/INACTIVE	Lenovo	discussion	Rel-19	NR_LPWUS-Core

- R2-2502659	Discussion on LP-WUS operation in RRC_IDLE/INACTIVE modes	InterDigital, Inc.	discussion	Rel-19	NR_LPWUS-Core

- R2-2502743	Discussion on LP-WUS in RRC_IDLE and RRC_INACTIVE	Sharp	discussion	Rel-19

- R2-2502901	Further Consideration on LP-WUS operation in IDLE/INACTIVE	CMCC	discussion	Rel-19	NR_LPWUS-Core

- R2-2502910	LP-WUS in idle and inactive	Ericsson	discussion

- R2-2502976	Discussion on the LP-WUS handling for Emergency call back	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core

- R2-2502977	Discussion on the LP-WUS capability issue within non-homogeneous deployment	NTT DOCOMO INC..	discussion	Rel-19	NR_LPWUS-Core