3GPP TSG-RAN WG2 #129bis	R2-2503053

Wuhan, China, 7 – 11 April 2025

Agenda Item:	7.3.1

Source:	Ericsson

Title:	Report of [AT129bis][302][TEI18] In-band operation for NB-IoT (Ericsson)

Document for:	Discussion

# 1	Introduction

RAN2 has received an LS from RAN4 about the introduction of inband operation for NB-IoT NTN and the potential RAN2 impact.

1. Overall Description:

RAN4 introduced the inband operation for NTN IoT in NR NTN in TEI18. RAN4 has assumed there is no RAN2 impact. RAN4 has made below agreements

- In Rel-18, UE is only required to support the same operation mode for anchor and non-anchor carriers

- For in-band operation, RRC signalling should indicate guardband-r13 in operationModeInfo IE in MasterInformationBlock-NB during UE conformance tests.

- in Rel-18 for in-band operation, UE is not expected to access a cell indicating inband-SamePCI or inband-DifferentPCI in operationMode IE in MasterInformationBlock-NB.

RAN4 would like to respectfully ask RAN2 to confirm above assumptions aligning with RAN2 specification.

2. Actions:

To RAN WG1/WG2 group.

ACTION: RAN4 would respectfully ask RAN2 to provide answer on the above.

# 2	Discussion

## 2.1	Stage 2 clarification

In the received LS, RAN4 states that the UE is only required to support the same operation mode for both anchor and non-anchor carrier. This means that mixed operation mode is not supported in NTN for both Release 17 (only standalone is supported in this release) and Release 18 (both standalone and in-band are supported). Companies are asked to comment whether they share the understanding that a clarification to indicate that mixed operation mode is not supported may be beneficial. An example is provided in section 3.1, please feel free to leave comments in the text proposal as well.

Is a clarification needed in Stage 2 to reflect that mixed operation mode is not supported in NTN for NB-IoT?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Mediatek | No | Mixed operation enables the standalone operation mode to be mixed with guardband and in-band. If we say mixed operation is not supported, it implies that combination of guardband and in-band is supported, which is not the intention of the RAN4 LS. Furthermore, the mixed operation mode is a terminology under the context of coexistence of NB-IoT and regular LTE. But we are talking about coexistence of NB-IoT NTN and NR NTN. A general description of the coexistence of NB-IoT in NTN and NR NTN is preferred, to capture the points in the RAN4 LS. Short introduction to this feature with the limiation on „same operation mode for anchor and non-anchor carriers“ would be better. |
| vivo | Comments | First of all, we think clarification is Stage-2 is needed anyway to reflect the Ran4 work. But we think the proposed text by the rapporteur is not aligned with RAN4 agreement. Specifically, in Rel-18, for inband operation, in-band mode is not supported, as per R4-2453030 (see below). The proposed text is not okay to us. We suggest the TP from R2-2501773. Anchor Carrier. In-band; Guard-band; Standalone. Non-Anchor Carrier; In-band; Yes (Rel-19); Yes (Rel-19); NO (Rel-18). Guard-band; NO; YES (Rel-18); NO (Rel-18). Standalone; NO; NO; YES (Rel-18). |
| Nokia | Yes | Stage 2 changes needed to indicate supported modes for anchor and non-anchor carriers for NR Inband operation. TP can be reviews via e-mail discussion |

Proposal 1	Discuss whether an update to Stage 2 is needed with either of the following changes:

Option 1) Add a new table reflecting what is supported in each release.

Option 2) Add a note clarifying the same operation mode is supported for anchor/non-anchor.

## 2.2	Introduction of a new UE capability

Provided the support of in-band has been introduced after the release is closed, a new UE capability without signalling may be needed to indicate that the UE supports the newly agreed in-band operation mode.

Introduce a new UE capability without signalling to indicate the support of in-band in NB-IoT NTN

| Company | Yes/No | Comments |
| --- | --- | --- |
| Mediatek | Yes |  |
| vivo | Wait for RAN4 input | Agree with the intention, but it is up to RAN4 to decide. |
| Nokia | No strong view | OK to introduce UE capability without signalling if it helps to clarify UE behaviour in NB-IoT NTN NR Inband cell |

Proposal 2	Introduce a new UE capability without signalling to indicate the support of in-band in NB-IoT NTN.

## 2.3	Further RAN2 impact

In addition to the previous two aspects (stage 2 and UE capability), there were no further comments on the potential impact in RAN2 specification. Still, companies are invited to comment whether any other specification changes may be expected

| Company | Yes/No | Comments |
| --- | --- | --- |
| MediaTek | yes | The current description of guardband in MIB is not correct anymore. It should be revised. guardband indicates a guard-band deployment.In NTN, it indicates in-band operation with NR NTN. See TS 36.102[113]. |
| vivo | No | Other stage-3 spec change is not necessary. |
| Nokia | No | Stage-3 changes not needed |

Proposal 3	From RAN2 perspective, the introduction of in-band support in NB-IoT NTN does not have any other Stage 3 impact.

In case the answer to the previous question is NO, companies are invited to provide feedback to the text proposal for the draft LS in section 3.2.

# 3	Text Proposals

## 3.1	TS 36.300

Table 5.5a-1: Anchor and non-anchor carrier deployment combinations in FDD

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Valid (Note 1) | Valid (Note 1) | Valid (Note 2, Note 3) |
|  | Guard-band | Valid (Note 1) | Valid (Note 1) | Valid (Note 2, Note 3) |
|  | Standalone | Valid (Note 2, Note 3) | Valid (Note 2, Note 3) | Valid (Note 2) |

NOTE 1:	Both carriers associated with the same LTE cell.

NOTE 2:	Total frequency span to not exceed 20MHz and both anchor and non-anchor carriers synchronised.

NOTE 3:	Not applicable to SC-PTM reception.

NOTE 4:	Mixed operation mode is not supported in NTN in this release of the specification.

## 3.2	Draft LS response

# 1. Overall Description:

RAN2 has discussed the RAN4 assumptions related to the introduction of in-band for NB-IoT NTN. RAN2 can confirm that the assumptions are aligned with RAN2 understanding and...

Option 1) RAN2 has decided to introduce a new UE capability without signalling to indicate the support of inband for NB-IoT NTN.

Option 2) RAN2 has decided to introduce a clarification in Stage 2 for the operation modes that can be supported in NTN.

Option 3) RAN2 sees there is no specification impact from the changes introduced by RAN4.

# 2. Actions:

# To RAN WG 4:

# RAN2 respectfully confirms RAN4 assumptions regarding the introduction of in-band operation mode for NB-IoT NTN.

# 4	Conclusion

Proposal 1	Discuss whether an update to Stage 2 is needed with either of the following changes:

Option 1) Add a new table reflecting what is supported in each release.

Table 5.5a-1: Anchor and non-anchor carrier deployment combinations in FDD

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band |  |  |  |
|  | Guard-band |  | Valid (Rel-18) |  |
|  | Standalone |  |  | Valid (Rel-18) |

Option 2) Add a note clarifying the same operation mode is supported for anchor/non-anchor.

Table 5.5a-1: Anchor and non-anchor carrier deployment combinations in FDD

|  | Anchor Carrier |  |  |  |
| --- | --- | --- | --- | --- |
|  |  | In-band | Guard-band | Standalone |
| Non-Anchor Carrier | In-band | Valid (Note 1) | Valid (Note 1) | Valid (Note 2, Note 3) |
|  | Guard-band | Valid (Note 1) | Valid (Note 1) | Valid (Note 2, Note 3) |
|  | Standalone | Valid (Note 2, Note 3) | Valid (Note 2, Note 3) | Valid (Note 2) |

NOTE 1:	Both carriers associated with the same LTE cell.

NOTE 2:	Total frequency span to not exceed 20MHz and both anchor and non-anchor carriers synchronised.

NOTE 3:	Not applicable to SC-PTM reception.

NOTE 4:	Same operation mode for anchor carrier and non-anchor carrier is supported for NTN in this release of the specification.

Proposal 2	Introduce a new UE capability without signalling to indicate the support of in-band in NB-IoT NTN.

Proposal 3	From RAN2 perspective, the introduction of in-band support in NB-IoT NTN does not have any other Stage 3 impact.