3GPP TSG-RAN WG2 Meeting #129bis	R2-2503066

# Wuhan, China, 7 - 11 Apr, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5335 | rev | 1 | Current version: | 18.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on ssb-TimeOffset |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-04-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN2#129bis, RAN2 agreed that for soft satellite switch “The starting boundaries of SFN0 of the source satellite and the target satellites are aligned at the uplink time synchronization reference point and ssb-TimeOffset can only be configured as 5ms or multiples of 5ms.” The understanding should be captured in the spec to guide the configuration. Impact analysis Impacted 5G architecture options: NR SA Impact analysis Impacted functionality: Soft satellite switch with re-synchronization Inter-operability: If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue and the Consequences if not approved remain. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point. The value is given in number of subframes and can only be configured as 5 subframes or multiples of 5 subframes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the network configures ssb-TimeOffset to a value which is not multiples of 5ms, the SSB pattern will contradict with what is defined in clause 4.1 in TS 38.213. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

–	SIB19

SIB19 contains satellite assistance information for NTN access.

SIB19 information element

-- ASN1START

-- TAG-SIB19-START

SIB19-r17 ::= SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

t-Service-r17                            INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

referenceLocation-r17                    ReferenceLocation-r17                           OPTIONAL,       -- Need R

distanceThresh-r17                       INTEGER(0..65525)                               OPTIONAL,       -- Need R

ntn-NeighCellConfigList-r17              NTN-NeighCellConfigList-r17                     OPTIONAL,       -- Need R

lateNonCriticalExtension                 OCTET STRING                                    OPTIONAL,

...,

[[

ntn-NeighCellConfigListExt-v1720         NTN-NeighCellConfigList-r17                     OPTIONAL        -- Need R

]],

[[

movingReferenceLocation-r18              ReferenceLocation-r17                           OPTIONAL,       -- Need R

ntn-CovEnh-r18                           NTN-CovEnh-r18                                  OPTIONAL,       -- Need R

satSwitchWithReSync-r18                  SatSwitchWithReSync-r18                         OPTIONAL        -- Need R

]]

}

NTN-NeighCellConfigList-r17 ::=          SEQUENCE (SIZE(1..maxCellNTN-r17))  OF NTN-NeighCellConfig-r17

NTN-NeighCellConfig-r17 ::=              SEQUENCE {

ntn-Config-r17                           NTN-Config-r17                                  OPTIONAL,       -- Need R

carrierFreq-r17                          ARFCN-ValueNR                                   OPTIONAL,       -- Need R

physCellId-r17                           PhysCellId                                      OPTIONAL        -- Need R

}

NTN-CovEnh-r18 ::=                       SEQUENCE {

numberOfMsg4HARQ-ACK-Repetitions-r18     BIT STRING (SIZE(4)),

rsrp-ThresholdMsg4HARQ-ACK-r18           RSRP-Range                                      OPTIONAL        -- Need R

}

SatSwitchWithReSync-r18 ::=              SEQUENCE {

ntn-Config-r18                           NTN-Config-r17,

t-ServiceStart-r18                       INTEGER (0..549755813887)                       OPTIONAL,       -- Need R

ssb-TimeOffset-r18                       INTEGER (0..159)                                OPTIONAL        -- Need R

}

-- TAG-SIB19-STOP

-- ASN1STOP

SIB19 field descriptions

distanceThresh Distance from the serving cell reference location and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. Each step represents 50m. This field is only present in an NTN cell.

movingReferenceLocation Reference location of the serving cell of an NTN Earth-moving cell at a time reference. It is used in the evaluation of eventD2 and condEventD2 criteria for the serving cell in RRC_CONNECTED, and location-based measurement initiation in RRC_IDLE and RRC_INACTIVE when distanceThresh is also configured, as defined in TS 38.304 [20]. The time reference of this field is indicated by epochTime in ntn-Config of the serving cell. This field is excluded when determining changes in system information, i.e., changes to movingReferenceLocation should neither result in system information change notifications nor in a modification of valueTag in SIB1. This field is only present in an NTN cell.

ntn-Config Provides parameters needed for the UE to access NR via NTN access such as Ephemeris data, common TA parameters, k_offset, validity duration for UL sync information and epoch time. In a TN cell, this field is only present in ntn-NeighCellConfigList and ntn-NeighCellConfigListExt.

ntn-NeighCellConfigList, ntn-NeighCellConfigListExt Provides a list of NTN neighbour cells including their ntn-Config, carrier frequency and PhysCellId. This set includes all elements of ntn-NeighCellConfigList and all elements of ntn-NeighCellConfigListExt. If ntn-Config is absent for an entry in ntn-NeighCellConfigListExt, the ntn-Config provided in the entry at the same position in ntn-NeighCellConfigList applies. Network provides ntn-Config for the first entry of ntn-NeighCellConfigList. If the ntn-Config is absent for any other entry in ntn-NeighCellConfigList, the ntn-Config provided in the previous entry in ntn-NeighCellConfigList applies.

referenceLocation Reference location of the serving cell provided via NTN (quasi)-Earth fixed cell and is used in location-based measurement initiation in RRC_IDLE and RRC_INACTIVE, as defined in TS 38.304 [20]. This field is only present in an NTN cell.

satSwitchWithReSync Provides parameters for the target satellite required to perform satellite switch with resynchronization. This field is only present in an NTN cell and its presence indicates that satellite switch without PCI change is supported in the cell.

t-Service Indicates the time information on when a cell provided via NTN is going to stop serving the area it is currently covering. This field applies for both service link switches in NTN quasi-Earth fixed cell and feeder link switches for both NTN quasi-Earth fixed and Earth-moving cell. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). The exact stop time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-Service is the uplink time synchronization reference point of the cell. This field is only present in an NTN cell.

NTN-CovEnh field descriptions

numberOfMsg4HARQ-ACK-Repetitions The number of repetition slots for PUCCH transmission with HARQ-ACK information for Msg4, see clause 9.2.6 in TS 38.213 [13]. The first/leftmost bit corresponds to the repetition factor 1, the second bit corresponds to the repetition factor 2, the third bit corresponds to the repetition factor 4, and the last/rightmost bit corresponds to the repetition factor 8. The repetition factor 1 shall be indicated together with at least one other repetition factor.

rsrp-ThresholdMsg4HARQ-ACK This threshold is used by the UE for determining the configuration of the MAC entity for PUCCH repetition for Msg4 HARQ-ACK, as specified in clause 6.2.1 in TS 38.321 [3].

SatSwitchWithReSync field descriptions

ssb-TimeOffset Indicates the time offset of the SSB from target satellite at its uplink time synchronization reference point with respect to the SSB from source satellite at its uplink time synchronization reference point. The starting boundaries of SFN0 of the source satellite and the target satellite are aligned at the uplink time synchronization reference point. The value is given in number of subframes and can only be configured as 5 subframes or multiples of 5 subframes.

t-ServiceStart Indicates the time information on when the target satellite is going to start serving the area currently covered by the serving satellite. The field indicates a time in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1st January 1900 (midnight between Sunday, December 31, 1899, and Monday, January 1, 1900). The exact start time is between the time indicated by the value of this field minus 1 and the time indicated by the value of this field. The reference point for t-ServiceStart is the uplink time synchronization reference point of the serving satellite.

End of Changes