3GPP TSG RAN WG2 Meeting #129bis            	                      R2-2503101

Wuhan, China, April 7th – 11th, 2025

Agenda Item:	8.10.1

Source:	Ericsson

Title:	[AT129bis][601][SONMDT] Address open issues for RRC

Document for:	Discussion and Decision

# Introduction

This document is to report the result of the following offline discussion. The discussion is to address the open issues in the running RRC SONMDT CR [1].

[AT129bis][601][SONMDT] Address open issues for RRC (Ericsson)

Scope:

    - Discuss offline how to proceed with the proposals in R2-2502974 and R2-2502927

Intended outcome:

    - Report in R2-2503101 with agreeable proposals which later can be used to progress running CRs.

Deadline:

    - Wednesday 17:00. The intention is to agree over email.

# Discussion

The following proposals from [2] are discussed during the offline session, to resolve the FFS and open issues in the running CR.

Observation 1: Two options exist for capturing PCell/PSCell measurements in RLF reports (or SCGFailureInformation):

Option 1: Reuse current stage-3 wording, including PCell/PSCell measurements in neighbour measurements.

Option 2: Include PCell/PSCell measurements in MeasResults IE and exclude them from neighbour measurements.

Proposal 2: For CHO with candidate SCGs, RAN2 to reuse the current stage-3 description, i.e.  include PCell and PSCell measurement results in neighbor measurements within RLF reports (or SCGFailureInformation).

Proposal 3: For CHO with candidate SCGs, RAN2 to clarify that the time duration between two fulfilled events shall only be included when both CHO and CPAC conditions are satisfied for failure cases.

Proposal 4: For CHO with candidate SCGs, RAN2 to explicitly define a new lastHO-Type for CHO with candidate SCGs.

Proposal 5: For CHO with candidate SCGs, logging of elapsed time between fulfilling the last triggering event and handover execution in SHR is not required when only one condition (CHO or CPAC) is fulfilled.

Proposal 6: For CHO with candidate SCGs, RAN2 to agree to include “firstFulfilledConfig”, “timeBetweenFulfillment”, and “timeBetweenLastFulfillmentAndEvent” in the choWithCandidateSCGInfoList IE within the SCGFailureInformation message.

Proposal 7: For CHO with candidate SCGs, RAN2 to discuss consolidating cell ID and measurement results in SHR (similar to the sourceCellInfo/targetCellInfo structure)

Proposal 8: RAN2 should discuss standardized wording for successful LTM execution in stage-3 specifications (e.g., "UE confirms network received first UL data" or "successful transmission of RRCReconfigurationComplete").

The following proposals from [2] are not discussed during the offline session and can be resubmitted to the next 3GPP meeting for the further discussion.

Proposal 9: RAN2 to send an LS to RAN3 to determine whether to implicitly introduce ltmCandidate in SHR, i.e. add a list indicating which LTM candidates the UE had in the SHR.

Proposal 10: For SDT, RAN2 to clarify that there are two scenarios that UE needs to log DL RSRP and UL data volume in the RA-report:

Case 1: UE initiated SDT but failed execution.

Case 2: UE failed to initiate RA-SDT and instead initiated RACH.

Proposal 11: For SDT, the maximum value of 172800 in timeSinceSdtExecution IE shall indicate 172800 seconds or longer.

We also discussed the following proposals based on the [3]

# for LTM

- Follow CHO like mechanism (i.e., a single rlf-Report for the RLF/HOF, including LTM recovery cell ID) for a consecutive LTM cell switch failure, i.e., an RLF/HOF at source or target cell followed by an HOF during LTM recovery.

RAN2 to adopt one of the following options to fetch the SHR possibly generated at later stage (i.e., after the RRCReconfigurationComplete message):
1. Leave it to network implementation, e.g., network blindly fetches the possible SHR later;
2. Adopt an enhanced way to allow UE to proactively provide successHO-InfoAvailable to network at later stage.

Adding beam information associated with ltm-RecoveryCellId.

Apply how to record L1 measurements in the running CR as in Section 6.

# for CHO with candidate SCG(s)

Apply how to capture the evaluation to the CHO with candidate SCG(s) in running CR as in Section 6.

To avoid duplication in case of two reports being generated CHO with candidate SCGs, any redundancy (e.g., measurements) are recorded in the reports for PCell (i.e., RLF Report, SHR).

Reuse CHO as the lastHO-Type for CHO with candidate SCG(s).

# Conclusion

The following conclusion on the FFS and open issues are made. It has been agreed to endorse them over email and no more online discussion is needed.

For agreement over email (no more online discussion (e.g., comeback session) is needed)

For CHO with candidate SCGs, RAN2 to reuse the current stage-3 description, i.e.  include candidate PCell and candidate PSCell measurements results in neighbor measurements within RLF reports (or SCGFailureInformation).

For CHO with candidate SCGs, RAN2 to clarify that the time duration between two fulfilled events (i.e., timeBetweenFulfillmen) shall only be included when both CHO and CPAC conditions are satisfied for failure cases.

For CHO with candidate SCGs, RAN2 explicitly define a new lastHO-Type for CHO with candidate SCGs.

For CHO with candidate SCGs, logging of elapsed time between fulfilling the last triggering event and handover execution in SHR is not required when only one condition (CHO or CPAC) is fulfilled.

For CHO with candidate SCGs, RAN2 to agree to include “firstFulfilledConfig”, “timeBetweenFulfillment”, and “timeBetweenLastFulfillmentAndEvent” in the choWithCandidateSCGInfoList IE within the SCGFailureInformation message. Keep the current ASN.1 structure and no change are needed.

Network is informed in the next RRC complete message about the SHR availability in case the UE performs RACH-less LTM cell switch. No need to change the current formulation of determining the SHR in the current running CR.

Follow CHO like mechanism (i.e., a single rlf-Report for the RLF/HOF, including LTM recovery cell ID) for a consecutive LTM cell switch failure, i.e., an RLF/HOF at source or target cell followed by an HOF during LTM recovery. Relevant FFS can be removed from the running CR. No change is needed in the relevant text of running CR.

FFS whether it is needed to avoid duplication of information in case of two reports being generated CHO with candidate SCGs, any redundancy (e.g., measurements) are recorded in the reports for PCell (i.e., in SHR, SPR).

# Reference

[1] R2-2501354 [Post129][601][SONMDT] Running NR RRC CR (Ericsson)

[2] R2-2502974	Discussion on Rel-19 SONMDT open issues	Xiaomi	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core

[3] R2-2502927	Remaining issues for MRO	ZTE Corporation, Sanechips	discussion	Rel-19	NR_ENDC_SON_MDT_Ph4-Core