3GPP TSG RAN WG2#130	R2-2503307

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG-CT WG4 Meeting #128	C4-251474

# Wuhan, P.R. China; 07th – 11th April 2025

# Title:	Reply LS on segmented UE capabilities

# Response to:	LS (R2-2501363) on segmented UE capabilities from RAN2

# Release:	Release 16

# Work Item:	RACS-RAN-Core, TEI17

Source:	CT4

To:	RAN2

Cc:

Contact Person:

Name:	Frank Yong Yang

Tel. Number:

E-mail Address:	frank.yong.yang@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

CT4 thanks RAN2 for their LS on segmented UE capabilities.

CT4 discussed the subject and would like to provide the following feedback:

- Over N26 interface GTPv2 protocol is used for communication between MME and AMF. GTPv2 is UDP based protocol, so a GTPv2 message cannot be larger more than 65535 bytes which is limited by the UDP payload length, i.e. a GTPv2 message cannot transfer any information element larger than 65535 bytes. In addition, transferring such a large message assumes to use IP fragmentation, which should be avoided whenever possible as specified by GTPv2;

- The UE Radio Access Capability may be transferred in a GTPv2 message e.g. during a handover procedure, however, it is transported by a container which is transparent to the MME, e.g. Source to Target Transparent container, where the content of such container is determined by the source RAN or target RAN which is under RAN2/RAN3 remit;

- SA2 has clearly specified in clause 5.11.2 of 3GPP TS 23.401, for UE Radio Capability Handling, that the source RAN shall ensure that the size of the UE Radio Capability information does not exceed the limits of interfaces involved in the handover:

"Hence, the source RAN shall ensure that the size of the UE Radio Capability information does not cause the size of the "source to target transparent container" to exceed the limits that can be handled by interfaces involved in the handover (e.g. Iu interface (TS 25.413 [22]) and, following SRVCC, E interface (TS 29.002 [86]))."

2. Actions:

To RAN2 group.

ACTION: 	CT4 kindly asks RAN2 group to take CT4 feedback into account.

3. Date of Next CT4 Meetings:

CT4 Meeting calendar can be found at:

https://www.3gpp.org/dynareport?code=Meetings-C4.htm