3GPP TSG RAN WG2#130	R2-2503330

St Julian’s, Malta, 19th - 23rd May 2025

3GPP TSG-RAN WG4 Meeting #114bis	R4-2505144

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title:	LS on introduction of new UE capabilities for FR2 MPR improvement

Response to:

Release:	Rel-19

Work Item:	NR_ENDC_RF_Ph4-Core

Source:	RAN4

To:	RAN2

Cc:

Contact Person:	Bozhi Li	bozhi.li@samsung.com

Sumant Iyer	sumanti@QTI.QUALCOMM.COM

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

The MPR of legacy UE is based on the cumulative aggregated channel BW (CABW*) i.e., MPR depends not only on the UL CA configuration but also on the DL CA configuration. For the Rel-19 NR_ENDC_RF_Ph4 work item, RAN4 has agreed to introduce two new UE capabilities for MPR enhancement, for FR2 intra-band contiguous CA. One capability is to indicate that the UE supports reduced MPR by removing dependence on DL CA configuration, the other is to indicate that UE supports MPR based on activation status of its configured CCs. The two new UE capabilities are optional and applicable from Rel-19. The applicable MPR for different UEs can be summarized in table below

|  | [fr2-MPR-ImprovementDownlinkIndependent-r19] supported? | [fr2-MPR-ImprovementActivationDependent-r19] supported? | MPR dependency |
| --- | --- | --- | --- |
| Legacy UE | No | No | Downlink+Uplink configured CCs （CABW） |
| Enhanced R19 UE | Yes | No | Uplink configured CCs |
| Enhanced R19 UE | No | Yes | Downlink+Uplink activated CCs (within CABW) |
| Enhanced R19 UE | Yes | Yes | Uplink activated CCs |

(*) The cumulative aggregated channel bandwidth is defined as the frequency band from the lowest edge of the lowest CC to the upper edge of the highest CC of all UL and DL configured CCs inside the UL/DL common spectrum in which the UE supports the configuration of uplink or downlink CCs.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully requests RAN2 to introduce the UE capabilities for FR2 intra-band contiguous CA MPR improvement by taking above into account.

3. Date of Next TSG WG RAN4 Meeting:

TSG RAN WG4 Meeting #115	19 – 23 May 2025	Malta

TSG RAN WG4 Meeting #116	25 – 29 May 2025	India