3GPP TSG-RAN WG2 Meeting #130	R2-2503344

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.2.3

Source:		Futurewei

Title:		Open issues on A-IoT random access

Document for:		Discussion and Decision

# Introduction

In RAN2#129, the following was agreed with respect to A-IoT random access [1]:

Agreements on msg1

- In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

- RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Agreements on new R2D (trigger) message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Agreements on msg 2

- A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

- Same Msg2 format is used for initial transmission and retransmission of Msg2.

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

In this contribution, we discuss some open issues of A-IoT random access.

# Discussions

## Content of the new R2D Trigger message

Message Type:

It is unclear at this point whether RAN1 can reach an agreement on using L1 signaling for the new R2D Trigger message at its last meeting for Rel-19 A-IoT WI. RAN2 can assume that it is a MAC message for now, and therefore, assume the Message Type field is included in the new R2D Trigger message.

Indications of X by Y Msg1 resources:

We think the configuration of X by Y numbers of access occasions for TDM/FDM Msg1s should remain stable within the same service session, which may include multiple paging cycles led by paging messages with the same Transaction ID, or at least within a paging cycle led by a paging message, no matter if the Transaction ID in it has changed or not. In this way, the X and Y values are indicated only in the paging message sent for a CBRA procedure, not in the R2D Trigger messages to reduce the signaling.

Proposal 1. RAN2 assume the Message Type field is included in the new R2D Trigger message unless RAN1 concludes to use L1 signaling.

Proposal 2. The X and Y values are indicated only in the paging message sent for a CBRA procedure, not in the R2D Trigger message.

## Whether R2D trigger message is needed in CFRA (Issue 2-2)

One main purpose of R2D Trigger message is to allow devices to count down its counter for selecting random access resource. Since there is no random-access resource assignment in CFRA, R2D trigger message should not be sent in CFRA; otherwise, it will cause devices, which are still counting, to miscount.

Proposal 3. R2D trigger message is not sent in CFRA procedure.

## R2D trigger message byte alignment (Issue 2-3)

Given the main purpose of R2D Trigger message, we do not think the MAC PDU for this particular message has to be byte-aligned. Furthermore, because the PHY signal needs to be OFDM symbol-aligned, and for OOK-4 waveform, the number of information bits that can be carried per OFDM symbol is a function of M value, where M is the M-chip per OFDM symbol. Therefore, padding may be needed to ensure that the PHY signal is OFDM symbol-aligned. It is unclear whether RAN1 will reach an agreement on doing this padding, when required, at PHY layer, or it will be left to RAN2. We suggest that RAN2 follow up with RAN1’s conclusion on how to provide padding to ensure OFDM symbol-alignment of PHY signals with various M values for OOK-4 waveform. If padding for OFDM symbol-alignment is also left for MAC layer to take care of, it is possible that MAC padding cannot ensure that the MAC PDU is byte-aligned and the corresponding PHY signal is OFDM symbol-aligned at the same time. In that case, RAN2 may need to reconsider whether MAC PDU byte-alignment can always be achieved.

Proposal 4. RAN2 follow up with RAN1’s conclusion on how to provide padding to ensure OFDM symbol-alignment of PHY signals with various M values for OOK-4 waveform.

Observation. If padding for OFDM symbol-alignment is also left for MAC layer to take care of, it is possible that MAC padding cannot ensure that the MAC PDU is byte-aligned and the corresponding PHY signal is OFDM symbol-aligned at the same time.

Proposal 5. If RAN1 leaves the padding for OFDM symbol-alignment to the MAC layer, RAN2 reconsider whether MAC PDU byte-alignment can always be achieved.

## How to address a device in NACK Feedback message

To define an explicit R2D NACK Feedback message, a question was raised about which ID between the RN16 submitted by a device and the AS ID assigned to the device in Msg2 should be included in the NACK Feedback message to identify the device as the intended recipient.

When RAN2 agreed on using Msg2 to assign AS ID for CBRA case in meeting#129-bis, we had cautioned the group that the implication of that agreement was that both the RN16 and the assigned AS ID of the device need to be included not only in a retransmitted Msg2 (sent for triggering a retransmission of Msg3 from the device), but also in a NACK Feedback message sent to cause the device to re-access. This is because when the reader fails to receive the expected Msg3 from the device, it could be due to a failure in the device’s receiving Msg2 (in which case, the device would hold onto its RN16) or due to a failure in the reader’s receiving Msg3 (in which case, the device would have replaced its RN16 with the value of the assigned AS ID in its memory). The reader cannot tell which way it is. Therefore, the reader needs to include both the RN16 value submitted by the device in Msg1 and the AS ID assigned to the device in Msg2 in the NACK Feedback message.

Proposal 6. If the reader has assigned an AS ID to a device in Msg2 and the reader fails to receive the Msg3 from the device, the reader can send a NACK Feedback message to the device to cause the device to re-access, by including the RN16 value and the assigned AS ID value of the device in separate fields (e.g., RN16 field and ASID field) in the NACK Feedback message.

Proposal 7. A device receiving its Msg2 correctly and with an AS ID assigned to it shall use its stored ID to compare with the value in the ASID field in the NACK Feedback message received after sending its Msg3.

Proposal 8. A device not receiving its Msg2 promptly shall use its stored ID to compare with the value in the RN16 field in the NACK Feedback message received after not receiving its Msg2, e.g., after an estimated end of the otherwise would-be Msg3 transmission.

Since the AS ID assignment is only optional for a device, from MAC PDU format point of view, the RN16 field should be a mandatory field in the NACK Feedback message, while the ASID field should be an optional field in the NACK Feedback message and is present only if the reader has assigned an AS ID to the device and the assignment is still valid.

Proposal 9. The RN16 field is a mandatory field in the NACK Feedback message, while the ASID field is an optional field in the NACK Feedback message and is present only if the reader has assigned an AS ID to the device and the assignment is still valid.

# Conclusions

Observation. If padding for OFDM symbol-alignment is also left for MAC layer to take care of, it is possible that MAC padding cannot ensure that the MAC PDU is byte-aligned and the corresponding PHY signal is OFDM symbol-aligned at the same time.

Therefore, we propose the following:

Proposal 1. RAN2 assume the Message Type field is included in the new R2D Trigger message unless RAN1 concludes to use L1 signaling.

Proposal 2. The X and Y values are indicated only in the paging message sent for a CBRA procedure, not in the R2D Trigger message.

Proposal 3. R2D trigger message is not sent in CFRA procedure.

Proposal 4. RAN2 follow up with RAN1’s conclusion on how to provide padding to ensure OFDM symbol-alignment of PHY signals with various M values for OOK-4 waveform.

Proposal 5. If RAN1 leaves the padding for OFDM symbol-alignment to the MAC layer, RAN2 reconsider whether MAC PDU byte-alignment can always be achieved.

Proposal 6. If the reader has assigned an AS ID to a device in Msg2 and the reader fails to receive the Msg3 from the device, the reader can send a NACK Feedback message to the device to cause the device to re-access, by including the RN16 value and the assigned AS ID value in both separate fields (e.g., RN16 field and ASID field) in the NACK Feedback message.

Proposal 7. A device receiving its Msg2 correctly and with an AS ID assigned to it shall use its stored ID to compare with the value in the ASID field in the NACK Feedback message received after sending its Msg3.

Proposal 8. A device not receiving its Msg2 promptly shall use its stored ID to compare with the value in the RN16 field in the NACK Feedback message received after not receiving its Msg2, e.g., after an estimated end of the otherwise would-be Msg3 transmission.

Proposal 9. The RN16 field is a mandatory field in the NACK Feedback message, while the ASID field is an optional field in the NACK Feedback message and is present only if the reader has assigned an AS ID to the device and the assignment is still valid.

# References

R2_129b_ChairNotes_25-04-11_16-45 final.