3GPP TSG-RAN WG2 Meeting #130	R2-2503345

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.2.4

Source:		Futurewei

Title:		Open issues on A-IoT data transmission and other aspects

Document for:		Discussion and Decision

# Introduction

In RAN2#129, the following was agreed with respect to A-IoT paging [1]:

Agreements on AS ID

1	AS ID is applied for Inventory + command case;

2	AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed.

3	For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits.

4	Do not specify the reader behaviour on how exactly the ASID is generated.

5	The device releases the AS ID upon power off (no stage 3 specification impact);

6	The device only keeps one AS ID at a time.

7	For CFRA, command message is used for AS ID assignment

8	For CBRA, Msg 2 is used for AS ID assignment

9	The device releases the AS ID at least:

- upon receiving Paging with new transaction id for that device, i.e. different session/service

- when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA)

- FFS other cases for release ASID to avoid keeping it indefinitely.

Agreements on segmentation

- For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again.  FFS whether offset zero is always included.

- FFS whether the reader always includes the command for retransmission of segments.

- 1-bit indication is sufficient to indicate whether more D2R data will be sent

- For inventory response, RAN2 assumes that segmentation is not applied.  RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size.  Notify SA2 about this assumption.

Agreements on MAC PDU format

- The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte.  The actual TBS value depends on RAN1.   FFS for R2D trigger message

- RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1.

- The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message

- The MAC padding is supported at least for D2R from RAN2 perspective.   The device includes padding bits if there is no more data and there is still space available in the TBS.

- In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

In this contribution, we discuss some open issues of A-IoT data transmission and other general aspects.

# Discussions

## Byte-alignment for D2R segmentation

RAN2 has the agreement that the MAC PDU should be byte-aligned, except FFS for R2D Trigger message. RAN2 also assumes that the upper layer data SDU is byte-aligned. However, the upper layer data SDU refers to the NAS PDU, which is still unsegmented. Therefore, a question remains whether D2R segmentation must be done byte-aligned. We understand that the earlier discussion on an indication of “offset bits” for segmentation retransmission, as highlighted below, is merely an example [2].

- Agreements on segmentation

- To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size.   If not possible, FFS if the 1 bit is sufficient.

- Segment retransmission is supported.

- For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start).  FFS This implies that unsegmented packet can also be retransmitted.   FFS if this applies to msg3

- R2D segmentation is not supported for R19 A-IoT.

A more important factor to be considered here is that if there is no guarantee that the control part in the MAC PDU carrying a D2R Upper Layer Data Transfer message is always byte-aligned, it appears more efficient to segment the upper layer data SDU in bits, not in bytes, to avoid using MAC padding bits for all non-last segments and thereby improving the MAC PDU fill-efficiency in the D2R direction. However, the price to pay is that the offset indication field in the R2D message triggering the segment retransmission requires three more bits due to the finer granularity. Because the reader is less concerned with its energy consumption, we think it is more important to reduce the number of bits that the device needs to transmit than to reduce the number of bits that the reader needs to transmit, if one has to be traded off for the other.

Proposal 1. When D2R segmentation occurs, the D2R upper layer data SDU is segmented in the unit of bit. And for segment retransmission, the offset is indicated in the unit of bit.

Proposal 2. When D2R segmentation occurs, MAC padding bits may be used only for the last segment.

## Command for non-first segment (Issue 3-1)

Regarding the question whether the upper layer command is included in the R2D message scheduling for non-first segment, we think that the device’s earlier transmission of the first D2R segment can serve as an acknowledgement that the upper layer command has been received correctly. Therefore, the upper layer command does not need to be  included in the R2D message scheduling for non-first segment. However, we think there is no need to introduce another R2D message for the purpose of scheduling for the non-first segment. The same R2D Upper Layer Data Transfer message should be used, except that there is no Data SDU field in the message in this case. The absence of the Data SDU field can be indicated by the presence bit or by value of 0 in an SDU Length field. If the device had previously sent a D2R Upper Layer Data Transfer message with the More Data Indication set to True, receiving the R2D Upper Layer Data Transfer message with no Data SDU field serves as both an acknowledgement of receiving the previous segment and a prompt for the next segment. If the device had previously sent a D2R Upper Layer Data Transfer message with the More Data Indication set to False, the reader should not send an R2D Upper Layer Data Transfer message with no Data SDU field in response. However, if the reader has another command to send to the device, the reader can send another R2D Upper Layer Data Transfer message with the Data SDU field carrying the next command, which also implicitly acknowledges the reception of the previous D2R transmission, as RAN2 had agreed before.

Observation: the device’s earlier transmission of the first D2R segment can serve as an acknowledgement that the upper layer command has been received correctly.

Proposal 3. The upper layer command does not need to be included in the R2D message scheduling for non-first segment.

Proposal 4. RAN2 consider using a presence bit or a Data SDU Length field set to value 0 to indicate the absence of the Data SDU field and hence the absence of the upper layer command.

Proposal 5. If the device had previously sent a D2R Upper Layer Data Transfer message with the More Data Indication set to True, receiving the R2D Upper Layer Data Transfer message with no Data SDU field serves as both an acknowledgement of receiving the previous segment and a prompt for the next segment.

Proposal 6. If the device had previously sent a D2R Upper Layer Data Transfer message with the More Data Indication set to False, the reader should not send an R2D Upper Layer Data Transfer message with no Data SDU field in response. However, if the reader has another command to send to the device, the reader can send another R2D Upper Layer Data Transfer message with the Data SDU field carrying the next command, which also implicitly acknowledges the reception of the previous D2R transmission, as RAN2 had agreed before.

## Offset for first segment (Issue 3-2)

Regarding the question whether offset is included in the R2D message scheduling for the first segment and unsegmented message, we think, as a uniformed design, all R2D Upper Layer Data Transfer messages scheduling for the first segment or unsegmented message, whether for initial transmission or retransmission, should not include an Offset field, but should include the Data SDU field to carry the command (again if it is for retransmission), as RAN2 has agreed. The device receiving the command will always respond by transmitting its upper layer response from the beginning, no matter whether the upper layer response is to be segmented or not and no matter whether it is the initial transmission or a retransmission, which is the intended behavior.

Proposal 7. As a uniformed design, all R2D Upper Layer Data Transfer messages scheduling for the first segment or unsegmented message, whether for initial transmission or retransmission, should not include an Offset field, but should include the Data SDU field to carry the command, as RAN2 has agreed.

Proposal 8. The device receiving the command will always respond by transmitting its upper layer response from the beginning, no matter whether the upper layer response is to be segmented or not and no matter whether it is the initial transmission or a retransmission, which is the intended behavior.

# Conclusions

Observation: the device’s earlier transmission of the first D2R segment can serve as an acknowledgement that the upper layer command has been received correctly.

Therefore, we propose the following:

Proposal 1. When D2R segmentation occurs, the D2R upper layer data SDU is segmented in the unit of bit. And for segment retransmission, the offset is indicated in the unit of bit.

Proposal 2. When D2R segmentation occurs, MAC padding bits may be used only for the last segment.

Proposal 3. The upper layer command does not need to be included in the R2D message scheduling for non-first segment.

Proposal 4. RAN2 consider using a presence bit or a Data SDU Length field set to value 0 to indicate the absence of the Data SDU field and hence the absence of the upper layer command.

Proposal 5. If the device had previously sent a D2R Upper Layer Data Transfer message with the More Data Indication set to True, receiving the R2D Upper Layer Data Transfer message with no Data SDU field serves as both an acknowledgement of receiving the previous segment and a prompt for the next segment.

Proposal 6. If the device had previously sent a D2R Upper Layer Data Transfer message with the More Data Indication set to False, the reader should not send an R2D Upper Layer Data Transfer message with no Data SDU field in response. However, if the reader has another command to send to the device, the reader can send another R2D Upper Layer Data Transfer message with the Data SDU field carrying the next command, which also implicitly acknowledges the reception of the previous D2R transmission, as RAN2 had agreed before.

Proposal 7. As a uniformed design, all R2D Upper Layer Data Transfer messages scheduling for the first segment or unsegmented message, whether for initial transmission or retransmission, should not include an Offset field, but should include the Data SDU field to carry the command, as RAN2 has agreed.

Proposal 8. The device receiving the command will always respond by transmitting its upper layer response from the beginning, no matter whether the upper layer response is to be segmented or not and no matter whether it is the initial transmission or a retransmission, which is the intended behavior.

# References

R2_129b_ChairNotes_25-04-11_16-45 final.

R2_129_ChairNotes_25-02-21_final.