3GPP TSG-RAN WG2 Meeting #130                                                                R2-2503352

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.8.2

Source:	vivo

Title:	Further Discussion on DL Coverage in NTN

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis [1], the issue of DL coverage enhancement was discussed and the following agreements were made.

Agreements: - From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits. - We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring - RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases). - We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking) - Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

However, it was unclear whether and what assistance information is needed for UE in the IDLE/INACTIVE state to select the SMTC or for NW to configure SMTC. In the contribution, we will continue to analyse the issue and give our views.

- Discussion

For RRC IDLE/RRC_INACTIVE state:

In RAN2#129bis, RAN2 concluded that network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode, and the assistance information can be the reference location and the threshold or the SSB index. According to the spec [2], UE can already support up to 4 SMTC configurations per frequency. Only in case the preconfigured number of SMTC(s) is more than 4, assistance information, i.e., the location, may be useful for the UE to select the corresponding SMTC. Otherwise, UE can just follow the legacy behaviour, i.e., monitor all of the configured SMTC(s) in the frequency carrier.

parallelSMTC-r17 Indicates whether the UE supports NTN SSB based RRM measurements on target cells belonging to 4 SMTC-s on a single frequency carrier. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports NTN SSB based RRM measurements on target cells belonging to 2 SMTC-s on a single frequency carrier.

By the way, for legacy, there is UE which only supports up to 2 SMTC(s) on a single frequency carrier, but it is totally up to UE implementation to decide which SMTC is used, e.g., no additional assistance information is provided to UE. From this perspective, UE implementation may also be enough to select the SMTC even if the number of SMTC(s) is more than 4, i.e., UE try each SMTC one by one and decides within which SMTC the measurement can be performed. How much benefit may be brought by the assistance information is unclear, especially compared with the specification impacts of introducing the assistance information.

smtc4list Measurement timing configuration list for NTN deployments, see clause 5.5.2.10. The offset of each SSB-MTC4 in smtc4list is based on the assumption that the gNB-UE propagation delay difference between the serving cell and neighbour cells equals to 0 ms, and UE can adjust the actual offset based on the actual propagation delay difference. For a UE that supports less SMTCs than what is included in this list, it is up to the UE to select which SMTCs to consider.

Moreover, even if RAN2 decide that the number of SMTC configurations can be extended to up to 6, the final decision is up to RAN4. Based on this, it was proposed that RAN2 postpone the discussion until RAN4 confirm to extend the number of SMTC configurations.

Proposal 1: If a new SMTC periodicity is not adopted and the number of SMTC configurations is not extended, UE apply legacy behaviour to select the SMTCs.

Proposal 2: If a new SMTC periodicity is adopted or the number of SMTC configurations is extended, RAN2 continue to discuss what assistance information can be provided.

For RRC CONNECTED state:

According to the spec [3], UE can report its coarse UE location to NW upon measurement reporting based on measurement configuration or provide its coarse UE location to NW via UEInformationResponse upon NW's request. As descripted in the Figure 1, along with the movement of UE, when UE moves towards the cell#3, at some time the event D1/D2 will be triggered, then UE can report its coarse UE location to the network correspondingly. Moreover, based on NW implementation, NW can also request the UE to report its coarse UE location via the UE information request/response procedure.

### 5.5.5 Measurement reporting #### 5.5.5.1 General ……Skip unrelated part…… 1> if the coarseLocationRequest is set to true in the corresponding reportConfig for this measId: 2> include coarseLocationInfo, if available; ……Skip unrelated part…… #### 5.7.10.3 Reception of the UEInformationRequest message ……Skip unrelated part…… 1> if the coarseLocationRequest is set to true: 2> include coarseLocationInfo, if available; ……Skip unrelated part……

Figure 1: Coarse UE location report

From our perspective, based on coarse location and deployment of the beam hopping, NW can decide how to configure each UE’s SMTC configurations correspondingly. So there is no need for any other enhancement in RRC CONNECTED to address the issue that the SSBs of surrounding cells may be transmitted at different times.

Observation 1: If the coarse UE location is needed, the legacy mechanism, i.e., UE measurement report or the UE information request/response procedure, can be used to acquire the location.

Observation 2: NW can decide the SMTC configuration for a specific UE based on the reported UE coarse location and knowledge of the beam hopping pattern.

Proposal 3: For UE in CONNECTED state, NW configures the corresponding SMTCs based on the reported coarse location and no further enhancement is needed.

- Conclusion

In this contribution, we have further discussed the DL coverage aspect. And we make the following observations and proposals:

Observation 1: If the coarse UE location is needed, the legacy mechanism, i.e., UE measurement report or the UE information request/response procedure, can be used to acquire the location.

Observation 2: NW can decide the SMTC configuration for a specific UE based on the reported UE coarse location and knowledge of the beam hopping pattern.

Proposal 1: If a new SMTC periodicity is not adopted and the number of SMTC configurations is not extended, UE apply legacy behaviour to select the SMTCs.

Proposal 2: If a new SMTC periodicity is adopted or the number of SMTC configurations is extended, RAN2 continue to discuss what assistance information can be provided.

Proposal 3: For UE in CONNECTED state, NW configures the corresponding SMTCs based on the reported coarse location and no further enhancement is needed.

- Reference

- Draft_RAN2_129bis_Meeting_Report_v1.

- TS38.300, NR; User Equipment (UE) radio access capabilities.

- TS38.331, NR; Radio Resource Control (RRC); Protocol specification.