3GPP TSG-RAN WG2 Meeting #130 	 R2-2503353

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.8.4

Source:	vivo

Title:	Remaining Issues on MBS Broadcast Provision in NTN

Document for:	Discussion and Decision

- Introduction

In the RAN2#129bis meeting, the following agreements were made for MBS broadcast within intended service area [1]:

1. RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts). 2. If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH. 3. If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies. 4. UE may prioritize the frequency(ies) for an interested service when UE can only receive the service on the frequency(ies) and the UE is in intended service area associated with the service provided in the frequency(ies). Otherwise, UE may de- prioritize the frequency. FFS whether and how the association between intended service area and frequencies is acquired.

With all the achieved agreements, the Rel-19 MBS broadcast function in NTN is almost ready. In this contribution, we would like to provide our consideration of some remaining issues.

- Discussion

  - 2.1	Signalling details

In the latest running RRC CR, the mapping between an MBS broadcast session (TMGI) and ISA ID(s) is achieved by extending the MBS-SessionInfoList. The signalling details are given below.

MBS-SessionInfoList-r17 ::=      SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfo-r17

MBS-SessionInfoListExt-r19 ::=   SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfoExt-r19

MBS-SessionInfo-r17 ::=          SEQUENCE {

mbs-SessionId-r17                TMGI-r17,

g-RNTI-r17                       RNTI-Value,

mrb-ListBroadcast-r17            MRB-ListBroadcast-r17,

mtch-SchedulingInfo-r17          DRX-ConfigPTM-Index-r17                      OPTIONAL, -- Need S

mtch-NeighbourCell-r17           BIT STRING (SIZE(maxNeighCellMBS-r17))       OPTIONAL, -- Need S

pdsch-ConfigIndex-r17            PDSCH-ConfigIndex-r17                        OPTIONAL, -- Need S

mtch-SSB-MappingWindowIndex-r17  MTCH-SSB-MappingWindowIndex-r17              OPTIONAL  -- Cond MTCH-Mapping

}

MBS-SessionInfoExt-r19 ::=       SEQUENCE {

mbs-SessionAreaList-r19          SEQUENCE (SIZE (1..maxNrofMBS-SessionPerArea-r19)) OF MBS-IntendedAreaID-r19 OPTIONAL, -- Need R

...

}

We would like to highlight that broadcast services delivered within a cell may have differing reception areas. For instance, some are intended for the entire cell, while others are restricted to the associated intended service areas. For services requiring reception across the full cell area, the network should configure the corresponding entry in the MBS-SessionInfoListExt as empty (i.e., not configuring explicit area IDs). From the UE point of view, upon association of an empty entry with a broadcast service in the MCCH, the UE should consider that the service is permissible for reception within the entire cell area. Concurrently, the legacy behavior remains applicable.

Proposal 1: If a broadcast session is intended for reception within the entire cell area, the network does not configure the intended area ID(s) associated with that service.

Proposal 2: RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is allowed for reception within the entire cell area, with legacy behavior applicable.

  - 2.2	Broadcast service continuity

Reselection priorities handling

In the existing MBS broadcast framework, for service continuity, the IDLE/INACTIVE UE may consider the frequency that providing the MBS broadcast service the UE interested in to be the highest priority or consider the frequency that not providing the MBS broadcast service to be the lowest priority, based on frequency mapping with FSAI in SIB21 or frequency indicated in USD.

With the introduction of ISA, to ensure the UE appropriately prioritizes the frequency for an interested service when UE can only receive the service on the frequency(ies) and the UE is in the intended service area associated with the service provided in the frequency, it is necessary to define a clear mapping between frequency, service, and intended area in SIB21. In the previous meeting, some companies have proposed additionally associating FSAI with ISA IDs. In our understanding, relying solely on a per-FSAI mapping to the ISA is insufficient.

Consider a common scenario where multiple services are announced with the same FSAI on the same frequency (e.g., Service#1 and Service#2 deployed on frequency f2 with the same FSAI but different ISAs). If the network associates both services’ ISAs with the same FSAI, a UE interested only in Service#2 might incorrectly prioritize f2 when entering the ISA of Service#1, leading to unintended frequency selection, as shown in the figure below.

Figure 1: Service continuity

Observation 1: Multiple broadcast sessions may be established with the same FSAI on the same frequency (e.g., the network deploys 2 sessions associated with the same service area on a given frequency).

Observation 2: The association between FSAI and ISA IDs fails to distinguish which service is associated with a specific intended service area, particularly when multiple services share the same FSAI on a given frequency.

To address this, it is proposed that SIB21 include a service area ID list explicitly associated with each TMGI. This allows the UE to resolve ambiguities in scenarios where multiple services share the same frequency and FSAI but have distinct ISAs, ensuring that frequency prioritization aligns with the UE’s actual service interest.

Proposal 3: SIB21 can include a service area ID list explicitly associated with TMGI.

With this, if UE is receiving or interested in receiving an MBS broadcast service associated with the intended service area and can only receive this MBS broadcast service by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session when its location is within the intended service area of the corresponding session after cell reselection. And once the UE goes outside of the intended service area, the UE should no longer prioritize the frequency that providing the MBS broadcast service.

Proposal 4: UE no longer prioritizes the frequency that providing the MBS broadcast service when the UE is outside the service area of that interested broadcast service.

An example of SIB21 signalling enhancment is given below,

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

...,

[[

mbs-FSAI-IntraFreq-SessionArea-r19       MBS-FSAI-List-SessionArea-r19   OPTIONAL,  -- Need R

mbs-FSAI-InterFreq-SessionAreaList-r19   MBS-FSAI-InterFreq-SessionAreaList-r19       OPTIONAL,  -- Need R

]]

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-List-SessionArea-r19 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-SessionArea-r19

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-SessionAreaList-r19 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-SessionArea-r19

MBS-FSAI-InterFreq-r17 ::= SEQUENCE {

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-FSAI-InterFreq-SessionArea-r19  ::= SEQUENCE {

MBS-FSAI-List-SessionArea-r19 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-SessionArea-r19

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

MBS-FSAI-SessionArea-r19 ::= SEQUENCE (SIZE (1..maxFSAI-SessionArea-r19)) OF SessionArea-r19

SessionArea-r19 ::= SEQUENCE {

mbs-SessionId-r19         TMGI-r17,     OPTIONAL,  -- Need R

mbs-SessionAreaList-r19          SEQUENCE (SIZE (1..maxNrofMBS-SessionPerArea-r19)) OF MBS-IntendedAreaID-r19

}

-- TAG-SIB21-STOP

-- ASN1STOP

Neighbour cell list

IDLE/INACTIVE UE may request unicast reception (thus triggering RRC setup/resume request procedure) of the broadcast service content before moving to a cell not providing the MBS broadcast service(s) via PTM mode, according to the neighbour cell list in MCCH.

For the unicast reception request for the broadcast service content, since it is up to UE implementation to do this without stage-3 spec normative text, we think we can leave it to pure UE implementation in the Rel-19 case. For example, when the UE is moving to a cell not providing the MBS broadcast service and will continue to stay within the intended service area, it may trigger an RRC setup/resume request for the unicast reception of the broadcast service content. No further enhancement is needed for the neighbour cell list solution.

Proposal 5: No enhancement is considered for the MCCH neighbour cell list mechanism in Rel-19.

MBS Interest Indication (MII) reporting

Last but not least, we would like to look at the MBS Interest Indication (MII) reporting mechanism. In TS 38.331, the following MBS Interest Indication reporting behaviour is specified for MBS broadcast,

5.9.4.2 Initiation An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell. If the UE does not have the CFR information and subcarrier spacing for MBS broadcast reception on a non-serving cell at the time it sends the MBS Interest Indication, the UE sends another MBS Interest Indication after it has acquired this information from the non-serving cell.

We can observe that the RRC_CONNECTED UE may initiate the MII reporting procedure upon entering or leaving the broadcast service area (i.e. cell/tracking area) based on the USD, as highlighted above. We think the same technical logic can be applicable to the Rel-19 case. That is the UE may report MII when entering or leaving the intended service area, as shown in Figure 1. With that, the gNB may provide the UE with an RRC reconfiguration, select a proper target cell, and deprioritize unicast/multicast scheduling, to allow the UE to receive the MBS services the UE is interested in.

Specifically, for CONNECTED UE, MII can be used for scheduling prioritization handling between broadcast and unicast/multicast (e.g. if the UE prioritizes broadcast service(s) over unicast/multicast, the network may release some DRBs/multicast MRBs to save more radio bearer capability for broadcast reception). Then the CONNECTED UE should report it to the network when it goes inside the intended service area so that the network can do the proper scheduling prioritization handling. Furthermore, if the UE goes outside the intended service area, the UE should also indicate the MII since the network has to know it does not need to consider scheduling prioritization handling between broadcast and unicast/multicast anymore for this UE.

Figure 2: MII reporting when moving inside/outside the intended service area

Based on the above, we propose,

Proposal 6: RRC_CONNECTED UE may initiate the MBS Interest Indication reporting procedure when UE is entering or leaving the intended service area.

3 Conclusion

In this contribution, we have discussed the remaining issues on MBS broadcast in Rel-19 NTN. And we make the following observations and proposals,

Singalling details:

Proposal 1: If a broadcast session is intended for reception within the entire cell area, the network does not configure the intended area ID(s) associated with that service.

Proposal 2: RAN2 confirms that if no intended area ID is explicitly indicated in MCCH for an MBS broadcast service the UE is interested in, the UE considers the service is allowed for reception within the entire cell area, with legacy behavior applicable.

Broadcast service continuity:

Observation 1: Multiple broadcast sessions may be established with the same FSAI on the same frequency (e.g., the network deploys 2 sessions associated with the same service area on a given frequency).

Observation 2: The association between FSAI and ISA IDs fails to distinguish which service is associated with a specific intended service area, particularly when multiple services share the same FSAI on a given frequency.

Proposal 3: SIB21 can include a service area ID list explicitly associated with TMGI.

Proposal 4: UE no longer prioritizes the frequency that providing the MBS broadcast service when the UE is outside the service area of that interested broadcast service.

Proposal 5: No enhancement is considered for the MCCH neighbour cell list mechanism in Rel-19.

Proposal 6: RRC_CONNECTED UE may initiate the MBS Interest Indication reporting procedure when UE is entering or leaving the intended service area.

4 Reference

[1] R2-2503302, RAN2#129bis Meeting Report, MCC.