3GPP TSG-RAN WG2 Meeting #130                                                                R2-2503357

St. Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.17

Source:	vivo

Title:	Further Discussion on IoT-NTN TDD mode

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis [1], the following open issues were made by RAN2 on IoT-NTN TDD mode. however, it is still FFS whether there is impact for the MAC timers in unit of ms.

Agreements: 1. Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe. 2. In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell. 3. Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes) 4. When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe 5. When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe 6. For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

Besides, RAN1 also discussed the issue in RAN1#120 [2], and the following conclusion was made.

Conclusion It is RAN1’s understanding that the issue of scheduling and transmitting SI messages in the presence of non-D subframes will be solved mainly (or completely) by RAN2 specifications. RAN1 may discuss potential impact on RAN1 specifications, if any.

In the contribution, we will continue to analyse the following issues and give our views.

- Impacts of invalid subframes within the PDCCH monitoring window;

- Impacts on MAC timers in units of subframes;

- Scheduling and transmission of the SI message.

- Discussion

## 2.1 Impacts of invalid subframes within the PDCCH monitoring window

Take the Msg2 reception as an example, UE begins to start monitoring the PDCCH reception addressed by RA-RNTI until it is received within the RAR window, or nothing is received upon expiration of the  RAR window. However, it is FFS whether UE shall keep monitoring the PDCCH even if it is within the UL duration or the guard time duration for the IoT-NTN TDD scenario. We agree that it is power consumption for UE to keep monitoring even in the non-DL duration, since there is definitely no response from the network. However, since the PDCCH monitoring is RAN1 issue, and RAN1 is now also discussing the issue of IoT-NTN TDD, it is proposed that RAN2 leave the issue to RAN1 to solve.

Proposal 1: It is RAN2’s understanding that the issue of UE behaviour within the invalid subframes within the PDCCH monitoring window will be solved mainly (or completely) by RAN1.

## 2.2 Impacts on MAC timers in units of subframes

Except for the timers in units of PP, there is also MAC times in unit of subframes, i.e., the HARQ RTT timer. And usch timers mainly depends on the valid DL/UL scheduling. The following is a description of how to set the corresponding value of the timers [3]. It can be seen that the duration of the timers depends on the valid scheduling time point, i.e., the start subframe of the next valid PDCCH occasion. Take the DL HARQ RTT timer under the scenario that the signle TB is scheduled by PDCCH or when multiple TBs are scheduled for the interleaved case when HARQ-ACK bundling is configured as an example, the length between the time point “k+3+N subframes plus RTToffset” and the time point “the potential 1st subframe of PDCCH occasion” already includes the duration of the UL duration and the guard time for the IoT-NTN TDD scenario.

For NB-IoT, when single TB is scheduled by PDCCH or when multiple TBs are scheduled for the interleaved case when HARQ-ACK bundling is configured the HARQ RTT Timer is set to k+3+N subframes plus RTToffset + deltaPDCCH, where k is the interval between the last subframe of the downlink transmission and the first subframe of the associated HARQ feedback transmission and N is the transmission duration in subframes of the associated HARQ feedback, and deltaPDCCH is the interval starting from the subframe following the last subframe of the associated HARQ feedback transmission plus 3 subframes plus RTToffset to the first subframe of the next PDCCH occasion. For NB-IoT, when multiple TBs are scheduled by PDCCH for the non-interleaved case or for the interleaved case when HARQ-ACK bundling is not configured, the HARQ RTT Timer is set to k+m*N+1 subframes plus RTToffset + deltaPDCCH where k is the interval between the last subframe of the downlink transmission and the first subframe of the first HARQ feedback transmission and N is the transmission duration in subframes of the associated HARQ feedback and m is the number of scheduled TBs as indicated in PDCCH whose associated HARQ process is with enabled HARQ feedback, and deltaPDCCH is the interval starting from the subframe following the last subframe of the last HARQ feedback transmission plus 1 subframe plus RTToffset to the first subframe of the next PDCCH occasion. ……Skip unrelated part…… For NB-IoT, when single TB is scheduled by PDCCH the UL HARQ RTT timer length is set to 4 subframes plus RTToffset + deltaPDCCH, where deltaPDCCH is the interval starting from the subframe following the last subframe of the PUSCH transmission plus 3 subframes plus RTToffset to the first subframe of the next PDCCH occasion. For NB-IoT, when multiple TBs are scheduled by PDCCH the UL HARQ RTT timer length is set to 2 subframes plus RTToffset + deltaPDCCH, where deltaPDCCH is the interval starting from the subframe following the last subframe of the PUSCH transmission plus 1 subframe plus RTToffset to the first subframe of the next PDCCH occasion.

Proposal 2: RAN2 to confirm that the legacy HARQ RTT timer is set based on scheduling, and no additional enhancement is needed for IoT-NTN TDD.

## 2.3 Scheduling and transmission of the SI message

As shown in Figure 1, due to the TDD pattern, the start position of the SI message or the duration of the SI message for a specific SI overlap with the guard time or the UL duration of the TDD pattern.

Figure 1: SI message resource conflictions with the FDD mode and TDD pattern

If the SI window is postponed to the next valid DL subframes of the TDD pattern, it may impact the next DL transmission in the next DL subframe. Correspondingly, it may lead to the timing relationship of the system becoming very complicated and even ambiguous. Moreover, there is no way to specify the UE behaviour from the spec perspective. Rather, one may argue that the SI window and the SI transmission can be dropped directly in case the SI window overlaps with the non-DL subframes, but this may impact the system performance due to the reduced resources for SI transmission. Further, as shown in Figure 1, the start position of the SI window is within the non-valid DL subframes of the TDD pattern, dropping means the whole SI window start position is ambiguous. Moreover, it is also very complicated from the UE perspective since UE may need to decide whether it is overlapped and how/whether to drop the resource.

Observation 1: The Postponement of a channel when it overlaps with non-U/D NB-IoT subframes may make the system timing relationship complicated and ambiguous.

Observation 2:  UE behaviour is unclear and cannot be specified from the spec perspective if the SI window is postponed to the next valid DL subframe.

Observation 3:  Dropping (full or partial) of SI windows when they overlap with non-valid DL NB-IoT subframes has impacts on system performance.

Observation 4:  Dropping (full or partial) of SI windows when they overlap with non-valid DL NB-IoT subframes may cause complicated UE behaviour.

Another solution is to introduce a new time resource allocation unit, i.e., the TDD frame and/or the hyper TDD frame. The  90ms length TDD frame is aligned with the IoT-NTN TDD pattern period, and the maximum of the TDD frame is X. As for the hyper TDD frame, the length is X TDD frame, and the maximum hyper TDD frame is Y. The following is an example of the TDD frame.

Figure 2: TDD frame/hyper TDD frame structure

Based on the newly introduced TDD frame and/or the hyper TDD frame, the network configures the resource periodicities and uses the new time unit to decide the SI window. More specifically, if we set the X=1024, and Y=1024, then the only change for the SI window calculation formula is to change the term “floor(x/10)” to “floor(x/90) within the formula of the SI window calculation if the SI window length configuration is ms” or “floor(x) within the formula of the SI window calculation if the SI window length configuration is TDD frame”.

Observation 5: New periodicities based on the TDD frame and/or the hyper TDD frame can save the issue of the SI window and all other resource overlap with the non-valid DL/UL subframe and have limited spec impacts on the resource calculation formula.

Proposal 3: Introduce new periodicities based on the time unit of the TDD frame and/or the hyper TDD frame to decide the SI window and SI transmission resource.

Proposal 4: Send LS to RAN1 to indicate RAN2 agreements on scheduling of the SI window and SI transmission resource.

- Conclusion

In this contribution, we have further discussed the IoT-NTN TDD mode. And we make the following observations and proposals:

Impacts of invalid subframes within the PDCCH monitoring window:

Proposal 1: It is RAN2’s understanding that the issue of UE behaviour within the invalid subframes within the PDCCH monitoring window will be solved mainly (or completely) by RAN1.

Impacts on MAC timers in units of subframes:

Proposal 2: RAN2 to confirm that the legacy HARQ RTT timer is set based on scheduling, and no additional enhancement is needed for IoT-NTN TDD.

Scheduling and transmission of the SI message:

Observation 1: The Postponement of a channel when it overlaps with non-U/D NB-IoT subframes may make the system timing relationship complicated and ambiguous.

Observation 2:  UE behaviour is unclear and cannot be specified from the spec perspective if the SI window is postponed to the next valid DL subframe.

Observation 3:  Dropping (full or partial) of SI windows when they overlap with non-valid DL NB-IoT subframes has impacts on system performance.

Observation 4:  Dropping (full or partial) of SI windows when they overlap with non-valid DL NB-IoT subframes may cause complicated UE behaviour.

Observation 5: New periodicities based on the TDD frame and/or the hyper TDD frame can save the issue of the SI window and all other resource overlap with the non-valid DL/UL subframe and have limited spec impacts on the resource calculation formula.

Proposal 3: Introduce new periodicities based on the time unit of the TDD frame and/or the hyper TDD frame to decide the SI window and SI transmission resource.

Proposal 4: Send LS to RAN1 to indicate RAN2 agreements on scheduling of the SI window and SI transmission resource.

- Reference

- Draft_RAN2_129bis_Meeting_Report_v1.

- Draft_RAN1_120_Meeting_Report_v1.

- TS36.321, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.