3GPP TSG-RAN WG2 Meeting #130	R2-2503368

St Julian’s, Malta, 19-23 May 2025

Agenda item:	8.7.6

Source:	Qualcomm Incorporated, MediaTek Inc.

Title:	Discussion on XR rate control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss the remaining issues for uplink rate control.

# Discussion

Maximum number of QoS flows that a UE supports for rate control

In 5G, each PDU session can include up to 64 QoS flows, and a UE can support up to 15 PDU sessions. Therefore, in theory, there can be up to 15x64=960 QoS flow in a UE.

This number obviously is way more than the typical number of QoS flows that need rate control. To simplify UE implementation, it is therefore desirable to define a maximum for this number.

The value of this maximum clearly depends on a UE’s capabilities. For example, a premium tier UE with more processing power and more memory can support more QoS flows than a low-tier UE (e.g. a RedCap UE). Therefore, it makes more sense to define this maximum as a UE capability instead of a constant in the spec for all types of UEs.

For most XR applications, it is safe to assume that the largest number of QoS flows subject to rate control is at most 64. This number can be used to set the range of the above UE capability.

Proposal 1.	The maximum number of QoS flows subject to rate control is a UE capability, whose range is 1~64.

[MAC-05] Whether the Rate Control MAC CE includes single or multiple QoS flows

One of the special characteristics of XR applications is that it generates multiple flows with different QoS requirements, e.g. video, audio, haptic, control, etc. When an application performs rate adaptation, it typically adjusts the rates of multiple flows simultaneously, especially during congestion when available bandwidth is low. Although it is possible to send multiple MAC CEs (one for each flow or LCH) at once in this case, it is more efficient and customary (like many other MAC CEs) to include all of them in a single MAC CE.

This enhancement is applicable to both rate indication MAC CE from gNB or rate query MAC CE from UE.

Proposal 2. 	[MAC-05] The Rate Control MAC CE (both rate indication and rate query) can include bit rates for multiple QoS flows.

[MAC-04] Which type of ID should be used to identify a QoS flow

It was left as an FFS how to identify a QoS flow in a Rate Control MAC CE, e.g. with DRB ID + QFI.

In our understanding, it is true that a QFI is unique only within a PDU session. Therefore, PDU session ID and QFI have to be used together to uniquely identify a QoS flow.

According to TS 24.007, PDU session ID is 8 bits long, and QFI is 6 bits long (a maximum of 64 QoS flows per PDU session). So the length of the target identifier is 14 bits.

If the Rate Control MAC CE can include multiple QoS flows, then 14 bits per rate indication seems a bit excessive from the perspective of signaling overhead. Especially if we assume that the typical number of QoS flows in a Rate Control MAC CE is expected to be no more than 64.

Therefore, in our view, it is more efficient to use RRC to configure a much shorter identifier for the purpose. More specifically, after core network provides the set of QoS flows subject to rate control (as agreed by SA2), RAN configures an identifier for each of these QoS flows for the UE, which then is used in the Rate Control MAC CE. This identifier is unique across all PDU sessions.

Proposal 3.	[MAC-04] Use an identifier configured by RRC, which is shorter than QFI, to identify QoS flows in the Rate Control MAC CE.

[MAC-06] Any additional fields that should be included in the Rate Control MAC CE

Based on the above analysis, we think the number of rate indications in a Rate Control MAC CE can vary. Hence it is necessary to include a length field in the MAC CE.

Proposal 4.	[MAC-06] The Rate Control MAC CE includes a length field.

[RRC-01] Granularity of rate query prohibit timer

In the legacy Recommended Bit Rate procedure, the rate query prohibit timer is configured per LCH. We think XR rate control may require finer granularity for this timer, because different QoS flows may differ in how often they adjust their rates. Audio vs video is a good example for that. Audio flows typically have low bit rates. Their rates do not need to be adjusted as often as video flows, as the latter’s large bandwidth are more sensitive to variations in available bandwidth. Therefore, if they are forced to share the same prohibit timer, audio flows can be penalized in their opportunities for rate adjustment.

Proposal 5.	[RRC-01] Rate query prohibit timer is configured per QoS flow.

[MAC-08] Handling of triggered UL rate queries

If it is agreed that the rate query prohibit timer is configured per QoS flow, then it is possible that multiple rate queries can be triggered and remain pending at the same time.

When an UL grant becomes available and it is large enough to accommodate all the pending rate queries in a Rate Control MAC CE, all pending rate queries should be multiplexed into the Rate Control MAC CE. To simplify the design and implementation, no truncation is supported, as rate queries are not super time critical. All pending rate queries are canceled after the Rate Control MAC CE is transmitted.

Proposal 6. 	[MAC-08] Multiple rate queries (for different QoS flows) can be pending at the same time.

Proposal 7.	[MAC-08] A pending rate query is canceled after a corresponding rate query is sent in a Rate Control MAC CE.

Proposal 8. 	[MAC-08] To keep the design simple, a Rate Control MAC CE is sent only if it can accommodate all pending rate queries. No truncation is supported.

[MAC-09] How UE should handle the Rate Control MAC CE in DC configuration

It has not been discussed yet whether, in DC configuration, bit rate recommendation sent in a Rate Control MAC CE is local to a specific cell group or “global” for the whole UE.

Before we analyze it, it is worth studying a very similar procedure, PSI based discard (congestion) de-/activation introduced in Rel-18. Congestion obviously is local to a specific cell group. However, RAN2 agreed that, in DC configuration, PSI discard de-/activation is “global”, i.e. a PSI discard de-/activation MAC CE received in ANY cell group de-/activates PSI-discard. UE does not need to do anything extra, because it is completely left to network to coordinate when/where to send PSI discard de-/activation MAC CEs.

The motivation of the Rate Control MAC CE is to adapt application’s bit rate during congestion. Therefore, the same design principle can be followed. More specifically, it is left to network to coordinate between MN and SN to determine the total bit rate available to the UE. On the UE side, when the UE receives a Rate Control MAC CE, it considers the indicated bit rate as the total bit rate across two cell groups, instead of the available bit rate in the specific cell group where the MAC CE is received. No additional handling is required for the UE.

Proposal 9. 	[MAC-09] UE considers the indicated bit rate in a Rate Control MAC CE received in any cell group as the total bit rate available to the application.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1.	The maximum number of QoS flows subject to rate control is a UE capability, whose range is 1~64.

Proposal 2. 	[MAC-05] The Rate Control MAC CE (both rate indication and rate query) can include bit rates for multiple QoS flows.

Proposal 3.	[MAC-04] Use an identifier configured by RRC, which is shorter than QFI, to identify QoS flows in the Rate Control MAC CE.

Proposal 4.	[MAC-06] The Rate Control MAC CE includes a length field.

Proposal 5.	[RRC-01] Rate query prohibit timer is configured per QoS flow.

Proposal 6. 	[MAC-08] Multiple rate queries (for different QoS flows) can be pending at the same time.

Proposal 7.	[MAC-08] A pending rate query is canceled after a corresponding rate query is sent in a Rate Control MAC CE.

Proposal 8. 	[MAC-08] To keep the design simple, a Rate Control MAC CE is sent only if it can accommodate all pending rate queries. No truncation is supported.

Proposal 9. 	[MAC-09] UE considers the indicated bit rate in a Rate Control MAC CE received in any cell group as the total bit rate available to the application.