3GPP TSG-RAN2 Meeting#130												R2-2503375

# Malta, Malta, May 19th - May 23rd, 2025

Agenda Item:	8.20.1

Source:	OPPO

Title:	Discussion on R4 LS on UE Architecture Type-4a/4b

Document for:	Discussion, Decision

- Introduction

This is to discuss the issue of UE type-4a/4b, as stated in R4 LS 2505161.

- Discussion

R4 asked R2 to define UE capability, as follows

UE capability:

Define one capability for NR CA and one capability for EN-DC

-	For EN-DC, Type 4a and Type 4b capability can be differentiated by legacy capability, such as MIMO-CapabilityDL-r10 on E-UTRA carrier

If UE reports capability for Type 4a/4b, Type 2 capability shall be deemed as supported by default, and UE still needs to report Type 2 capability if UE supports it in order to keep backward compatibility (i.e., for legacy BS)

Yet considering that the legacy type-2 capability is a per-BC capability, and MIMO layer capability is a per-FSPC capability, it seems sufficient to rely on legacy signaling to define RF and RRM requirements for the type-4a/b cases, where the only missing part is the support of the new BS signaling.

|  | For a BC | intraBandNR-CA-non-collocated-r18 | interBandMRDC-WithOverlapDL-Bands-r16 | <supportedMIMO-CapabilityDL-MRDC-r15 for CC1, maxNumberMIMO-LayersMulticastPDSCH-r17 for CC2> | <maxNumberMIMO-LayersMulticastPDSCH-r17 for CC1, maxNumberMIMO-LayersMulticastPDSCH-r17 for CC2> |
| --- | --- | --- | --- | --- | --- |
| NR-CA | Type-1 | absent |  |  | <1,1> <1,2> <1,4> <2,2> <2,4> <4,4> |
|  | Type-2 | present |  |  | <1,1> <1,2> <2,2> |
|  | Type-4b | present |  |  | <1,4> <2,4> <4,4> |
| EN-DC | Type-1 |  | absent | <1,1> <1,2> <1,4> <2,2> <2,4> <4,4> |  |
|  | Type-2 |  | present | <1,1> <1,2> <2,2> |  |
|  | Type-4a |  | present | <1,4> <2,4> |  |
|  | Type-4b |  | present | <4,4> |  |

- R2 confirm the new capability only needs to cover the support of new BS signaling part, while the associated intraBandNR-CA-non-collocated-r18, interBandMRDC-WithOverlapDL-Bands-r16, supportedMIMO-CapabilityDL-MRDC-r15, maxNumberMIMO-LayersMulticastPDSCH-r17 are defined as pre-requisite to reflect the RF/RRM performance.

The highlighted ones are those not included in R4 definition (R4-2217734), which is unclear since R4 clarified that For NR-CA, type-4a (MIMO layer = <2,4>) is not considered;

However, R2 capability always assume fallback applicability, i.e., if a BC reports to support 4 MIMO layer, 2 MIMO layer would be supported as well, i.e., type-4a and also MIMO layer combination of <1,4> should be supported as well.

In our understanding, there is no additional work from R4 to support these cases.

- R2 sends reply LS to R4, to clarify R2 capability signaling assume fallback applicability, and check whether MIMO layer combination of <1,4> and <2,4> can be supported for UE architecture type-4b, and whether MIMO layer combination of <1,4> can be supported for UE architecture type-4a.

For BS signaling, R4 asked that

BS signalling:

When maxMIMO-layers > 4, Type 1(collocated) requirements apply. No new BS signaling is needed

When maxMIMO-layers=2, UE follow Rel-18 specification. No new BS signaling is needed

When maxMIMO-layers=4, new BS signalling(s) is needed for NR-CA and EN-DC, and the details are up to RAN2

NOTE: maxMIMO-layers intends for the legacy BS signaling

In legacy spec, the BS signaling is limited to MIMO layer <= 2, i.e., applicable to MIMO layer combination of

<1,1>

<1,2>

<2,2>

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements when indicating support of interBandMRDC-WithOverlapDL-Bands-r16.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

Considering BS signaling is not limited to 1/2/4, but allows setting of 3/5/6/7/8 as well,

MaxMIMO-LayersDL-r16 ::=                INTEGER (1..8)

New BS signaling should not be limited to MIMO layer = 4, but should be applied to MIMO layer at least covering

<1,3>

<1,4>

<2,3>

<2,4>

<3,3>

<3,4>

<4,4>

In addition, considering the support of 6 MIMO layers, it should be applied to the following combinations as well

<1,5>

<2,5>

<3,5>

<1,6>

<2,6>

- R2 sends reply LS to R4, to check if any concern if the new BS signaling is applied for UE configured with maxMIMO-Layers with value larger than 2, and smaller than or equal to 4for at least one corresponding serving cell.

- Conclusion

Proposal 1	R2 confirm the new capability only needs to cover the support of new BS signaling part, while the associated intraBandNR-CA-non-collocated-r18, interBandMRDC-WithOverlapDL-Bands-r16, supportedMIMO-CapabilityDL-MRDC-r15, maxNumberMIMO-LayersMulticastPDSCH-r17 are defined as pre-requisite to reflect the RF/RRM performance.

Proposal 2	R2 sends reply LS to R4, to clarify R2 capability signaling assume fallback applicability, and check whether MIMO layer combination of <1,4> and <2,4> can be supported for UE architecture type-4b, and whether MIMO layer combination of <1,4> can be supported for UE architecture type-4a.

Proposal 3	R2 sends reply LS to R4, to check if any concern if the new BS signaling is applied for UE configured with maxMIMO-Layers with value larger than 2, and smaller than or equal to 4for at least one corresponding serving cell.