3GPP TSG-RAN WG2 Meeting #130	R2-2503386

St Julian’s, Malta, 19-23 May 2025

Agenda Item:	8.11.2

Source:	NEC

Title:	Remaining open issues on random access in SBFD

Document for:	Discussion, Decision

# Introduction

In RAN2#129bis meeting, the following agreements were reached and a good progress on random access in SBFD was achieved.

Agreements:

On RO type signaling for CFRA

-	For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig.

-	For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated.

When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type.

Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold.

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

This paper further discuss the remaining open issues for random access in SBFD.

# Discussion

## Msg.3 based early indication

On Msg.1 based early indication, RAN2#127bis meeting reached the following agreement.

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

The remaining issue is whether to support Msg.3 based early indication for SBFD-aware UE in Rel-19. This topic was discussed in RAN2#128 meeting however no consensus was reached.

The main motivation of supporting Msg.3 based early indication for SBFD-aware UE is to allow the network schedule subsequent transmissions/receptions in SBFD symbols before enquiring UE capability which especially brings benefits of latency reduction for initial attach.

Regarding the details of supporting Msg.3-based early indication, a reserved eLCID can be allocated to SBFD-aware UEs to indicate support for SBFD. This approach has almost no impact on legacy UEs and specification. Therefore, we proposed to support Msg.3 based early indication for SBFD-aware UE in Rel-19.

Proposal 1:	Support Msg.3 based early indication for SBFD-aware UE in Rel-19.

## RA-RNTI collision

RA-RNTI is used by the UE to receive Random Access Response (RAR) once PRACH is transmitted to the network. Different RACH occasions within a system frame are associated with different RA-RNTI values to ensure that RAR for a given PRACH transmission can be properly addressed to only the UE which has used that given RACH occasion.

If RA-RNTI values are same for 2 RACH occasions, then there is a chance of confusion where 2 UEs which have used different RACH occasions act on the same RAR message. To ensure this uniqueness, RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

When additional RACH resources for SBFD are configured to the UE (especially when FDMed with legacy RACH resources) then there is a chance that RA-RNTI confusion may occur as one legacy RACH resource and an additional RACH resource will get the same RA-RNTI value.

When additional RACH resources do not share the same time domain resource as a legacy RACH resource then the RA-RNTI confusion may not occur. Hence, one simple solution is to ensure that additional RACH resources are always TDMed with legacy RACH resources. In specification, this may be restricted by UE side behavior where UE may not expect to receive a configuration of additional RACH resource where the starting symbol of the RO of additional RACH resources is not the same as starting symbol of any RO of legacy RACH resource set.

In some cases for SBFD, it may be beneficial to perform FDM of additional RACH resources and legacy RACH resource to compress the PRACH occasion occurrences in time domain. To resolve RA-RNTI confusion for such case, there may be two options:

Option-1: RAR message itself indicates whether the associated PRACH was transmitted on legacy RACH resources or additional RACH resources;

Option-2: RA-RNTI calculation is updated to also consider whether the RACH resource belongs to legacy RACH resources or additional RACH resources:

For example,

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 x 2 x RACH_config

Here, RACH_config is 0 for the case off legacy RACH resource and has a non-zero value for additional RACH resource set.

With Option-1, it may involve a different message format of RAR to contain the information of RACH resource set corresponding to the PRACH transmission, which presents a more specification impact than Option-2. Hence we prefer Option-2.

Proposal 2:	RA-RNTI calculation is updated to also consider whether the RACH resource belongs to legacy RACH resources or additional RACH resources:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 x 2 x RACH_config

## LTM Consideration

For SBFD-aware UE, we think there is no reason to forbid the UE to select the additional RACH resources to perform LTM handover. And our understanding is that the SBFD-aware UE can also benefit the configuration of these additional RACH resources during LTM handover. With this understanding, it is proposed to support the LTM cell switch via Random access procedure in SBFD symbols for SBFD-aware UEs.

Proposal 3:	Support LTM cell switch via Random access procedure in SBFD symbols for SBFD-aware UEs.

Before the execution of LTM cell switch, the UE will receive the LTM Cell Switch Command MAC CE, it would be beneficial to indicate the exact RO type for CFRA triggered RACH-based LTM cell switch. One reserved bit in the LTM Cell Switch Command MAC CE may be used for this purpose.

Proposal 4:	For CFRA triggered RACH-based LTM cell switch, one reserved bit in the LTM Cell Switch Command MAC CE can be used to indicate the RO type.

## RO type switch

According to current MAC procedure, the Random Access procedure is performed as follows:

- Step 1) UL carrier selection between NUL and SUL.

- Step 2) a set of Random Access resources (i.e., RACH partition) associated with feature selection

- Step 3) RA type selection between 4-step RA and 2-step RA

- Step 4) initialization of variables specific to Random Access type

- Step 5) Random access Resource selection, i.e., SSB, preamble, and RO selection

In addition, when the random access attempt from 2-step RA switch to 4-step RA, as shown in the following, the UE fallback to perform the step 4) initialization procedure and step 5) random access resource selection procedure, i.e., the UE fallback to perform the procedure after the RA type selection.

5.1.3a MSGA transmission …… 3> if the Random Access procedure is not completed: 4> if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1: 5> set the RA_TYPE to 4-stepRA; 5> perform initialization of variables specific to Random Access type as specified in clause 5.1.1a; 5> if the Msg3 buffer is empty: 6> obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer; 5> flush HARQ buffer used for the transmission of MAC PDU in the MSGA buffer; 5> discard explicitly signalled contention-free 2-step RA type Random Access Resources, if any; 5> perform the Random Access Resource selection procedure as specified in clause 5.1.2. 4> else: 5> perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a). 5.1.4a MSGB reception and contention resolution for 2-step RA type …… 2> if the Random Access procedure is not completed: 3> if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1: 4> set the RA_TYPE to 4-stepRA; 4> perform initialization of variables specific to Random Access type as specified in clause 5.1.1a; 4> if the Msg3 buffer is empty: 5> obtain the MAC PDU to transmit from the MSGA buffer and store it in the Msg3 buffer; 4> flush HARQ buffer used for the transmission of MAC PDU in the MSGA buffer; 4> discard explicitly signalled contention-free 2-step RA type Random Access Resources, if any; 4> perform the Random Access Resource selection procedure as specified in clause 5.1.2. 3> else: 4> select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF; 4> if the criteria (as defined in clause 5.1.2a) to select contention-free Random Access Resources is met during the backoff time: 5> perform the Random Access Resource selection procedure for 2-step RA type Random Access (see clause 5.1.2a). 4> else: 5> perform the Random Access Resource selection procedure for 2-step RA type Random Access (see clause 5.1.2a) after the backoff time. 5.1.5 Contention Resolution … … 3> else (i.e. the RA_TYPE is set to 2-stepRA): 4> if msgA-TransMax is applied (see clause 5.1.1a) and PREAMBLE_TRANSMISSION_COUNTER = msgA-TransMax + 1: 5> set the RA_TYPE to 4-stepRA; 5> perform initialization of variables specific to Random Access type as specified in clause 5.1.1a; 5> flush HARQ buffer used for the transmission of MAC PDU in the MSGA buffer; 5> discard explicitly signalled contention-free 2-step RA type Random Access Resources, if any; 5> perform the Random Access Resource selection as specified in clause 5.1.2. 4> else: 5> select a random backoff time according to a uniform distribution between 0 and the PREAMBLE_BACKOFF; 5> if the criteria (as defined in clause 5.1.2a) to select contention-free Random Access Resources is met during the backoff time: 6> perform the Random Access Resource selection procedure for 2-step RA type as specified in clause 5.1.2a. 5> else: 6> perform the Random Access Resource selection for 2-step RA type procedure (see clause 5.1.2a) after the backoff time.

Considering the additional RO type is introduced for SBFD, and RAN2 working assumption that “For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources”. Therefore, the RA procedure for SBFD-aware UE is performed as follows:

-	Step 1) UL carrier selection between NUL and SUL.

-	Step 2) RO type selection between the additional RO and legacy RO.

-     Step 3) a set of Random Access resources (i.e., RACH partition) associated with feature selection

-	Step 4) RA type selection between 4-step RA and 2-step RA

-	Step 5) initialization of variables

-	Step 6) Random access Resource selection, i.e., SSB, preamble, and RO selection

Based on the above analysis, similar to RA type switch, if the RO type switch happens, the UE need to fallback to perform the procedure after the RO type selection, i.e., the UE perform the step 2) to step 6) after the RO type switch.

Proposal-5: When the RO type switch happens, the UE fallbacks to perform: a set of random access selection, RA type selection, initialization of variables and random access resource selection procedure.

RAN1 agreed that for RACH configuration Option 1 and Option 2, the UE increase the power ramping counter when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure.

RAN1#120bis Agreement For RACH configuration Option 1 and Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, support Alt-1: Increase the power ramping counter.

In addition, RAN2 agreed that RO type switch from legacy RO to additional RO in SBFD symbols is supported. Therefore, UE should also increase the power ramping counter when a SBFD-aware UE switches from legacy-ROs to additional ROs in one RACH procedure.

Proposal-6: Increase the power ramping counter when a SBFD-aware UE switches RO type (i.e., RO type switches from legacy-ROs to additional ROs or from additional ROs to legacy ROs) in one RACH procedure.

At least for RACH configuration option 2, the separate preamble_power_ramping_step for SBFD may be configured. Therefore, upon the RO type switching in PRACH transmission re-attempt in one RACH procedure, similar rules as switching from 2-step RACH to 4-step RACH, a power offset can be introduced to compensate the power ramping difference, which is determined by (PREAMBLE_POWER_RAMPING_COUNTER–1) × (PREAMBLE_POWER_RAMPING_STEP_SBFD – PREAMBLE_POWER_RAMPING_STEP_Legacy).

Proposal-7: For RACH configuration Option 2, when RO type switch is performed in one RACH procedure, a power offset is introduced to compensate the power ramping difference.

Proposal-7a: If the proposal 7 is agreed, the power offset is determined by (PREAMBLE_POWER_RAMPING_COUNTER–1) × (PREAMBLE_POWER_RAMPING_STEP_SBFD – PREAMBLE_POWER_RAMPING_STEP_Legacy)

# Conclusion and Proposal

We have the following proposals:

Proposal-1:	Support Msg.3 based early indication for SBFD-aware UE in Rel-19.

Proposal-2:	RA-RNTI calculation is updated to also consider whether the RACH resource belongs to legacy RACH resources or additional RACH resources:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id + 14 × 80 × 8 x 2 x RACH_config

Proposal-3:	Support LTM cell switch via Random access procedure in SBFD symbols for SBFD-aware UEs.

Proposal-4:	For CFRA triggered RACH-based LTM cell switch, one reserved bit in the LTM Cell Switch Command MAC CE can be used to indicate the RO type.

Proposal-5: When the RO type switch happens, the UE fallbacks to perform: a set of random access selection, RA type selection, initialization of variables and random access resource selection procedure.

Proposal-6: Increase the power ramping counter when a SBFD-aware UE switches RO type (i.e., RO type switches from legacy-ROs to additional ROs or from additional ROs to legacy ROs) in one RACH procedure.

Proposal-7: For RACH configuration Option 2, when RO type switch is performed in one RACH procedure, a power offset is introduced to compensate the power ramping difference.

Proposal-7a: If the proposal 7 is agreed, the power offset is determined by (PREAMBLE_POWER_RAMPING_COUNTER–1) × (PREAMBLE_POWER_RAMPING_STEP_SBFD – PREAMBLE_POWER_RAMPING_STEP_Legacy)