3GPP TSG-RAN WG2 Meeting #130		R2-2503399

St Julian’s, Malta, 19th – 23rd, May 2025

Source:	vivo

Title:		Discussion on LCM for UE-sided model for Beam Management

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

# Introduction

At the RAN2#129bis meeting, RAN2 achieved the following agreements regarding LCM for UE-sided model for the beam management use case:

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior - Agreements on applicability reporting - Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec - Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. - No prohibit timer is introduced - Agreements on data collection configuration The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In this contribution, we will further discuss the LCM for UE-sided model for beam management use case based on RAN2 agreements and RAN1’s reply LS[1]. Note that CR Rapporteur has listed some open issues to further discuss in [3], and we will mark the related open issues in the paper.

# Discussion

## 2.1 Open issue RRC-9: Definition of ‘applicable AI/ML functionality’

In the RAN1’s reply LS [1]:

In RAN1’s discussion of RAN 2 terminologies on beam management, - The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs - The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters - The Activated functionalities may be enabled based on CSI framework. Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same. … Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

In the current TS 38.300/331 running CRs, there is also the following editor’s note:

Editor's Note: FFS how to update the definition, e.g., replace 'functionality'

According to RAN1’s LS [1], we think that RAN2 can keep the “functionality” concept at least for applicability reporting for the convenience/brevity of RAN2 specification description. From the view of RAN2, the “functionality” can be defined as “inference configuration or a set of inference related parameters”.

- Adopt the following definition of “AI/ML functionality” in RAN2 specifications:

AI/ML functionality: Inference configuration or a set of inference related parameters.

## 2.2 Open issue RRC-8: Coexistence between option A and option B

There are the following RAN1’s agreements in [2]:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

RAN1 agreed on two options: Option A and Option B. The full inference configuration is configured in Step 3 for Option A and in Step 5 for Option B. For Option A, the UE needs to report applicability for the full inference configuration. For Option B, the UE does not need to report applicability for the full inference configuration in Step 5. One issue is how the UE knows whether to report applicability upon receiving a full inference configuration since full inference configuration can be configured via the RRCReconfiguration message in both options. To address the issue, we think that there needs to be an indicator associated with the configured full inference configuration to indicate whether to feedback the applicability report for the full inference configuration.

- An indicator associated with configured full inference configuration is introduced to indicate whether to feedback the applicability report for the full inference configuration.

For option A, the CSI-ReportConfigId is used to refer to the functionality (or full inference configuration). We think that set ID can be introduced to refer to a set of inference related parameters for option A. The following example of ASN.1 can be considered:

ApplicabilityReportConfigId	::=    CHOICE {

csi-ReportConfigId 	CSI-ReportConfigId,			-- for option A

setId 				InferenceParameterSetId		-- for option B

}

InferenceParameterSetId ::=          INTEGER (0..maxNrofInferenceParameterSet-1)

maxNrofInferenceParameterSet-1       INTEGER ::= FFS      -- Maximum number of inference parameter sets minus 1

Therefore, we suggest:

- For option B, introduce the set ID for a set of inference related parameters, which is separate from CSI-ReportConfigId. The following ASN.1 is considered as the baseline:

ApplicabilityReportConfigId	::=    CHOICE {

csi-ReportConfigId 	CSI-ReportConfigId,			-- for option A

setId 				InferenceParameterSetId		-- for option B

}

InferenceParameterSetId ::=          INTEGER (0..maxNrofInferenceParameterSet-1)

maxNrofInferenceParameterSet-1       INTEGER ::= FFS      -- Maximum number of inference parameter sets minus 1

For option A, full inference configuration is configured via CSI-MeasConfig. For option B, RAN2 assumes that a set of reference related parameters is configured via otherConfig. One issue is whether to support the coexistence between option A and option B. We think that option A and option B have their respective advantages and disadvantages. Therefore, it is beneficial to support the coexistence of the two options. There is no clear reason to forbid the coexistence. We do not identify some problems about the coexistence.

- Option A and option B can be configured simultaneously in a single RRCReconfiguration message.

## 2.3 Open issue RRC-16: UE behaviour when the associated ID is not provided by the network

It is still FFS whether associated ID in Step 3 is mandatory or optional. We think that the issue should be discussed in RAN1, and RAN2 should wiat for RAN1’s conclusion before discussing the open issue RRC-16.

- Wait for RAN1’s conclusion about whether associated ID in Step 3 is mandatory or optional.

## 2.4 Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

At the RAN2#129bis meeting, RAN2 agreed that upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. We understand that the full inference configuration(s) in the RAN2’s agreement includes the case of periodic CSI-report. In the case of periodic CSI-report, the periodic CSI-report is not activated. The network can immediately release the periodic CSI-report after receiving the applicability reporting. Also the network wants the periodic CSI-report and keeps monitoring the applicability of the periodic CSI-report, and once it becomes applicable the gNB can know the information and activate the periodic CSI-report as soon as possible. We think that the two types of network implementation should be supported.

- In case that configured full inference configuration with periodic CSI-report is non-applicable, it is up to network implementation whether to immediately release it or keep monitoring the applicability of the periodic CSI-report.

If the network does not release the periodic CSI-report and the periodic CSI-report becomes applicable from non-applicable, the UE should send the updated applicability reporting to the network. One issue is how to activate the full inference configuration with periodic CSI-report in the case. Three potential alternatives are as below:

- Alt 1: The UE autonomously activates the periodic CSI-report when becoming applicable from non-applicable;

- Alt 2: The network activates the periodic CSI-report via an MAC CE;

- Alt 3: The network activates the periodic CSI-report by reconfiguring the periodic CSI-report with the same CSI-report ID.

With Alt 1, the time point of becoming applicable is uncertain and may be any time. The autonomous activation by the UE may be undesirable since it is not direct network control. With Alt 2, the existing SP CSI reporting on PUCCH Activation/Deactivation MAC CE is only used for SP CSI reporting and can not be used for periodic CSI-report. A new MAC CE needs to be introduced to activate the periodic CSI-report. With Alt 3, there is no specification impact. Therefore, we have a little preference for Alt 3.

- When configured full inference configuration with periodic CSI-report becomes applicable from non-applicable, the UE sends the updated applicability reporting to the network.

- When configured full inference configuration with periodic CSI-report becomes applicable from non-applicable, RAN2 to discuss the following three potential alternatives:

- Alt 1: The UE autonomously activates the periodic CSI-report when becoming applicable from non-applicable;

- Alt 2: The network activates the periodic CSI-report via an MAC CE;

- Alt 3: The network activates the periodic CSI-report by reconfiguring the periodic CSI-report with the same CSI-report ID.

## 2.5 Open Issue RRC-5: Reporting behaviour of inapplicable periodic beam prediction configuration

Another scenario is that activated full inference configuration with periodic CSI-report becomes non-applicable. It is unclear what is the UE behavior in the scenario. Anyway, the network needs to know the situation. Therefore, the UE should send the updated applicability reporting to the network. Based on the updated applicability reporting, the network can immediately release the inference configuration. So we think that the UE should continue to perform the inference and reporting until receiving the release of inference configuration.

- When activated full inference configuration with periodic CSI-report becomes non-applicable, the UE sends the updated applicability reporting to the network. The UE continues to perform the inference and reporting until receiving the release of inference configuration.

## 2.6 Open issue RRC-6: Handling of inference, applicability reporting and UE data collection preference configurations

According to RAN1/RAN2 agreements, inference configuration, applicability configuration and UE data collection preference configuration are configured in the CSI-MeasConfig and otherConfig, which are one part of UE context. During RRC state transition, handover and RRCReestablishment, we think that the handling of inference configurations, applicability configurations and UE data collection preference configuration can follow the legacy handling for CSI-MeasConfig and otherConfig, i.e., no specific handling is needed unless it is justified.

- During RRC state transition, handover and RRCReestablishment,  the handling of inference configurations, applicability configurations and UE data collection preference configuration follows the legacy handling for CSI-MeasConfig and otherConfig.

## 2.7 Open issue RRC-7: Applicability reporting for option B in RRCReconfigurationComplete

At the RAN2#129bis meeting, RAN2 reached the assumption that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. Upon receiving inference related parameters for Option B, the UE needs to immediately feedback the applicability reporting. Similar to other “other configuration”, e.g., overheating assistance config, the UE should feedback the applicability reporting via a UAI message.

- For option B, with the assumption that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig, the UE feedbacks the applicability reporting via a UAI message upon receiving one set or multiple sets of inference related parameters.

## 2.8 Open issue RRC-2: Content of otherConfig for enabling applicability reports in UAI

At the RAN2#127 meeting, RAN2 agreed that UE is allowed to do UAI reporting via OtherConfig. According to RAN1 agreements, the UE may be configured with multiple CSI-ReportConfig for Option A or multiple sets of inference related parameters for Option B. One issue is how to configure the triggering of applicability reporting, e.g., whether it is necessary to configure the triggering per CSI-ReportConfig or per set of inference related parameters? We think that if the network wants to know the applicability status change, it should be for all the configured CSI-ReportConfig or sets of inference related parameters, not just for part of them. Therefore, it is sufficient to configure the triggering per UE, i.e., 1 bit is sufficient..

- The allowance of applicability reporting for applicability status change of inference configuration is configured per UE, i.e., 1 bit is sufficient.

## 2.9 Open issue RRC-1: Cause of inapplicability

At the RAN2#129bis meeting, RAN2 reached the following agreement regarding the inapplicability cause:

Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

Since the agreed cause is simple and is related to model availability, we think that it is straightforward that the cause value is 1 bit, which is used to indicate whether there is an available model.

- The simple cause value of inapplicability is 1 bit, which is used to indicate whether there is an available model.

## 2.10 Open issue RRC-3: UE data collection request

At the RAN2#129bis meeting, RAN2 reached the following agreements regarding UE sided data collection:

- Agreements on data collection configuration The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: • An indication on start/stop of data collection • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

For each candidate configuration provided by NW, a configuration index should be associated with the configuration. If one of the candidate configurations is preferred by the UE, the UE can indicate the index of preferred configuration in the request message. If none of the candidate configurations is desired for the UE, the UE can include the detailed measurement configuration in the request message.

- The candidate measurement configurations, if provided by the network, are associated with configuration indexes, and the UE can indicate the index of preferred configuration or the detailed measurement configuration in the data collection configuration request.

We think that the detailed parameters in candidate measurement configuration should be determined by RAN1. An LS can be sent to RAN1 to inform the RAN2’s agreements about candidate measurement configurations, and ask RAN1 to provide the detailed parameters for candidate measurement configuration.

- LS to RAN1 to inform the RAN2’s agreements about candidate measurement configurations, and ask RAN1 to provide the detailed parameters for candidate measurement configuration.

## 2.11 Open issue RRC-13: CSI prediction LCM framework

In the existing specification, the same configuration framework, i.e., CSI-MeasConfig, is used for both beam management and CSI report. The main difference between the two is the setting of report quantity in the CSI-ReportConfig:

reportQuantity                          CHOICE {

none                                    NULL,

cri-RI-PMI-CQI                          NULL,

cri-RI-i1                               NULL,

cri-RI-i1-CQI                           SEQUENCE {

pdsch-BundleSizeForCSI                  ENUMERATED {n2, n4}                                         OPTIONAL    -- Need S

},

cri-RI-CQI                              NULL,

cri-RSRP                                NULL,

ssb-Index-RSRP                          NULL,

cri-RI-LI-PMI-CQI                       NULL

},

Observation: In the existing specification, beam management and CSI report uses the same configuration framework, i.e., CSI-MeasConfig.

For beam management use case, RAN2 has agreed reusing the CSI report framework for inference configuration and applicability reporting. For CSI prediction use case, it is natural to also resue the CSI report framework. We think that RAN2 can assume that all the LCM related RAN2 agreements for beam management use case are also applicable to the CSI prediction use case. If the exceptions are identified, RAN2 can further dicuss them. Note that there may be the different detailed parameters between the two use cases, which can be left to RAN1.

- RAN2 assumes that all the LCM related RAN2 agreements for UE-sided model of beam management use case are applied to the CSI prediction use case. The exceptions, if identified, can be further discussed.

# Conclusion

In the contribution, we have the following observations and proposals:

Open issue RRC-9: Definition of ‘applicable AI/ML functionality’

- Adopt the following definition of “AI/ML functionality” in RAN2 specifications:

AI/ML functionality: Inference configuration or a set of inference related parameters.

Open issue RRC-8: Coexistence between option A and option B

- An indicator associated with configured full inference configuration is introduced to indicate whether to feedback the applicability report for the full inference configuration.

- For option B, introduce the set ID for a set of inference related parameters, which is separate from CSI-ReportConfigId. The following ASN.1 is considered as the baseline:

ApplicabilityReportConfigId	::=    CHOICE {

csi-ReportConfigId 	CSI-ReportConfigId,			-- for option A

setId 				InferenceParameterSetId		-- for option B

}

InferenceParameterSetId ::=          INTEGER (0..maxNrofInferenceParameterSet-1)

maxNrofInferenceParameterSet-1       INTEGER ::= FFS      -- Maximum number of inference parameter sets minus 1

- Option A and option B can be configured simultaneously in a single RRCReconfiguration message.

Open issue RRC-16: UE behaviour when the associated ID is not provided by the network

- Wait for RAN1’s conclusion about whether associated ID in Step 3 is mandatory or optional.

Open issue RRC-4: Activation of a periodic CSI report configuration upon change from inapplicable to applicable

- In case that configured full inference configuration with periodic CSI-report is non-applicable, it is up to network implementation whether to immediately release it or keep monitoring the applicability of the periodic CSI-report.

- When configured full inference configuration with periodic CSI-report becomes applicable from non-applicable, the UE sends the updated applicability reporting to the network.

- When configured full inference configuration with periodic CSI-report becomes applicable from non-applicable, RAN2 to discuss the following three potential alternatives:

- Alt 1: The UE autonomously activates the periodic CSI-report when becoming applicable from non-applicable;

- Alt 2: The network activates the periodic CSI-report via an MAC CE;

- Alt 3: The network activates the periodic CSI-report by reconfiguring the periodic CSI-report with the same CSI-report ID.

Open Issue RRC-5: Reporting behaviour of inapplicable periodic beam prediction configuration

- When activated full inference configuration with periodic CSI-report becomes non-applicable, the UE sends the updated applicability reporting to the network. The UE continues to perform the inference and reporting until receiving the release of inference configuration.

Open issue RRC-6: Handling of inference, applicability reporting and UE data collection preference configurations

- During RRC state transition, handover and RRCReestablishment,  the handling of inference configurations, applicability configurations and UE data collection preference configuration follows the legacy handling for CSI-MeasConfig and otherConfig.

Open issue RRC-7: Applicability reporting for option B in RRCReconfigurationComplete

- For option B, with the assumption that UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig, the UE feedbacks the applicability reporting via a UAI message upon receiving one set or multiple sets of inference related parameters.

Open issue RRC-2: Content of otherConfig for enabling applicability reports in UAI

- The allowance of applicability reporting for applicability status change of inference configuration is configured per UE, i.e., 1 bit is sufficient.

Open issue RRC-1: Cause of inapplicability

- The simple cause value of inapplicability is 1 bit, which is used to indicate whether there is an available model.

Open issue RRC-3: UE data collection request

- The candidate measurement configurations, if provided by the network, are associated with configuration indexes, and the UE can indicate the index of preferred configuration or the detailed measurement configuration in the data collection configuration request.

- LS to RAN1 to inform the RAN2’s agreements about candidate measurement configurations, and ask RAN1 to provide the detailed parameters for candidate measurement configuration.

Open issue RRC-13: CSI prediction LCM framework

Observation: In the existing specification, beam management and CSI report uses the same configuration framework, i.e., CSI-MeasConfig.

- RAN2 assumes that all the LCM related RAN2 agreements for UE-sided model of beam management use case are applied to the CSI prediction use case. The exceptions, if identified, can be further discussed.

# Reference

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model, RAN1

- Draft Report of 3GPP TSG RAN WG1 #119 v0.3.0

- [POST129bis][016][AI PHY] 38.331 Running CR (Ericsson) - RRC open issues AIML PHY_v15_Rapp.docx