3GPP TSG-RAN WG2 Meeting#130	R2-2503400

St Julian’s, Malta, 19th – 23rd, May 2025

Source:	vivo

Title:	Discussion on open issues of AI/ML enhanced positioning Case 1

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

RAN2#129bis meeting has made progress on functionality-based LCM of UE-sided model for AI/ML enhanced positioning as follows [1].

Agreements 1 LMF is responsible for functionality management 2 UE reports the applicable functionality to the LMF by the LPP provide capabilities message without any additional LMF control. 3 Switching/fallback to non-AI/ML positioning can be supported by including multiple positioning methods in a LPP Request Location Information message. No additional specification work is foreseen specifically for supporting "switching/fallback operation". 4 An AIML positioning functionality is considered “activated” once UE receives an LPP RequestLocationInformation from the LMF requesting inferred location information. 5 For triggered and periodical reporting, we rely on existing positioning framework mechanisms to deactivate AI/ML positioning (no spec impact is foreseen)

In this paper, we mainly focus on one leftover issue regarding error causes for AI/ML based positioning Case 1.

- .Discussion

## On issue LPP-13: Location server error causes

Open issue LPP-13: Location server error causes Issue description: The IE NR-AI-ML-Positioning-Error is used by the location server or target device to provide NR AI/ML positioning error reasons to the target device or location server, respectively. RAN2#128: - As a baseline, if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered. RAN2#129 - The content of error cause is discussed while drafting stage3 CRs.. Status in running CR: The IE NR-DL-AI-ML-LocationServerErrorCauses currently includes the error causes from DL-TDOA. Captured as "Editor's Note" in IE NR-DL-AI-ML-LocationServerErrorCauses. Proposed resolution: Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-LocationServerErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.)

Regarding the error causes provided by the location server in LPP Provide Assistance Data, we consider the fields in the newly introduced IE, i.e., NR-DL-AI-ML-LocationServerErrorCauses, can directly reuse the ones in legacy NR DL positioning methods, e.g., fields in NR-DL-TDOA-LocationServerErrorCauses or NR-DL-AOD-LocationServerErrorCauses. Because over the discussion of AI/ML LCM for positioning, there is no new reason from NW-side to cause an error for positioning. In this sense, we consider the current version IE NR-DL-AI-ML-LocationServerErrorCauses in LPP running CR includes all the needed error causes from location server.

- For error causes provided by the location server to the UE via LPP Provide Assistance Data, no need to introduce any new cause value in addition to the legacy reasons that the current running CR captures.

## On issue LPP-14: Target device error causes

Open issue LPP-14: Target device error causes Issue description: The IE NR-AI-ML-Positioning-Error is used by the location server or target device to provide NR AI/ML positioning error reasons to the target device or location server, respectively. RAN2#128: - As a baseline, if the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered. RAN2#129 - The content of error cause is discussed while drafting stage3 CRs.. Status in running CR: The IE NR-DL-AI-ML-TargetDeviceErrorCauses currently includes the error causes from DL-TDOA. Captured as "Editor's Note" in IE NR-DL-AI-ML-TargetDeviceErrorCauses. Proposed resolution: Companies to provide contributions to the following meeting on error causes for IE NR-DL-AI-ML-TargetDeviceErrorCauses (the proposed error causes should be accompanied by some justification (e.g., why needed? Expected receiver behaviour? etc.)

For error causes provided by UE to LMF in LPP Provide Location Information, similar to the above, some legacy reasons can still be supported as captured in the running CR, such as lack of signals or TRPs to measure.

NR-DL-AIML-TargetDeviceErrorCauses-r19 ::= SEQUENCE { cause-r19 ENUMERATED { undefined, assistance-data-missing, unableToMeasureAnyTRP, attemptedButUnableToMeasureSomeNeighbourTRPs, thereWereNotEnoughSignalsReceived, locationCalculationAssistanceDataMissing, ... },

Furthermore, RAN2#128 meeting came to the following agreements regarding how UE reports inapplicability.

As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered.

Based on our understanding, this illustrates a scenario that the applicable functionality becomes inapplicable during an ongoing positioning session. From our understanding, none of the existing causes can be reused for this scenario. The LMF should be aware of this scenario to avoid triggering another location request with DL AI/ML positioning method. UE may employ a new error cause to report that the positioning procedure using the AI/ML positioning method fails due to the functionality becoming inapplicable. Therefore, we propose:

- For error causes provided by the UE to the location server via LPP Provide Location Information, introduce a new cause value as “AI/ML positioning method becomes nonapplicable”.

## On issue LPP-15: Applicability of Positioning Integrity to AI/ML positioning

Open issue LPP-15: Applicability of Positioning Integrity to AI/ML positioning Issue description: Per RAN1 agreement: "For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE." The RAN1 agreement already includes info #15 for positioning integrity: Data facilitating the integrity results determination of the calculated location However, companies have different views on whether this implies support for integrity.

NR positioning system introduces the integrity evaluation in R18, to help the network evaluate whether the estimated location is within the acceptable range. It requires the UE owns the capability to analyse the positioning integrity based on some error sources about positioning assistance data provided from the network, and therefore generate the decision whether the target integrity risk can be accepted.

However, for AI/ML positioning, on the one hand, WID does not include the objective to evaluate integrity towards AI/ML positioning. On the other hand, the underlying principle of how the model generates location estimation is based on UE implementation, therefore the legacy error sources to evaluate NR positioning methods may not be applicable anymore for AI/ML-based positioning. If we want to support this, a study about whether the principle and assistance data can be reused should be carried out. In this sense, we do not consider positioning integrity mechanism can be directly introduced for AI/ML positioning.

- Positioning integrity evaluation is not applicable for AI/ML-based positioning at least for R19. If not, send LS to RAN1 ask whether the error sources are applicable for AI/ML-based positioning methods.

## On issue LPP-16: Signalling of Monitoring Outcome

Open issue LPP-16: Signalling of Monitoring Outcome Issue description: In terms of monitoring for AI/ML positioning, RAN1 agrees to support: - Option A: The target UE side performs monitoring metric calculation. The target UE may signal the monitoring outcome to the LMF. - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method. This includes all the sub-options A-1, A-2 and A-3. The details are upon RAN1 still, but there are expected RAN2 impact.

At RAN1#120bis meetings [2], the following agreements were made regarding the performance monitoring of AI/ML positioning Case 1.

Agreement For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

With the previous RAN2 agreement on “LMF is responsible for functionality management”, the performance monitoring and functionality management can be realized in a way that UE performs monitoring metric calculation and LMF performs functionality management.

Furthermore, based on previous RAN1 input, LMF may provide some assistance information to UE to achieve performance monitoring metric calculation at UE, e.g., ground truth label of the target UE, position calculation assistance data, PRU measurement, or PRU location. Logically speaking, if LMF configures UE to perform performance monitoring, the information can be provided via LPP Assistance Data Transfer procedure. Or the UE needs its location as ground truth label, UE may initiate MO-LR request to obtain its current location.

- RAN2 to confirm that it is UE that performs monitoring metric calculation.

- To enable performance monitoring metric calculation at target UE, LMF may provide some assistance information to UE, e.g., ground truth label of target UE, position calculation assistance data, PRU measurement, or PRU location.

- To enable performance monitoring at target UE, the target UE obtains assistance data, and PRU measurement/location from LMF via LPP Request/Provide Assistance Data.

- To enable performance monitoring at target UE, the target UE obtains its current location with legacy positioning methods from LMF via MO-LR procedure.

However, UE would not transfer the performance metrics back to LMF. It judges whether the AI positioning performance satisfies the positioning KPI on itself. As a consequence, if the result is satisfactory, UE will report the location normally. On the contrary, if UE determines the performance does not fit the LMF requirement, it would provide feedback with an error. However, for the stage 3 details of field information, we are open to differentiating between the monitoring result not being satisfactory and the inapplicability due to other reasons.

- RAN2 to confirm that, if the AI positioning performance does not satisfy the positioning KPI, UE would report an error cause about not being capable of AI positioning via LPP Provide Location Information.

- If P7 is agreed, RAN2 to discuss the error cause related to performance monitoring, i.e., whether introducing a dedicated cause value (e.g., ‘performance requirement not satisfied’) or reusing the error cause introduced for non-applicable indication.

- Conclusion

In summary, we make the following proposals on functionality-based LCM for AI/ML enhanced positioning:

Issue LPP-13

- For error causes provided by the location server to the UE via LPP Provide Assistance Data, no need to introduce any new casue value in addition to the legacy reasons that the current running CR captures.

Issue LPP-14

- For error causes provided by the UE to the location server via LPP Provide Location Information, introduce a new cause value as “AI/ML positioning method becomes nonapplicable”.

Issue LPP-15

- Positioning integrity evaluation is not applicable for AI/ML-based positioning at least for R19. If not, send LS to RAN1 ask whether there are other error sources specific for AI/ML-based positioning methods.

Issue LPP-16

- RAN2 to confirm that it is UE that performs monitoring metric calculation.

- To enable performance monitoring metric calculation at target UE, LMF may provide some assistance information to UE, e.g., ground truth label of target UE, position calculation assistance data, PRU measurement, or PRU location.

- To enable performance monitoring at target UE, the target UE obtains assistance data, and PRU measurement/location from LMF via LPP Request/Provide Assistance Data.

- To enable performance monitoring at target UE, the target UE obtains its current location with legacy positioning methods from LMF via MO-LR procedure.

- RAN2 to confirm that, if the AI positioning performance does not satisfy the positioning KPI, UE would report an error cause about not being capable of AI positioning via LPP Provide Location Information.

- If P7 is agreed, RAN2 to discuss the error cause related to performance monitoring, i.e., whether introducing a dedicated cause value (e.g., ‘performance requirement not satisfied’) or reusing the error cause introduced for non-applicable indication.

- Reference

- RAN2#129bis meeting Chair’s Notes

- TS 37.355	LTE Positioning Protocol (LPP)

- RAN1#118bis meeting Chair’s Notes