3GPP TSG-RAN WG2 Meeting#130	       	R2-2403401

St. Julian’s, Malta, 19th – 23rd, May 2025

Source:	vivo

Title:	Discussion on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

# Introduction

At the RAN2#129bis[1], on NW side data collection, RAN2 made the following agreements:

Agreements on data collection configuration

- The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc)

Agreements on Idle/inactive and HO

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

and

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

In this contribution, we will further discuss some remaining issues on NW-side data collection for model training for beam management,as follows:

- Data collection configuration

- Data logging

- NW control on retaining logged data at HO

- Open issue RRC-8: Reporting assist. info related to logged meas

- Open issue RRC-28: Handling of configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE etc

- Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state

- Open issue RRC-9: Further procedures for UE assist, info related

- Open issue RRC-14: New SRB usage

# Discussion

## Data collection configuration

RAN1 discussed and reached the following WA regarding associated ID.

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

The above WA is related to the generalization of the model for beam management. Specifically, when the associated ID of a specific cell is the same during the training and inference phases, the UE assumes the NW-side additional conditions for the UE-sided model are fulfilled.

Correspondingly, for the NW-sided model, only the data collected by the UE under specific cell(s) with the specific associated ID(s) can be used for model training. In other words, when the UE moves to a serving cell that is not associated with an associated ID, it should stop data collection.

- Introduce an area (e.g., NCGI, cell PCI) configuration to restrict the area in which the UE performs training data logging. And each cell within the area should be associated with an associated ID.

Open issue RRC-25: Dynamic activation/deactivation of data collection configurations for logging

Issue description: RAN2#127 made the following agreement regarding NW-side data collection:

1 As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

As agreed at the RAN2#130 meeting, data collection can configure measurements for multiple sets of resources and use cases (e.g., BM, Mobility, etc.), in other words, the configuration may include data collection configuration containing L1 and L3 measurement related reconfigurations. At least for L3 measurement configuration, we do not see any motivation for dynamic activation/deactivation. We think RRC configuration is enough, and as soon as the UE is configured, the UE can apply the configurations for data collection.

Therefore,

- Dynamic activation/deactivation is NOT supported for data collection measurement configurations.

## Data logging

RAN2 has agreed that for gNB-centric and OAM-centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement results from UE to gNB via an RRC message as configured by gNB is an optional feature. When reporting the logged data, NW should be aware of the type of data collection, i.e., gNB-centric or OAM-centric, to allow corresponding logged data forwarding. Thus, information such as gNB ID should be included in the logged data report. Therefore,

- For gNB centric data collection, gNB ID is included in the logged data reporting.

As summarized by the Rapporteur, it has not been clarified what type of cell ID the UE needs to log along with the logged data in order to unambiguously identify the cell in which the UE performed the data logging, e.g., CGI, PCI-ARFCN etc. Therefore,

- Along with logged data, UE also logs the area where the data are logged (e.g., NCGI, PCI-ARFCN).

## Open issue RRC-7: NW control on retaining logged data at HO

The open issue is summarized by the Rapporteur as follows:

It should be clarified whether/how the 1-bit indication can be sent during or before HO, taking into account that the source gNB decides whether the data should be kept. From the rapporteur perspective, it is not possible for the source gNB to add the 1-bit indication during HO, in the same RRCReconfiguration that encapsulates the RRCReconfiguration from the target gNB.

Two possible solution options are identified:

- Option 1: The source gNB sends the 1-bit indication to the UE before HO

This option seems simple, as the NW can configure the UE when providing the UE with data collection configuration. This approach can be similar to successHO-Config configuration for the UE to report the successful handover information to the network. With this approach, the 1-bit configuration does not have to affect the HO procedure.

- Option 2: The source gNB decides if the 1-bit indication is needed and, if so, sends it to the target gNB. This approach would require including the 1-bit indication in the RRCReconfiguration sent to the UE during HO.  Additional RAN3 impact is expected.

Based on the above, option 1 is preferred; therefore,

- The source gNB sends the 1-bit indication to the UE before HO, when configuring with the data collection configuration.

## Open issue RRC-8: Reporting assist. info related to logged meas

RAN2 has agreed that the measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g., BM, Mobility, etc.). Consequently, availability indications, such as full buffer and buffer threshold being reached, and low power state, should also be applicable to AI mobility. Therefore,

- With regard to data collection related availability indications from UE to gNB, full buffer and buffer threshold being reached, and low power state indications are also applicable to other use cases , i.e, AI mobility.

On how UE sends a UAI with data available indication and reason as full buffer, buffer threshold reached, or low power state, the following approach can be considered:

- 1-bit for buffer threshold being reached or full buffer being reached. When configured to report buffer threshold being reached or full buffer being reached:

  - If the buffer threshold is being reached, the UE reports the 1-bit indication and continues logging data;

  - If the full buffer is being reached, the UE reports the 1-bit indication and stops logging data.

- 1-bit for low power state.

Therefore,

- UE sends a UAI containing data available indication and reason for full buffer, buffer threshold reached, or low power state as follows:

- 1-bit for buffer threshold being reached or full buffer being reached;

- 1-bit for low power state.

## Open issue RRC-28: Handling of configuration to report data availability and low power state during RRCReestablishment, transition to RRC_INACTIVE etc

Issue description: Handling of configuration for UE assistance information, to report data availability and low power state (LoggedDataCollectionAssistanceConfig in RRC running CR) during RRCReestablishment and in transition to RRC_INACTIVE state needs to be specified.

When performing RRCReestablishment, the UE may already be in a low power state; thus, naturally, the UE can indicate a low power state during RRCReestablishment.

- The UE is allowed to report low power state during RRCReestablishment.

UE transitioning to RRC-INACTIVE is network controlled. When releasing the UE, configured to perform data collection, into RRC_INACTIVE, the UE may have already stored some data. Thus, before going into RRC_INACTIVE, the UE can indicate to the network available data if the available data has reached the configured buffer state. Reporting available data that has not reached the buffer threshold may not be useful to the network.

- UE, configured to perform data collection, can indicate available data, if available data has reached the configured buffer threshold to the network, before the UE transitions to RRC_INACTIVE.

## Open issue RRC-29: Whether data availability indication should be sent when the UE has data below the threshold and low power state

Whether a data availability indication should be sent when the UE has data and a low power state is sent, and what cause should be included then. The UE may have some data when indicating low power state. Even though this data volume may be lower than full buffer/threshold, it is still useful to let the network know about that so that the data can be fetched.

For the UE in a low power state, we can consider the case as follows:

- When the UE receives data collection configuration, if the UE is already in a low power state, the UE should not start data logging and can instantly report its low power state to the network. As the UE has not started data logging, there is no available logged data to indicate to the network.

- When the UE receives data collection configuration, if the UE is not in a low power state, the UE can start data logging. In the meantime, if the UE experiences a low power state, the UE may have already stored some data. If the network has configured a data threshold, and if the stored data has not reached the configured threshold, the UE can just report the low power state to the network, and the UE is not required to report the available data. Because those data may not be useful to the network, subsequent data reporting would just deepen the UE low power state unnecessarily.

- When the UE is reporting a low power state, the UE just report lowpower state and the UE is not required to report available data, if the data has not reached the configured buffer threshold.

## Open issue RRC-9: Further procedures for UE assist, info related

As for the availability indication of logged data due to low power state, RAN2 has come to the following agreements.

- Data collection is controlled by the network. The UE will not autonomously stop when low power state is detected. - The UE reports to the network when the power state is low. We will not specify how the UE determines low power state. The network should de-configure the data collection (this can be captured in stage 2).

It is clearly agreed that, after indicating the logged data is available due to the low power state, the UE is expected to receive the de-configuration of data collection but keeps logging data. However, in case UE does not receive any reaction for NW addressing UE’s availability indication, or if NW provides UE with some configuration that UE cannot apply, UE may unnecessarily waste its power on logging data.

- UE keeps power consumption down if NW cannot stop UE from logging data after indication of UE low power state and has no enough power for other services with higher priority.

Considering that UE may not be de-configured for the data collection with low power, an NW-controlled timer can be introduced for UE to release data collection. Specifically, when configuring the availability indication due to low power state in otherconfig via RRCReconfiguration, NW can also configure UE with a time period as well, which can be used to control when UE is able to release the data collection configuration after reporting the data availability triggered by the low power state. That is, UE should start a timer upon detecting its low power state and reporting the data availability. UE would stop the timer when receiving the de-configuration of data collection. At the timer expiry, UE can stop the data collection procedure directly. In this solution, the UE is able to save some power from logging data following NW control.

- The network can configure UE with a timer, which is started when UE indicates the logged data availability due to low power state; and is stopped when UE receives de-configuration of data collection. At the timer expiry, UE can stop logging the data.

## Open issue RRC-14: New SRB usage

On whether to multiplexe legacy logged data and AIML logged data in the new SRB, our understanding behind introducing the new SRB is to design an SRB for AI/ML specific data reporting and avoid coupling with legacy SON/MDT and QOE data reporting procedures. SRB2 and SRB4 are still available to report those SON/MDT and QOE legacy data. Therefore,

- Legacy logged data and AIML logged data are not multiplexed in new SRB.

# Conclusion

In this contribution, some remaining issues on NW-side data collection for model training for beam management and we have the following obsetvations and proposals:

- UE keeps power consumption down if NW cannot stop UE from logging data after indication of UE low power state and has no enough power for other services with higher priority.

And

Data collection configuration

- Introduce an area (e.g., NCGI, cell PCI) configuration to restrict the area in which the UE performs training data logging. And each cell within the area should be associated with an associated ID.

- Dynamic activation/deactivation is NOT supported for data collection measurement configurations.

Data logging

- For gNB centric data collection, gNB ID is included in the logged data reporting.

- Along with logged data, UE also logs the area where the data are logged (e.g., NCGI, PCI-ARFCN).

Retaining logged data a HO

- The source gNB sends the 1-bit indication to the UE before HO, when configuring with the data collection configuration.

Open issue RRC-8: Reporting assist. info related to logged meas

- With regard to data collection related availability indications from UE to gNB, full buffer and buffer threshold being reached, and low power state indications are also applicable to other use cases , i.e, AI mobility.

- UE sends a UAI containing data available indication and reason for full buffer, buffer threshold reached, or low power state as follows:

- 1-bit for buffer threshold being reached or full buffer being reached;

- 1-bit for low power state.

Open issue RRC-28: Data availability and low power state during RRCReestablishment, RRC_INACTIVE

- The UE is allowed to report low power state during RRCReestablishment.

- UE, configured to perform data collection, can indicate available data, if available data has reached the configured buffer threshold to the network, before the UE transitions to RRC_INACTIVE.

Open issue RRC-29: Whether data availability indication when data below the threshold and low power state

- When the UE is reporting a low power state, the UE just report low power state and the UE is not required to report available data, if the data has not reached the configured buffer threshold.

Open issue RRC-9: Further procedures for UE assist, info related

- The network can configure UE with a timer, which is started when UE indicates the logged data availability due to low power state; and is stopped when UE receives de-configuration of data collection. At the timer expiry, UE can stop logging the data.

New SRB usage

- Legacy logged data and AIML logged data are not multiplexed in new SRB.

- Reference

- RAN2#129bis Chairman notes