3GPP TSG-RAN WG2 Meeting #130		R2-2503402

St Julian’s, Malta 19th – 23rd, May 2025

Source:	vivo

Title:	Discussion on UE side data collection

Agenda Item:	8.1.4

Document for:	Discussion and Decision

- Introduction

At the RAN2#127 meeting, the following options were summarized for UE side data collection:

1a. UE collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for UE-side model training. No 3GPP specification impact is expected. 1b. UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO). 2. UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server. 3. UE collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

At the RAN#106 meeting, TSG RAN sent LS [1] to SA2 and SA5 to trigger the discussion on UP-based data transfer and agreed that RAN2 will continue to study UE-sided data collection during Rel-19 with the following scope. (i.e., option 1b was excluded)

For RAN2 - During Rel-19 RAN2 continues studies on AIML UE sided data collection (excluding 1b) - Study RAN aspects related to data transfer over UP - Discuss level of NG-RAN involvement in the control and configuration of UE side data collection. - Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion). - Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain. - Discuss on the scalability aspects of CP - RAN2 can Liaise with other WGs if/when needed

At the RAN2#129-bis meeting, the following agreements were made regarding the UE-side data collection configuration.

Agreements on data collection configuration

- The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW.  Details of signaling are FFS.  It is up to network what it configures at the end.

- Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

In this contribution, we will provide our analysis of the identified issues for options 1a, 2 and 3.

# Discussion

### 2.1 RAN aspects related to data transfer over UP

For MO-data, the UE may initiate the UE-requested PDU session establishment procedure to setup UP tunnel with CN:

- Step 1: UE sends PDU SESSION ESTABLISHMENT REQUEST to the CN;

- Step 2: CN sends a PDU SESSION RESOURCE SETUP REQUEST message to the NG-RAN node to assign resources on Uu;

- Step 3: The NG-RAN node decides the resource allocation and may reconfigure the DRBs to UE for the PDU session.

From our understanding, the existing PDU session management procedure can be reused for data transfer over UP. In addition, the data transfer procedure can be transparent to NG-RAN node, i.e., NG-RAN node is not aware that the PDU session is utilized for training data transfer.

- With existing PDU session establishment procedure to setup UP tunnel, NG-RAN node is only involved in resource allocation and configuration of the DRBs to UE for the PDU session. NG-RAN node is not aware whether the PDU session is used for training data transfer purpose.

RAN2 may revisit the potential impact on RAN after SA2/SA5 feedback on the UP-based data transfer solution.

- For UE data transfer over UP, RAN2 assumes that the existing PDU session management procedure can be reused and the NG-RAN node is not aware that the PDU session is utilized for training data transfer. RAN2 will revisit this assumption after receiving SA2/SA5 feedback on the UP-based data transfer solutions.

### 2.2 NG-RAN involvement in UE side data collection

Data collection refers to the UE collecting data that involves the UE performing measurements (e.g., radio measurements). At the RAN2#128 meeting, the following agreements were reached regarding UE-side data collection for beam management:

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

The data collection configuration can be further clarified with multiple types. From the UE perspective, UE side data collection includes two phases:

- Phase 1: Acquire the RS configuration for measurement;

- Phase 2: Perform measurement and data logging;

Correspondingly, three levels of NG-RAN involvement in the control and configuration of UE side data collection can be considered:

- Level 0: Not involved. Level 0 is applicable for positioning use case, in which the configuration is controlled by LMF.

- Level 1: Control the RS configuration provision. Level 1 is applicable for Option 1a of beam management.

- Level 2: Control the RS configuration provision and data logging configuration. Level 2 is applicable for Option 2/3 of beam management.

- The following three levels of NG-RAN involvement in the control and configuration of UE side data collection can be considered:

- Level 0: Not involved. Level 0 is applicable for positioning use case.

- Level 1: Control the RS configuration provision. Level 1 is applicable for Option 1a of beam management use case.

- Level 2: Control the RS configuration provision and data logging configuration. Level 2 is applicable for Options 2/3 of beam management use case.

In the agreement of last RAN2#129-bis meeting, the candidate configurations provided by NW is still FFS. From our understanding, the FFS depends on which configuration parameter is critical for model generalization and UE capability (e.g., the UE may only support specific beam number of Set A), and should be decided by RAN1. Therefore, we propose:

- LS to RAN1 to ask which parameters can be provided to UE as candidate data collection configuration for UE to request.

### 2.3 NG-RAN involvement in UE side data transfer

Data transfer refers to the sending/transfer of the collected data from the UE to Server for data collection for UE-side model training/OTT server. The data transfer path summarized in [2] is copied in Table 2.3-1.

Table 2.3-1 AI/ML-specific Data Transfer Path of Candidate Options

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |

Based on Table 2.3-1, NG-RAN may or may not be involved in UE side data transfer for each option:

- For UP-based solutions 1a, NG-RAN node is not aware of the model transfer procedure, and the data is not visible to NG-RAN node.

- For UP-based solution 2/3, whether NG-RAN is aware of model transfer procedure and the data content relies on SA2/SA5 feedback.

- For CP-based Option 2, the data is transferred from UE to CN first as NAS signaling. NG-RAN is involved in NAS information transfer from UE to CN, while the data content is specified and can be visible to CN while is not visible to NG-RAN.

- For CP-based Option 3, the data is transferred from UE to NG-RAN node first as RRC message, which is the same with NW-side data collection, i.e., the NG-RAN can control the reporting configuration and the data content specified and is is visible to NG-RAN. Then, NG-RAN further transfers the data to OAM. Optionally, the NG-RAN node may refine the data content/format before transferring them to OAM. Similar to MDT, the interaction between NG-RAN and OAM is in the scope of SA5.

- For CP-based UE side data collection Option 2, NG-RAN is only involved in NAS information transfer from UE to CN. The data content is specified and can be visible to CN while is not visible to NG-RAN.

- For CP-based UE side data collection Option 3, NG-RAN can control the reporting configuration for data collection and the data content is specified and is visible to NG-RAN. After acquiring the data from UE, NG-RAN will further transfer the data to OAM, which is in the scope of SA5.

### 2.4 Scalability Aspects of CP

For CP-based solutions of UE side data collection, the framework/agreement of NW side data collection can be reused. The main difference is whether the NW will further transfer the training data to the server for data collection for UE-side model training/OTT server.

- The main difference between CP-based solutions of UE side data collection and NW side data collection is only whether the NW will further transfer the training data to the server for data collection for UE-side model training/OTT server.

For NW side data collection, taking MDT as an example, the network can configure multiple UEs to perform the data collection and reporting, and low-priority SRB will be used for reporting, thus no scalability issue for CP-based data transfer. Therefore, no scalability issue is foreseen for CP-based UE data collection and data transfer.

- The framework/agreement of NW side data collection is reused for CP-based UE side data collection.

- No scalability issue for CP-based UE data collection and data transfer.

- Conclusion

This contribution concludes with the following:

RAN aspects related to data transfer over UP

- With existing PDU session establishment procedure to setup UP tunnel, NG-RAN node is only involved in resource allocation and configuration of the DRBs to UE for the PDU session. NG-RAN node is not aware whether the PDU session is used for training data transfer purpose.

- For UE data transfer over UP, RAN2 assumes that the existing PDU session management procedure can be reused and the NG-RAN node is not aware that the PDU session is utilized for training data transfer. RAN2 will revisit this assumption after receiving SA2/SA5 feedback on the UP-based data transfer solutions.

NG-RAN involvement in UE side data collection

- The following three levels of NG-RAN involvement in the control and configuration of UE side data collection can be considered:

- Level 0: Not involved. Level 0 is applicable for positioning use case.

- Level 1: Control the RS configuration provision. Level 1 is applicable for Option 1a of beam management use case.

- Level 2: Control the RS configuration provision and data logging configuration. Level 2 is applicable for Options 2/3 of beam management use case.

- LS to RAN1 to ask which parameters can be provided to UE as candidate data collection configuration for UE to request.

NG-RAN involvement in UE side data transfer

- For CP-based UE side data collection Option 2, NG-RAN is only involved in NAS information transfer from UE to CN. The data content is specified and can be visible to CN while is not visible to NG-RAN.

- For CP-based UE side data collection Option 3, NG-RAN can control the reporting configuration for data collection and the data content is specified and is visible to NG-RAN. After acquiring the data from UE, NG-RAN will further transfer the data to OAM, which is in the scope of SA5.

Scalability Aspects of CP

- The main difference between CP-based solutions of UE side data collection and NW side data collection is only whether the NW will further transfer the training data to the server for data collection for UE-side model training/OTT server.

- The framework/agreement of NW side data collection is reused for CP-based UE side data collection.

- No scalability issue for CP-based UE data collection and data transfer.

- Reference

- RP-243316	LS on AI/ML UE sided data collection

- R2-2407807	CR on RAN2 inputs to TR 38.843