3GPP TSG-RAN WG2 Meeting#130	R2-2503403

St. Julian’s, Malta, 19th – 23rd May, 2025

Source:	vivo, Qualcomm Incorporated, ZTE Corporation, Sanechips, Lenovo, Ericsson, CATT, OPPO, Fraunhofer IIS, Fraunhofer HHI, CEWiT

Title:	Discussion on consistency between training and inference for AI POS

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

For the UE-sided model, it is critical to ensure the consistency between training and inference to guarantee acceptable AI model performance.

RAN1 has discussed how to guarantee the consistency between training and inference for AI/ML positioning Use Case 1. However, the progress in RAN1 is not sufficient to guarantee consistency. To finalize the WI in time, we provide our views on this consistency issue from RAN2 perspective.

- Discussion

## RAN1 current progress

To ensure the consistency between training and inference of configuration/information of specific TRP, RAN1 has come to the following agreements [1][2] on which specific assistance data can be provided for AI/ML positioning Use Case 1.

Agreement @RAN1#118bis meeting For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference, - for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other; - Companies can provide inputs on further enhancements of existing assistance data, including new information - Note: this does not mean that training and inference phases are mentioned in assistance data. Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable. Information; UE-based DL-TdoA; UE-based DL-AoD. 1; Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement. 2; Timing relative to the serving (reference) TRP of candidate NR TRPs. 3; DL-PRS configuration of candidate NR TRPs. 4; Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation. 5; SSB information of the TRPs (the time/frequency occupancy of SSBs). 6; Spatial direction information (e.g., azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB. 7; Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs). 8; Fine Timing relative to the serving (reference) TRP of candidate NR TRPs. 9; PRS-only TP indication. 10; The association information of DL-PRS resources with TRP Tx TEG ID. 11; LOS/NLOS indicators. 12; On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation. 13; Validity Area of the Assistance Data. 14; PRU measurements together with the location information of the PRU. 15; Data facilitating the integrity results determination of the calculated location. 16; TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP). 17; Expected Angle Assistance information. 18; PRS priority list. Agreement @RAN1#119 meeting For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following: - Alternative 1. Info #7 is provided implicitly via associated ID. - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly. - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference. - Alternative 3. Info #7 is not be provided from LMF to UE. - If info #7 is not provided, UE may assume info #7 is consistent between training and inference. - Alternative 4. Info #7 is provided explicitly from LMF to UE.

Seeing from above, almost all the assistance data used in DL positioning methods except for Info#7 (i.e., Geographical coordinates of the TRPs) are agreed to be explicitly indicated to UE. RAN1 also discussed the alternatives on how to provide the TRP location info to UE explicitly or implicitly. However, the consistency is in the granularity of TRP information (e.g., TRP location, and TRP related configurations).

- RAN1 ensures consistency between training and inference of a specific TRP based on assistance data from LMF to UE in the two phases.

During the study item phase, simulation evaluation in TR 38.843 [3] has shown that: for direct AI/ML positioning, identification of the active TRPs is beneficial for positioning accuracy. Otherwise, the model suffers from poor performance in terms of positioning accuracy. Specifically, evaluation results from 4 sources show that the horizontal positioning accuracy is greater than 10 m if TRP identification is not included as model input.

- TR 38.843 captures that involved TRPs have a strong impact on AI positioning accuracy, i.e., the change of TRP identification between training and inference would cause the performance degradation of AI model.

- RAN2 to discuss how to ensure the consistency of involved TRPs between training and inference, as it strongly impacts the performance of positioning accuracy.

## Potential solutions

Inspired by the on-demand PRS scheme in the current positioning architecture, we come out with a solution to ensure the consistency of TRP information, in order to make the AI model functionally well.

The current on-demand PRS scheme is designed to ensure the positioning QoS, therefore only considering certain characteristics (e.g., bandwidth, periodicity) of the PRS, without requesting specific cells to transmit PRS. From our understanding, seeing that AI-enhanced positioning is area-specific, which requires the high consistency of the TRP location and its configuration during training and inference stage. If the UE moves to an area which has a different group of TRPs from the one it trains the model, the model would be totally inapplicable. In this sense, to avoid redundant interaction and help UE with applicability determination, the target UE should be able to request on-demand PRS for a desired group of TRPs. The desired group of TRPs could be identified by a list of global cell IDs, e.g., NCGI.

- To ensure the consistency between training and inference, the UE should be able to explicitly request assistance data associated with a specific group of TRPs, e.g., include a list of NCGI in on-demand PRS request.

- Conclusion

On a basis of the above analysis, we make the following proposals regarding how to ensure the consistency of AI enhanced positioning between training and inference:

- RAN1 ensures consistency between training and inference of a specific TRP based on assistance data from LMF to UE in the two phases.

- TR 38.843 captures that involved TRPs have a strong impact on AI positioning accuracy, i.e., the change of TRP location/identification between training and inference would cause the performance degradation of AI model.

- RAN2 to discuss how to ensure the consistency of involved TRPs between training and inference, as it strongly impacts the performance of AI positioning accuracy.

- To ensure the consistency between training and inference, the UE should be able to explicitly request assistance data associated with a specific group of TRPs, e.g., include a list of NCGI in on-demand PRS request.

- Reference

- Report of 3GPP TSG RAN WG1 meeting#118bis meeting

- Report of 3GPP TSG RAN WG1 meeting#119 meeting

- TR 38.843	Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface