3GPP TSG-RAN WG2 Meeting#130	R2-2503404

St Julian’s, Malta, 19th – 23rd May, 2025

Source:	vivo

Title:	Discussion on A-IoT Paging

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

RAN2#129 meeting has made some progress in terms of A-IoT paging [1].

- FFS which solution if any for device behaviour if it gets a new service request while one procedure is still ongoing or leave it to implementation. - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. - Introduce an explicit 1-bit indication to indicate whether it is CFRA or CBRA per paging message. - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - Send an LS to SA2 to take this into account for their design. - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number.

In this contribution, with the guidance of issues [3] left with FFS, we investigate A-IoT paging message from the following perspectives:

- Format;

- Reader visibility of paging identifier.

- Discussion

## Format

Drawing forth the design of A-IoT paging message format, here cites the fields captured in the MAC running CR [2].

#### 6.2.1.1 A-IoT Paging message The fields in this message are defined as follows: - R2D Message Type: This field indicates the message type. See the Table 6.1-1. - RA Type: This field indicates CBRA or CFRA. - Indication of Paging ID presence: This field indicates whether Paging ID and Length of Paging ID are present (when set to 1) or absent (when set to 0). - Length of Paging ID: This field indicates the Paging ID length information when Paging ID field is present. - Paging ID: xxx - Transaction ID: xxx - Number of Access Occasions: This field indicates the number of access occasions. - D2R Scheduling Info: This field indicates the physical layer parameters used for D2R scheduling. Editor’s Note: FFS if CFRA can omit the fields of transaction ID, Indication of Paging ID present/absence, Number of access occasions. Editor’s Note: FFS the length of transaction ID.

Based on the content of the paging message and aligning with the overall design of A-IoT MAC PDU illustrated in the MAC running CR, we came up with the possible format of the paging message in Figure 2.2-1. Note that, except for fields of message type (based on the number of identified R2D messages, it requires 3 bits), RA type, Indication of Paging ID presence (simplified as IPIDP) with recommended length, the number of occupied bits in other fields and whether the reserved field is needed are, for example, to be further discussed.

Figure 2.1-1: A-IoT MAC format for A-IoT paging message with the paging identifier

To page all the devices, the reader does not include the paging identifier field in the paging message, which corresponding MAC format is demonstrated in Figure 2.2-2. Compared with Figure 2.1-1, the length of paging identifier and the paging identifier itself are omitted.

Figure 2.1-2: A-IoT MAC format for A-IoT paging message without the paging identifier

### On Issue 1-2: Transaction ID size and generation

| Issue 1-2: transaction ID | Whether/how to specify how the reader generate Transaction ID, and the size - Relevant agreements: - The “transaction ID” can be generated by reader based on CN correlation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size - Status in running CR: captured as Editor’s Note in 6.2.1.1. | To be discussed by company contributions |
| --- | --- | --- |

As is agreed that the transaction ID is used by the device to avoid repeated responses for the same A-IoT service, which is unique to an A-IoT service over some certain time for some certain area. In this regard, it should be of relatively long size to prevent a potential wrap-around situation. Besides, since RAN2 assumed that the transaction ID is generated by the reader based on the correlation ID from CN, how this generation works and whether this generation needs to be specified should be up to RAN3 and SA2 determination. To sum up, it is preferable that RAN2 just inform the usage and requirement of the transaction ID to the relevant WGs with a suggestion of 4-bit length and let them make the final decision on the reasonable size and how to generate such a transaction ID.

- RAN2 to send LS to SA2 and RAN3 about the usage and requirement on transaction ID over the air interface in order to avoid duplicate responses for the same A-IoT service, with a suggested length of 4-bit. Let them decide on whether and how to specify the generation scheme.

### On Issue 1-4: Configuration for access occasion

| Issue 1-4: AO number field | How to indicate the number of access occasions, e.g. the maximum number, the length of field, format design. - Relevant agreements: - the A-IoT paging message can include a number of msg1 resources - Status in running CR: the field name is captured in 6.2.1.1 without the detailed format. |
| --- | --- |

Based on the whole picture of how a device could get an occasion to access in a paging round, RAN2 has come to a basic conclusion that the device could know the number of MSG1 resources from the paging message. However, the detailed format of this information and how the device uses such information to determine the access occasion have not been discussed.

Focusing on what detailed information is carried in the paging message, we would like to analyze how the device can randomly pick out an access occasion based on such information. For a clear understanding, we illustrate the draft for a set of access occasions provided in one paging round. Except for the number of access occasions N, the device also has to know two of three component parameters {Z (the number of resource sets following one R2D message), X (the number of the time domain resource(s) within resource set), Y (the number of the frequency domain resource(s) within resource set)}, to figure out the FDMA and TDMA resource pattern. Since RAN1 is discussing how to indicate X and Y in MAC layer, the left thing for us is to decide whether to indicate Z or N in the paging message. Considering the signaling overhead, N is (X*Y) times of Z, it can save more bits if we simply indicate the number of slots for one paging round, and let device calculate the number of access occasions based on these parameters. As a starting point of inventory 1000-3000 devices in the air interface for one time, with the assumption that around 16 access occasions can be provided in one slot from the current RAN1 progress (X=1 or 2, Y is FFS at around 7), 6-bit to 8-bit length of Z is needed. However, the exact length depends on the value range determined by RAN1. Seeing that there is only one meeting left for RAN1, we prefer to wait for their decision and therefore compose each possible field length.

Figure 2.1-3: Draft for a set of access occasions provided in the paging

- X, Y and Z are respectively included in the paging message, where X/Y= the number of time/frequency domain resource(s) in each resource set for access occasions and Z= the number of resource sets within one paging procedure. The device obtains the number of access occasions N by X*Y*Z.

- RAN2 to wait RAN1 decision on the value ranges of X and Y, and then to decide the reasonable length of field to indicate the number of resources sets, Z.

### On Issue 1-5: Situation for CFRA

| Issue 1-5: Paging content for CFRA | Whether paging in CFRA can omit the CBRA related fields, such as transaction ID, Indication of Paging ID present/absence, Number of access occasions in Paging message. - In last meeting, there was a discussion whether transaction ID is needed for CFRA in Paging message. Then during the CR drafting, the Rapp identifies other fields like no paging ID indication, number of access occasions are not useful for CFRA. Since RAN2 also agreed to introduce an explicit indication to indicate CFRA and CBRA, it should be feasible to have different fields in the paging message for CFRA and CBRA. Thus, this can be discussed further. - Status in running CR: captured as Editor’s Note in 6.2.1.1. |
| --- | --- |

When it comes to the paging message for CFRA case, since it only supports paging for one specific device in R19, there is no need to indicate whether paging identifier exists or not and to indicate the contention-based access occasions related fields. Furthermore, since there is no so-called subsequent paging for CFRA, that is, if the device fails to access, the reader needs to re-send a paging message for such device. Therefore, no duplicate responses needs to be avoided in CFRA, and the signalling overhead caused by transaction ID can be relieved.

- The fields of transaction ID, indication of paging ID presence and the number of access occasions can be omitted in the paging message when RA type indicates CFRA.

Figure 2.1-4: A-IoT MAC format for A-IoT paging message for CFRA

### Access Occasion Trigger message rld.

As for the access occasion trigger message, firstly, we would like a clarification on the functionality of it. As agreed in 129bis meeting, this message is modelled as another message individual from the paging message.

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number.

However, the definition of “determining MSG1 resources” is not crystal clear, and there is also an EN in the running CR waiting to be addressed with regard to this access occasion trigger message.

Editor’s Note: More details may be added later according to further agreement if any, e.g., how the device determine the selected access occasion based on the Access Occasion Trigger message.

In our understanding, this Access Occasion Trigger message is used for timing calibration, i.e., when receiving this message, the device who has received the initial paging message and determine the corresponding access occasion for itself, should consider there is a set of resources for MSG1 transmissions coming, and should therefore figure out whether its selected access occasion belongs to the coming resource set. In this understanding, we find the running MAC CR does not capture the corresponding UE behavior when it receives this message, which is not expected from device implementation perspective.

- The Access Occasion Trigger message is to indicate the start of the following set of access occasions (except the first set) in slotted-aloha CBRA scheme. RAN2 to capture the device behaviour upon reception of such Access Occasion Trigger message.

However, one special case is that for the first set of access occasions following the paging message, we think it does not need to be indicate by a dedicated Access Occasion Trigger message following the paging message. The reason is that RAN1 has not evaluated or analyzed the requirements for device to receive two consecutive R2D messages, e.g., interval and timing. Hence, it is preferable and simple that the paging message, i.e., not the R2D trigger, indicates the start of the first set of MSG1 resources.

- The start of the first set of access occasions is directly indicated by the A-IoT paging message. If not, RAN2 to send LS to RAN1 on feasibility evaluation for the device to receive two continuous R2D messages.

Regarding the content of the Access Occasion Trigger message, except for the message type field, we think the transaction ID field also needs to be included. The significant reason is due to RAN1 CRC design [5] for an R2D message, it should be no less than 6 bits because it requires at least 6-bit message for register to perform CRC-6. Therefore, only including 3-bit message type does not work for CRC check.

Agreement When CRC is attached to a PRDCH or PDRCH transmission, when the number of information bits is ≤ 24 bits, CRC-6 is used. Otherwise, when the number of information bits is > 24 bits, CRC-16 is used.

On the other side, to let the device aware of whether the received Access Occasion Trigger message is associated with the former paging message, this field is also needed. In the case that transaction ID field is not introduced, if the next paging message starts a new service and it is miss detected by the device, the device may consider the following Access Occasion Trigger messages belonging to the current paging round. It may cause mal-function to the subsequent access, which has fairness issue.

Figure 2.1-5: A-IoT MAC format for Access Occasion Trigger message

- Access Occasion Trigger message should not be smaller than 6 bits according to RAN1 CRC-6 requirement.

- Device cannot recognize the Access Occasion Trigger message associated with which Paging message if no transaction ID is not included in the Access Occasion Trigger message, which causes access collision issue when miss-detect of the latter paging message happens.

- The MAC format of Access Occasion Trigger message contains:

- Message type to indicate Accession Occasion Trigger;

- Transaction ID.

## On Issue 1-6: Reader visibility of paging identifier

| Issue 1-6: Paging ID visibility | Whether Paging ID is invisible or visible to MAC. - Relevant agreements: - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer. - From the previous discussion, there are some motivations to make paging ID visible to MAC: - 1. Reader can operate on the paging ID for further sub-grouping. The Rapp understands this can be considered as an enhancement from reader side for better system efficiency. From device side, since there is an explicit indication for CBRA and CFRA, the device (even in multi-device CFRA) can determine how to perform random access instead of paging ID/group ID. In this case, such visibility is not an essential function. And according to guidance from chair lady, such enhancement can be considered with lower priority. - 2. Reader can associate the paging ID/device ID and AS ID for a given device within a service request. The Rapp understands according to RAN3 LS R3-252481, reader will allocate NGAP device ID for each device and maintain the per-session per-device context, via which the reader can associate the command receiving from the NG interface with the AS ID assigned for a device, i.e. such device management/association does not rely on the paging ID/device ID. - 3. The Temp ID may have impact on this visibility discussion. The Rapp understands SA3 has not concluded on the solution of Temp ID. But majority seems think this Temp ID is maintained/managed between CN and device, since they already concluded there is no AS security in A-IoT. Therefore, no RAN2 discussion is needed before SA3 further inputs. |
| --- | --- |

As Rapp suggested, RAN2 has assumed that paging identifier is transparent to A-IoT MAC layer [1].

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer.

The email discussion [2] investigated that the visibility issue is in essence about how the reader associate the assigned AS ID maintained, to the further command service from CN.

Company views are diverse on whether the paging identifier should be ‘visible’ to MAC layer. Some companies also raised that question was not very clear (i.e., what does being ‘visible’ mean?). Given the comments are diverse, Rapporteur does not intend to make any specific proposal on the ‘visibility’ of the paging identifier to the MAC layer. This can be handled based on further progress on the discussion of paging message MAC PDU formats. However, multiple companies raised how the reader knows the association between the paging identifier (presumably provided by the CN to the reader) and the AS-ID (presumably used over the air for at least the inventory+command use case.) Discuss whether and how the reader knows the association between the paging identifier (presumably used by the CN in the request to the reader) and the AS-ID (presumably used over the air for command delivery to device in inventory+command use case.)

In our understanding, the reason why such issue exists lies in how the inventory procedure is triggered and performed for a command service. In SA TS 23.369, the command procedure from the AIoT server can trigger firstly an inventory procedure and then multiple per-device command procedures between devices and AIOTF [3] as in Annex. However, due to the inventory procedure may not target for specific devices, i.e., paging identifier in the air interface (or information about the target AIoT Device(s) among CN nodes) can be in the form of “filtering criteria” to filter a number of devices in one paging message with a number of AS ID assigned for each paged device, it cannot help this problem even if the reader is aware of the paging identifier. If the reader is exposed with device ID in MSG3, this problem can somehow be solved within RAN interaction. However, there is no way for such a NAS layer information of device ID to be exposed to the reader for security consideration.

- The paging identifier can be in the form of filtering criteria to filter one or multiple devices, which cannot be associated to a specific AS ID assigned for one device for the further command service.

With regard to this issue, we find that RAN3 reach one related agreement in RAN3#127bis meeting.

- Introduce per-session per-device Device Association (“RAN NGAP Device ID”, and “(FFS) CN NGAP Device ID” pair) between gNB and AIOTF, in case of Command after inventory. - In case of command after inventory, the gNB allocates and provides a “RAN NGAP Device ID” in the Inventory Report Transfer IE for each device.

This RAN NGAP device ID between RAN reader and AIOTF is similar to RAN NGAP UE ID maintained between gNB and AMF, to dedicatedly identify a device in NGAP interface.

- RAN3 introduces RAN NGAP device ID maintained per device by the reader, reported together with inventory report to AIOTF, where inventory report is based on MSG3 in the air associated to a specific AS ID.

In this sense, the reader can allocate this RAN NGAP Device ID for a device together with the inventory report, which is to transfer the permanent device ID via MSG3 over the air to the AIOTF. And since the AS ID (if assigned) is assigned in MSG2, indicating a dedicated MSG3 occasion for such device. Therefore, when receiving MSG3, the reader can know it is associated with which specific device’s AS ID. As illustrated in Figure 2.2-1, with such a mapping relationship, the reader can associate the AS ID assigned for the device and the command targeted for this device, and then use the corresponding AS ID for later command scheduling.

Figure 2.2-1: how reader attach AS ID and NGAP RAN device ID for a specific device

- RAN2 to confirm that the paging identifier is invisible at reader’s MAC layer, i.e., the reader maintains the mapping between RAN NGAP device ID used in the interface and AS ID assigned for the device in the air, to ensure that the command is toward which specific target device.

- Conclusion

On a basis of above analysis, we make the following proposals regarding A-IoT paging:

Format

- Issue 1-2

- RAN2 to send LS to SA2 and RAN3 about the usage and requirement on transaction ID over the air interface in order to avoid duplicate responses for the same A-IoT service, with a suggested length of 4-bit. Let them decide on whether and how to specify the generation scheme.

- Issue 1-4

- X, Y and Z are respectively included in the paging message, where X/Y= the number of time/frequency domain resource(s) in each resource set for access occasions and Z= the number of resource sets within one paging procedure. The device obtains the number of access occasions N by X*Y*Z.

- RAN2 to wait RAN1 decision on the value ranges of X and Y, and then to decide the reasonable length of field to indicate the number of resource sets, Z.

- Issue 1-5

- The fields of transaction ID, indication of paging ID presence and the number of access occasions can be omitted in the paging message when RA type indicates CFRA.

- Access occasion trigger message rld.

- The Access Occasion Trigger message is to indicate the start of the following set of access occasions (except the first set) in slotted-aloha CBRA scheme. RAN2 to capture the device behaviour upon reception of such Access Occasion Trigger message.

- The start of the first set of access occasions is directly indicated by the A-IoT paging message. If not, RAN2 to send LS to RAN1 on feasibility evaluation for the device to receive two continuous R2D messages.

- Access Occasion Trigger message should not be smaller than 6 bits according to RAN1 CRC-6 requirement.

- Device cannot recognize the Access Occasion Trigger message associated with which Paging message if no transaction ID is not included in the Access Occasion Trigger message, which causes access collision issue when miss-detect of the latter paging message happens.

- The MAC format of Access Occasion Trigger message contains:

- Message type to indicate Accession Occasion Trigger;

- Transaction ID.

Issue 1-6

- The paging identifier can be in the form of filtering criteria to filter one or multiple devices, which cannot be associated to a specific AS ID assigned for one device for the further command service.

- RAN3 introduces RAN NGAP device ID maintained per device by the reader, reported together with inventory report to AIOTF, where inventory report is based on MSG3 in the air associated to a specific AS ID.

- RAN2 to confirm that the paging identifier is invisible at reader’s MAC layer, i.e., the reader maintains the mapping between RAN NGAP device ID used in the interface and AS ID assigned for the device in the air, to ensure that the command is toward which specific target device.

- Reference

- Report of 3GPP TSG RAN WG2 meeting#129bis

- R2-2503997	A-IoT MAC running CR

- R2-2503998	Summary of A-IoT MAC open issues

- R2-2502211	Email discussion report: [POST129][035][AIoT] Paging	Qualcomm

- TS 23.369		Architecture support of Ambient power-enabled Internet of Things

- Report of 3GPP TSG RAN WG3 meeting#127bis

- Report of 3GPP TSG RAN WG1 meeting#120bis

- Annex

### 6.2.3	Procedure for Command

Editor’s Note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor’s Note:	Alignment is required for how to document and describe Direct Path and Indirect Path options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

Figure 6.2.3-1: Command Procedure

<skipped the irrelevant part>

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT Command request with an appropriate cause code. The AIOTF performs AIoT RAN and optionally AIoT RAN Reader selection as specified in clause 5.3. If no AIoT RAN or AIoT RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

Editor’s Note:	The authorisation split between the NEF and AIOTF needs alignment with the other clauses in this TS.

5.	AIOTF sends the Naiotf_AIoT_ Command Response message (accept or reject result for the AIoT Command service operation request) to the NEF.

6.	NEF sends the Nnef_AIoT_ Command Response message (accept or reject result for the AIoT Read/Write/PermanentDisable service operation request) to the AF.

Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed.

Editor's note:	It is FFS whether the AIoT RAN performs aggregation for Inventory Report during the Command procedure.

Editor's note:	It is FFS how to handle the scenarios that the Inventory Response received for Command Request sent for many individual devices are in parallel.

Editor's note:	Further clarification and enhancement on step 7 to step 11 of procedure for Inventory specified in clause 6.2.2 is FFS.

For Direct path as shown in clause 4.2.2.2, step 12a is performed.

12a.	AIOTF sends Command Request message (correlation identifier, NAS Command Request) to the AIoT RAN. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

For Indirect path via AMF as shown in clause 4.2.2.3, step 12b and 12c are performed.

12b.	The AIOTF sends Namf_AIoT_MessageDelivery (AIoT RAN ID, Command Request message (correlation identifier, NAS Command Request)) to AMF. The correlation identifier is corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request..

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

12c.	The AMF sends the Command Request message (correlation identifier, NAS Command Request)) to AIoT RAN.

<skipped the irrelevant part>