3GPP TSG-RAN WG2 Meeting #130   	       	R2-2503405

St Julian’s, Malta, 19th – 23rd, May 2025

Source:	vivo

Title:	Random Access Procedure for A-IoT Device

Agenda Item:	8.2.3

Document for:	Discussion and Decision

# Introduction

In the last RAN2 meeting, random access procedure of A-IoT device was discussed and some agreements were achieved as follows [1]:

- Agreements on msg1

      - In case of CBRA, only 16 bits random ID is included in Msg1.  FFS can be revisited if message type will be needed for other D2R messages purposes

      - RN16 is not included in the first D2R message in the CFRA procedure.  AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader.   FFS if we can assume or need to support multiple device scenario.

Agreements on new R2D message

- A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling.   The R2D message indicates the start of a set of MSG1 resources that were configured in paging message.

- Assumption: The R2D message does not include slot number/count down number.

Agreements on msg 2

- A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices.

- Same Msg2 format is used for initial transmission and retransmission of Msg2.

Agreements on NACK

- For CBRA, as a baseline, NACK based mechanism is applied only to the Msg3.   May come back for D2R data, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure” and release the AS ID accordingly (depending on other later discussion).

- For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access.    No need for a timer.   FFS whether subsequent R2D message is trigger message or paging

- For CFRA, NACK feedback and re-access is not supported.  FFS how to achieve

- FFS on end of procedure

In this contribution, we continue to discuss A-IoT random access procedure, which includes detailed procedure handling and format for messages in CBRA and CFRA.

# Discussion

## 2.1 3-step CBRA

In the following figure 1, we provide basic 3-step CBRA procedure and examples of some failure cases.

Figure 1  3-step CBRA

MSG0:

In the last RAN2 meeting, a new R2D trigger other than the paging message was introduced for A-IoT device to indicate the start of a set of MSG1 resources that were configured in the paging message. Then the next issue is how to use this new R2D trigger, such as:

- Whether the first set of MSG1 resources needs this R2D trigger or not;

- Whether each R2D trigger can change the MSG1 resource parameters configured in paging message or not.

For the first issue, we think that the paging message already has the necessary function of R2D trigger as the starting point of the first set of MSG1 resources. Furthermore, RAN1 has not evaluated or analyzed the requirements for two consecutive R2D messages, e.g., interval and timing. Hence, it is preferable and simple that the paging message, i.e., not the R2D trigger, indicates the start of the first set of MSG1 resources. From the second set of MSG1 resources, the R2D trigger can indicate the start of the following set of MSG1 resources.

For the second issue, the simplest RACH modeling is that there are no MSG1 resource parameters in the R2D trigger, i.e., not allowed to change RO parameters in each RACH slot. In this way, access and collision probability are uniform and controllable. The complexity of the device and reader is lower. That is to say, the number of MSG1 occasions in each resource set in the Time Domain and/or Frequency Domain (i.e., X value and Y value in RAN1 agreements) is consistent and only indicated in the paging message, and the R2D trigger should not change these numbers.

- The paging message directly indicates the start of the first set of MSG1 resources, i.e., no separate R2D trigger before the first set of MSG1 resources. If not agreed, send LS to RAN1 for feasibility evaluation.

- From the second set of MSG1 resources, the R2D trigger indicates the start of the following set of MSG1 resources.

- The number of MSG1 occasions in each resource set in Time Domain and/or Frequency Domain (i.e., X value and Y value in RAN1 agreements) is consistent and only indicated in the paging message, and the R2D trigger should not carry any RO parameter.

Furthermore, for the detailed format of the R2D trigger, message type is the necessary information, which may be 3-4 bits. However, from RAN1 CRC perspective, the R2D trigger should be no smaller than 6 bits if a reliability guarantee of CRC-6 is required. Hence, the R2D trigger is better to keep byte alignment as well, e.g., 1-byte. The transaction ID in paging message may also be included in it.

- R2D trigger should be no smaller than 6 bits according to RAN1 CRC-6 requirement.

- The R2D trigger (i.e., Access Occasion Trigger message in MAC running CR) is byte alignment, e.g., 1-byte.

MSG1:

The number of MSG1 occasions in each resource set in the Time Domain and/or Frequency Doman (i.e., X value and Y value in RAN1 agreements) is decided by RAN1 progress and agreements. It is in the scope of RAN2 to decide on how to randomly select a RO from them for an A-IoT device.

The assumption is: X is the number of MSG1 resources in the Time domain and Y is the one in the Frequency domain in each resource set; Z is the number of MSG1 resource sets in this paging round. There will be two options for randomly selecting an MSG1 resource for an A-IoT device:

- One-step:  directly select a random number from the total MSG1 resources in one paging round, e.g., from the range of [0, X*Y*Z-1];

- Two-step: firstly, select a random resource set, e.g., from the range of [0, Z-1], and secondly, select a random MSG1 occasion in that set, e.g., from the range of [0, X*Y-1];

In terms of selection probability and effectiveness, these two options are similar. From the perspective of device complexity, the one-step method requires a larger random number, and the two-step method requires two random operations and maintaining two numbers. RAN2 can discuss and choose one of them. For both methods, the index and order of ROs in each MSG1 resource set must be predefined or specified in MAC/RAN1 spec.

- RAN2 to decide how the device randomly selects one MSG1 resource:

        - One-step: directly select a random number from the total MSG1 resources in one paging round, e.g. from the range of [0, X*Y*Z-1];

        - Two-step: firstly, select a random resource set, e.g., from the range of [0, Z-1], and secondly, select a random MSG1 occasion in that set, e.g., from the range of [0, X*Y-1];

Note: X is the number of MSG1 resources in the Time Domain and Y is the one in the Frequency Domain in each resource set; Z is the number of resource sets in this paging round.

- The index and order of ROs in each MSG1 resource set must be predefined or specified in MAC/RAN1 spec.

MSG2:

After a device sends MSG1 with its RN16, it will monitor the expected MSG2. The MSG2 monitoring needs a deadline to avoid deadlock. The MSG2 window mechanism in the Uu interface is not suitable for an A-IoT device since the A-IoT interface is not a system with strict time synchronization, and the timing capability of the device is not guaranteed. To allow moderate space for reader scheduling implementation, the next R2D trigger is a suitable deadline for MSG2 monitoring. In other words, failure of contention resolution is determined on the device side when the device does not receive the MSG2 carrying its RN16 same as MSG1, until the next R2D trigger, i.e., indicating the start of the next set of MSG1 resources.

- Failure of contention resolution is determined on the device side when the device does not receive the MSG2 carrying its RN16 same as MSG1, until the next R2D trigger.

According to the previous RAN2 agreements and current MAC running CR, RN16 is a mandatory field in MSG2 and AS ID is optional and present only in the RN16 collision case. At least for the RN16 collision case, the mapping relationship or index of RN16 with MSG1 resources is very important for a correct understanding of MSG2, especially for multiplexing MSG2.

For example, in the following figure 2, when there are 4 ROs in one set of MSG1 resources, two devices send the same RN16 values separately in RO2 and RO3. Other ROs are blank. If the reader responds to a multiplexing MSG2 without a resource index as the MSG2 option 1, two devices cannot distinguish two identical RN16 values and get their correct AS ID values. The AS ID re-assignment mechanism for collision recovery is invalid and not workable. However, if the reader responds to a multiplexing MSG2 with the MSG1 resource index as the MSG2 option 2, the two devices can each correctly get their AS ID values according to their resource index. The AS ID re-assignment mechanism for collision recovery is valid and workable.

Figure 2  An example of RN16 collision

- The multiplexing MSG2 format without resource index may be invalid and not workable for RN16 value collision and AS ID re-assignment cases.

The content of MSG2 for 3-step CBRA may include the following fields:

        - Message Type: 4 bits for R2D messages differentiation;

        - A or Ai: presence indicator for the corresponding AS ID field, e.g., 0 means not presence and 1 means presence. Ai is only for multiplexing case and A for single RO case;

        - Ni: presence indicator for the corresponding Random ID field, e.g., 0 means not presence and 1 means presence. The order of Ni is the same as the one of MSG1 ROs/resources. Ni is only for multiplexing case;

        - Random IDi: 16-bit number and same as MSG1;

        - AS IDi: 16-bit number and optionally present, i.e., allocated by reader if necessary;

        - D2R Scheduling Info: up to RAN1 progress.

Figure 3  MSG2 multiplexing format for 4 ROs case

After successful reception of its MSG2, the device should only store its AS ID, i.e., instead of RN16, since RN16 is not useful anymore. The AS ID is the confirmation of RN16 or reassigned by the reader. Only this AS ID is used for the following scheduling and indication.

- Multiplexing MSG2 format should indicate the corresponding MSG1 resource index, e.g., via bitmap.

- After successful reception of its MSG2, the device should only store its AS ID (i.e., instead of RN16), which is the confirmation of RN16 or re-assigned by the reader for following scheduling and indication.

MSG3:

RAN2 agreed that as a baseline, the NACK-based mechanism is applied only to the MSG3. This NACK indication should have a separate code point in the Message Type field to distinguish it from other R2D messages. An MSG3 NACK indication for a device should include the AS_ID of the corresponding device (i.e., instead of RN16) since this NACK is only applicable for the device whose MSG2 is successfully received. The device that does not successfully receive its MSG2 needs not to detect an MSG3 NACK indication, and re-access also occurs due to contention resolution failure determination.

- The device that does not successfully receive its MSG2 needs not to detect an MSG3 NACK indication, and re-access also occurs due to contention resolution failure determination.

Furthermore, from the reader’s side, there is no need to differentiate between MSG2 failure and MSG3 failure, and it can always trigger an MSG3 NACK indication to a specific device if its MSG2 is sent.

- The reader can always trigger an MSG3 NACK indication without differentiation between MSG2 and MSG3 failure.

MSG3 NACK indication may include the following fields: Message Type and AS ID(s). And multiple AS ID fields are possible for the case of multiple device failures. No length field is needed if padding is not supported in the R2D direction.

Figure 4  MSG3 NACK indication format

Hence, we propose:

- MSG3 NACK indication should carry the AS ID of failure device(s).

- Only the device that successfully receives its MSG2 and stores AS ID needs to process an MSG3 NACK indication carrying its AS ID.

The end of procedure:

After MSG3 transmission, the reader will forward the upper layer data in MSG3, e.g., EPC ID, to CN, and CN’s authentication, security handling, and response round trip may need a little longer time. Before the subsequent upper layer command is transmitted to the reader, A-IoT radio resources may be used for other devices’ access or transmission, e.g., cross next R2D trigger slot or even next paging re-access round.

Figure 5 Command procedures

- The upper layer command transmission may cross the next R2D trigger slot since CN’s authentication, security, and response round trip may need a little longer time, and multi-process operations with multiple devices in the A-IoT radio interface are beneficial to resource efficiency.

- The upper layer command transmission may also cross the next paging re-access round since there may be a short-paging round case or an initialized paging round for access parameter adjustment before previous round termination.

Overall, if cross-R2D trigger slot transmission and cross-paging round transmission with the same transaction ID are allowed, resource efficiency in the A-IoT radio interface will be greatly increased, and the whole service latency will also be reduced. From the perspective of complexity of device, continuous monitoring is similar, and it is merely that the usage cycle of the AS ID has become longer. In addition to implicitly starting consequent paging with a new transaction ID to indicate AS ID release, an explicit R2D Release message is also introduced to indicate the end of the procedure. Details are up to reader implementation.

- The end of the procedure is indicated by an explicit R2D Release message in addition to the implicit start of consequent paging with a new transaction ID.

## 2.2 CFRA

In the following figure, we provide basic CFRA procedure and some special cases.

Figure 6 CFRA procedure

The first left issue is whether CFRA for multi-devices is supported or not. In our understanding, according to Study Item conclusions of RAN and SA, and WID of RAN, the scope is “Support the options that a paging message contains one identifier, and that a paging message contains no identifier”. In the case of a paging message containing one identifier, it should trigger a CFRA procedure for a single device if this ID is a specific device ID, and it may trigger a CBRA procedure for multiple devices if this ID is a group ID or filter information. Some companies may argue that the latter may also trigger a CFRA procedure for a group of devices. However, a CFRA procedure for multi-devices should have a pre-configured group size and definite order of each member within the group, which has not been discussed and progressed in SA2 and it cannot be supported in R19.

- CFRA for multi-devices is not in R19 WI scope according to SA2 and RAN agreements.

Hence,

- RAN2 to confirm that CFRA for multi-devices is not supported in R19.

In a basic CFRA procedure, there is no need for failure or success indication of MSG1 because initiating a new CFRA procedure is more preferred when the previous MSG1 is not successfully received by the reader. Neither the failure indication of CFRA MSG1 nor an R2D trigger has a clear and effective guidance regarding the subsequent behaviors of the device. A new CFRA trigger is more beneficial for re-attempt and potentially with updated resource or parameter allocation, which increases the probability of success in the subsequent retry. Furthermore, the device must consistently reply with MSG1 for duplicated CFRA paging since the reader can be fully responsible for and control the previous access result. The device does not need to make a judgment about the transaction ID in the CFRA paging message. Hence, transaction ID is omitted in the CFRA paging message.

- If previous CFRA MSG1 fails, the device may receive a retransmitted CFRA paging trigger for recovery/re-access.

Hence,

- The R2D trigger is not supported in the CFRA case.

- Device always responds with MSG1 for CFRA paging and transaction ID is omitted in the CFRA paging message.

For the end of the CFRA procedure, a unified solution is possible for CFRA and CBRA. For the inventory only case, MSG1 is the end of the whole procedure. For command case, after AS ID is successfully allocated with the MSG2 command, the end of the procedure is indicated by an explicit R2D Release message or implicitly by the start of consequent new paging.

- After AS ID is successfully allocated with the MSG2 command, the end of the procedure is indicated by an explicit R2D Release message or implicitly by the start of consequent new paging, i.e., unified between CFRA and CBRA.

# Conclusion

In this contribution, we discuss further details about CBRA and CFRA and have the following observations:

- R2D trigger should be no smaller than 6 bits according to RAN1 CRC-6 requirement.

- The multiplexing MSG2 format without resource index may be invalid and not workable for RN16 value collision and AS ID re-assignment cases.

- The device that does not successfully receive its MSG2 needs not to detect an MSG3 NACK indication, and re-access also occurs due to contention resolution failure determination.

- The reader can always trigger an MSG3 NACK indication without differentiation between MSG2 and MSG3 failure.

- The upper layer command transmission may cross the next R2D trigger slot since CN’s authentication, security, and response round trip may need a little longer time, and multi-process operations with multiple devices in the A-IoT radio interface are beneficial to resource efficiency.

- The upper layer command transmission may also cross the next paging re-access round since there may be a short-paging round case or an initialized paging round for access parameter adjustment before previous round termination.

- CFRA for multi-devices is not in R19 WI scope according to SA2 and RAN agreements.

- If previous CFRA MSG1 fails, the device may receive a retransmitted CFRA paging trigger for recovery/re-access.

Based the above discussions and observations, we have the following proposals:

MSG0 for 3-step CBRA

- The paging message directly indicates the start of the first set of MSG1 resources, i.e., no separate R2D trigger before the first set of MSG1 resources. If not agreed, send LS to RAN1 for feasibility evaluation.

- From the second set of MSG1 resources, the R2D trigger indicates the start of the following set of MSG1 resources.

- The number of MSG1 occasions in each resource set in Time Domain and/or Frequency Domain (i.e., X value and Y value in RAN1 agreements) is consistent and only indicated in the paging message, and the R2D trigger should not carry any RO parameter.

- The R2D trigger (i.e., Access Occasion Trigger message in MAC running CR) is byte alignment, e.g., 1-byte.

MSG1 for 3-step CBRA

- RAN2 to decide how the device randomly selects one MSG1 resource:

        - One-step: directly select a random number from the total MSG1 resources in one paging round, e.g. from the range of [0, X*Y*Z-1];

        - Two-step: firstly, select a random resource set, e.g., from the range of [0, Z-1], and secondly, select a random MSG1 occasion in that set, e.g., from the range of [0, X*Y-1];

Note: X is the number of MSG1 resources in the Time Domain and Y is the one in the Frequency Domain in each resource set; Z is the number of resource sets in this paging round.

- The index and order of ROs in each MSG1 resource set must be predefined or specified in MAC/RAN1 spec.

MSG2 for 3-step CBRA

- Failure of contention resolution is determined on the device side when the device does not receive the MSG2 carrying its RN16 same as MSG1, until the next R2D trigger.

- Multiplexing MSG2 format should indicate the corresponding MSG1 resource index, e.g., via bitmap.

- After successful reception of its MSG2, the device should only store its AS ID (i.e., instead of RN16), which is the confirmation of RN16 or re-assigned by the reader for following scheduling and indication.

MSG3 for CBRA

- MSG3 NACK indication should carry the AS ID of failure device(s).

- Only the device that successfully receives its MSG2 and stores AS ID needs to process an MSG3 NACK indication carrying its AS ID.

The end of CBRA procedure

- The end of the procedure is indicated by an explicit R2D Release message in addition to the implicit start of consequent paging with a new transaction ID.

CFRA

- RAN2 to confirm that CFRA for multi-devices is not supported in R19.

- The R2D trigger is not supported in the CFRA case.

- Device always responds with MSG1 for CFRA paging and transaction ID is omitted in the CFRA paging message.

- After AS ID is successfully allocated with the MSG2 command, the end of the procedure is indicated by an explicit R2D Release message or implicitly by the start of consequent new paging, i.e., unified between CFRA and CBRA.

- Reference

- RAN2#130, Chairman report;

- R2-2503404, Discussion on AIoT Paging, vivo, RAN2#130;

- R2-2503406, AIoT Data Transmission, vivo, RAN2#130;