3GPP TSG-RAN WG2 Meeting #130	R2-2503406

St.Julian’s,  Malta, 19th – 23rd, May 2025

Source:	vivo

Title:	AIoT Data Transmission

Agenda Item:	8.2.4

Document for:	Discussion and Decision

# Introduction

In this contribution, we will discuss the following topics regarding AIoT data transmission and other general aspects, including the open issues listed in the email discussion of [POST129bis][017][AIoT] [1] and potential new issue identified.

- AS ID

- D2R Segmentation

- D2R MAC PDU format

# Discussion

# AS ID

In the email discussion of [POST129bis][017][AIoT] [1], the AIoT MAC CR Rapporteur provides the following open issues on AS ID assignment as described below:

| Issue 2-9: AS ID assignment in multi-device CFRA | Whether to consider multiple device scenario as to the AS ID in CFRA. - Relevant agreements: - AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. - Status in running CR: not captured. | To be discussed by company contributions |
| --- | --- | --- |
| Issue 3-3: AS ID release | Whether to specify any additional AS ID release method - Relevant agreements: - FFS other cases for release ASID to avoid keeping it indefinitely. - Status in running CR: captured as Editor’s Note in 5.2. | To be discussed by company contributions |

# Issue 2-9: AS ID assignment in multi-device CFRA

With regard to the above Issue 2-9, at least the following aspects need to be taken into account in a multi-device scenario:

- Firstly, it is clearly stated in the WID that RAN2 scope is to support the options that a paging message contains one identifier and that a paging message contains no identifier [2]. In other words, even if the multiple device scenario is to be supported in this Release, no multiple identifiers can be contained in one paging message. Instead, the reader would send the paging message one by one with only one identifier each time.

- Secondly, given that the R2D command message is used for AS ID assignment, some company raised a potential issue that there may be AS ID collision from a device perspective, e.g., when there is more than one device monitoring the R2D messages on PRDCH and receiving this R2D command message with an AS ID, all of the devices may consider the received R2D command message be for addressing itself. However, we think the collision issue can be avoided by smart reader’s implementation. For example, the reader scheduling can guarantee that the procedures, e.g., at least including transmitting a D2R message containing the upper layer device ID and receiving a R2D command message containing the assigned AS ID, are all executed in the same access slot for a single device before triggering other devices to access this reader. As a consequence, there are no multiple devices performing the procedures as to AS ID assignment with the given reader at the same time.

Based on the above analysis, we believe that multi-device scenario for CFRA is not supported in this Release. In other words, as to the AS ID assignment for CFRA, it is under reader control and implementation to ensure that only one device is performing the procedure with the given reader.

- RAN2 to confirm that, for CFRA multi-device scenario is NOT supported in this Release, i.e., as to the AS ID assignment for CFRA, only consider the scenario that a single device is performing the procedure with the given reader.

# Issue 3-3: AS ID release

According to RAN2#129bis agreement, RAN2 has already agreed to the following two conditions to release AS ID:

- upon receiving paging with a new transaction ID for that device, i.e., a different session/service;

- when it triggers new MSG1 transmission as a result of receiving paging message (i.e., it has to generate a random ID for CBRA).

It is observed that both of the above two conditions happen along with receiving another paging message, i.e., from the device’s perspective, the assigned AS ID is always to be released in the next paging round. Thus, the remaining issue is whether there are other cases for the reader to release the AS ID earlier in the same paging round. In our understanding, at least the following use case can be considered:

- Use case description: the reader has successfully received the D2R message containing command response from a device, and there are no more commands arriving for this device (i.e., the reader considers the end of the procedure for the device); then the assigned AS ID for this device can be released so that the device can save some power to process only paging messages afterwards.

If the above use case to early release the AS ID is agreeable, then there are two candidate options for the signaling design of AS ID release.

- Option 1: Reuse R2D trigger message

- Option 2: Introduce a new explicit release message

Between the two options, we think Option 2 is more flexible and future-proof because it allows transmitting the command over the AIoT radio interface across different access slots or even different paging rounds of the current session/service for a device. While Option 1 brings negative impacts to the efficiency of AIoT inventory and command procedures. For example, in Option 1, in case multiple commands (e.g., one is for Read operation and the other is for Write operation) arrive or a command arrives later after transmitting MSG3 for a device, the R2D trigger message needs to be on hold by the reader for some time to make sure the command procedure for this device is completed, which prolongs the overall duration of the current access slot. Since the R2D trigger message is mainly introduced for triggering MSG1 for devices performing CBRA, it also brings unfairness and access delay to the other devices that have not triggered MSG1 yet. In addition, we think Option 2, if agreed can also be used as an implicit indication for indicating the end of the procedure of the current session/service for a specific device.

- Introduce an explicit release message that includes an AS ID field to indicate the AS ID release for a given device. This explicit release message also indicates the end of the current session/service for this given device.

# New issue: Random ID and AS ID storage for CBRA

Basically, the Random ID and AS ID storage issue is mainly for CBRA since for CFRA there is no Random ID and only AS ID. As the expected device behavior on Random ID and AS ID storage may have impacts on real device implementation, we believe it is essential to be clarified. Moreover, our considerations of the device behavior on Random ID and AS ID storage are illustrated as follows:

- Firstly, the device can store the Random ID after transmitting MSG1 in order to compare it with the value of the Random ID field in subsequent MSG2.

- Then upon receiving MSG2, if the value of the Random ID field in MSG2 is the same as the stored Random ID, the device can store the AS ID, i.e., either set to the value of the Random ID field or AS ID field.

- Lastly, the question is whether the stored Random ID can be released immediately once the AS ID is stored by the device. Given that the AS ID will be used to address the device afterwards, there is no usage of the stored Random ID anymore. It can also save device memory and reduce device cost by enabling the device to only store Random ID or the AS ID but not store both. We prefer that the stored Random ID can be released immediately once the AS ID is stored by the device.

Based on the above analysis, our views on Random ID and AS ID storage are summarized by the following observations:

- For CBRA, the device can store the Random ID after transmitting MSG1.

- For CBRA, the device can store the AS ID, which is promoted based on the value of the Random ID field or AS ID field after receiving MSG2.

- For CBRA, it is beneficial that the device only stores Random ID or AS ID but not both, for the sake of saving device memory and reducing device cost.

In accordance with the above obseravations, we propose to agree the following Proposal as the basic principle for Random ID and AS ID storage for CBRA.

- A device performing a CBRA procedure is not required to store both Random ID and AS ID, i.e., the device can store the AS ID and release the stored Random ID immediately after successful contention resolution by MSG2.

If Proposal 3 is agreeable, then the device behavior can be further defined, focusing on two cases, i.e., Case 1 with only stored Random ID and Case 2 with only stored AS ID. Our understanding of the device monitoring and R2D message reception behaviors is summarized below:

- Case 1: If the device has a stored Random ID only (i.e., no stored AS ID), only MSG2, wherein the value of the random ID field included in MSG2 is matched with its stored Random ID is further processed, while other R2D messages are ignored.

- Case 2: If the device has a stored AS ID only (i.e., no stored Random ID), only MSG2, wherein the value of the AS ID field in MSG2 is matched with its stored AS ID, and R2D command MSG, wherein the value of the AS ID field in R2D command MSG is addressed to its stored AS ID, are further processed, while other R2D messages are ignored.

- For CBRA, if the device has a stored Random ID only (i.e., no stored AS ID), only MSG2, wherein the value of the Random ID field included in MSG2 is matched with its stored random ID, is further processed, while other R2D messages are ignored.

- For CBRA, if the device has a stored AS ID only (i.e., no stored Random ID), only MSG2, wherein the value of the AS ID field in MSG2 is matched with its stored AS ID, and R2D command MSG, wherein the value of the AS ID field in R2D command MSG is addressed to its stored AS ID, are further processed, while other R2D messages are ignored.

# D2R Segmentation

In the email discussion of [POST129bis][017][AIoT] [1], the remaining open issues regarding segment retransmission of D2R message are summarized as below:

| Issue 3-1: command for non-first segment | Whether upper layer command is included in the R2D message scheduling for non-first segment. - Relevant agreements: - FFS whether the reader always includes the command for retransmission of segments. - Status in running CR: captured as Editor’s Note in 5.4.2. | To be discussed by company contributions |
| --- | --- | --- |
| Issue 3-2: offset for first segment | Whether offset is included in the R2D message scheduling for the first segment and unsegmented message - Relevant agreements: - For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. - Status in running CR: captured as Editor’s Note in 5.4.3. | To be discussed by company contributions |

# Issue 3-1: command for non-first segment

If a CMD message is included in the R2D message for transmission/retransmission of the segments after the first segment, it means that the device should process the same CMD message multiple times. Additionally, the following issues should be further considered:

- Firstly, it is not sure whether the D2R messages generated in response to different receptions of the same CMD are completely the same. As one concern, the command procedure may involve the upper layer security handling, including ciphering and/or integrity protection. After security protection of the original D2R message, the output D2R message content is typically changed for each D2R transmission, e.g., due to the generated integrity protection info for the original D2R message being variational to avoid message tampering attacks. As another concern, the device could have updated the stored information in the upper layer for D2R message generation between different receptions of the same CMD. For instance, when the device is used for temperature monitoring, the device may update the temperature data in the upper layer because the connected temperature sensor has output a new temperature value between different receptions of the same CMD. Anyway, the handling of repeated CMD messages is up to the upper layer. To resolve these concerns, an LS to SA2/SA3 is needed.

- R2D resource overhead for repeated transmissions of the same command.

- The processing delay and additional energy consumption of the device, due to:

  - the device needs more energy to receive the R2D message with CMD included;

  - the upper layer needs longer time to receive/process the same CMD multiple times. The upper layer processing delay could be unpredictably as described in clause  “2.3.2 MAC padding” in [4].

- Command in every R2D message for transmission of the segments after the first segment has the following concerns:

- Whether the generated D2R messages in response to different receptions of the same CMD are completely the same;

- R2D resource overhead increase for repeated transmissions of the same command;

- The processing delay and additional energy consumption of the device.

While if the CMD message is not included in the R2D message for transmission/retransmission of the segments after the first segment, i.e., the CMD is only transmitted in the R2D message for transmission/retransmission of the first segment/unsegmented D2R message, all the above concerns disappear. The cost is that the device is required to store the D2R message for transmission/retransmission of the segments after the first segment. However, we do not think this increases the complexity/cost of the device, as the device has to generate the whole D2R message and store it temporarily for transmission even though no segmentation is needed. The only difference caused by segmentation is that the device needs to store the D2R message for a longer time compared to the case without segmentation.

- The Command is not included in the R2D messages for transmission/retransmission of the segments after the first segment of a D2R message. If not agreeable, RAN2 should send an LS to ask SA2/SA3 to confirm whether the D2R messages generated by the same device in response to different CMD repetitions are completely the same.

# Issue 3-2: offset for first segment

From the perspective of the device, there should be no differentiation between the initial transmission and the retransmission of the first segment/unsegmented D2R message, i.e., whether the R2D message is for transmission or retransmission of the first segment/unsegmented D2R message should be transparent to the device. From the device’s perspective, when it receives an R2D message with an SDU (i.e., CMD) included, it determines there is no offset zero in the R2D message and transmits the corresponding D2R message from the start. In such a way, there is no need to include an offset zero in the R2D message of the first segment/unsegmented D2R message.

- For the transmission/retransmission of the first segment/unsegmented D2R message, offset zero is not included in the R2D message.

As the maximum message size for AIoT service is ~1000 bits, the offset value range can be 0 to ~1000 bits. Considering that the D2R message is byte aligned, the offset indication can also be in bytes instead of bits. Hence, the offset indication in the scheduling message can be defined as an 8-bit field in granularity of bytes.

- The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 8-bit length in bytes.

# D2R MAC PDU format

For D2R MAC PDU format for data transmission, there are the following two open issues according to email discussion on running CR of 38.391 [1]:

| Issue 3-4: D2R padding indication | How to indicate padding and the Length field for SDU (segmentation or non-segmentation) or padding and its size - Relevant agreements: - In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS. - Status in running CR: captured as Editor’s Note in 6.2.2.2. | To be discussed by company contributions |
| --- | --- | --- |
| Issue 3-5: D2R message type | Whether to support D2R message type - Relevant agreements: - FFS whether we introduce D2R message type. Discuss after looking at the overall MAC header design and space before deciding whether we introduce message type or reserved bits - Status in running CR: captured as Editor’s Note in 6.1.1. | To be discussed by company contributions |

# Issue 3-4: D2R padding indication

Regarding the above Issue 3-4, if padding length is provided, the device has to derive the SDU length by subtracting the padding length from the available capacity (i.e., the TBS excluding the MAC header size) for SDU, which means the complexity increases at the device side. While if SDU length is provided, the device can derive the SDU following the MAC header part directly. For simplicity of the device, SDU length instead of padding length should be provided in D2R MAC PDU for upper layer SDU transmission.

- SDU length field is used to indicate how many SDU bytes are present in D2R MAC PDU.

# Issue 3-5: D2R message type

Regarding the above Issue 3-5, as far as our understanding, the D2R message is generated in response to an R2D MSG, i.e., the format and the content of the D2R message is determined by the corresponding R2D message format and content. Upon reception of a D2R message in response to an R2D message transmitted to a device, the reader can exactly determine the format and content of the D2R message received from a device according to the format and content of the corresponding R2D message transmitted to the device. In such a sense, it is not necessary to introduce an MSG type field in D2R MAC PDU.

- Message type field is not needed for D2R messages.

Based on the existing agreements and the above discussions, the following D2R MAC PDU contents are proposed:

- The D2R MAC PDU for data transmission contains the following fields:

- More Data Indication;

- Length indication of a Data SDU;

- Data SDU;

- MAC Padding, if any.

# Conclusion

In this contribution, remaining issues on AIoT data transmission and other general aspects are discussed, and we have the following observations:

- For CBRA, the device can store the Random ID after transmitting MSG1.

- For CBRA, the device can store the AS ID, which is promoted based on the value of the Random ID field or AS ID field after receiving MSG2.

- For CBRA, it is beneficial that the device only stores Random ID or AS ID but not both, for the sake of saving device memory and reducing device cost.

- Command in every R2D message for transmission of the segments after the first segment has the following concerns:

- Whether the generated D2R messages in response to different receptions of the same CMD are completely the same;

- R2D resource overhead increase for repeated transmissions of the same command;

- The processing delay and additional energy consumption of the device.

Based the above discussions and observations, we have the following proposals:

AS ID

- RAN2 to confirm that, for CFRA multi-device scenario is NOT supported in this Release, i.e., as to the AS ID assignment for CFRA, only consider the scenario that a single device is performing the procedure with the given reader.

- Introduce an explicit release message that includes an AS ID field to indicate the AS ID release for a given device. This explicit release message also indicates the end of the current session/service for this given device.

- A device performing a CBRA procedure is not required to store both Random ID and AS ID, i.e., the device can store the AS ID and release the stored Random ID immediately after successful contention resolution by MSG2.

- For CBRA, if the device has a stored Random ID only (i.e., no stored AS ID), only MSG2, wherein the value of the Random ID field included in MSG2 is matched with its stored random ID, is further processed, while other R2D messages are ignored.

- For CBRA, if the device has a stored AS ID only (i.e., no stored Random ID), only MSG2, wherein the value of the AS ID field in MSG2 is matched with its stored AS ID, and R2D command MSG, wherein the value of the AS ID field in R2D command MSG is addressed to its stored AS ID, are further processed, while other R2D messages are ignored.

D2R Segmentation

- The Command is not included in the R2D messages for transmission/retransmission of the segments after the first segment of a D2R message. If not agreeable, RAN2 should send an LS to ask SA2/SA3 to confirm whether the D2R messages generated by the same device in response to different CMD repetitions are completely the same.

- For the transmission/retransmission of the first segment/unsegmented D2R message, offset zero is not included in the R2D message.

- The offset indication for transmission/retransmission of the segments after the first segment of a D2R message is 8-bit length in bytes.

D2R MAC PDU format

- SDU length field is used to indicate how many SDU bytes are present in D2R MAC PDU.

- Message type field is not needed for D2R messages.

- The D2R MAC PDU for data transmission contains the following fields:

- More Data Indication;

- Length indication of a Data SDU;

- Data SDU;

- MAC Padding, if any.

- Reference

- Report of [POST129bis][017][AIoT] 38.391 Running CR (Huawei).

- RP-250796 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.

- RAN2#129bis Chairman Notes.

- R2-2502269	A-IoT data transmission	Huawei, HiSilicon.