3GPP TSG-RAN WG2 Meeting #130                                                                                  R2-2503410

St.Julians, Malta,  May 19th – 23rd , 2025

Source:                 CATT

Title:	                L1 event triggered measurement reporting

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

In this paper, we further explore the open issues regarding L1 event-triggered measurement reporting and also the support of SP CSI-RS resource for LTM.

The organization of this contribution is as follows. In Section 2, we discuss the following aspects.

- MR MAC CE

- Signalling Configuration aspects

- Support of SP CSI-RS resource for LTM

Our proposals are summarized in Section 3.

Discussion

MR MAC CE

Issue: RS used for L1 event LTM2 evaluation

Regarding the RS) used for evaluation in event-triggered measurement, there is no ambiguity regarding event LTM3/LTM5. For these events, the UE conducts measurements on the beam or RS of the candidate cell, which is configured within the associated LTM CSI resource configuration. However, in the case of event LTM2, it remains unclear how the UE can obtain the RS of the serving beam(s). Fundamentally, there are two interpretations:

- Understanding 1: The RSs of the serving cell within the LTM-CSI-ResourceConfig associated with event LTM2.

- Understanding 2: The RS associated with the indicated TCI state.

In the previous RAN1 meeting, RAN1 reached an agreement that at least one candidate RS should be configured. This configuration is to let the UE to determine the RS type for event LTM2.

Agreement For the RS type determination for event triggered reporting with event LTM2, Alt 1-2: At least one candidate RS shall be configured.

It is worth to note that this RAN1 agreement is only for RS type determination but it does not clarify on whether the RS of serving beam(s) to be evaluated for LTM2 should be configured in the associated LTM-CSI-ResourceConfig.

In our view, in accordance with the following agreement, only the current beam should be evaluated for event LTM2,

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

Therefore, the UE does not have to assess other beams of the serving cell. As such, Option 1(i.e., to configure the RSs of serving cell in the associated LTM-CSI-ResourceConfig) is not necessary. In contrast, Option 2 is consistent with the existing agreement.

Proposal 1a: For event LTM2, UE only evaluates the RS associated with the indicated TCI state(i.e., no need to the RSs of serving cell in the associated LTM-CSI-ResourceConfig).

Proposal 1b: For event LTM2, UE only performs the evaluation when the RS type of the indicated TCI state matches the candidate RS configured in the associated LTM-CSI- ResourceConfig.

Issue MAC-5:The content of L1 MR MAC CE triggered by LTM2

In MAC running CR, it is a FFS whether to only include the current beam information in the MR MAC CE for MR triggered by LTM2 or the MR can include measurements for LTM candidates.

Event LTM2 will be evaluated on serving beam only and we have agreed LTM MR MAC CE only include one triggered measurement event,

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

Even though RAN1 has agreed that at least one candidate RS shall be configured in the associated resource configuration as follows,

Agreement For the RS type determination for event triggered reporting with event LTM2, Alt 1-2: At least one candidate RS shall be configured.

However, the RAN1 agreement is only for the determination of the RS type for event LTM2. Consequently, the UE is not required to evaluate or report the measurement results of the candidate RS within the associated resource configuration.

Given the above - mentioned considerations, it is evident that only the information of the current beam should be incorporated into the MR MAC CE triggered by event LTM2.

Proposal 2: For an MR triggered by event LTM2, the MR MAC CE includes only the current beam information.

Issue MAC-15:Whether the triggered MR should be cancelled if all the triggered beam(s) have been reported

Regarding issue MAC-15 (i.e., Whether the triggered MR should be cancelled in the below case: if all the triggered beam(s), i.e. Type#00 and Type#01, have been reported, no matter whether truncated MR MAC CE or regular MR MAC CE is used before, the trigger MR should be cancelled.), our understanding is as follows.

The triggered MR should only be cancelled upon the transmission of the original complete MR for the corresponding ltm-CSI-ReportConfigId. This is because all the information contained in the initial complete MR is valuable and intended for transmission to the NW. For instance, when reportonleave is not configured, the NW relies on type3 beam information to determine whether an unreported beam has met the leaving condition.

Proposal 3: The triggered MR is only cancelled once the original complete MR for the corresponding ltm-CSI-ReportConfigId has been successfully transmitted.

Issue:Whether to count the Truncated MR MAC CE for event-triggered periodic measurement reporting,

In the MAC running CR, MR_SENT_COUNTER(i.e., the number of L1 event triggered measurement report performed by UE) is incremented by 1 when a Truncated L1 measurement report MAC CE is sent,

2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE; 3> increment the MR_SENT_COUNTER by 1; 3> stop the periodical reporting timer, if running; 3> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount: 4> start the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig;

In our view, when counting the number of L1 event-triggered measurement reports, the intention is to count the original complete measurement report MAC CEs. Truncated L1 measurement report MAC CEs and subsequent MR MAC CEs of the same triggered event should not be double-counted. Therefore, it would be beneficial to clarify this.

Proposal 4: For event-triggered periodic measurement reporting, the reporting of the Truncated L1 measurement report MAC CE are not counted as the number of L1 event-triggered measurement reports.

Signaling Configuration aspects

FFS whether it is possible to configure CSI-RS without SSB in the LTM-CSI-ResourceConfig

In the RRC Running CR, there is a FFS on whether it is possible to configure CSI-RS without SSB in the LTM-CSI-ResourceConfig and it is indicated that RAN2 waits for R1’s progress.

ltm-CSI-RS-ResourceSet This field indicates the resource set for LTM measuremenet based on CSI-RS. When the field is present, the UE shall ignore the field ltm-SSB-ResourceSet. Editor's NOTE: FFS whether it is possible to configure CSI-RS without SSB and wait for R1’s progress.

However, it seems RAN1 has not held any discussions on this matter.

Observation 1: RAN1 has not engaged in discussions regarding the possibility of configuring CSI-RS in the absence of a valid ltm-SSB-ResourceSet in the LTM-CSI-ResourceConfig.

Considering the CSI-RS may has QCLed with SSB, and it’s time domain configuration may be determined based on the timing of the SSB, whether it should be configured in the same LTM-CSI-ResourceConfig should be determined by RAN1, we suggest to send LS to RAN1 to ask them the question.

Proposal 5: Send LS to RAN1 to check whether it is allowed to configure CSI-RS in the LTM-CSI-ResourceConfig without a valid ltm-SSB-ResourceSet.

Support of SP CSI-RS resource for LTM

Issue: Granularity for the SP CSI-RS activation/deactivation

For the Activation/deactivation of SP CSI-RS for LTM, the following agreement was reached in last RAN2 meeting,

Agreements on L1 event-triggered MR

- Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource.

- The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id.

In the RAN1 LS [1], it mentioned that the new MAC CE is similar as the legacy mechanism for serving cell.

MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

In Legacy, the MAC CE is used to activate/deactivate one SP CSI-RS resource set of a serving cell.

However, it is not straightforward that same granularity is used for the LTM SP CSI-RS activation/deactivation. According to RAN1 LS [2] for L1 parameters, the SP CSI RS resource is provided under LTM-CSI-ResourceConfig-r18 via LTM-NZP-CSI-RS-ResourceSet-r19, which defines one NZP-CSI-RS resource set from one or more candidate cells. This is different from the legacy SP CSI-RS resource set which is for only one cell.

| RAN2 Parent IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specification | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTM-CSI-ResourceConfig-r18 |  | LTM-NZP-CSI-RS-ResourceSet-r19 | New |  | Defines one NZP-CSI-RS resource set from one or more candidate cells | SEQUENCE { ltm-NZP-CSI-RS-ResourceList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI- NZP-CSI-RS -ResourcesPerSet-r19)) OF NZP-CSI-RS-ResourceId, ltm-CandidateIdList-r19 SEQUENCE (SIZE (1..maxNrofLTM-CSI- NZP-CSI-RS-ResourcesPerSet-r19)) OF LTM-CandidateId-r18, } |  | UE | UE-specific | 38.331 | Agreement (RAN1 120) An LTM report configuration for L1-RSRP is associated with a single resource config that includes: o Alt.A: a single resource set containing CSI-RS resources corresponding to multiple candidate cells. i.e., the same design as that of SSB in Rel-18 LTM. |
| LTM-CSI-ResourceConfig-r18 |  | resourceType-r19 | New |  | Time domain behavior of resource configuration (see TS 38.214 [19], clause 5.2.1.2). | ENUMERATED {periodic, semipersistent} |  | UE | UE-specific | 38.331 |  |

Therefore, it is necessary to consult with RAN1 regarding the granularity of the LTM SP CSI-RS activation/deactivation. This includes determining whether the activation or deactivation is managed at the level of,

- Option 1: per LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. all RSs(from same candidate cell or different candidate cells) in one LTM-NZP-CSI-RS-ResourceSet-r19 are activated/deactivated by the new MAC CE).

- Option 2: per candidate cell in one LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. all RSs of certain candiate cell in one LTM-NZP-CSI-RS-ResourceSet-r19 are activated/deactivated by the new MAC CE).

- Option 3: per RS in one LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. a certain RS in one LTM-NZP-CSI-RS-ResourceSet-r19 is activated/deactivated by the new MAC CE).

Proposal 6: Send LS to RAN1 to clarify the granularity of the SP CSI-RS activation/deactivation, with the following options:

- Option 1: per LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. all RSs(from same candidate cell or different candidate cells) in one LTM-NZP-CSI-RS-ResourceSet-r19 are activated/deactivated by the new MAC CE).

- Option 2: per candidate cell in one LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. all RSs of certain candiate cell in one LTM-NZP-CSI-RS-ResourceSet-r19 are activated/deactivated by the new MAC CE).

- Option 3: per RS in one LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. a certain RS in one LTM-NZP-CSI-RS-ResourceSet-r19 is activated/deactivated by the new MAC CE).

Issue: How to handle the activated semi-persistent CSI-RS resources after LTM cell switch

Regarding how to handle the activated semi-persistent CSI-RS resources after LTM cell switch,the following options was suggested by company and discussed last RAN2 meeting,

- Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch

- Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch

- Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch

In our understanding,it is natual that the activated semi-persistent CSI-RS resources after LTM cell switch are sill in activated state after LTM cell switch,unless it is deactivated explicitly. Consequently, it is logical for the UE to continue measuring these resources after the LTM cell switch. Regarding potential RAN3 impacts, it may depends on the RAN3 network interface design for the semi-persistent CSI-RS activation phase. As such, this aspect can be deferred to RAN3 for further consideration.

Given the above, Option 2 is the most favourable choice.

Proposal 7: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch.

Conclusion

According to the previous analysis in section 2, our observations and proposals are summarized as follows:

Observation 1: RAN1 has not engaged in discussions regarding the possibility of configuring CSI-RS in the absence of a valid ltm-SSB-ResourceSet in the LTM-CSI-ResourceConfig.

MR MAC CE

Proposal 1a: For event LTM2, UE only evaluates the RS associated with the indicated TCI state(i.e., no need to the RSs of serving cell in the associated LTM-CSI-ResourceConfig).

Proposal 1b: For event LTM2, UE only performs the evaluation when the RS type of the indicated TCI state matches the candidate RS configured in the associated LTM-CSI- ResourceConfig.

Proposal 2: For an MR triggered by event LTM2, the MR MAC CE includes only the current beam information.

Proposal 3: The triggered MR is only cancelled once the original complete MR for the corresponding ltm-CSI-ReportConfigId has been successfully transmitted.

Proposal 4: For event-triggered periodic measurement reporting, the reporting of the Truncated L1 measurement report MAC CE are not counted as the number of L1 event-triggered measurement reports.

Signaling Configuration aspects

Proposal 5: Send LS to RAN1 to check whether it is allowed to configure CSI-RS in the LTM-CSI-ResourceConfig without a valid ltm-SSB-ResourceSet.

Support of SP CSI-RS resource for LTM

Proposal 6: Send LS to RAN1 to clarify the granularity of the SP CSI-RS activation/deactivation, with the following options:

- Option 1: per LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. all RSs(from same candidate cell or different candidate cells) in one LTM-NZP-CSI-RS-ResourceSet-r19 are activated/deactivated by the new MAC CE).

- Option 2: per candidate cell in one LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. all RSs of certain candiate cell in one LTM-NZP-CSI-RS-ResourceSet-r19 are activated/deactivated by the new MAC CE).

- Option 3: per RS in one LTM-NZP-CSI-RS-ResourceSet-r19 (i.e. a certain RS in one LTM-NZP-CSI-RS-ResourceSet-r19 is activated/deactivated by the new MAC CE).

Proposal 7: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch.

Reference

- LS on activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells, R1- 2501500

- Consolidated Rel-19 higher layers parameter list Post RAN1#120bis, R1-2503155