3GPP TSG-RAN WG2 Meeting #130                                                        R2-2503432

St.Julians, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Discussion on the Control Plane Procedures

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

In the past RAN2 meetings, the control plane of multi-hop Relay were discussed. In this contribution, we will further discuss the following issues based on approach 1:

- How to handle RLF?

- Whether assistant information is needed for QoS split？

Discussion

RLF handling

With the introduction of multi-hop, there are three kinds of links as below:

- Sidelink 1: The sidelink between Remote UE and first Relay UE;

- Sidelink 2: The sidelink between Relay UEs;

- Uu link: The Uu link between the last Relay UE and gNB.

Figure-1  Multi-hop relay

When analyzing the RLF handling, we should discuss based on the above three kinds of links:

Table-1 RLF handling for multi-hop Relay

| Failed link | Legacy Rel-17 behaviors | Rel-19 behaviors |
| --- | --- | --- |
| Sidelink 1 | Remote UE detected PC5 RLF, key behaviors: - Release the PC5 Relay RLC channels between Remote UE and Relay UE; - Consider the PC5-RRC connection between Remote UE and U2N Relay is released; - If Remote UE is in CONNECTED, initiate RRC connection re-establishment. Relay UE detected PC5 RLF: - Release the PC5 Relay RLC channels between Remote UE and Relay UE; - Consider the PC5-RRC connection between Remote UE and U2N Relay is released; If Relay UE is in CONNECTED, send SUI to gNB. | Remote UE detected PC5 RLF: same as Rel-17 U2N Remote UE. First Relay UE detected PC5 RLF: same as Rel-17 U2N Relay UE. |
| Sidelink 2 | N/A | First Relay UE detected PC5 RLF: enhancement is possible. Send NotificationMessageSidelink message to Remote UE to indicate the failure between first Relay UE and Intermediate Relay (in case of more than 2 Relays) or between first Relay UE and last Relay UE (in case of only 2 Relays). Last Relay UE detected PC5 RLF: same as Rel-17 U2N Relay UE. |
| Uu link | Relay UE detected Uu RLF: - Send NotificationMessageSidelink message to remote UE. | Relay UE detected Uu RLF: enhancement is possible. - Send NotificationMessageSidelink message to its child Relay UE and its child relay UE should forward it the Remote UE. |

Based on the analysis in the above table, the following are proposed:

Proposal 1: In approach 1, when Remote UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Remote UE.

Proposal 2: In approach 1, when the first Relay UE detects PC5 RLF between the Remote UE and the first Relay UE, its behaviors are same as Rel-17 U2N Relay.

Proposal 3: In approach 1, when the first Relay UE detects PC5 RLF between the first Relay UE and Intermediate Relay/last Relay UE, it can send NotificationMessageSidelink to the Remote UE to indicate the failure between first Relay UE and Intermediate Relay/last Relay UE.

Proposal 4: In approach 1, when the last Relay UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Relay UE.

QoS split for multi-hop relay

Regarding to the QoS split, in RAN2#128 meeting, the following agreement was reached for approach 1:

Agreements:

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

Based on the above agreement, it should further discuss whether the assistance information from the UE(s) is needed in order to assistant the NW to perform QoS split.

In Rel-17 U2N Relay, the NW performs the QoS split based on its implementation, no UE assistant information is reported. In case of multi-hop, although the number of hops is higher than Rel-17 U2N relay, it is still unnecessary to report assistance information. The main reason is that considering both the remote UE and the Relay UE may be moving, the PC5 link quality between two UEs is not stable and can be changed dynamically. However, the QoS split is semi-static. Hence, the PC5 link channel quality is not very trustable when performing QoS split.

Proposal 5:  In approach 1, UE assistance information is not needed for QoS split.

Conclusion

According to the analysis in section 2, for Approach 1, it is proposed:

RLF handling

Proposal 1: In approach 1, when Remote UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Remote UE.

Proposal 2: In approach 1, when the first Relay UE detects PC5 RLF between the Remote UE and the first Relay UE, its behaviors are same as Rel-17 U2N Relay.

Proposal 3: In approach 1, when the first Relay UE detects PC5 RLF between the first Relay UE and Intermediate Relay/last Relay UE, it can send NotificationMessageSidelink to the Remote UE to indicate the failure between first Relay UE and Intermediate Relay/last Relay UE.

Proposal 4: In approach 1, when the last Relay UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Relay UE.

QoS split

Proposal 5:  In approach 1, UE assistance information is not needed for QoS split.

Reference

- R2-2502984 Report from session on positioning and sidelink relay	Session chair (MediaTek)	report