3GPP TSG-RAN WG2 Meeting #130	R2-2503453

St.Julians, Malta, 19th – 23rd May, 2025

Agenda Item:	8.7.4.2

Source: 	OPPO

Title:  	Discussion on DSR enhancements for XR

Document for:	Discussion and Decision

# Introduction

In Rel-19 XR WID [1] revised in RANP#105 meeting, the objectives of scheduling enhancements are updated as follows.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This contribution would like to provide our considerations on DSR enhancements for scheduling.

# Discussion on DSR enhancements

In the last RAN2 meetings, the following agreements were made. Here we further discuss the leftover issues.

RAN2#129 Agreements on DSR enhancements - One extension bit (e.g. by redefining the reserved R bit) can be used to indicate whether a further pair of remaining time and buffer size information is present for the associated LCG in the enhanced DSR MAC CE. - FFS New DSR MAC CE will (always) be used when at least one LCG is configured with multiple thresholds. - We do not support truncated DSR nor fallback to legacy DSR in case of limited PUSCH grant size. - Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold. - Triggering threshold is not used as a reporting threshold (but one of reporting thresholds can be configured to the same value as triggering threshold). - Do not support a configuration of an LCG without any triggering threshold but with DSR reporting threshold(s).

RAN2#129bis Agreements on DSR enhancements - To avoid the case that there is no delay-reporting data for DSR, at least one configured reporting threshold should be no lower than the DSR triggering threshold. This means we do not have to address “empty DSR” issue, but it is FFS whether we need to capture this restriction in specifications - During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one. - Clarify RAN2#128 agreement as “the UE may also support including non-delay-reporting data ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication” (the exact terminology to be discussed as part of CR review) - We will try to find a way to describe how the UE determines non-delay-reporting data ahead of delay-reporting data for delay-reporting data volume calculation. The aim is to have consistent UE behaviour to avoid fairness issues, but also consider different UE implementations. Agreements related to RRC CR - Introduce RRC configuration to enable/disable the inclusion of non-delay critical data ahead of delay critical data in the buffer size calculation for DSR - The maximum number of entries in the dsr-ReportingThresList is 4. Agreements related to PDCP CR - In PDCP specifications, use “PDU Set remaining time” to describe the shortest remaining time till discardTimer expiry among the PDCP SDUs belonging to the PDU Set. - Keep the text “The dsr-ReportingThresholds configured for the PDCP entity are ordered in ascending order.” in the RRC specification. - Specify delay-reporting indication in the PDCP specification. - Do not specify explicitly an update of delay-reporting indication in the PDCP. FFS (when discussing the CR) whether some explanation/note can be added to clarify that the UE should ensure the indicated data volume is up to date. Agreements on UE capabilities - RAN2 assumes that we define separate UE capabilities for different XR features (as in Rel-18 XR). - An optional UE capability with signalling (e.g. enhancedDelayStatusReport-r19) is introduced to indicate the support of enhanced delay status report of the buffered data associated with multiple thresholds. FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18. - For the above Rel-19 XR UE capabilities defined by RAN2 are per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

[UE Cap-1] The pre-requisite of enhanced delay status report

One open issue on UE capability is the pre-requisite for the capability of Rel-19 DSR, i.e., whether a UE supporting Rel-19 DSR shall also indicate support of delayStatusReport-r18 or not, as shown below.

Open issue 1: the pre-requisite of enhanced delay status report This was discussed in RAN2#129bis meeting (as shown below) and an FFS was captured: “FFS A UE supporting this feature shall also indicate support of delayStatusReport-r18.” - LGE, QCM thinks we do not need to couple R18 and R19 DSR. Vivo thinks reasonable UE will implement R18 if implements R19, but does not want to couple in specs. - Huawei does not think it is possible to support R19 without R18

In our understanding, the UE behavior for Rel-18 DSR is actually a special case of Rel-19 DSR. For example, for a LCG with Rel-19 DSR, if only one DSR-ReportingThreshold equal to remainingTimeThreshold-r18 is configured in dsr-ReportingThresList-r19, at the same time dsr-ReportNonDelayCriticalData-r19 is not enable, the UE behaviors for this LCG are fully same as Rel-18 DSR. Therefore, it is not possible for a UE only support Rel-19 DSR without Rel-18 DSR.

Although a proper UE implementation should support both Rel-18 and Rel-19 DSR together, but we still understand no need to couple them from UE capability signaling perspective in the specification.

- From UE implementation perspective, considering UE behaviour for Rel-18 DSR is actually a special case of Rel-19 DSR, it is not possible for a UE only support Rel-19 DSR without Rel-18 DSR.

- [UE Cap-1] From UE capability signalling perspective, no need to have the pre-requisite for the capability of Rel-19 DSR.

[PDCP-1, RLC-5] Control PDUs and retransmitted data

Another open issues on PDCP and RLC, as shown below, are how to deal with some Control PDUs and retransmitted data in data volume calculation mechanism, including PDCP control PDU, PDCP SDUs to be retransmitted, PDCP Data PDUs to be retransmitted, RLC data PDUs that are pending for retransmission and RLC STATUS PDU.

# 5. Open issues The following editor’s NOTE have been kept in the current running CR. It is asked for companies to discuss following open issues during the RAN2#130 meeting. Proposal: Discuss the following issues in the RAN2#130 meeting. - Issue 1: It is FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted. - Issue 2: It is FFS for which PDCP SDU the UE shall trigger remaining-time-based RLC retransmission and polling, if pdu-SetDiscard is configured.

Index; Issue description; Rapporteur suggestion. RLC-5; Editor’s Note: Same as PDCP open issue: it is FFS which delay-reporting RLC data volume shall consider RLC data PDUs to be retransmitted. Editor’s Note: It is FFS which delay-reporting RLC data volume shall consider STATUS PDU to be transmitted.; Issue Type: Essential How to address it: based on companies’ contribution.

In our understanding, there are two options put on the table, i.e., both PDCP and RLC consider Control PDU and retransmitted data into the data volume calculation of either

- Option-1: the first reported portion in DSR, or

- Option-2: the first configured portion in DSR.

We need to analyze whether additional effort is needed or not for both Option-1 and Option-2.

With the current DSR mechanism, the DSR report in MAC layer is per LCG which includes multiple LCHs associated to different PDCP/RLC entities, and data volume calculation in PDCP/RLC layer is per PDCP/RLC entity.

If Option-1, i.e., first reported portion, is adopted, additional effort is needed for cross-layer/entity interaction. For example,

1/ if LCHs within one LCG have delay-reporting data but the first reported portions of these LCHs are different, it is problematic on which portion to capture Control PDU and retransmitted data. At least there are two different interpretations, i.e., either the first reported portion of each LCH or the first reported portion of whole LCG, where the latter case introduces the dependency between different PDCP/RLC entities during data volume calculation.

2/ for the LCH within one LCG but does not have delay-reporting data, i.e., only contains Control PDU and retransmitted data, it is also problematic on which portion to capture Control PDU and retransmitted data. The potential approaches include either it is captured in the first reported portion of the whole LCG which also requires the interaction between different PDCP/RLC entities during data volume calculation, or it is not calculated during data volume calculation which leads to missing data volume.

- If first reported portion adopted, additional effort is needed for cross-layer/entity interaction for the cases including
1/ LCHs within one LCG have delay-reporting data but the first reported portions of these LCHs are different;
2/ LCH within one LCG but does not have delay-reporting data.

If Option-2, i.e., first configured portion, is adopted, the only small issues are that in case of no delay-reported data associated to the first configured portion, the first configured portion still needs to be present in DSR MAC CE to capture Control PDU and retransmitted data if any, which leads to 2-octet signaling overhead per LCG for DSR MAC CE, and needs to consider how to set its remaining time without the delay-reporting data.

Therefore, we propose to go for Option-2, i.e., both PDCP and RLC consider Control PDU and retransmitted data into the data volume calculation of the first configured portion of the associated LCG.

As discussed in [4], in case of only PDCP/RLC Control PDUs and/or retransmission data without delay-reported data contained in the first configured portion, we understand that the following alternatives to set the remaining time field in the DSR MAC CE may be considered:

- Alt-1: Set the remaining time field to a reserved value (e.g., 0);

- Alt-2: Set the remaining time field to the smallest reporting threshold (i.e., dsr-ReportingThreshold with i=1).

- [PDCP-1, RLC-5] For Rel-19 DSR data volume calculation, all PDCP/RLC control PDUs and retransmission data should always be considered into the first configured portion of the associated LCG.

- [PDCP-1, RLC-5] In case of only PDCP/RLC Control PDUs and/or retransmission data without delay-reported data contained in the first configured portion of the associated LCG, RAN2 discusses the alternatives to set its remaining time field in the DSR MAC CE:
Alt-1: A reserved value (e.g., 0);
Alt-2: The smallest reporting threshold (i.e., dsr-ReportingThreshold, i=1).

[MAC-03] DSR cancellation in case of DC

One open issue on MAC is [MAC-03] DSR cancelation in case of DC.

In RAN2#129 meeting, DSR cancellation issue in case of DC was raised for Rel-18 XR with Option-1/2/3/4 [2]. After the online/offline discussion [3], since companies have different understanding on whether the current DSR cancellation conditions allow the UE to cancel the pending DSR based on data being sent in another MAC entity, no clarification for Rel-18 DSR is made, but it is open to make specifications clearer for DSR in Rel-19.

As in PHR shown below, a lot of events to trigger a PHR comes from “any MAC entity”. Although the current MAC specification is written from MAC entity perspective, it still allows to perform action based on other MAC entities, i.e., cross-MAC entity interaction has been supported.

### 5.4.6 Power Headroom Reporting The Power Headroom reporting procedure is used to provide the serving gNB with the following information: - Type 1 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell; - Type 2 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH and PUCCH transmission on SpCell of the other MAC entity (i.e. E-UTRA MAC entity in EN-DC, NE-DC, and NGEN-DC cases); - Type 3 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for SRS transmission per activated Serving Cell; - MPE P-MPR: the power backoff to meet the MPE FR2 requirements for a Serving Cell operating on FR2; - DPC: the adjustment to maximum output power for a given power class for a Serving Cell operating on FR1; - DPCBC: the adjustment to maximum output power for a given power class for a Band Combination operating on FR1. ( omit some texts ) A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5]. - phr-PeriodicTimer expires; - upon configuration or reconfiguration of the power headroom reporting functionality by upper layers, which is not used to disable the function; - activation of an SCell of any MAC entity with configured uplink of which firstActiveDownlinkBWP-Id is not set to dormant BWP; - activation of an SCG; - addition of the PSCell except if the SCG is deactivated (i.e. PSCell is newly added or changed); - phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink: - there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell. - Upon switching of activated BWP from dormant BWP to non-dormant DL BWP of an SCell of any MAC entity with configured uplink

- Although the current MAC specification is written from MAC entity perspective, cross-MAC entity interaction has been supported.

In the current DSR procedure, the cancellation mechanisms of a DSR are specified in MAC specification as shown below.

DSR cancellation: A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

From UE point of view, it is reasonable to cancel DSR if a MAC PDU contains all SDUs associated with the DSR is transmitted from MAC entity itself or other MAC entity. Meanwhile, we understand the current specification, as highlighted above, does not restrict that the MAC PDU is transmitted by the MAC entity itself or other MAC entity. Up to UE implementation, if UE’s MAC entity has got the trigger internally, e.g., from the other MAC entity or from PDCP layer, the MAC entity shall cancel the pending DSR.

- [MAC-03] RAN2 confirms that the MAC entity cancels a pending DSR in case that all PDCP SDUs associated with the DSR is transmitted by its own or other MAC entity.

Regarding the MAC specification, we prefer to keep the current MAC specification on DSR cancellation as it is. But, if majority of companies prefer to make it clearer in Rel-19 DSR, we prefer to go for Option-2 in [2], i.e.,

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted by any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.

- [MAC-03] For DSR cancellation in case of DC in Rel-19, RAN2 chooses either one between the following two alternatives 
Alt-1: No MAC specification change;
Alt-2: Option-2 in R2-2500894, i.e., the MAC entity shall cancel a pending DSR when a MAC PDU is transmitted by any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.

# 3. Conclusion

Based on the discussion above, we made the following observations and proposals:

[UE Cap-1] The pre-requisite of enhanced delay status report

- From UE implementation perspective, considering UE behaviour for Rel-18 DSR is actually a special case of Rel-19 DSR, it is not possible for a UE only support Rel-19 DSR without Rel-18 DSR.

- [UE Cap-1] From UE capability signalling perspective, no need to have the pre-requisite for the capability of Rel-19 DSR.

[PDCP-1, RLC-5] Control PDUs and retransmitted data

- If first reported portion adopted, additional effort is needed for cross-layer/entity interaction for the cases including
1/ LCHs within one LCG have delay-reporting data but the first reported portions of these LCHs are different;
2/ LCH within one LCG but does not have delay-reporting data.

- [PDCP-1, RLC-5] For Rel-19 DSR data volume calculation, all PDCP/RLC control PDUs and retransmission data should always be considered into the first configured portion of the associated LCG.

- [PDCP-1, RLC-5] In case of only PDCP/RLC Control PDUs and/or retransmission data without delay-reported data contained in the first configured portion of the associated LCG, RAN2 discusses the alternatives to set its remaining time field in the DSR MAC CE:
Alt-1: A reserved value (e.g., 0);
Alt-2: The smallest reporting threshold (i.e., dsr-ReportingThreshold, i=1).

[MAC-03] DSR cancellation in case of DC

- Although the current MAC specification is written from MAC entity perspective, cross-MAC entity interaction has been supported.

- [MAC-03] RAN2 confirms that the MAC entity cancels a pending DSR in case that all PDCP SDUs associated with the DSR is transmitted by its own or other MAC entity.

- [MAC-03] For DSR cancellation in case of DC in Rel-19, RAN2 chooses either one between the following two alternatives 
Alt-1: No MAC specification change;
Alt-2: Option-2 in R2-2500894, i.e., the MAC entity shall cancel a pending DSR when a MAC PDU is transmitted by any MAC entity and this MAC PDU includes all the PDCP SDUs associated with the DSR.

# 4. Reference

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia

[2] R2-2500894, Discussion on DSR cancellation in DC, Huawei, HiSilicon

[3] R2-2501350, Report of [AT129][014][XR] DSR cancellation (Huawei) , Huawei, HiSilicon

[4] R2-2502181, On Data Volume Calculations for Rel-19 DSR, Apple, CATT, LG Electronics, OPPO, Samsung, Xiaomi