3GPP TSG-RAN2 Meeting #130	R2- 2503477

# St.Julians, Malta, May 19th – 23rd, 2025

Agenda item:		8.11.2 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Remaining issues on Random Access procedure for SBFD

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 SBFD[1], it is agreed to support the Random Access procedure using the SBFD symbols.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - … - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - …

In RRC and MAC running CR discussion [2-3], following open issues are remained:

RRC running CR [Editor’s note: where to place sbfd-RACH-SingleConfig-r19 is FFS] [Proposal for RRC-2] RAN2 to decide whether ” Early UL synchronization with an LTM candidate cell” and/or ”RACH-based LTM” can be supported in SBFD symbols and the RRC signallings if to support. (based on companies’ meeting contributions, 11/16 companies prefer not to support.) MAC running CR Open issue 1: Solicit comments on the preference between: Option 1) current wording (i.e., “satisfies rsrp-ThresholdSBFD”), given that RRC captures the satisfying conditions, i.e., ‘below’/’above’ the threshold. Option 2) explicitly capture two conditions, i.e., above/below the threshold based on above/below signalling, in MAC spec. Open issue 2: Solicit input on whether it is acceptable from RAN2 point of view to support SBFD RO for CSI-RS based CFRA. We can consider proposing a RAN2 working assumption, based on the discussion result. Open issue 3: Solicit input focusing on whether RA-RNTI collision issue should be addressed or not in RAN2. Open issue 4: Solicit input for whether RA resource set reselection can be performed or not after RO type switching. Open issue 5: Solicit input for whether Msg1 repetition number fallback can be supported with SBFD RO. Open issue 6: Solicit input for whether RO type signalling is supported by LTM cell switch command MAC CE or not.

This document discusses highlighted open issues on RA procedure for SBFD.

2.	Discussion

Signaling of RACH configuration Option 1

For RACH configuration Option 1, in RAN1#119 Meeting [4], it is agreed to indicate whether the SBFD RA procedure is available or not using an explicit indication.

Agreement Introduce explicit indications (1-bit SIB1 indication and/or dedicated signalling to convey cell-specific configuration) to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not from network side.

Based on the above, in updated RAN1 parameter list [5], sbfd-RACHSingleConfig IE is defined in order to indicate whether RACH configuration Option 1 is enabled.

On the other hand, according to the current RAN2 signaling structure for RA resource configuration, there are multiple RACH configurations based on triggering feature.

- RACH configuration included in directly within BWP configuration (i.e., RACH-ConfigCommon IE and MsgA-ConfigCommmon IE in BWP-UplinkCommon), which includes RA resource not associated with any feature (blue-colored)

- RACH configurations included in additionalRACH-ConfigList, for separated RACH configurations for feature or feature combinations (purple-colored)

Therefore, it should be discussed whether the explicit indication (i.e., sbfd-RACHSingleConfig) is indicated per RACH configuration, per BWP, or per cell.

In Rel-17, additionalRACH-ConfigList is defined in order to configure the separated RACH occasion for feature/feature combination (e.g., SDT/RedCap/…). Therefore, it would cause network restriction to configure the additional RACH occasion for feature or feature combination, if the SBFD RACH configuration Option 1 is available for all ROs configured in the cell. In addition, for some feature/feature combinations, SBFD RO may not be needed, i.e., no need to allow SBFD RACH configuration Option 1 for some additional RACH configuration associated with features. Therefore, it would be better to allow the network flexibility to configure Random Access resources, using the explicit indication configured for each RACH configuration.

Proposal 1. The explicit indication to indicate whether RACH configuration Option 1 is enabled or not (i.e., sbfd-RACHSingleConfig) is indicated per RACH configuration (e.g., in RACH-ConfigCommon IE).

RA resource selection procedure for initial transmission

In RAN2#129 and RAN2#129bis meeting [2-3], it was discussed on which step RO type should be selected, and it was agreed as a working assumption to select the RO type prior to the selection of the set of Random Access resources:

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

In our understanding, the main reason for this way forward is to simplify the RA resource selection procedure considering separated RSRP threshold to determine Msg1 repetition number in SBFD RO. Otherwise, additional complexity on UE operation and specification is expected due to selecting two sets of Random Access resources based on the separated threshold, two RA type selection, and re-determination of feature combination based on the selected RO type. Given that current RA procedure with RACH partitioning framework is already complicated, the additional complexity of RA resource selection should be minimized as much as possible.

Since the same issue was discussed for two meetings already and all the pros and cons are identified, there is no need to duplicate the whole discussion. In this sense, it is suggested to confirm the above working assumption.

Proposal 2. Confirm the working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

CSI-RS based CFRA support

During the RAN2#129bis meeting[6], it was briefly discussed on whether CSI-RS based CFRA can be supported in SBFD symbol, for handover and BFR cases. However, due to different understanding on RAN1/4 impact, it was remained as an open issue in order to allow some time for internal checking with other WGs.

R2-2502510 Remaining issues for RACH in SBFD Apple discussion Rel-19 NR_duplex_evo-Core Noted Proposal 7: RAN2 should consult with RAN1 and RAN4 whether to support CSI-RS based CFRA on SBFD symbols. DISCUSSIOS - OPPO think R1 already agreed CSI-RS meas on SBFD symbol - LG E think this issue is valid, and this requires more discussions in R1/R4 regarding measurement and reporting. - Ericsson think we do not need to discuss this, and it is up to UE implementation which is quite simple. - ZTE think this concern is valid but think we can FFS and do not think a LS is needed. Xiaomi agree. - Apple think current running CR already covers CSI-RS based RACH so this is not right. Apple encourages companies to do checking internally and can discuss in the future. - HW has sympathy on Apple comments and think MAC spec editor can remove this related part if that is controversial.

Based on the internal check with our RAN1/4 colleagues, there is no identified issue from RAN1 perspective. From RAN4 point of view, there are just a couple of possible impacts of CSI-RS measurement on SBFD symbol for RA resource selection as follows, which are not so critical:

- The CSI-RSRP for CSI-RS in SBFD symbol is same as CSI-RSRP for CSI-RS in non-SBFD symbol, although the accuracy of CSI-RSRP for CSI-RS in SBFD symbol may be different.

- Given that UE measures CSI-RS occasion on the valid symbol type indicated by network, RAN4 is discussing on extension of measurement period for CSI-RS based L1-RSRP [R4-2504904], which may cause a latency issue for CSI-RS measurement.

In addition, given that the CSI-RS based CFRA is an optional feature, it could be handled by the proper network configuration e.g., by not configuring CSI-RS resources if it is not appropriate to perform CSI-RS based CFRA in SBFD symbol for the UE. In other words, any impact on CSI-RS based CFRA procedure can be handled by the network anyway.

Therefore, CSI-RS based CFRA using SBFD symbol can be supported from RAN2 perspective, which can be optionally configured by the network. If it is really needed to check with other WGs, it is suggested to agree with CSI-RS based CFRA in SBFD symbol from RAN2 point of view, and send an LS to RAN4 whether there is any issue to support CSI-RS based CFRA.

Proposal 3. From RAN2 perspective, CSI-RS based CFRA in SBFD symbol is supported. Send an LS to RAN4 whether there is any issue to support CSI-RS based CFRA, if needed.

RO type switch details

For re-attempt in one random access procedure, it is agreed to support the RO type switching from SBFD RO to legacy RO, after the certain number of RA attempt using SBFD RO[7]:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

In addition, in RAN2#129bis meeting, it is agreed to support the RO type switching from legacy RO to SBFD RO as well.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

Regardless of the direction of RO type switch, as listed in the open issue, it should be discussed how to select the set of Random Access resources (i.e., RACH partition) and RA type after the RO type switch.

According to the current MAC procedure, any fallback from feature combination A to feature combination B is not supported during the same Random Access procedure. Even though when the fallback from low repetition number to high repetition number is supported in Rel-18, it was allowed only within same feature combination, in order to minimize additional complexity of RA procedure with RACH partitioning framework.

If the feature combination is changed during the Random Access procedure, additional UE complexity on whole procedure is expected, given that current RACH partitioning framework is defined for unified procedure with different features.

For example, assume that the network configures RACH partition for RA-SDT as follows, due to RA resource limitation in SBFD RO:

- In legacy RO,

  - RACH partition 1: common RACH partition (i.e., RACH partition not associated with any feature)

  - RACH partition 2: RA-SDT (i.e., RA resource to perform RA-SDT)

- In SBFD RO,

  - RACH partition 3: common RACH partition

In this case, if the SBFD-aware UE initiated for RA-SDT and selects the legacy RO, RA-SDT partition (i.e., RACH partition 2) can be selected and RA-SDT can be performed. After that, if the SBFD-aware UE switches to SBFD RO based on the number of RA attempt and re-selects the RACH partition, the SBFD-aware UE would select the common RACH partition (i.e., RACH partition 3, without RA-SDT indication) and the SBFD-aware UE needs to perform the legacy RRC resume procedure (i.e., fallback from RA-SDT to legacy RRC resume procedure). However, since the SDT procedure requires interaction between MAC layer and RRC layer (due to some different RRC procedures for SDT, e.g., resuming SDT bearer, T319a handling, etc), fallback from SDT procedure to legacy RRC resume procedure is not simple and needs lots of further discussion. Note that it was extensively discussed whether to support the fallback from SDT procedure to non-SDT procedure in Rel-17, but it was not adopted due to additional complexity.

On the other hand, based on unified procedure to select the set of Random Access resources and feature priority handling, it is also not easy to define any special handling for any specific feature, e.g., allowing Msg1 repetition re-selection while not allowing SDT to non-SDT fallback. In addition, given that Msg1 repetition number is selected during the selection of the set of Random Access resources, it would be hard to re-select only the Msg1 repetition number, e.g., based on the RSRP threshold, while the same feature combination is selected.

Therefore, in order to avoid any additional complexity to change the feature combination during the same Random Access procedure and to keep the same principle as in Rel-17 RACH partitioning framework, it is proposed to select the same feature combination and Msg1 repetition number after the RO type switch, i.e., no re-selection of the set of Random Access resources. In other words, the RO type switch is allowed only if the set of Random Access resources with the same feature combination with the same Msg1 repetition number is configured for the different RO type.

Proposal 4. For PRACH transmission re-attempt in one RA procedure, UE is allowed to switch between SBFD RO and non SBFD-RO only in the same feature combination and the same repetition number.

Similarly, for RA type selection, it is not needed to re-select the RA type after the RO type switch, i.e., 4-step RA will be performed even though the RO type is switched from SBFD RO to legacy RO.

Proposal 5. After the RO type switch, same RA type is selected for the current Random Access procedure, i.e., no re-select the RA type upon RO type switch.

Msg1 repetition with SBFD RO

During the MAC running CR review, following open issue is remained on whether/how Msg1 repetition number fallback can be supported with SBFD RO:

Open issue 5: Solicit input for whether Msg1 repetition number fallback can be supported with SBFD RO.

In the current MAC procedure, increasing Msg1 repetition number (i.e., fallback from low Msg1 repetition number to high Msg1 repetition number) is supported within the same feature combination.

Given that the fallback from low Msg1 repetition number to high Msg1 repetition number is supported in order to increase the likelihood of successful RA attempt, it is also beneficial to support the fallback from low Msg1 repetition number to high Msg1 repetition number in SBFD RO as well. There is no reason to restrict the Msg1 repetition number fallback only in legacy RO.

Regarding the details for the Msg1 repetition number fallback for SBFD RO, same procedure can be reused for SBFD RO, i.e., by applying the criteria to increase the Msg1 repetition number based on the existing number threshold. Then, even though RO type switch is supported based on the number threshold, it would be simply implemented with Msg1 repetition fallback if the criteria to switch the RO type is applied independently from the criteria of Msg1 repetition number fallback.

For example, assume that the number threshold to increase the Msg1 repetition number (i.e., preambleTransMax-Msg1-Repetition) is set to n4 and the number threshold to switch the RO type (i.e., preambleTransMaxRO-Type) is set to n10), and the sets of Random Access resources for Msg1 repetition number 2, 4, 8 are configured in SBFD RO and legacy RO, respectively, within the same RACH configuration (i.e., shared RO case). Then, if the SBFD-aware UE selects the SBFD RO and Msg1 repetition partition with Msg1 repetition number = 2 at the initial RA attempt,

- When the PREAMBLE_TRANSMISSION_COUNTER = 5 (i.e., [preambleTransMax-Msg1-Repetition] + 1), the SBFD-aware UE selects the SBFD RO and Msg1 repetition partition with Msg1 repetition number = 4 (i.e., increases Msg1 repetition number in SBFD RO);

- When the PREAMBLE_TRANSMISSION_COUNTER = 9 (i.e., 2 × [preambleTransMax-Msg1-Repetition] + 1), the SBFD-aware UE selects the SBFD RO and Msg1 repetition partition with Msg1 repetition number = 8 (i.e., increases Msg1 repetition number in SBFD RO); and

- When the PREAMBLE_TRANSMISSION_COUNTER = 11 (i.e., [preambleTransMaxRO-Type] + 1), the SBFD-aware UE selects the legacy RO and Msg1 repetition partition with Msg1 repetition number = 8 (i.e., RO type switch);

Proposal 6. Support the Msg1 repetition number fallback, i.e., fallback from low Msg1 repetition number to high Msg1 repetition number in SBFD RO.

Regarding the order of RO type switch and Msg1 repetition number fallback, the result of the fallback procedure will be same in general, unless the number threshold of the RO type switch is same as the number threshold of Msg1 repetition number fallback (i.e., preambleTransMax-Msg1-Repetition) or two times of the number threshold of Msg1 repetition fallback (i.e., 2 × [preambleTransMax-Msg1-Repetition]).

For this specific case, in order to identify any difference on the order of each fallback, the current procedure for Msg1 repetition number fallback should be reviewed. According to current MAC procedure, increasing Msg1 repetition number (i.e., fallback from low Msg1 repetition number to high Msg1 repetition number) is supported within the same feature combination as follows[8]:

5> if set of Random Access resources configured with the same prach-ConfigurationIndex and associated with a higher Msg1 repetition number with the same feature or feature combination as the current set of Random Access resources is available: 6> select the set of Random Access resources associated with the next higher Msg1 repetition number with the same feature or feature combination for this Random Access procedure; 6> initialize startPreambleForThisPartition, numberOfPreamblesPerSSB-ForThisPartition, numberOfRA-PreamblesGroupA and msg1-RepetitionTimeOffsetROGroup parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources.

In other words, there are couple of conditions to support the fallback from low Msg1 repetition number to high Msg1 repetition number are as follows:

- The fallback from low repetition Msg1 repetition number to high Msg1 repetition number is supported only in shared RO case (as in blue-colored condition). In this sense, after increasing the Msg1 repetition number, only the preamble information and Msg1-repetition specific parameters are re-initialized (e.g., ssb-perRACH-OccasionAndCB-PreamblesPerSSB is NOT initialized again since it will be same for shared RO case). It is the intended behaviour, as agreed in RAN2#123bis meeting as follows[9]:

- Separate RO for different number is supported; - Fallback from lower number to higher number is performed within the selected set of RACH resources. - Alt1: Fallback is only supported for sharedRO case

- The fallback from low repetition Msg1 repetition number to high Msg1 repetition number is supported only in the same feature combination (as in purple-colored condition).

Based on the current condition to support Msg1 repetition number fallback, depending on the order of Msg1 repetition fallback, the result of the Msg1 repetition number after the RO type switch will be different.

For example, assume that RACH partition is configured as follows, and both the number threshold of the RO type switch and the number threshold of Msg1 repetition number fallback is configured as n4 (i.e., preambleTransMaxRO-Type = preambleTransMax-Msg1-Repetition = 4):

- In SBFD RO,

  - In RACH configuration 1:

    - RACH partition A1: common RACH partition (i.e., RACH partition not associated with any feature)

    - RACH partition A2: Msg1 repetition with repetition number 2

  - In RACH configuration 2:

    - RACH partition A3: Msg1 repetition with repetition number 4

- In legacy RO,

  - In RACH configuration 3:

    - RACH partition B1: common RACH partition

    - RACH partition B2: Msg1 repetition with repetition number 2

    - RACH partition B3: Msg1 repetition with repetition number 4

    - RACH partition B4: Msg1 repetition with repetition number 8

Then, if the UE selects the SBFD RO and selects the Msg1 repetition partition with Msg1 repetition number 2 (i.e., RACH partition A2) for initial RA attempt (based on RSRP), after 4 RA attempts, the UE checks the condition for RO type switch and Msg1 repetition number fallback. Then based on the order of each fallback, the UE will select different partition as follows:

- If the UE first performs RO type switch, UE first switch the SBFD RO to legacy RO and then checks the condition of the Msg1 repetition number fallback. Since the RACH partition B2 and RACH partition B3 are configured within the same RACH configuration (i.e., shared RO), the UE will be able to increase the Msg1 repetition number after the RO type switch.

- If the UE first performs Msg1 repetition number fallback, UE checks the condition of the Msg1 repetition number fallback before the RO type switch. Since the RACH partition A2 and RACH partition A3 are not in the same RACH configuration (i.e. NOT shared RO case), the UE is not able to increase the Msg1 repetition number. Then the UE will perform the RO type switch and then selects the same Msg1 repetition number.

Although it is not a general case to configure two number thresholds as a same value, but it is not reasonable to prevent the Msg1 repetition number fallback due to RACH partition which will never be used after the RO type switch (i.e., RACH partition A3). In addition, since the RO type selection is performed prior to Msg1 repetition number selection, it seems straightforward to switch the RO type first and then check whether the Msg1 repetition number can be increased.

In this sense, it is slightly preferred to switch the RO type and then determine the Msg1 repetition number fallback.

Proposal 7. RO type switch is performed prior to Msg1 repetition number fallback.

RA-RNTI handling

In RAN2#129bis meeting, it was discussed online on whether any specific handling is needed on RA-RNTI for RACH configuration Option 2. However, even though several companies would like to handle RA-RNTI issue, e.g., by adding an offset on existing RA-RNTI equation (e.g., in s_id, f_id, etc), it was not agreed since more companies would like to leave up to the network implementation.

In our understanding, RA-RNTI collision is not new issue, since the multiple RACH configuration is already supported in various features. Especially, in the RACH partitioning framework defined in Rel-17, RA-RNTI issue was extensively discussed for two meeting, since up to 16 additional RACH configuration is supported for various feature/feature combinations [10-11]. However, although various methods are proposed to alleviate RA-RNTI collision issue[12-14], it was concluded to leave up to the network implementation since it is the network’s responsibility to consider the possible RA-RNTI collision as well when the network configures the separated RACH configuration.

Therefore, given that only two meetings left for SBFD WI and this issue is already discussed in last meeting, there is no critical need to duplicate the discussion for RA-RNTI optimization.

Proposal 8. It is up to network implementation to handle the RA-RNTI collision issue for RACH configuration Option 2.

3.	Conclusion

In this contribution, it is discussed about RAN2 aspects on Random Access operation for SBFD. This document includes following proposals.

Proposal 1. The explicit indication to indicate whether RACH configuration Option 1 is enabled or not (i.e., sbfd-RACHSingleConfig) is indicated per RACH configuration (e.g., in RACH-ConfigCommon IE).

Proposal 2. Confirm the working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

Proposal 3. From RAN2 perspective, CSI-RS based CFRA in SBFD symbol is supported. Send an LS to RAN4 whether there is any issue to support CSI-RS based CFRA, if needed.

Proposal 4. For PRACH transmission re-attempt in one RA procedure, UE is allowed to switch between SBFD RO and non SBFD-RO only in the same feature combination and the same repetition number.

Proposal 5. After the RO type switch, same RA type is selected for the current Random Access procedure, i.e., no re-select the RA type upon RO type switch.

Proposal 6. Support the Msg1 repetition number fallback, i.e., fallback from low Msg1 repetition number to high Msg1 repetition number in SBFD RO.

Proposal 7. RO type switch is performed prior to Msg1 repetition number fallback.

Proposal 8. It is up to network implementation to handle the RA-RNTI collision issue for RACH configuration Option 2.

4.	Reference

[1] RP‑241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] RRC running CR for Evolution of NR duplex operation (SBFD)

[3] R2-2504444 MAC running CR for Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[4] Report of 3GPP TSG RAN WG1 #119

[5] R1-2503155 Consolidated Rel-19 higher layers parameter list Post RAN1#120bis

[6] Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China, 7 - 11 April, 2025

[7] Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024

[8] TS38.321 v.18.5.0

[9] R2-2311701 Report of 3GPP TSG RAN WG2 meeting #123bis, Xiamen, China

[10] R2-2200049 [Post116-e][515][RACH partitioning] MAC Procedure aspects (ZTE)

[11] R2-2203736 Report of [AT117-e][506] [RA Part] UP additional open issues (Intel)

[12] R2-2110560 RNTI collision problem for Rel-17 features

[13] R2-2200206 Preamble and RACH resource configuration

[14] R2-2200917 RNTI collision issue for different features in NR