3GPP TSG-RAN WG2 Meeting #130	R2-2503478

Malta, Malta, May. 19th – May. 23th, 2025

Source: 	NTU

Title:  	Further discussion on CB-Msg3 and Msg-4 enhancement

Agenda Item:	8.9.3

Document for:	Discussion and Decision

- Introduction

At RAN2 #129bis, the configuration of the Msg4 monitoring window was discussed, and the following agreements were reached.

Agreements (part 2):

1.	For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window).

2.	The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure)

3.	The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

4.	FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

For CB-Msg3 EDT, a Single Msg4 monitoring window and a Single RNTI are adopted. However, whether it is possible to start after the transmission of the first replica plus UE-eNB RTT is an FFS. In the proposal, we will further discuss the issue.

- Discussion

  - UL/DL collision problem

In CB-Msg3 EDT, there are two possible choices for the UE to start the msg4 monitoring window. One is starting at the end of CB-Msg3 EDT transmission window plus UE-eNB RTT, and the other is starting after the transmission of the first replica plus UE-eNB RTT. For the latter choice, it leads to a shorter delay for starting monitoring earlier. However, it also leads to the UL/DL collision problem.

Opting for the latter approach offers the advantage of a reduced delay in commencing monitoring, potentially leading to a faster completion of the random access procedure. However, this early start introduces the problem of a UL/DL collision. Specifically, if the UE begins its msg4 monitoring window after transmitting only the first replica, it might still be in the process of transmitting subsequent replicas. This overlap between uplink transmission and downlink monitoring creates a conflict that necessitates a prioritization decision by the UE. A clear trade-off arises when a UL/DL collision occurs. If the UE chooses to transmit a replica, it forfeits the chance to end the random access procedure early by receiving Msg4 for a previous successful replica. Conversely, if it opts to monitor for Msg4, it sacrifices the opportunity to send another replica.

To determine the optimal UE strategy, a key issue is understanding how Msg4 is designed on the eNB side. If the eNB is configured to respond for a replica only after the period of the replica has ended, then missing the Msg4 monitoring window results in the failure of a potentially successful previous replica. In this scenario, prioritizing transmission during a collision can directly make the replica useless. However, if the eNB instead responds to all UEs at the end of the DSA transmission window, then missing the monitoring window merely results in the loss of an opportunity for early termination—it does not invalidate a previously successful replica. In this case, the impact of prioritizing monitoring is less severe.

Observation 1: If the eNB responds to all UEs at the end of the DSA transmission window, it can prevent failures caused by UEs missing the Msg4 corresponding to a successful replica transmission.

  - Proposed method for Msg4 monitoring window configuration

To reduce delay and mitigate the impact of UL/DL collisions, a balanced approach is proposed. On the eNB side, Msg4 is transmitted after each replica reception period to inform all UEs that successfully accessed the network during that specific period. Additionally, at the end of the DSA transmission window, the eNB sends a Msg4 message covering all UEs that successfully accessed the network during the entire window, regardless of which replica was used. This strategy prevents scenarios where a UE successfully accesses the network using an earlier replica but fails to receive the corresponding Msg4 due to prioritizing the transmission of subsequent replicas. Taking Figure 1 as an example, when the first and second replica periods end, the eNB responds with Msg4 to UEs that successfully accessed during those specific periods. However, after the last replica period ends, the eNB must respond not only to the UE that accessed during that final period (e.g., UE “D”) but also to all UEs that successfully accessed within the entire DSA transmission window.

Figure 1: the design for the eNB side

On the UE side, to support early termination of the random access procedure, the Msg4 monitoring window can begin after the first replica's transmission plus the UE-eNB RTT. However, since only a single Msg4 monitoring window is adopted, and to avoid missing any opportunity for replica transmission, the monitoring window should be delayed until after the last replica is transmitted. Thus, the Msg4 monitoring window starts at the latest of the following two times:

- The subframe containing the last (N)PUSCH repetition of the first replica (tf) plus the UE-eNB RTT

- The subframe containing the last (N)PUSCH repetition of the last replica (tl)

That is, the msg4 monitoring window starts at time max⁡(tf+RTT,tl).

Figure 2: The msg4 monitoring window configuration

Figure 2 illustrates our proposed Msg4 window configuration in CB-Msg3 EDT. If the transmission of the last replica ends later than the first replica plus the UE-eNB RTT (top), monitoring is delayed until the last replica finishes to avoid UL/DL collision. As shown in the figure, the UE cannot receive an early response because it is still transmitting replicas. However, it can still receive the Msg4 sent after the entire DSA transmission window. If the last replica ends earlier than the first replica plus the RTT (bottom), monitoring can start right after the first replica plus RTT. In this case, the UE can receive the early response and reduce delay.

Proposal 1: On the eNB side, the eNB not only transmits Msg4 after each replica reception period to notify all UEs that have successfully accessed the network during that specific period, but also transmits Msg4 at the end of the DSA transmission window to inform all UEs that have successfully accessed the network during the entire DSA transmission window.

Proposal 2: On the UE side, the Msg4 monitoring window should begin at the latest of the following two time points:

- The subframe containing the last (N)PUSCH repetition of the first replica plus the UE-eNB RTT

- The subframe containing the last (N)PUSCH repetition of the final replica.

- Conclusions

Observation 1: If the eNB responds to all UEs at the end of the DSA transmission window, it can prevent failures caused by UEs missing the Msg4 corresponding to a successful replica transmission.

Proposal 1: On the eNB side, the eNB not only transmits Msg4 after each replica reception period to notify all UEs that have successfully accessed the network during that specific period, but also transmits Msg4 at the end of the DSA transmission window to inform all UEs that have successfully accessed the network during the entire DSA transmission window.

Proposal 2: On the UE side, the Msg4 monitoring window should begin at the latest of the following two time instants:

- The subframe containing the last (N)PUSCH repetition of the first replica plus the UE-eNB RTT

- The subframe containing the last (N)PUSCH repetition of the final replica.

Reference

[1] Chairman_notes of 3GPP TSG RAN WG2 meeting #129bis