3GPP TSG-RAN WG2 Meeting #130	                                 	                   R2-2503485

St. Julians, Malta, May 19th – 21st, 2025

Agenda Item:	8.5.3

Source: 	Lenovo

Title: 	Providing Cell Reselection Assistance information

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and decision

# Introduction

In the previous RAN2 meeting [RAN2#129bis] it was discussed if cell access related information should be included in the UL WUS configuration to avoid unnecessary SIB1 requests on a cell where UE will not have access. Apart from CellAccessRelatedInfo from SIB1 of the target cell, some other information could be useful to ensure that SIB1 Request is used only when the UE may also access the target cell. This documents looks into the details.

# Discussion

Below excerpts from [1] indicate that cell reselection happens in two phases:

5.2	Cell selection and reselection

5.2.1	Introduction

UE shall perform measurements for cell selection and reselection purposes as specified in TS 38.133 [8].

When evaluating Srxlev and Squal of non-serving cells for reselection evaluation purposes, the UE shall use parameters provided by the serving cell and for the final check on cell selection criterion, the UE shall use parameters provided by the target cell for cell reselection.

…

5.2.4.6	Intra-frequency and equal priority inter-frequency Cell Reselection criteria

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst - Qoffsettemp Rn = Qmeas,n -Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2.

5.2.4.7	Cell reselection parameters in system information broadcasts

5.2.4.7.0	General reselection parameters

Cell reselection parameters are broadcast in system information and are read from the serving cell

Based on the cited specification text it is clear that cell reselection procedure follows mainly two phases:

In the first (let’s call it “evaluation”) phase the UE shall use parameters provided by the serving cell. In the second phase (let’s call it “final check”), parameters (SIB1, SIB2) from target cell will be needed.

Observation 1: Reselection procedure follows mainly two steps: In the first phase (let’s call it “evaluation” phase) the UE shall use parameters provided by the serving cell. In the second phase (let’s call it “final check”), parameters (SIB1, SIB2) from target cell will be needed to check the cell selection criterion.

The SIB1 Request should be made after the first phase, for the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5 of [1]. We observe that this is in line with the agreements made so far to not use any new triggers for SIB1 request or change the reselection procedure.

Observation 2: Previous RAN2 agreements rightly assume that SIB1 Request is made after the first phase.

However, as observed in the previous meeting, it is possible that a target cell out of first phase may not provide access to the UE. This can be due to a number of reason like target cell being a barred cell for the UE. Let us next look at all such assistance information that can impact the decision to make SIB1 Request.

## What information can assist to minimize SIB1 Request

Here, mainly feature-specific barring like cellBarredFeature1 was mentioned, which is included (or absent) in SIB1. In the following we show other information equally (or even more) important to minimize SIB1 Request.

### MIB cellBarred

MIB cellBarred should be regarded as the most important barring information for deciding on OD-SIB1 Request for following reasons:

- Emergency Calls: The MIB cellBarred set to ‘barred’ prohibits even emergency calls. So, a SIB1 Request to a cell that can’t even support emergency calls is of no use.

- Even specific feature-devices need MIB cellBarred information. It is true that some features (e.g., NTN, ATG ) ignore the MIB cellBarred bit (ignored by IAB-MT and NCR-MT as well) but other features do need the MIB cellBarred bit in case “their” cellBarredFeature is not broadcasted in SIB1 e.g., cellBarredNES.

- A bulk of devices are not feature-specific devices. These rely on MIB cellBarred to determine if these will get service in a potential target cell. So, a SIB1 Request to a cell that can’t provide regular service is of no use.

Based on the above arguments, it should be clear that MIB cellBarred can be regarded as the most important barring information.

Observation 3: MIB cellBarred is the most important barring information for deciding on OD-SIB1 Request and as an example, there’s no use of requesting SIB1 from a target cell that can’t even support emergency call.

Does reading MIB cellBarred come for free?

The next question is if the MIB cellBarred should also be included in the WUS configuration (SIB1 Request config). One argument against this could be that a UE can anyway acquire the MIB (generally PBCH) during SSB measurement (i.e., in the first phase of reselection “evaluation”). Let’s examine this belief.

Figure 1: SSB Block

Though MIB/ PBCH is embedded around the synchronization signals (PSS/ SSS) as shown in Figure 1, acquiring MIB/ PBCH while evaluation (measurement in the evaluation phase) does not come for free esp. since the UE in the first phase might be evaluating many neighbouring cell. PBCH DMRS is only optionally (“may”) used for SS-RSRP (up to UE implementation), please check the excerpts from TS 38.215 below.

However, to reduce latency (delay due to waiting for another SSB burst), it is expected that UE uses PBCH DMRS. To extract PBCH DM RS REs, FFT is needed to every PBCH symbol (DM RS exists in every PBCH symbol). But since the system design can’t assume that all UEs use PBCH DMRS SS-RSRP measurements, it can’t be taken for granted that MIB/ PBCH is read by every UE (implementation) in the evaluation phase. It is therefore efficient to perform MIB and SIB1 acquisition (even in rel. 15) for only best cell or highest ranked cell on a frequency.

5.1.1	SS reference signal received power (SS-RSRP)

| Definition | SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable. For SS-RSRP determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in TS 38.213 [5]. If SS-RSRP is not used for L1-RSRP, the additional use of CSI reference signals for SS-RSRP determination is not applicable. SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches. |
| --- | --- |

For measurement reporting, UE should know an SSB index of the measured SSB. Thus, PBCH decoding is needed for the case of the max. SSB in a SSB burst being larger than 8. If SSB indices of the neighbor cell can be derived from the serving cell, PBCH decoding is not necessary.

Observation 4: Acquiring MIB/ PBCH during “evaluation” phase is not necessary/ mandatory and some UEs may not do it as it does not come for free.

Observation 5: It is more efficient to perform MIB and SIB1 acquisition (even in rel. 15) for only best cell or highest ranked cell on a frequency, and therefore as part of the “final check” (and not in evaluation phase).

Further, we note here that RAN1 has already agreed to include pdcch-ConfigSIB1 from MIB in the UL WUS configuration (since the MIB “might” include different coreset#0; searchSpace#0 for legacy UEs). In addition, RAN1 had already previously agreed to include KSSB in the UL WUS configuration since the KSSB in the MIB may just be used to indicate the OD-SIB1 scenario and therefore the real value of KSSB must be provided in the WUS Configuration. Please see below:

RAN1#120bis Agreement From RAN1 perspective, for agreed UL WUS parameters, regarding their mandatory or optional presence and applicability to TDD and/or FDD, adopt the followings: - PhysCellId and ARFCN-ValueNR are mandatory - frequencyBandList and absoluteFrequencyPointA are present in IE FrequencyInfoUL for FDD (as in the legacy specification) - K_SSB is mandatory - searchSpaceZero and controlResourceSetZero are mandatory - ra-PreambleStartIndex, od-sib1-duration, offsetToTimeWindow are mandatory

Observation 6: Including information from MIB/ PBCH in the UL WUS configuration is acceptable: RAN1 has already agreed to include some content of MIB/ PBCH (KSSB and pdcch-ConfigSIB1) in the UL WUS configuration, although for different reasons.

### However!

Why to provide WUS configuration (by a cell A) for a MIB barred (NES) neighbour cell, as anyway this cell can't provide service?

An NTN UE can camp on a cellBarred cell if the NTN barring included in SIB1 is notBarred. Even if the neighbour cell is an NTN Cell (NTN UEs ignore the MIB barring bit), including the MIB barring bit is actually not required since based on the cellBarredNTN-r17 in SIB1 the NTN UE will know that NTN access is allowed, assuming cellBarredNTN-r17 is included in the WUS configuration. So, a WUS Configuration should only be provided for a MIB notBarred cell; or an NTN Cell. Similar arguments can be made for IFRI bit.

Observation 7: A WUS Configuration should only be provided for a MIB notBarred cell; or an NTN Cell.

Observation 7 is indeed a network implementation and therefore, we make the following proposal:

Proposal 1: Including the cellBarred of the NES Cell's MIB in the UL WUS configuration is not necessary.

### Other information

Apart from barring info, providing some other information could be useful to restrict SIB1 request. Here, please see the following excerpts from [1].

5.2.4.4	Cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to one or more of the following reasons:

-	this cell belongs to a PLMN which is not indicated as being equivalent to the registered PLMN, or

-	this cell is a CAG cell that belongs to a PLMN which is equivalent to the registered PLMN but with no CAG-ID that is present in the UE's allowed CAG list being broadcasted, or

-	this cell is not a CAG cell and the CAG-only indication in the UE is set, or

-	this cell does not belong to a SNPN that is equal to or indicated as being equivalent to the registered or selected SNPN of the UE in SNPN access mode,

the UE shall not consider this cell and, for operation in licensed spectrum, other cells on the same frequency as candidates for reselection for a maximum of 300 seconds.

For operation with shared spectrum channel access, when the highest ranked cell or best cell is not a candidate for reselection per the previous paragraph, the UE should continue to consider other cells on the same frequency for cell reselection, however if the second highest ranked cell on this frequency is also not suitable due to one or more of the above reasons, the UE may consider this frequency to be the lowest priority for a maximum of 300 seconds.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of 5GS forbidden TAs for roaming", the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300 seconds.

Based on the above highlighted excerpts, one can easily see that the following (basically the CellAccessRelatedInfo from SIB1) can potentially render not suitable an otherwise highest ranked cell or best cell!

- PLMN ID(s)

- CAG list

- SNPN(s)

- Tracking Area

These can exceed 1000 bits ((24 bits TAC + 44 NID bits) *12 + other Info) and there is always the possibility (or need) to include further info cellReservedForOtherUse, cellReservedForFutureUse, imsEmergencySupportForSNPN and the following feature bits:

- cellBarredATG

- cellBarred-eRedCap1Rx

- Barred-eRedCap2Rx

- cellBarredNES

- cellBarredNTN

- cellBarred-RedCap1Rx

- cellBarred-RedCap2Rx

As this information will likely not be common across cells for which UL WUS configuration is being included in SIBXX, these need to be included separately for each cell and therefore this could be really huge (up to 8 times CellAccessRelatedInfo!) in each Cell A!! Therefore, RAN2 should carefully consider the energy saving aspect.

Observation 8: Providing exhaustive assistance information to minimize SIB1 request could be really huge (up to 8 times CellAccessRelatedInfo! + other bits) in each Cell A!

## What should RAN2 do?

RAN2 can use one of the following strategies:

- Provide all or none: So, all barring information + PLMN/ SNPN + …. Or nothing at all

- Provide a subset of information that can help some UEs at least and thus restrict SIB1 Request in some cases. The subset of information can include barring bits from SIB1

The second strategy above may not be widely helpful and in some cases just to a fraction of UEs. But it is based on the principle of “something is better than nothing”. On the other hand, the first strategy can be helpful if we can optimize signaling to avoid transmitting all PLMNs/ SNPNs/ TAs if all/ most of these are the same as in the current serving cell A, so use some kind of delta signaling.

Proposal 2: Include feature specific barring bits from SIB1.

Proposal 3: Other information (PLMN IDs, SNPNs, CAG List and TA) of the NES Cell's SIB1 in the UL WUS configuration can be provided to restrict SIB1 Request if signalling load is managed using e.g., delta signalling with respect to the serving cell.

# Conclusion

This doc

Observation 1: Reselection procedure follows mainly two steps: In the first phase (let’s call it “evaluation” phase) the UE shall use parameters provided by the serving cell. In the second phase (let’s call it “final check”), parameters (SIB1, SIB2) from target cell will be needed to check the cell selection criterion.

Observation 2: Previous RAN2 agreements rightly assume that SIB1 Request is made after the first phase.

Observation 3: MIB cellBarred is the most important barring information for deciding on OD-SIB1 Request and as an example, there’s no use of requesting SIB1 from a target cell that can’t even support emergency call.

Observation 4: Acquiring MIB/ PBCH during “evaluation” phase is not necessary/ mandatory and some UEs may not do it as it does not come for free.

Observation 5: It is more efficient to perform MIB and SIB1 acquisition (even in rel. 15) for only best cell or highest ranked cell on a frequency, and therefore as part of the “final check” (and not in evaluation phase).

Observation 6: Including information from MIB/ PBCH in the UL WUS configuration is acceptable: RAN1 has already agreed to include some content of MIB/ PBCH (KSSB and pdcch-ConfigSIB1) in the UL WUS configuration, although for different reasons.

Observation 7: A WUS Configuration should only be provided for a MIB notBarred cell; or an NTN Cell.

Proposal 1: Including the cellBarred of the NES Cell's MIB in the UL WUS configuration is not necessary.

Observation 8: Providing exhaustive assistance information to minimize SIB1 request could be really huge (up to 8 times CellAccessRelatedInfo! + other bits) in each Cell A!

Proposal 2: Include feature specific barring bits from SIB1.

Proposal 3: Other information (PLMN IDs, SNPNs, CAG List and TA) of the NES Cell's SIB1 in the UL WUS configuration can be provided to restrict SIB1 Request if signalling load is managed using e.g., delta signalling with respect to the serving cell.

# Reference

[1]: TS 38.304