3GPP TSG-RAN WG2 Meeting #130	                                     R2-2503487

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.6.3

Source: 	OPPO

Title:  	Open issues for event-triggered L1 measurement reporting

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss the remaining open issues for L1 event-triggered measurement reporting.

Discussion

MAC-5

| MAC-5 | The content of L1 MR MAC CE triggered by LTM2 Editor’s NOTE: FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

Event LTM2 is defined as the beam of serving cell becomes worse than threshold. For L1 event triggered measurement reporting, it is questionable whether the triggered MR MAC CE can include measurement of other candidate cells. The triggered MR MAC CE is aiming to assist NW to select the candidate beam/cell for early synchronization and select the target beam/cell for an LTM cell switch procedure. If the measurement report contains only the measurements of serving cell, NW may not be able to select the candidate cell/beam based on only serving cell measurements. Thus, even for MR triggered by LTM2, the MR MAC CE can also include the measurements for LTM candidates.

For MR triggered by LTM2, the MR MAC CE can include measurements for LTM candidates.

MAC-6

| MAC-6 | Truncated L1 MR MAC CE details. E.g. Except the triggered beam, whether any beam should be prioritised to be included in the truncated L1 MR MAC CE? In case the UL grant could include at least 2 beams in truncated L1 MR MAC CE, which beam should be included as the first beam in the truncated L1 MR MAC CE? Based on following agreement: - For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

Another issue is about the truncated MR MAC CE format, i.e. whether to put triggered beam measurement at the beginning of the MR MAC CE. As we agreed, differential RSRP is used to indicate the L1-RSRP measurement result for MR MAC CE, which implies the first beam in the MR MAC CE is the one with the highest measurement quantity.

While another agreement on truncated MR MAC CE has also been agreed. That is, the truncated MR MAC CE include at least one triggered beam measurement.

Truncated MR MAC CE includes the triggered beam information:

  - One report config ID (event config ID)

  - At least one triggered beam id (SSBRI or CRI) + L1 RSRP

Then, it is questionable whether the first is the triggered beam or best beam in the truncated MR MAC CE if the UL grant can accommodate more than 1 beam measurements.

- Option1: The first beam is triggered beam with absolute RSRP, the second beam is the best beam (absolute RSRP), and remaining beams can use differential RSRP based on the absolute RSRP of best beam.

- Option 2: The first beam is best beam with absolute RSRP, and remaining beams (including triggered beam) can use differential RSRP based on the absolute RSRP of best beam. With this option, UE can only use truncated MR MAC CE if UL grant can accommodate first Nth beams and triggered beam.

We understand both options can work, while option1 is more aligned with previous agreement. With option1, UE can report the triggered beam measurement to NW even the UL grant can only accommodate one beam measurement information.

For truncated MR MAC CE, the triggered beam takes precedence over the best beam.

For truncated MR MAC CE, absolute RSRP is used for both triggered beam and best beam.

MAC-7

| MAC-7 | Whether one UL TB could include more than one truncated MAC CE or only one? For the case more than 1 triggering events are pending, if UL grant is sufficient for one regular L1 MR MAC CE or for two truncated L1 MR MAC CEs but insufficient for two regular L1 MR MAC CEs, the UE behaviour should be discussed. | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

Another issue is whether one UL TB could include more than one truncated MAC CE for the case more than 1 triggering events are pending, and the UL grant is sufficient for one regular L1 MR MAC CE or for two truncated L1 MR MAC CEs but insufficient for two regular L1 MR MAC CEs. In legacy, prioritization among MAC CEs of same priority is up to UE implementation. For triggered measurement reports, we can follow similar way, i.e. whether to report a full MR MAC CE or multiple truncated MR MAC CE is up to UE implementation.

Whether one UL TB includes more than one truncated MR MAC CE is up to UE implementation.

MAC-8

| MAC-8 | FFS the coexistence between event triggered report and mTRP, e.g. how UE determine the current beam or candidate beam in this case? | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

In previous RAN1 meeting, there is a conclusion on that it is up to RAN2 to decide whether the coexistence of LTM event triggered reporting and mTRP operation at serving cells is supported or not.

Conclusion

The following is up to RAN2: Coexistence of LTM event triggered reporting and mTRP operation at serving cells

In general, we understand supporting LTM event triggered L1 measurement reporting should not cause any restrictions to serving cell configuration. If the coexistence is not supported, it means NW should disable the mTRP related configuration when providing an event triggered L1 measurement reporting configuration for LTM candidates. And LTM support subsequent cell switch execution, the event configuration is provided to UE during LTM preparation period. That is, the system may need to turn off the mTRP operation for a long period as UE may keep LTM config among multiple cell coverages.

Coexistence of LTM event triggered reporting and mTRP operation at serving cell is supported.

Supporting the serving cell to configure mTRP requires minimal additional specification effort, which is how to define the beam of the serving cell since there are two indicated TCI states in this case.

In previous meeting, RAN2 agreed that the current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. For mTRP case, there are serval options to define the current beam:

- Option1: current beam corresponds to one of the indicated TCI states that selected/configured by NW.

- Option2: current beam corresponds to best beam of the two indicated beams.

- Option3: current beam corresponds to both indicated beams.

RAN2 further study how to define the current beam of serving cell for LTM L1 event evaluation if mTRP is configured based on the following options:

- Option1: current beam corresponds to one of the indicated TCI states that is selected/configured by NW.

- Option2: current beam corresponds to best beam of the two indicated beams.

- Option3: current beam corresponds to both indicated beams.

MAC-17

| MAC-17 | FFS how to handle the case if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, when MR is triggered Editor’s NOTE: FFS how to handle the case if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, when MR is triggered, e.g. trigger RACH directly. [Rapp]: Based on comments from ZTE and Samsung. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

In earlier meeting, it has agreed that NW can configure a dedicated SR configuration for MR MAC CE transmission. And the legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

And there is an FFS on how to handle the case if the dedicated SR configuration for L1 measurement report MAC CE transmission is not configured, when MR is triggered. In legacy, the RACH procedure will be triggered if there is no available SR configuration. For the triggered MR, if no dedicated SR is provided, RACH procedure can be triggered to require the UL grant for MR MAC CE transmission.

If no dedicated SR is provided, RACH procedure is triggered to request the UL grant for MR MAC CE transmission.

There are still open issues related to SR handling which need to be further studied. The first one is SR cancelation condition. If the SR is triggered for L1 event-triggered measurement report, and if a MAC PDU is transmitted and this PDU includes L1 measurement report MAC CE, the triggered SR should be cancelled accordingly.

The SR triggered for L1 event-triggered measurement report is cancelled if a MAC PDU is transmitted and this PDU includes L1 MR MAC CE or truncated MR MAC CE.

Another issue is about SR overlapping. In current spec, there are some note to clarify the SR/PUCCH resource handling for the cases that PUCCH resources are overlapped among the triggered SRs.

- Case 1: non-BFR SR overlapping

Except for SR for SCell beam failure recovery, the selection of which valid PUCCH resource for SR to signal SR on when the MAC entity has more than one overlapping valid PUCCH resource for the SR transmission occasion is left to UE implementation.

- Case 2: BFR SR& non-BFR SR

When the MAC entity has pending SR for SCell beam failure recovery and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery of a BFD-RS set) overlapping with PUCCH resource for SCell beam failure recovery for the SR transmission occasion, the MAC entity considers only the PUCCH resource for SCell beam failure recovery as valid. When the MAC entity has pending SR for beam failure recovery of a BFD-RS set of Serving Cell and the MAC entity has one or more PUCCH resources (other than PUCCH resources of pending SR for beam failure recovery) overlapping with PUCCH resource for beam failure recovery of that BFD-RS set for the SR transmission occasion, the MAC entity considers only the PUCCH resource for beam failure recovery of that BFD-RS set as valid.

- Case 3 Scell BFR SR & BFD-RS set BFR SR

When the MAC entity has PUCCH resource for pending SR for SCell beam failure recovery overlapping with PUCCH resource for pending SR for beam failure recovery of a BFD-RS set for the SR transmission occasion, it is up to UE implementation to select PUCCH resource for SCell beam failure recovery or PUCCH resource for beam failure recovery of a BFD-RS set.

For L1 event triggered measurement report, there may be similar issue. When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, which PUCCH resource is considered as valid.

In general, there are three ways:

- Option 1: left to UE implementation.

- Option2: MAC entity considers only the PUCCH resource for L1 event triggered MR MAC CE as valid.

- Option3: if “other triggered SR” is for BFR, the MAC entity considers only the PUCCH resource for beam failure recovery as valid.

When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, UE selects PUCCH resource based on:

- Option 1: left for UE implementation.

- Option2: MAC entity considers the PUCCH resource for L1 event triggered MR MAC CE as valid.

- Option3: if “other triggered SR” is for BFR, the MAC entity considers the PUCCH resource for beam failure recovery as valid.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1	For MR triggered by LTM2, the MR MAC CE can include measurements for LTM candidates.

Proposal 2	For truncated MR MAC CE, the triggered beam takes precedence over the best beam.

Proposal 3	For truncated MR MAC CE, absolute RSRP is used for both triggered beam and best beam.

Proposal 4	Whether one UL TB includes more than one truncated MR MAC CE is up to UE implementation.

Proposal 5	Coexistence of LTM event triggered reporting and mTRP operation at serving cell is supported.

Proposal 6	RAN2 further study how to define the current beam of serving cell for LTM L1 event evaluation if mTRP is configured based on the following options:

-	Option1: current beam corresponds to one of the indicated TCI states that is selected/configured by NW.

-	Option2: current beam corresponds to best beam of the two indicated beams.

-	Option3: current beam corresponds to both indicated beams.

Proposal 7	If no dedicated SR is provided, RACH procedure is triggered to request the UL grant for MR MAC CE transmission.

Proposal 8	The SR triggered for L1 event-triggered measurement report is cancelled if a MAC PDU is transmitted and this PDU includes L1 MR MAC CE or truncated MR MAC CE.

Proposal 9	When the MAC entity has PUCCH resource for pending SR for L1 event triggered MR and the MAC entity has one or more PUCCH resources overlapping with PUCCH resource for “other triggered SR”, UE selects PUCCH resource based on:

-	Option 1: left for UE implementation.

-	Option2: MAC entity considers the PUCCH resource for L1 event triggered MR MAC CE as valid.

-	Option3: if “other triggered SR” is for BFR, the MAC entity considers the PUCCH resource for beam failure recovery as valid.

Reference

[1] RP-234036 New WID-R19 Mobility Enhancements-v5.docx