3GPP TSG-RAN WG2 Meeting #130	R2-2503491

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.13.4

Source:	OPPO

Title:	Service continuity of multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the service continuity related issues of multi-hop U2N relay.

Specify the following intra-gNB service continuity scenarios for multi-hop U2N relay based on Rel-17/18 procedures (for remote UE): First Priority: - Intra-gNB multi-hop indirect to direct path switching using existing framework - Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused.

Discussion

Case C/D: path switching from multi-hop indirect path

Open issue RRC-1: Measure Events for scenario C and D

For the path switch to multi-hop indirect path, it is restricted in the WID that all relays at the target multi-hop relay link should be in RRC connected state to the same cell.

Second Priority in order of importance: - Intra-gNB direct to multi-hop indirect path switching - Intra-gNB single-hop indirect to multi-hop indirect path switching The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

With this restriction, the target multi-hop indirect path consisting of all the relays in RRC Connected mode to the same cell is identical to the target single-hop indirect path since:

- From UE perspective, it measures and reports the measurement results only for the directly detected first relay UE, which is similar to single hop case.

- From network perspective, since all the relay UEs are RRC connected and multi-path multi-hop relay is not supported, the network can identify the target path via measurement report from the remote UE as in single hop case.

With the restriction in the WID that all the relay UEs on the target multi-hop path, from both UE and network perspective, there is no much difference for multi-hop target path and single-hop target path.

For the path switching preparation procedure, Y1/Y2/Z1 are defined for the path switching to indirect path:

- Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2)

- Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold)

- Event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2)

Like the discussion for Case A and B, with the understanding that the “first relay” in multi-hop relay is the serving/candidate relay to be reported, Y1/Y2/Z1 can be reused.

With the understanding that the “first relay” in multi-hop relay is the serving or candidate relay to be reported, Y1/Y2/Z1 can be reused for the path switching to multi-hop indirect path.

(RRC-1) RAN2 to confirm event Y1/Y2/Z1 can be reused in path switching to multi-hop indirect path with the understanding that the “first relay” in multi-hop relay is the “candidate” relay to be reported.

Open issue RRC-2: Contents of report for scenario C and D

For the measurement report, in the current path switching to single-hop indirect path procedure, the L2 U2N Remote UE reports one or multiple candidate L2 U2N Relay UE(s) (L2 ID, Cell ID, SD-RSRP) that fulfil both AS and upper layer criteria. The principle should be reused. And the candidate L2 U2N Relay UE(s) in multi-hop relay case should be the first relay, since only the RSRP towards the first relay can be measured by the remote UE. It is also sufficient to report L2 ID, Cell ID, and the RSRP of the first relay since all the relay UEs at the multi-hop relay link are in RRC connected state to the same cell in the same gNB, network can identify the target link via the same information as legacy.

It is sufficient to report L2 ID, Cell ID, and the RSRP of the first relay (as candidate L2 U2N Relay) since all the relay UEs at the multi-hop relay link are in RRC connected state to the same cell in the same gNB.

(RRC-2) RAN2 to confirm that the measurement reporting from remote UE which includes L2 ID, serving cell ID, and SD-RSRP of the candidate first relay, as in legacy, is sufficient for the path switching to multi-hop indirect path.

Then for the path switching execution, gNB can decide the target path based on the information from the remote UE and do the reconfiguration for the relay UEs at the target path and the remote UE as in legacy.

For the path switching execution to multi-hop indirect path, gNB can decide the target path based on the information from the remote UE and do the reconfiguration for the relay UEs at the target path and the remote UE as in legacy.

Open issue RRC-3: Source relay becomes target for scenario D

For scenario D, similar to Case-B, UE just needs to perform the measurement and report based on gNB configuration. It doesn’t need to consider if the candidate relay UE is the U2N Relay at the source side.

(RRC-3) RAN2 to confirm for scenario D, no differentiation is needed for the case where the intermediated/last relay at the target side 1) is, or 2) is not the U2N relay UE at the source side.

Case A/B: path switching from multi-hop indirect path

In the RAN2 128 meeting, the following agreements for the cases to be supported/not supported in scenario B were made:

Agreements: For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused. For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching. Spec impact will be minimised. For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

For scenario B, it is agreed to reuse the R18 i2i path switch procedure as baseline.

The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

Based on the current path switching procedure, the remote UE measures the serving and candidate relay, and reports measurement results based on the measurement event. The path switch decision is made by the gNB.

As agreed for the baseline procedure, the remote UE just measures the serving/candidate relays and trigger measurement report based on network configuration.

Regarding the measurement of serving/candidate relay and measurement report at the UE side, there is no special handling regarding whether it is the old last/intermediate relay at the source side.

RAN2 to confirm for scenario B, no differentiation is needed for the cases where the U2N Relay at the target side 1) is, or 2) is not the intermediate/last relay at the source side.

Conclusion

We have the following observations:

Observation 1	With the restriction in the WID that all the relay UEs on the target multi-hop path, from both UE and network perspective, there is no much difference for multi-hop target path and single-hop target path.

Observation 2	With the understanding that the “first relay” in multi-hop relay is the serving or candidate relay to be reported, Y1/Y2/Z1 can be reused for the path switching to multi-hop indirect path.

Observation 3	It is sufficient to report L2 ID, Cell ID, and the RSRP of the first relay (as candidate L2 U2N Relay) since all the relay UEs at the multi-hop relay link are in RRC connected state to the same cell in the same gNB.

Observation 4	For the path switching execution to multi-hop indirect path, gNB can decide the target path based on the information from the remote UE and do the reconfiguration for the relay UEs at the target path and the remote UE as in legacy.

Observation 5	As agreed for the baseline procedure, the remote UE just measures the serving/candidate relays and trigger measurement report based on network configuration.

We have the following proposals:

Proposal 1	(RRC-1) RAN2 to confirm event Y1/Y2/Z1 can be reused in path switching to multi-hop indirect path with the understanding that the “first relay” in multi-hop relay is the “candidate” relay to be reported.

Proposal 2	(RRC-2) RAN2 to confirm that the measurement reporting from remote UE which includes L2 ID, serving cell ID, and SD-RSRP of the candidate first relay, as in legacy, is sufficient for the path switching to multi-hop indirect path.

Proposal 3	(RRC-3) RAN2 to confirm for scenario D, no differentiation is needed for the case where the intermediated/last relay at the target side 1) is, or 2) is not the U2N relay UE at the source side.

Proposal 4	RAN2 to confirm for scenario B, no differentiation is needed for the cases where the U2N Relay at the target side 1) is, or 2) is not the intermediate/last relay at the source side.