3GPP TSG-RAN WG2 Meeting #130                                        R2-2503500

St Julian’s, Malta, 19th – 23rd May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues for CB-msg3-EDT in IoT NTN

Agenda Item:	8.9.3

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. The WID is further refined and the latest one is approved in [1]. The main objectives related to support of capacity enhancements for uplink are as following:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In previous meetings, RAN2 has made much progress on the CB-msg3-EDT procedure. The related agreements can be found in the Annex. In this document, we will discuss the remaining issues for CB-msg3 transmission and CB-msg4 reception. Based on the discussion, some proposals are given.

Discussion

The resources configuration for CB-msg3-EDT

The time-domain (N)PUSCH resources configuration

Before and during RAN2#129bis meeting, several offline discussions [R2-2502773, R2-2503058] have been performed for the basic resources configuration for CB-msg3-EDT. Till now, only a LS has been sent out to RAN1 [R2-2503175] for check and confirmation, meanwhile, there is no detailed resources configuration parameters in the RRC running CR yet.

For the configuration of time domain (N)PUSCH resource, RAN2 has agreed that the configuration includes the periodicity and start time. As reference, the periodicity of NPRACH for EDT is {ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560}. Considering that the number of UEs using CB-msg3 EDT will generally be less than the number of UEs using the EDT, thus, it’s reasonable to assume that the periodicity in the time domain resources for CB-msg3 EDT can be sparer than NPRACH for EDT. Hence, the periodicity of the resource for CB-msg3 EDT could be in the unit of tens of, or hundreds of, even thousands of milliseconds. So taking the radio frame as unit (resulting in the length of periodicity between 10~10240 ms) may be enough for the definition of periodicity. Meanwhile, taking subframe as unit of periodicity means more dense time resource, and seems not so necessity.

There is also some thoughts that due to the possible large size of Msg3 and large repetitions, the transmission of Msg3 may need long time-domain resources, so it may need to consider taking H-SFN duration (i.e., 10.24s) as unit of periodicity. However, per our understanding, there would be seldom usage of large repetitions in NTN, or in other word, if large repetitions are needed, it’s not suitable to directly trigger CB-msg3 EDT. Also, taking H-SFN duration as unit of periodicity would lead to increased transmission latency (on top of the original large RTT in NTN). So we think very large periodicity for (N)PUSCH resource is not needed. In final, only SFN is suggested to be used as unit of the periodicity of CB-msg3 EDT resources. For start time point, the start subframe and start SFN are generally needed to represent a specific time occasion within a periodicity.

Proposal 1: To introduce the following parameters for the configuration of time domain (N)PUSCH resources for CB-msg3 EDT:

- a start time point expressed by a start SFN and a start subframe

- a periodicity in units of radio frame

The configuration for CB-msg3 transmission window

For supporting the transmission of multiple replicas of one Msg3 packet, a CB-msg3 transmission window has been agreed. According to the agreement in last meeting, the start time of CB-msg3 transmission window aligns with the start of time domain (N)PUSCH resource, hence, this start time could be deduced by the configuration of (N)PUSCH resource.

In last meeting, there was some discussion on how to define the CB-msg3 transmission window size but no agreement. Considering the following two aspects:

- The purpose of introducing this window is to provide a suitably sized resource pool with sufficient transmission occasions from which UEs can randomly select the required number of resources for transmitting multiple Msg3 replicas, trying to minimize collisions between different UEs' selections;

- If an absolute time duration is configured for the length of the transmission window, the time duration couldn’t guarantee to include multiple complete (N)PUSCH transmission occasions;

We think the most suitable way is to define the length of CB-msg3 transmission window with the unit of (N)PUSCH resources periodicity.

Proposal 2a: It’s suggested to define the length of CB-msg3 transmission window in units of (N)PUSCH resources periodicity.

Furthermore, typically, in order to provide sufficient randomness or dispersion, the number of resources in the resource pool can be several times greater than the required number of resources (e.g., the number of replicas), so we think a multiplication factor for parameter of replicas, e.g., K can be introduced, and the final value of CB-msg3 transmission window size in units of periodicity of (N)PUSCH resource can be the result of [(the number of replicas) multiplied by the factor of K]. Considering that the resource pool also cannot be too large that would cause resource waste, it’s suggested to define the maximum of K to be 4.

Proposal 2b: It’s suggested to introduce a multiplication factor for parameter of replicas, e.g., K as the configurable parameter for the length of CB-msg3 transmission window. The final value of the length of CB-msg3 transmission window is the result of [(the number of replicas) multiplied by the factor of K], that is:

Length of CB-msg3 transmission window = [(the number of replicas) * K] * (N)PUSCH resources periodicity

Proposal 2c: The number of K can be configured by the NW with the maximum value of 4.

In last meeting, RAN2 also agree to introduce another parameter, e.g. the periodicity of CB-msg3 transmission window, which can be same as or different from the length of CB-msg3 transmission window. When the periodicity of CB-msg3 transmission window is different from the window length, it means there is “empty” space between two consecutive windows, which can be used to schedule other services.

For simplicity, we think the periodicity of CB-msg3 transmission window only needs to be one or only a few periodicities of (N)PUSCH resource longer than the length of CB-msg3 transmission window. Therefore, it is also suggested to configure the periodicity of CB-msg3 transmission window as a value in units of (N)PUSCH resources periodicity.

Proposal 2d: It’s suggested to also define the periodicity of CB-msg3 transmission window as a value in units of (N)PUSCH resources periodicity.

Observation: Compared with the way to define the length and the periodicity of CB-msg3 transmission window directly in time units, e.g., ms, the proposed way in above Proposal 2a~2d of configuring the length and the periodicity of CB-msg3 transmission window, e.g., in units of (N)PUSCH resources periodicity, can offer the benefit of reducing signaling overhead.

The following Figure 1 gives an example to explain the way for configuring the length and the periodicity of CB-msg3 transmission window:

Figure 1

There is another discussion point on how to define the end of the CB-msg3 transmission window. According to the agreement in last meeting, by default UE can start the Msg4 monitoring window after the end of the CB-msg3 transmission window plus UE-eNB RTT. Such scheme can be simple for UE process in the case of multiple replicas transmission, but with obvious issue of causing additional latency. So in order to try to shorten the waiting time required before receiving Msg4, the end of the CB-msg3 transmission window can be at the end of the last (N)PUSCH transmission occasion, no need to be at the end of the (N)PUSCH resources periodicity, as illustrated as Figure 2. For this way, it only requires a minor adjustment to the calculation of length of CB-msg3 transmission window.

Figure 2

Proposal 2e: It’s suggested to adjust the end of the CB-msg3 transmission window to at the end of the last (N)PUSCH transmission occasion within the window. The final value of the length of CB-msg3 transmission window is as below:

Length of CB-msg3 transmission window = [(the number of replicas) * K - 1] * (N)PUSCH resources periodicity + Length of (N)PUSCH transmission occasion

Please note, it’s no need to change the calculation of the periodicity of the CB-msg3 transmission window.

Based on the discussion above, we also give the following signaling example for the basic resources configuration for CB-msg3-EDT (without OCC) for eMTC:

CB-Msg3-ParametersList-r19 ::= SEQUENCE (SIZE (1.. maxCB-Msg3-Resources-r19)) OF CB-Msg3-Parameters-r19 CB-Msg3-Parameters-r19 ::= SEQUENCE { cb-Msg3-NumReplicas INTEGER(1..4), cb-Msg3-MPDCCH-Config-r19 CB-Msg3-MPDCCH-Config-r19, //waiting for RAN1 cb-Msg3-PUCCH-Config-r19 CB-Msg3-PUCCH-Config-r19, //waiting for RAN1 cb-Msg3-PUSCH-Config-r19 CB-Msg3-PUSCH-Config-r19, cb-Msg3-PDSCH-Config-r19 CB-Msg3-PDSCH-Config-r19, //waiting for RAN1 cb-Msg3-TransmissionWindow-r19 SEQUENCE { startSFN-19 INTEGER (0.. 1023), factorForWindowSize-19 INTEGER(1..4), windowPeriodicity-r19 INTEGER(1..16) } OPTIONAL, --Need OP ........ } CB-Msg3-PUSCH-Config-r19 ::= SEQUENCE { cb-Msg3-StartTimeParameters-r19 SEQUENCE { pusch-periodicity-r19 ENUMERATED {FFS}, pusch-startTime-r19 INTEGER (0..1023), pusch-startSubframe-r19 INTEGER (0..9) }, cb-Msg3-PUSCH-ModeAConfig-r19 CB-Msg3-PUSCH-ModeAConfig-r19 } CB-Msg3-PUSCH-ModeAConfig-r19::= SEQUENCE { numRUs-r19 BIT STRING (SIZE(2)), prb-AllocationInfoSet-r19 SEQUENCE (SIZE(1..FFS)) OF BIT STRING (SIZE(10)), mcs-r19 BIT STRING (SIZE(4)), numRepetitions-r19 BIT STRING (SIZE(3)) PUSCH-FreqHopping-r19 BOOLEAN, p0-UE-PUSCH-r19 INTEGER (-8..7), alpha-r19 Alpha-r12 } ……

Resources configuration and selection for the case of replica = 1

When configured number of replica equals to 1, since only one replica of Msg3 packet needs to be transmitted, there is no need to configure the CB-msg3 EDT transmission window. Similar as the preamble transmission, the UE can select the nearest (N)PUSCH resource for CB-msg3 transmission. Furthermore, as the UE has no other replica to be transmitted, it’s feasible for the UE to start the Msg4 monitoring window as early as possible, e.g., after the end of last repetition of CB-msg3 transmission (e.g., the end of (N)PUSCH transmission occasion) plus UE-eNB RTT. We see this option (Option 1) is the most suitable way for CB-msg3 transmission when the configured replica =1.

There may be some thoughts that similar CB-msg3 EDT transmission window as that for the case of replica larger than 1 can be configured. However, we see both of the following possible ways have problems, so neither of them is preferred:

- (Option 2) to configure a CB-msg3 EDT transmission window with only one periodicity of (N)PUSCH resource or only one (N)PUSCH resource: this option has same resource selection result as Option 1 (e.g., to select the only and also the nearest (N)PUSCH resource), but has a disadvantage of additional delay if the Msg4 monitoring window starts after the end of the CB-msg3 transmission window plus UE-eNB RTT.

- (Option 3) to configure a CB-msg3 EDT transmission window with several periodicities of (N)PUSCH resource: That means UE can randomly select a (N)PUSCH resource from multiple (N)PUSCH resources for the only one Msg3 replica transmission. At first glance, this option seems to reduce the probability of collisions among multiple UEs initiating Msg3 transmission at the same time, as they may select different (N)PUSCH resources within a certain range. However, this is not actually the case — these UEs can still collide with other UEs that trigger services at different times. Moreover, another issue arises that, UEs cannot always choose the nearest (N)PUSCH resource to initiate the Msg3 transmission, which introduces unnecessary latency.

Based on the above analysis, Option 1 is preferred.

Proposal 3: When configured number of replica equals to 1, there is no need to configure the CB-msg3 EDT transmission window. The UE can select the nearest (N)PUSCH resource for CB-msg3 transmission. Furthermore, UE starts the Msg4 monitoring window after the end of last repetition of CB-msg3 transmission (e.g., the end of (N)PUSCH transmission occasion) plus UE-eNB RTT.

The CB-msg3-EDT resources configuration on non-anchor carrier

For NB-IoT and also NB-IoT over NTN, it supports to configure multiple non-anchor carriers to provide NPRACH and EDT resources with intention to increase the PRACH and Paging capacity. In last meeting, RAN2 high-level agreed that for NB-IoT, the shared resources for CB-Msg3 can also be configured in SIB22-NB for non-anchor carriers.

One possible way is to introduce separate UL and DL non-anchor carriers list to only configure the CB-msg3 EDT resources. However, considering the limited spectrum resources of operators, it’s suggested reusing the existing UL and DL non-anchor carriers list with up to 15 entries to also provide CB-msg3 EDT resources configuration.

Proposal 4a: It’s suggested to reuse the existing UL and DL non-anchor carriers list to provide CB-msg3 EDT resources configuration.

It can be seen that within current UL non-anchor carriers list, for each UL carrier, the NPRACH resources configuration is optional. That means, even the existing UL non-anchor carriers list is reused, it’s still feasible to configure only CB-msg3 EDT resources on a certain UL carrier. Meanwhile, it’s also feasible to configure both NPRACH resources and CB-msg3 EDT resources on a certain UL carrier. Such configuration flexibility can be left to NW implementation.

Proposal 4b: It’s feasible to configure only CB-msg3 EDT resources on a certain UL carrier. Meanwhile, it’s also feasible to configure both NPRACH resources and CB-msg3 EDT resources on a certain UL carrier. Such configuration flexibility can be left to NW implementation.

During post-RAN2#129bis offline discussion, there is some discussion on whether the legacy rules based on selection probability for NPRACH anchor/non-anchor carrier selection can also be reused for carrier selection for CB-msg3 EDT. As reference, the existing rules are cited as below:

TS 36.321: ………………………………. - when multiple carriers provide PRACH resources for the same enhanced coverage level, the UE will randomly select one of them using the following selection probabilities: - the selection probability of the anchor carrier PRACH resource for the given enhanced coverage level, nprach-ProbabilityAnchor, is given by the corresponding entry in nprach-ProbabilityAnchorList - the selection probability is equal for all non-anchor carrier PRACH resources and the probability of selecting one PRACH resource on a given non-anchor carrier is (1- nprach-ProbabilityAnchor)/(number of non-anchor NPRACH resources)

Per our understanding, the legacy rules is simple and has its benefit of distributing the random access load more evenly. Therefore, it can be leveraged for carrier selection for CB-msg3 EDT, but selection probability can be a new parameter which is different from the existing one for NPRACH. Moreover, there is an implicit requirement that, as the selection probability for all non-anchor carriers are equal, it generally needs to provide the CB-msg3 EDT resources evenly among these non-anchor carriers.

Proposal 4c: The legacy rules based on selection probability for NPRACH anchor/non-anchor carrier selection can also be reused for anchor/non-anchor carrier selection for CB-msg3 EDT. A separate selection probability different from that for NPRACH can be introduced.

The resources configuration for CB-msg3-EDT with OCC

In previous meetings, whether to support using OCC code for CB-msg3 transmission has been discussed but remained undecided. Although to support OCC for CB-msg3-EDT have the benefits of reducing collisions and increasing capacity, there are concerns that power imbalance issue might lead to degraded reception performance. Till now, RAN2 only agreed that if we finally decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission. From RAN2 perspective, we assume most of the resources configuration for CB-msg3-EDT without OCC can be leveraged for CB-msg3-EDT with OCC, furthermore, besides the addition of OCC code, the following configuration needs to be considered.

According to the information from RAN1, if two UEs without power imbalance select different OCCs and share the same time-frequency resources, the better gain could be achieved in the receiver. Conversely, if two UEs with power imbalance select different OCCs, and share the same time-frequency resources, the gain might decrease. Therefore, in previous meeting, it has been suggested to introduce specific RSRP range for using OCC for CB-msg3 transmission to control the power imbalance issue. However, setting a single RSRP range will greatly limit the number of UEs which can use CB-msg3-EDT with OCC as the number of UEs meeting the RSRP range might be very small. In order to expand usage scenarios for CB-msg3-EDT with OCC, it's reasonable to set multiple RSRP ranges.

Moreover, for UEs in two different coverage areas (e.g., cell edge or middle part of the cell), due to different transmission power, they may cause different levels of interference to other UEs who select the same time and frequency resource at the receiver. For example, compared with UE with -140dBm RSRP, UE with -100dBm RSRP may cause more interference to UEs who select the same time and frequency resources. Therefore, in order to match the situation in different coverage areas, the separate RSRP ranges can be flexibly configured per CE level.

Proposal 5: One or multiples RSRP ranges could be introduced to control power imbalance for the UEs to use CB-msg3-EDT with OCC. And the separate RSRP ranges can be flexibly configured per CE level.

Efficient CB-msg3 transmission

Backoff/Fallback schemes for CB-msg3

In last meeting, the Backoff mechanism has been agreed, and Backoff Indicator could be included in Msg4. Generally, a range of waiting time information needs to be included in the Backoff Indicator. Based on this information, the UEs can wait for varying durations before retriggering the same procedure, e.g., CB-msg3 EDT. That also means the UEs that faced collisions during previous transmission can retrigger another CB-msg3 EDT at least in another time-domain resource location. This could improve the likelihood of successful access by the UE in subsequent attempt.

Proposal 6a: A range of waiting time needs to be included in the Backoff Indicator in Msg4. Based on this information, the UEs can wait for varying durations before retriggering the same procedure, e.g., CB-msg3 EDT.

RAN2 also agreed that UEs that select the same transmission window will monitor the same CB-RNTI. In other words, if many UEs select a same transmission window, some may transmit Msg3 successfully, while some fails to transmit. But both successful and unsuccessful UEs will monitor the same CB-RNTI and see the same Backoff Indicator in Msg4. Then the UEs which have successfully transmitted Msg3 may also think it fails and needs to backoff to another time to re-access. This is also a typical issue with using a Single Msg4 monitoring window and Single RNTI that only takes into account the start transmit resource of the transmission window - the original filtering function by RNTI is faded away (taking legacy RA-RNTI as example, it generally includes as many resource location information as possible, thereby enabling the differentiation of UEs that trigger access using different resources). Therefore, merely including waiting time information in the Backoff Indicator isn’t sufficient.

Therefore, in order to only trigger the unsuccessful UE to access again, the Backoff Indicator in Msg4 also needs to include the information for targeting those unsuccessful UE. A straightforward way is to include the resource information of the transmission location in the Backoff Indicator, if a collision is detected at that transmission position by the eNB. In other word, such resource information of the transmission location that identifies the time occasion, frequency occasion and OCC sequence can be used to distinguish UEs that had conflicts or failed CB-msg3 transmissions. Hence, Backoff Indicator also needs to include the information of time occasion, frequency occasion and/or OCC sequence where collisions are detected.

Proposal 6b: For CB-msg3 EDT procedure, Backoff Indicator also needs to include the information of time occasion, frequency occasion and/or OCC sequence where collisions are detected.

In last meeting, the fallback mechanism to higher CE level has also been agreed. However, if the deterioration of channel quality of the UE leads to the failed reception of CB-msg3, the CB-msg3-EDT procedure may still fail even after UE switches to higher CE level. Considering the decoding requirements of CB-msg3 are more stringent than PRACH, the reception condition of PRACH may still be satisfied and so switching to RACH based procedure could increase the successful access probability. Hence, fallback to RACH based EDT procedure is feasible and also needs to be supported. In a summary, with the fallback mechanism, if multiple transmissions of the CB-msg3 in a given CE level fail, firstly a higher CE level can be selected and CB-msg3 is triggered in this level. Till the CB-msg3 transmissions in highest CE level fail, a RACH based EDT procedure can be triggered.

Proposal 6c: For CB-msg3 EDT procedure, fallback to RACH based EDT procedure also needs to be supported.

HARQ for CB-msg3 transmission

In the post-RAN2#129bis offline discussion for MAC running CR/remaining issues, one discussion point is whether the HARQ operation is applicable to transmission of CB-Msg3. Some companies mentioned that, there is no “NACK” for msg3, thus it obviously cannot add “retransmission of msg3” as there may be many UEs that send in a CB-msg3 transmission window, and all will then retransmit causing new collisions.

We agree such observation, and also think such issue is partially related to the use of common CB-RNTI. When a common CB-RNTI that is only associated with the transmission window is applied, it’s difficult/impossible to exactly schedule the required retransmissions as there is no PDCCHs scrambled by different RNTIs (for such scheduling, at least RNTIs need to be associated with different transmission resources). Therefore, it’s suggested not to support PDCCH triggered re-transmission of CB-msg3. Also considering that backoff/fallback schemes have been agreed to handle the failure case of CB-msg3 transmission, we think the PDCCH triggered re-transmission of CB-msg3 is also not so necessary.

Proposal 7: PDCCH triggered re-transmission of CB-msg3 is not supported.

Efficient delivery of Msg4 response to CB-msg3

CB-msg4 monitoring window

In last meeting, RAN2 has agreed for CB-msg3-EDT, a Single Msg4 monitoring window and Single RNTI are adopted. And the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not). There is also a FFS on whether NW can configure earlier start of the CB-msg4 monitoring window, e.g., starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time).

In this section, we will further discuss the start of CB-msg4 monitoring window in following two scenarios:

- Under the scenario 1 with large RTT, e.g. GEO 540 ms, the length of transmission window may be smaller than the RTT, as illustrated in Figure 2.

Figure. 3

In this case, the earliest time that UE could receive CB-msg4 is larger than the end time of transmission window. Hence, there isn’t any overlapping between CB-msg3 transmission window and CB-msg4 monitoring window. If the CB-msg4 monitoring window starts at the end of CB-msg3 transmission window plus UE-eNB RTT (e.g., the window in the blue box), the reception of Msg4 will be delayed, e.g. at subframe 19. In fact, UE is able to receive CB-msg4 earlier, e.g. at subframe 13. Moreover, NW can know that there is no issue of overlapping between CB-msg4 monitoring window and CB-msg3 transmission window. Therefore, for this scenario, in order to avoid unnecessary delayed reception of CB-msg4, it’s feasible and also reasonable to allow NW configure earlier start of the CB-msg4 monitoring window, e.g., at the end of CB-msg3 EDT transmission occasion of the first replica plus UE-eNB RTT (e.g., the window in the red box).

Observation 1a: In the scenario 1 with large RTT and not long CB-msg3 transmission window, as there would be no issue of overlapping between CB-msg4 monitoring window and CB-msg3 transmission window, it’s feasible and also reasonable to allow NW configure earlier start of the CB-msg4 monitoring window, e.g., at the end of CB-msg3 EDT transmission occasion of the first replica plus UE-eNB RTT.

- Under the scenario 2 with large RTT, the length of transmission window may be larger than the RTT due to the large repetition number, as illustrated in Figure 3.

Figure. 4

In this case, the earliest time that UE could receive CB-msg4 is earlier than the end time of transmission window. Hence, there is overlapping between transmission window and monitoring window. Considering of the larger RTT, if the Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT, the reception of Msg4 will be largely delayed, e.g. at subframe 27. For this scenario, as NW can know it may cause the issue of overlapping between CB-msg4 monitoring window and CB-msg3 transmission window, one option is that, the NW can choose not to configure earlier start of the CB-msg4 monitoring window.

However, in this scenario, since the reception of CB-msg4 may experience significant delay with default start of monitoring window (larger than that in scenario 1), it also implies that another option of starting monitoring window earlier can bring greater gains, e.g., largely shorten the delay for CB-msg4 reception. In addition, in such scenario, since the transmission window is relatively long, by the time the monitoring window starts, the first or even the first few replicas may have already been transmitted and successfully received. Therefore, canceling the subsequent replica transmissions would not be a major issue. Hence, it is reasonable to allow higher priority for CB-msg4 reception within the overlapped part of transmission window and monitoring window. In a summary, for this scenario, it’s also feasible and reasonable to allow NW configure earlier start of the CB-msg4 monitoring window.

Observation 1b: In the scenario 2 with large RTT and long CB-msg3 transmission window, to start CB-msg4 monitoring window earlier can bring obvious gain, e.g., largely shorten the delay for CB-msg4 reception. Meanwhile, even there may be issue of overlapping between CB-msg4 monitoring window and CB-msg3 transmission window, as the transmission window is relatively long, by the time the monitoring window starts, the first or even the first few replicas may have already been successfully transmitted, it’s also feasible and reasonable to allow NW configure earlier start of the CB-msg4 monitoring window, and prioritize CB-msg4 monitoring within the overlapped window part.

Based on the above analysis, it’s suggested to support NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT. Such configuration can be just a simple enable indication to enabling earlier start of CB-msg4 monitoring window. Without such enable indication, the UE can follow the default way to start CB-msg4 monitoring window. Moreover, enabling earlier start of CB-msg4 monitoring window also implies that UE needs to prioritize CB-msg4 monitoring within the overlapped part of CB-msg3 transmission window and CB-msg4 monitoring window.

Proposal 8a: Based on network configuration, the CB-msg4 monitoring could start earlier at the end of CB-msg3 EDT transmission occasion of the first replica plus UE-eNB RTT.

Proposal 8b: The network configuration can be an enable/disable indication to enabling/disabling earlier start of CB-msg4 monitoring window.

Proposal 8c: When network configures earlier start of CB-msg4 monitoring window, if there is overlapping between CB-msg3 transmission window and CB-msg4 monitoring window, UE prioritizes the CB-msg4 monitoring.

Another FFS for starting CB-msg4 monitoring window is that, besides taking UE-eNB RTT into account, whether we need to consider additional delay e.g. for the processing time. Taking the following descriptions from RAN1 and RAN2 specs as reference, it’s suggested to consider adding processing time with fixed value of 4 subframes.

TS 36.213: 9.1.5 MPDCCH assignment procedure //skip unrelated parts If the UE has initiated a PUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the MPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-MPDCCH-SS-window-duration where is provided by higher layer parameter K-mac, otherwise Kmac=0. Upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]), the UE is not required to monitor the MPDCCH UE-specific search space for the remaining search space window duration. //skip unrelated parts - 16.6 Narrowband physical downlink control channel related procedures //skip unrelated parts If the UE has initiated a NPUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the NPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-SS-window-duration. Upon detection of a NPDCCH with DCI format N0 with CRC scrambled by PUR-RNTI intended for the UE within the search space window and the value of "modulation and coding scheme" field () in the corresponding DCI is set to '14', the UE is not required to monitor the NPDCCH UE-specific search space for the remaining search space window duration. //skip unrelated parts TS 36.321: 5.4.7.1 Transmission using PUR //skip unrelated parts After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT. - else: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes. //skip unrelated parts

Proposal 8d: It’s suggested to consider adding processing time with fixed value of 4 subframes for determining the start of CB-msg4 monitoring window.

For the length of CB-msg4 monitoring window, it can be configured by the eNB. we assume the length of Msg3 response window in PUR as below can be referred:

| for eMTC | pur-ResponseWindowTimer-r16 ENUMERATED {sf240, sf480, sf960, sf1920, sf3840, sf5760, sf7680, sf10240} |
| --- | --- |
| For NB-IoT | pur-ResponseWindowTimer-r16 ENUMERATED {pp1, pp2, pp3, pp4, pp8, pp16, pp32, pp64} |

Due to that UE may need to wait for the responses of the multiple msg3 replicas, the length of CB-msg4 monitoring window may need to be further extended.

Proposal 8e: To take the value range of pur-ResponseWindowTimer as baseline for the length of CB-msg4 monitoring window and further discuss whether extended values are needed.

CB-RNTI calculation

In last meeting, RAN2 has agreed to introduce a single CB-RNTI that is derived on the transmit resource for the transmission window. Hence, the calculation of CB-RNTI could introduce the parameters identifying the selected transmission window.

For the design of CB-RNTI, to try to reduce the false alarm issue, the important thing is to make sure that there is no same CB-RNTI during a CB-msg4 monitoring window. If UEs select different transmission window, the SFN index of the first time-domain transmission occasion within the transmission window, which is also the start SFN index of the transmission window, could be used to distinguish UEs. Thus, it could be used for the calculation of CB-RNTI. In order to compress the part where SFN index is involved, SFN index can be divided by a minimum periodicity of the CB-msg3 transmission window, e.g., factor A, in the formula. Besides, if the length of CB-msg4 monitoring window can be larger than a H-SFN duration, the H-SFN index of the first time-domain transmission occasion may also need to be considered.

As a result, referring to the following formula for RA-RNTI for NB-IoT TDD case (which also takes the H-SFN into account),

For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2) where SFN_id is the index of the first radio frame of the specified PRACH and H-SFN is the index of the first hyper frame of the specified PRACH. The PDCCH transmission and the PRACH resource are on the same carrier.

The formula for CB-RNTI can be as following, the square brackets means the H-SFN may be optional:

CB-RNTI = 1 + floor(SFN_id/A) + [C*(H-SFN mod B)] Where, SFN_id is the index of the radio frame of the first time-domain transmission occasion within the selected CB-msg3 transmission window and H-SFN is the index of the hyper frame of the first time-domain transmission occasion within the selected CB-msg3 transmission window.

Here factor A is determined by the minimum periodicity of the CB-msg3 transmission window in units of radio frame (e.g., 10ms). B is determined by the maximum length of CB-msg4 monitoring window in units of H-SFN duration. C is determined by the maximum value of floor(SFN_id/A)+1.

Based on the above discussion, the following proposals are given for CB-RNTI calculation:

Proposal 9: The CB-RNTI calculation can be as below:

CB-RNTI = 1 + floor(SFN_id/A) + [C*(H-SFN mod B)]

Where, SFN_id is the index of the radio frame of the first time-domain transmission occasion within the selected CB-msg3 transmission window and H-SFN is the index of the hyper frame of the first time-domain transmission occasion within the selected CB-msg3 transmission window.

A is determined by the minimum periodicity of the CB-msg3 transmission window in units of radio frame (e.g., 10ms). B is determined by the maximum length of CB-msg4 monitoring window in units of H-SFN duration. C is determined by the maximum value of floor(SFN_id/A)+1.

Type of CB-msg4

Similar as that in legacy contention-based procedures, the collision caused by using shared resources for Msg3 transmission cannot be completely resolved by RNTI design. The additional contention resolution scheme is still needed.

The straightforward way is to reuse the contention resolution scheme in legacy random access procedure. That is, after reception of CB-msg3, eNB can send a DL MAC PDU in which a Contention Resolution Identity is included. For UE side, it needs to check whether the UE Contention Resolution Identity included in the DL MAC PDU matches the 48 first bits of the CCCH SDU transmitted in Msg3. In last meeting, RAN2 also agreed that the UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity. We think the details need to be captured a bit further.

Proposal 10a: A UE Contention Resolution Identity MAC CE can be included in CB-msg4. UE checks whether the received UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in CB-msg3 and if matched, UE determines that contention is resolved.

It’s still FFS in RAN2 that, besides Contention Resolution Identity MAC CE, whether RRC message must be included together in the CB-msg4.

In the procedure for transmission using PUR for CP solution, L1 ACK and L2 ACK (e.g., TA command MAC CE) can be used to early terminate the procedure if there is no data to be sent to the UE. Similarly as PUR, for CB-msg3 for CP solution, upon the reception of the CB-msg3, the network is able to determine that there is no DL data for UE, thereby it can choose to terminate the procedure efficiently. Moreover, considering that the UE Contention Resolution Identity is essential to resolve the possible collision, the UE Contention Resolution Identity MAC CE would be always needed for CB-msg3 EDT procedure. Therefore, it’s very straightforward to use L2 ACK as a way to terminate the CB-msg3 EDT procedure. Meanwhile, as there would be no case to use L1 ACK, RAN2 has excluded this way in last meeting.

In last meeting, there was also a view that CB-msg3 EDT procedure should be terminated by RRC message. We understand such view is unreasonable since PUR procedure already supports termination by the ways other than RRC message. Moreover, sending an empty RRC message when there is no any data to send will cause unnecessary signaling overhead. Further taking into account CB-msg4 multiplexing, we think the main viable scenario is that multiple UEs having no DL data and merely needing to match UE Contention Resolution Identity MAC CE to complete the procedure. For such case, requiring the transmission of RRC messages may restrict the applicability of CB-msg4 multiplexing.

For UP solution, even there is no data to be sent to the UE, the eNB still need to update the security information and may also the resume ID via the RRC message, therefore, the RRC message is still needed in CB-msg4.

Proposal 10b: For CP solution, if there is no data to be sent to the UE, UE Contention Resolution Identity MAC CE can be merely sent to early terminate the CB-msg3 EDT procedure, no need of RRC message together in the CB-msg4.

CB-msg4 multiplexing

In previous meeting, RAN2 has agreed to support msg4 multiplexing. Also during post-RAN2#129bis offline discussion for MAC running CR/remaining issues, more companies agree to define new MAC PDU format for CB-msg4 multiplexing.

If the CB-msg4 of multiple UEs are multiplexed into the same MAC PDU, the important issue is how to let a certain UE knows which MAC CE or MAC SDU in the MAC PDU is intended for it. There are two alternatives to address this issue:

- Alt1: leveraging the current MAC PDU design in LTE but introducing new format: according to previous discussion, a UE Contention Resolution Identity is essential to resolve the collision in Msg4. For the multiplexing of Msg4, the UE Contention Resolution Identity also can be used to differentiate the MAC CE or MAC SDU targeting for different UEs in a MAC PDU. To be specially, if the CB-msg4 of multiple UEs are multiplexed into the same MAC PDU, the MAC CE or MAC SDU targeting for the same UE should be placed after the UE Contention Resolution Identity of that UE. At UE side, as long as a decoded contention resolution identity carried in MAC PDU matches its own CCCH, the UE will assume the following MAC CE or MAC SDU until the next contention resolution identity or the end also belong to it. As illustrated in Figure 5, two CB-msg4 are multiplexed in a MAC PDU. For UE#1, if UE Contention Resolution Identity MAC Control Element#1 matches, MAC SDU#1 is targeted to it. For UE#2, if UE Contention Resolution Identity MAC Control Element#1 matches, C-RNT1 MAC CE#2 is targeted to it.

| MAC subheader #1-1 | MAC subheader #1-2 | MAC subheader #2-1 | MAC subheader #2-2 | UE Contention Resolution Identity MAC Control Element#1 | MAC SDU#1 | UE Contention Resolution Identity MAC Control Element#2 | C-RNTI MAC Control Element#2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 5. MAC PDU design for Alt1

- Alt2: introducing MAC subPDU similar as the scheme in NR: a MAC PDU consists of one or more MAC subPDUs, as illustrated in Figure 6. Each subPDU could target to a certain UE, and include MAC subheader, RAR or MAC SDU. This method is conducive to distinguish the information of UE. However, on one hand, the new MAC subPDU format will introduce additional complexity. It requires UE to decode different MAC formats for different access processes. For example, for CB-msg3-EDT procedure and PUR procedure, UE needs to decode different MAC formats for Msg4. On the other hand, the RAR with fixed size in NR can’t support all cases of Msg4 for CB-msg3-EDT procedure. For example, the RAR with fixed size is not suitable for both cases carrying or not carrying the C-RNTI.

| MAC subPDU #1 | MAC subPDU #2 |
| --- | --- |

Figure 6. MAC PDU design for Alt2

According to above analysis, we think Alt1 has better compatibility and low complexity and therefore be preferred.

Proposal 11: For CB-msg3-EDT procedure, if the CB-msg4 of multiple UEs are multiplexed into the same MAC PDU, the MAC CE or MAC SDU targeting for the same UE should be placed after the UE Contention Resolution Identity of that UE.

HARQ feedback for CB-msg4

In last RAN2 meeting, RAN2 has agreed that HARQ feedback is adopted to acknowledge Msg4. It’s FFS on how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.

Per our understanding, as a certain UE can identify which response in CB-msg4 MAC PDU is targeted for it, it’s feasible to give HARQ feedback for its own response, e.g., give HARQ feedback per UE. How to give HARQ feedback needs to wait for RAN1 progress.

Proposal 12: HARQ feedback should be given for each response in CB-msg4.

Conclusion

Based on the above discussion, the following proposals are given:

The time-domain (N)PUSCH resources configuration

Proposal 1: To introduce the following parameters for the configuration of time domain (N)PUSCH resources for CB-msg3 EDT:

- a start time point expressed by a start SFN and a start subframe

- a periodicity in units of radio frame

The configuration for CB-msg3 transmission window

Proposal 2a: It’s suggested to define the length of CB-msg3 transmission window in units of (N)PUSCH resources periodicity.

Proposal 2b: It’s suggested to introduce a multiplication factor for parameter of replicas, e.g., K as the configurable parameter for the length of CB-msg3 transmission window. The final value of the length of CB-msg3 transmission window is the result of [(the number of replicas) multiplied by the factor of K], that is:

Length of CB-msg3 transmission window = [(the number of replicas) * K] * (N)PUSCH resources periodicity

Proposal 2c: The number of K can be configured by the NW with the maximum value of 4.

Proposal 2d: It’s suggested to also define the periodicity of CB-msg3 transmission window as a value in units of (N)PUSCH resources periodicity.

Proposal 2e: It’s suggested to adjust the end of the CB-msg3 transmission window to at the end of the last (N)PUSCH transmission occasion within the window. The final value of the length of CB-msg3 transmission window is as below:

Length of CB-msg3 transmission window = [(the number of replicas) * K - 1] * (N)PUSCH resources periodicity + Length of (N)PUSCH transmission occasion

Resources configuration and selection for the case of replica = 1

Proposal 3: When configured number of replica equals to 1, there is no need to configure the CB-msg3 EDT transmission window. The UE can select the nearest (N)PUSCH resource for CB-msg3 transmission. Furthermore, UE starts the Msg4 monitoring window after the end of last repetition of CB-msg3 transmission (e.g., the end of (N)PUSCH transmission occasion) plus UE-eNB RTT.

The CB-msg3-EDT resources configuration on non-anchor carrier

Proposal 4a: It’s suggested to reuse the existing UL and DL non-anchor carriers list to provide CB-msg3 EDT resources configuration.

Proposal 4b: It’s feasible to configure only CB-msg3 EDT resources on a certain UL carrier. Meanwhile, it’s also feasible to configure both NPRACH resources and CB-msg3 EDT resources on a certain UL carrier. Such configuration flexibility can be left to NW implementation.

Proposal 4c: The legacy rules based on selection probability for NPRACH anchor/non-anchor carrier selection can also be reused for anchor/non-anchor carrier selection for CB-msg3 EDT. A separate selection probability different from that for NPRACH can be introduced.

The resources configuration for CB-msg3-EDT with OCC

Proposal 5: One or multiples RSRP ranges could be introduced to control power imbalance for the UEs to use CB-msg3-EDT with OCC. And the separate RSRP ranges can be flexibly configured per CE level.

Backoff/Fallback schemes for CB-msg3

Proposal 6a: A range of waiting time needs to be included in the Backoff Indicator in Msg4. Based on this information, the UEs can wait for varying durations before retriggering the same procedure, e.g., CB-msg3 EDT.

Proposal 6b: For CB-msg3 EDT procedure, Backoff Indicator also needs to include the information of time occasion, frequency occasion and/or OCC sequence where collisions are detected.

Proposal 6c: For CB-msg3 EDT procedure, fallback to RACH based EDT procedure also needs to be supported.

HARQ for CB-msg3 transmission

Proposal 7: PDCCH triggered re-transmission of CB-msg3 is not supported.

CB-msg4 monitoring window

Observation 1a: In the scenario 1 with large RTT and not long CB-msg3 transmission window, as there would be no issue of overlapping between CB-msg4 monitoring window and CB-msg3 transmission window, it’s feasible and also reasonable to allow NW configure earlier start of the CB-msg4 monitoring window, e.g., at the end of CB-msg3 EDT transmission occasion of the first replica plus UE-eNB RTT.

Observation 1b: In the scenario 2 with large RTT and long CB-msg3 transmission window, to start CB-msg4 monitoring window earlier can bring obvious gain, e.g., largely shorten the delay for CB-msg4 reception. Meanwhile, even there may be issue of overlapping between CB-msg4 monitoring window and CB-msg3 transmission window, as the transmission window is relatively long, by the time the monitoring window starts, the first or even the first few replicas may have already been successfully transmitted, so it’s also feasible and reasonable to allow NW configure earlier start of the CB-msg4 monitoring window, and prioritize CB-msg4 monitoring within the overlapped window part.

Proposal 8a: Based on network configuration, the CB-msg4 monitoring could start earlier at the end of CB-msg3 EDT transmission occasion of the first replica plus UE-eNB RTT.

Proposal 8b: The network configuration can be an enable/disable indication to enabling/disabling earlier start of CB-msg4 monitoring window.

Proposal 8c: When network configures earlier start of CB-msg4 monitoring window, if there is overlapping between CB-msg3 transmission window and CB-msg4 monitoring window, UE prioritizes the CB-msg4 monitoring.

Proposal 8d: It’s suggested to consider adding processing time with fixed value of 4 subframes for determining the start of CB-msg4 monitoring window.

Proposal 8e: To take the value range of pur-ResponseWindowTimer as baseline for the length of CB-msg4 monitoring window and further discuss whether extended values are needed.

CB-RNTI calculation

Proposal 9: The CB-RNTI calculation can be as below:

CB-RNTI = 1 + floor(SFN_id/A) + [C*(H-SFN mod B)]

Where, SFN_id is the index of the radio frame of the first time-domain transmission occasion within the selected CB-msg3 transmission window and H-SFN is the index of the hyper frame of the first time-domain transmission occasion within the selected CB-msg3 transmission window.

A is determined by the minimum periodicity of the CB-msg3 transmission window in units of radio frame (e.g., 10ms). B is determined by the maximum length of CB-msg4 monitoring window in units of H-SFN duration. C is determined by the maximum value of floor(SFN_id/A)+1.

Type of CB-msg4

Proposal 10a: A UE Contention Resolution Identity MAC CE can be included in CB-msg4. UE checks whether the received UE Contention Resolution Identity matches the 48 first bits of the CCCH SDU transmitted in CB-msg3 and if matched, UE determines that contention is resolved.

Proposal 10b: For CP solution, if there is no data to be sent to the UE, UE Contention Resolution Identity MAC CE can be merely sent to early terminate the CB-msg3 EDT procedure, no need of RRC message together in the CB-msg4.

CB-msg4 multiplexing

Proposal 11: For CB-msg3-EDT procedure, if the CB-msg4 of multiple UEs are multiplexed into the same MAC PDU, the MAC CE or MAC SDU targeting for the same UE should be placed after the UE Contention Resolution Identity of that UE.

HARQ feedback for CB-msg4

Proposal 12: HARQ feedback should be given for each response in CB-msg4.

Reference

- RP-250472 Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#107, March 2025

Annex

For reference, the agreements in previous RAN2#125bis~RAN2#129bis meetings are listed as below:

RAN2#125bis agreements: - Both NB-IoT and eMTC are within scope of uplink capacity enhancements - Both C-plane and U-plane solutions are within scope of uplink capacity enhancements. - Only CIoT EPS is within scope of uplink capacity enhancements RAN2#126 agreements: - RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). - RAN2 can continue the discussion on Diversity Slotted ALOHA (DSA) and Contention Resolution Diversity Slotted Aloha (CRDSA) for Msg3-EDT transmissions without msg1/ RAR, evaluating possible impacts on the specification, in the next RAN2 meeting (RAN2 might send an LS to RAN1 later on this) - If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3. RAN2#127 agreements: - RAN2 will continue working on a CB-msg3 EDT-like mechanism. - RAN2 assumes that a DSA based scheme would not have RAN1 impacts, while RAN2 thinks that a CRDSA based scheme would necessarily have RAN1 impacts - In the next meeting continue the comparison (e.g. in terms of packet loss ratio, usage of UL/DL radio resources) between existing CB mechanism (Slotted Aloha) and other mechanisms (DSA, CRDSA, others) and try to decide on which way to go and in case whether/what to ask to RAN1. - For DSA and CRDSA, RAN2 can consider in the evaluation how to integrate them with repetition RAN2#127bis agreements: - RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19. - RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details. - CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA) - RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA) - At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission - The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA) - CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN. RAN2#128 agreements: - Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B) - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels) - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource. - RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started) RAN2#129 agreements: - It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used - RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC - RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - Frequency domain resources for (N)PUSCH occasions - repetition number - (N)PDCCH resource - MCS - For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. - The UE shall at most have one ongoing CB EDT procedure at any time. - The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. - The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. - CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. - As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. - RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature Working assumption: - One CB-MSG4 can target multiple UEs simultaneously (FFS on the details) - For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicasinside the window. RAN2 assumes that a pointer solution is not needed in Rel-19 RAN2#129bis agreements: Agreements (part 1) - Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT - We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure - For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). - We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case) - We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation. - If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission. - The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource. - The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same. - RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply (CB-Msg3-EDT configuration for eMTC) - For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2. - For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed). - We will not support TDD related parameters. - For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources). - For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration - For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (CB-Msg3 configuration for NB-IoT) - For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier. - For NB-IoT, introduce below physical layer parameters in shared resources configuration as below: - Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16) - Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16) - Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set. - MCS configuration for NPUSCH (as in npusch-MCS-r16). - PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13) - The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used. NOTE: confirm with RAN1 is needed - For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration Agreements (part 2) - For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). - The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure) - The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not). - FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring Agreements (part 3) - The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific. - The Msg4 monitoring window configuration (e.g. length) is CE level specific - RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized. - RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT - For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject - For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject. - RAN2 also intends to support CB-msg3-EDT for MT cases - The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI) - Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1 - The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4. - Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached. - Backoff information could be included in CB-Msg4. - L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported. - HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).