3GPP TSG-RAN WG2 Meeting #130	R2-2503501

St Julian’s, Malta, 19th – 23rd May, 2025

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues for PWS support in IoT NTN

Agenda Item:	8.9.4

Document for:	Discussion and Decision

Introduction

In RAN#102, a new WID for further enhancements on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) has been agreed. In RAN#105, the WID is further refined in [1] and a new objective for PWS support has been added as below:

The aim of this WI is enhancement of IoT-NTN with the following objectives: <<skip the unrelated parts>> - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: <skip the unrelated part> - Support of Capacity enhancements for uplink <skip the unrelated part> - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In RAN#107, the WID is further refined in [2] and a note was added “PWS notifications in paging, direct indication, and system information in RRC connected mode is not supported”. According to this clarification for WID objective, it can be seen that the NB-IoT cannot acquire PWS notifications when it’s in connected mode.

For this objective, in RAN2#127bis meeting, RAN2 focus the discussion on new SIB for PWS and new PWS indication in direct indication information, and achieved the following agreements:

- RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS - The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes. - For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields): - SystemInformationBlockType10-NB for primary ETWS notification; - SystemInformationBlockType11-NB for secondary ETWS notification; - SystemInformationBlockType12-NB for CMAS notification. - Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB): - etws-Indication; - cmas-Indication. - Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means) - Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

In RAN2#128 meeting, RAN2 focus the discussion on the remaining issues, and achieved the following agreements:

- We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. - We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed - Add the following PWS indication in Paging-NB: - etws-Indication; - cmas-Indication. - We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed - We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements. - Send a LS to RAN1 asking whether it’s possible to specify support for PWS reception in RRC_CONNECTED for NB-IoT in Rel-19.

In RAN2#129 meeting, the following agreements are achieved:

- Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells - Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information. - Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB - We don’t introduce a new SIB for carrying geographical information for emergency areas

In RAN2#129bis meeting, the following agreements are achieved:

- In case of PWS notification the network may release a RRC_CONNECTED UE to idle, in case the UE reports the capability to receive PWS in idle (no other spec impact other than the introduction of a UE capability for supporting PWS reception in idle)

In this document, we will discuss the remaining issues for PWS support and give our proposals.

Discussion

  - Scheduling info acquisition before PWS acquisition

In general, when the network changes (some of the) system information, it first notifies the UEs about this change, e.g., via paging message. And then in the next modification period, the network transmits the updated system information. But for ETWS notification and/or CMAS notifications, the network can transmit the SIB10 for ETWS primary notification and/or SIB11 for ETWS secondary notification, and/or SIB12 for one or more CMAS notifications in the nearest scheduled subframes, without waiting for the next modification period.

The corresponding UE behavior in the specification is as below:

TS 36.331 5.3.2.3 Reception of the Paging message by the UE //skip the unrelated part 1> if the etws-Indication is included and the UE is ETWS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary; 2> if the schedulingInfoList indicates that SystemInformationBlockType10 is present: 3> acquire SystemInformationBlockType10; NOTE 2: If the UE is in CE, it is up to UE implementation when to start acquiring SystemInformationBlockType10. 2> if the schedulingInfoList indicates that SystemInformationBlockType11 is present: 3> acquire SystemInformationBlockType11; 1> if the cmas-Indication is included and the UE is CMAS capable: 2> re-acquire SystemInformationBlockType1 immediately, i.e., without waiting until the next system information modification period boundary as specified in 5.2.1.5; 2> if the schedulingInfoList indicates that SystemInformationBlockType12 is present: 3> acquire SystemInformationBlockType12;

As above, the scheduling information for SIB10, SIB11 and SIB12 is contained in SIB1 (e.g., schedulingInfoList). In order to simplify the receiving process and quickly acquire PWS, some companies suggested that UE can bypass the acquisition of SIB1-NB prior to obtaining the PWS information. Instead, they suggest that NW by implementation can provide kind of pre-configured PWS scheduling information within SIB1-NB, even in the absence of an actual PWS notification. Meanwhile, UE could store the scheduling information in advance and utilize it to require SIB10~12 upon receiving of ETWS notification and/or CMAS notifications. However, due to the following reasons, we think the way is infeasible:

- Firstly, we think it is not practical for NW to determine pre-configured PWS scheduling info as the network has NO idea about the size of PWS contents.

- Secondly, even assuming that kind of fixed scheduling configuration can be provided by eNB, considering the implementation of eNB can adjust the scheduling for SIB based on the latest network load and the scheduling information may keep changed, then, when the UE receives PWS notification, if it uses previously stored (may even be stored a long time ago) scheduling information to obtain SIB10~12, the risk of message reception failure due to outdated scheduling information would be relatively high.

- Finally, considering PWS notification is rare case, to continuously provide its scheduling information when there is no PWS notification at all would be very bad for the aspect of signaling overhead.

So it’s suggested to still follow the legacy acquisition scheme, e.g., upon receiving of ETWS notification and/or CMAS notifications, ETWS or CMAS capable UE firstly needs to re-acquire SIB1 immediately without waiting until the next system information modification period boundary, and then acquire SIB10, SIB11 and SIB12 according to the latest scheduling information in SIB1.

Proposal 1a: It’s suggested to follow the legacy scheme to acquire SIB10-NB~SIB12-NB, e.g., upon receiving the PWS notification from eNB, the PWS-capable NB-IoT UE firstly needs to re-acquire schedulingInfoList contained in SIB1 immediately without waiting until the next system information modification period boundary, and then acquire the corresponding PWS messages.

Proposal 1b: No need to support any scheme of providing pre-configured scheduling information for SIB10-NB~SIB12-NB when there is no PWS notification.

  - Meeting PWS requirement for NB-IoT

According to the description in section “10.1.4 Paging and C-plane establishment” in TS 36.300, when extended DRX (eDRX) is used in idle mode, the following exceptional case about meeting ETWS, CMAS, PWS requirement is mentioned for LTE/eMTC UE regarding PWS function:

TS 36.300 10.1.4 Paging and C-plane establishment <skip the unrelated parts> When extended DRX (eDRX) is used in idle mode, the following are applicable: - …… - ETWS, CMAS, PWS requirement may not be met when a UE is in eDRX. For EAB, if the UE supports SIB14, when in extended DRX, it acquires SIB14 before establishing the RRC connection; - ……

Also according to TS 22.268, the requirement for ETWS is as below

5.2 Duration of delivery time Duration of the delivery time for PLMN operators is the time from the receipt of the Warning Notification by the PLMN operator, i.e. the edge of the 3GPP network, to the time that the Warning Notification is successfully delivered to the UEs. Provisioning of delivery of Primary and Secondary Notification may be required: - Primary Notification shall be delivered within 4 seconds to the UE in the Notification Area even under congestion situation. - Secondary Notification is delivered to the users in the Notification Area even under congestion situation.

For NB-IoT, the defaultPagingCycle value is {rf128, rf256, rf512, rf1024}. If the defaultPagingCycle is set as rf512, or rf1024, it is longer than 4s. In this case, the requirement of EMTS primary notification also can’t be satisfied. Furthermore, if PSM is enabled and UE enters into sleep mode, any notification will be missed. Hence, we suggest to add two above specific cases where ETWS, CMAS, PWS requirement may not be met.

Proposal 2a: For NB-IoT UE, the ETWS, CMAS, PWS requirement may not be met in some cases. Besides the case when the UE is in eDRX, there are also other cases, e.g., when the UE is configured with a large DRX cycle (e.g., rf512, rf1024), or when PSM is enabled.

Since in section “10.1.4”, only the case of using eDRX is listed, it may not be the suitable section to capture the above Proposal 2a. Further considering in the 36.300 running CR, a very general description about the limitations of supporting PWS in NB-IoT NTN has been introduced in section “4.10 NB-IoT”, we suggest to elaborate the description a bit as following:

Proposal 2b: It’s suggested to capture Proposal 2a in section “4.10 NB-IoT” in TS 36.300 running CR as below:

TS 36.300 4.10 NB-IoT NB-IoT provides access to network services using physical layer optimized for very low power consumption (e.g. full carrier bandwidth is 180 kHz, subcarrier spacing can be 3.75 kHz or 15 kHz). As indicated in the relevant clauses in this specification, a number of E-UTRA protocol functions supported by all Rel-8 UEs are not used for NB-IoT and need not be supported by eNBs and UEs only using NB-IoT. In this version of the specification, a number of functions including inter-RAT mobility, handover, measurement reports, GBR, CSG, support of HeNBs, relaying, carrier aggregation, dual connectivity, NAICS, real-time services, interference avoidance for in-device coexistence, RAN assisted WLAN interworking, sidelink communication/discovery, V2X sidelink communication, MDT, emergency call, CS fallback, ACB, EAB, ACDC, SSAC, aerial UE Communication, EN-DC and RRC_INACTIVE are not supported for NB-IoT. This is not further stated in the corresponding procedures. In NTN, public warning system requirements may not be met for some scenarios, e.g., when the UE is in eDRX, or when the UE is configured with a large DRX cycle (e.g., rf512, rf1024), or when PSM is enabled.

  - Acceptable cell/any cell camping for NB-IoT

In previous meeting, some companies [3] gave the observation that, in eMTC and normal LTE, a UE can camp on an acceptable cell to receive PWS broadcasting and originate emergency calls, meanwhile, NB-IoT UE support neither acceptable cell nor camped on any cell state as it doesn’t support emergency calls or PWS broadcasting.

- Considering that for NB-IoT NTN, it’s already agreed to introduce support for PWS reception, e.g., for the notification or rescue in emergency situations, also, there is plan to support voice call in future release, we think now it makes sense to introduce support for acceptable cell and also support for “Camped on Any Cell state” for NB-IoT NTN UE.

Proposal 3: It’s suggested to introduce support for acceptable cell and also support for “Camped on Any Cell state” for NB-IoT NTN UE.

  - PWS acquisition for UE in RRC_CONNECTED

In previous meeting, RAN2 has agreed that, upon reception of ETWS/CMAS notification, eNB could release the PWS-capable NB-IoT UEs in RRC_CONNECTED to RRC_IDLE to trigger them to acquire SIB10-NB~SIB12-NB messages in RRC_IDLE.

For this way, firstly NW needs to know the UE capacities, then NW can choose to only release the UEs that are capable of receiving PWS messages in idle state. In the current TS 36.306, the related capabilities for support of the ETWS and CMAS for LTE UEs are defined as follows:

6.2.1 ETWS It is optional for UE to support ETWS reception as specified in TS 36.331 [5]. 6.2.2 CMAS It is optional for UE to support CMAS reception as specified in TS 36.331 [5]. It is optional for a CMAS-capable UE to support Geofencing information (warningAreaCoordinates-r15) as specified in TS 36.331 [5].

In R19, those two capabilities can also be introduced for NB-IoT UE.

Proposal 4a: Two capabilities of supporting ETWS or CMAS reception can be introduced for NB-IoT UE.

Furthermore, the subsequent challenge after releasing the PWS-capable UEs is that, those UEs may trigger access to the network later almost at the same time, causing serious signaling storm problem.

To mitigate the potential signaling storm issue, one straightforward option can be based on network implementation, e.g., eNB configures different extendedWaitTime for different UEs via the RRC release message. However, considering that the eNB may not have clear knowledge of which UEs will need to re-access in the future, and also considering the number of UEs to be released may be large, it will be difficult for the eNB to set suitable waiting time for each UE.

Another option can be to include a waiting time range (e.g., similar as the backoff parameter in RACH procedure) in SIB or RRC release message. The UEs that are released due to PWS reception and need to re-access in the future can randomly select a waiting time within this waiting time range on their own, thereby also achieving an effect of distributing accesses from UEs.

Proposal 4b: It’s suggested RAN2 to discuss the following potion to mitigate the potential signaling storm issue caused by releasing UEs to idle state to acquire PWS messages:

- To introduce a waiting time range (e.g., similar as the backoff parameter in RACH procedure) in SIB or RRC release message to facilitate the UEs to randomly select a waiting time in this range on their own.

Conclusion

Based on the above analysis, the following proposals are given:

Proposal 1a: It’s suggested to follow the legacy scheme to acquire SIB10-NB~SIB12-NB, e.g., upon receiving the PWS notification from eNB, the PWS-capable NB-IoT UE firstly needs to re-acquire schedulingInfoList contained in SIB1 immediately without waiting until the next system information modification period boundary, and then acquire the corresponding PWS messages.

Proposal 1b: No need to support any scheme of providing pre-configured scheduling information for SIB10-NB~SIB12-NB when there is no PWS notification.

Proposal 2a: For NB-IoT UE, the ETWS, CMAS, PWS requirement may not be met in some cases. Besides the case when the UE is in eDRX, there are also other cases, e.g., when the UE is configured with a large DRX cycle (e.g., rf512, rf1024), or when PSM is enabled.

Proposal 2b: It’s suggested to capture Proposal 2a in section “4.10 NB-IoT” in TS 36.300 running CR as below:

TS 36.300 4.10 NB-IoT NB-IoT provides access to network services using physical layer optimized for very low power consumption (e.g. full carrier bandwidth is 180 kHz, subcarrier spacing can be 3.75 kHz or 15 kHz). As indicated in the relevant clauses in this specification, a number of E-UTRA protocol functions supported by all Rel-8 UEs are not used for NB-IoT and need not be supported by eNBs and UEs only using NB-IoT. In this version of the specification, a number of functions including inter-RAT mobility, handover, measurement reports, GBR, CSG, support of HeNBs, relaying, carrier aggregation, dual connectivity, NAICS, real-time services, interference avoidance for in-device coexistence, RAN assisted WLAN interworking, sidelink communication/discovery, V2X sidelink communication, MDT, emergency call, CS fallback, ACB, EAB, ACDC, SSAC, aerial UE Communication, EN-DC and RRC_INACTIVE are not supported for NB-IoT. This is not further stated in the corresponding procedures. In NTN, public warning system requirements may not be met for some scenarios, e.g., when the UE is in eDRX, or when the UE is configured with a large DRX cycle (e.g., rf512, rf1024), or when PSM is enabled.

Proposal 3: It’s suggested to introduce support for acceptable cell and also support for “Camped on Any Cell state” for NB-IoT NTN UE.

Proposal 4a: Two capabilities of supporting ETWS or CMAS reception can be introduced for NB-IoT UE.

Proposal 4b: It’s suggested RAN2 to discuss the following potion to mitigate the potential signaling storm issue caused by releasing UEs to idle state to acquire PWS messages:

- To introduce a waiting time range (e.g., similar as the backoff parameter in RACH procedure) in SIB or RRC release message to facilitate the UEs to randomly select a waiting time in this range on their own.

Reference

[1] RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc., RAN#105, September, 2024

[2] RP-250472, Revised WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. March, 2025

[3] R2-2502458 Acceptable cell camping for NB-IoT	Samsung, Iridium, Vivo, discussion, Rel-19, IoT_NTN_Ph3-Core, RAN2#129bis, April, 2025

[4] R2-2503306, Reply LS on maximum warning message size (C4-251432; contact: CICT Mobile), RAN2#130, May, 2025