3GPP TSG-RAN WG2 Meeting #130	R2-2503506

St. Julian’s, Malta, May 19th - 23rd, 2025

Agenda item:		8.18.2

Source:		Samsung

Title:	Configuration and scheduling aspects for LTE-based 5G Broadcast Phase 2

Document for:	Discussion & Decision

# 1 Introduction

RAN#106 (latest WID in [1]) approved a new work item on ‘Introduction of Time and Frequency Interleaving (TFI) for LTE-based 5G Terrestrial Broadcast’ with the following objective:

For MBMS-dedicated cells, specify time-frequency interleavers [RAN1] and corresponding signaling [RAN2, RAN3] - For the time-interleaving part of 5G Broadcast TFI, one transport block is mapped to multiple non-consecutive subframes, following the principles of NR TBoMS. The following steps are followed: - Scale the TBS by a factor of N, where N is the number of subframes over which the TB is mapped. - RAN1 to discuss the details of the scaling, the value(s) for N and the set of subframes over which the TB is mapped, including how to schedule the TBs. - Within one subframe, the transmitted rate matched bits are read consecutively from the circular buffer. - In a given subframe, the procedure in TS 36.212 Section 5.1.4.1.2 applies (except for the parameter k0k0 which needs to be modified according to point 3 below) - Before frequency interleaving (if applied), the mapping from coded bits to RE follows the current specification. - Across subframes belonging to the same TB, the starting point for reading from the circular buffer (k0(k0 in TS 36.212 Section 5.1.4.1.2) is to be decided by RAN1 - NOTE 1: The maximum TBS after scaling and soft buffer size should follow limitations imposed by current LTE categories. RAN1 to discuss how to capture this limitation. - NOTE 2: Time interleaving is not applied for PMCH carrying MSI and/or MCCH. - For the frequency-interleaving part of 5G Broadcast TFI, interleaving is performed via a row-column interleaving with the following steps: - Data symbols in an OFDM symbol are written column-wise. - RAN1 to decide what is the unit over which data symbols are written (e.g. a single RE, four REs, etc.) - The rows and elements in each row may be permuted, with details to be decided by RAN1. - Data symbols in an OFDM symbol are read row-wise. - RAN1 to decide the number of {rows, columns} of the interleaver. NOTE: Time-frequency interleaving is not applicable to 0.37kHz SCS.

In this paper we discuss the following aspects for Rel-19 5G Broadcast:

- Co-existence for Rel-19 5G Broadcast and legacy (Pre-Rel-19) 5G Broadcast

- Time Interleaver configuration

- Impact on MCH scheduling period

- Impact on MCH reception and MSI

- HARQ handling for Rel-19 5G Broadcast

- UE capability signalling for Rel-19 5G Broadcast

# 2 Discussion

Figure 1 depicts a time interleaving operation for Rel-19 LTE-based 5G Broadcast. The group of TBs can be referred to as pmch-TimeInterleaving-M or simply, M and the number of redundancy versions can be referred to as pmch-TimeInterleaving-N or simply, N. As shown, M TBs are time interleaved with N redundancy versions in a basic transmission unit of (M x N) MBSFN subframes, excluding MCCH and MSI subframes, in order to enhance the reception performance reliability.

Figure 1: Time Interleaver for R19 5G Broadcast – M TBs and N RVs

### Co-existence for Rel-19 5G Broadcast and legacy 5G Broadcast

As Time Interleaving is being introduced in R19, while 5G Broadcast has been specified since R15, it is essential that coexistence of legacy and R19 transmissions, and diverse UEs, be ensured in the new system.

Defining separate R19 PMCHs in addition to legacy PMCHs can ensure co-existence for legacy and new set of UEs which differ in their reception capabilities with respect to Time and Frequency Interleaving (TFI) based transmission, configurations and scheduling requirements. An extension list for R19 PMCHs (PMCH-InfoExt-r19) can provide a clean co-existence approach as the PMCH-InfoExt-r19 ASN configurations are not accessible to legacy UEs while legacy configurations can still be accessed, whereas new set of UEs can access both legacy configurations (w/o TFI) and R19 configurations (w/ TFI). The system can, thus, be judiciously configured to meet a diversity of UEs and transmission requirements on need basis.

Proposal 1: Co-existence for legacy and R19 transmission and UEs is achieved with defining R19 PMCHs to cater to Time interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs.

### Time Interleaver configuration

RAN1 Agreement [2]: The agreement in the previous meeting is updated as follows: Regarding configuration for time interleaving transmission, the following options can be considered: - Option 1: Per PMCH configuration, e.g., via pmch-Config - Option 2: Per MBMS session, e.g., via MBMS-SessionInfo or via MAC CE - Alt1: If per MBMS session configuration is done, a T_proc of 3 ms between the end of the MSI and the first MTCH in the MCH scheduling period is needed for UE de-rate matching. - Alt2: If per MBMS session configuration is done, other options (than Alt1 above) are not precluded, e.g. that allow the UE to know the M, N values of the first MTCH scheduled after the MSI

We understand the motivation for per session approach (Option 2) is more flexibility for Time Interleaving parameters M/N configuration per service. However, service requirements (mobility, coverage and reliability) and diversity of M/N configurations should be achieved by mapping service MTCH to appropriate PMCH (specifically, SCS/FFT size is configured per MBSFN area and data MCS is configured per PMCH). Moreover, broadcast system can provide a couple of PMCHs (up to 15 PMCHs allowed per MBSFN area) that can fulfil diverse transmission charateristerics.

There are the serious drawbacks with TI configuration per session approach:

- dataMCS is configured per PMCH and fundamentally limits the range of M and N values possible for the associated MTCHs. Conversely, if one service has large M and N, it will reduce dataMCS applicable for all other MTCHs, which is a big drawback.

- Tproc is not a desirable interruption and we can avoid this with per PMCH approach

- Excessive MCCH signalling burden with per session configuration. For example, a theoretical maximum 1024 sessions requiring a large increase in the size of MCCH information message with per session TI configuration. This may be drastic with requiring potential segmentation and spreading of MCCH message over more than one Transport Block, especially in case of smaller carrier bandwidths.

- Scheduler operation and reception operation is difficult with many different M, N based MTCHs in same MSI periodicity (e.g. if UE has to share HARQ resources between broadcast and unicast, it is difficult to pursue w/o prior knowledge of M and N in MSI period)

Therefore, we understand option 1 is more preferable (i.e., Per PMCH configuration for TI parameters, e.g., via pmch-Config). However, RAN2 should wait for RAN1 decision and accordingly, prepare the configuration signaling.

Further, we illustrate how RAN2 can define the time interleaving configuration for different options as follows:

Assuming option 1:

- Define configurable parameters pmch-TimeInterleaving-M (i.e., M value) and pmch-TimeInterleaving-N (i.e., N value) for a PMCH

PMCH-InfoList information element PMCH-InfoListExt-r19 ::= SEQUENCE (SIZE (0..maxPMCH-PerMBSFN)) OF PMCH-InfoExt-r19 PMCH-InfoExt-r19 ::= SEQUENCE { pmch-Config-r19 PMCH-Config-r19, mbms-SessionInfoList-r19 MBMS-SessionInfoList-r9, ... } PMCH-Config-r19 ::= SEQUENCE { <omitted> pmch-TimeInterleaving-v1900 SEQUENCE { pmch-TimeInterleaving-M-r19 ENUMERATED {4,8,16,32..}, pmch-TimeInterleaving-N-r19 ENUMERATED {2,4,8,16,..} } ... }

Assuming option 2:

- In order to have a signalling efficient approach, pmch-Config provides a list of Time Interleaver configurations and individual MBMS-SessionInfo carries an index to one entry of the pmch-TimeInterleaver-ConfigList

Proposal 2: Wait for RAN1 decision on the configuration for the time interleaving transmission and accordingly, incorporate in to the configuration signaling.

### Impact on MCH scheduling period

RAN1 Agreement [2]: In case of time interleaving, in order to reduce the resource wastage due to the potential mismatch between MSI periodicity and the (MxN) pattern: - Adopt an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs that includes the intermediate values among the periodicities defined for the legacy configurations. - May include the case of scheduling different M/N within the same MSI periodicity - FFS the values of the new periodicities. - RAN1 to specify UE behavior, if needed, in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity - Option 1: Drop the time-interleaved TBs which do not have all the N RVs transmitted within the MSI periodicity - Option 2: Drop the time-interleaved TBs which could not be successfully decoded. - It is up to network to decide whether to schedule or not schedule MTCH transmission in the residual mismatch part of the MSI periodicity. UE knows this from MSI (legacy behaviour) - Other Options not precluded

As agreed in RAN1, adopting an extended set of configurable periodicities provide a simple and effective way to reduce the resource wastage. MCH-Scheduling period is configured per PMCH and scheduler can readily accommodate corresponding M/N for service MTCHs of the PMCH.

For new periodicities, we understand it is up to RAN1 to do detailed analysis and adopt intermediate values that provides better match for typical M/N configurations. Accordingly, RAN2 can incorporate an extended set of periodicities for MCH scheduling period in the configuration signalling.

Proposal 3: RAN2 to incorporate an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs once RAN1 provides the relevant set of values.

For the case of residual mismatch, we understand following cases are to be considered:

Figure 1: Padding of the residual mismatch part (< M x N subframes) in the MCH Scheduling Period

Case 1: Network scheduler only transmits those number of MTCHs for which M/N transmission pattern can be fully accommodated within the MCH-SchedulingPeriod and remaining (residual mismatch) part of the MCH-SchedulingPeriod is unused / padded.

Figure 2: Transmission of last MTCH Z in the residual mismatch part (< M x N subframes) in the MCH Scheduling Period

Case 2: Network scheduler utilizes the remaining part to still schedule the last MTCH with partial M/N transmission pattern until the end of the MCH-SchedulingPeriod. UE drops TBs which could not be successfully decoded (successful decoding need not require all RVs) and processes the successfully decoded TBs. Performance may vary based on the amount of the mismatch and loss perceived by UE

We think for case 2, a satisfactory reception performance needs to be ensured and is dependent on amount of remaining part (residual mismatch) available in the MCH-SchedulingPeriod. In our view, the decision whether to schedule or not schedule MTCH transmission in the remaining part of the MCH-SchedulingPeriod can be left to network scheduler and if scheduled, UE needs to decode until the end of the MCH-SchedulingPeriod as per MSI (legacy behaviour).

That is, option 2 is suitable for the UE behaviour. Option 1 is not reasonable as UE should not drop time-interleaved TBs just on basis of not all the N RVs transmitted within the MSI periodicity, as successful decoding not necessarily require all RVs and UEs cannot pre-judge decoding result.

However, RAN2 should wait for RAN1 to decide on UE behaviour in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity and accordingly, if needed, RAN2 specify MAC protocol.

Proposal 4: Wait for RAN1 to decide on UE behaviour in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity and accordingly, if needed, RAN2 specify the MAC protocol.

### Impact on MCH reception and MSI

TS 36.321 5.12 MCH reception ... The MAC entity shall assume that the first scheduled MTCH starts immediately after the MCCH or the MCH Scheduling Information MAC control element or the Extended MCH Scheduling Information MAC control element if the MCCH is not present, and the other scheduled MTCH(s) start immediately after the previous MTCH, at the earliest in the subframe where the previous MTCH stops. …

In order to support basic transmission pattern of (M x N) MBSFN subframes for Time interleaver, it seems essential to bring a few enhancements for MSI as compared to legacy specifications (shown in spec snippet), including

- Not allowing multiplexing of two MTCHs in same subframe

- Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH

- Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to TI difference

- Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

As these enhancements are not backward compatible with legacy specifications, it is required that such enhancements are kept as minimal as possible and are applied only for scheduling of R19 PMCHs based MTCHs. Further, these enhancements need to be captured in the MAC specification.

Proposal 5: RAN2 to adopt and specify minimal multiplexing enhancements for MCH reception that are applied only for scheduling of R19 PMCHs based MTCHs and include:

- Not allowing multiplexing of two MTCHs in same subframe

- Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH

- Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference

- Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

### HARQ handling for Rel-19 5G Broadcast

Since HARQ buffers would be used for time-interleaved transmission for R19 5G Broadcast, there seems to be a need to clarify between new transmission and retransmission/repetition, allowing for soft combining and addressing HARQ feedback generation aspects. Below we proposed the impact to the MAC specifications to address this aspect.

#### 5.3.2.2 HARQ process For each TTI where a transmission takes place for the HARQ process, one or two (in case of downlink spatial multiplexing) TBs and the associated HARQ information are received from the HARQ entity. For each received TB and associated HARQ information, the HARQ process shall: - if the NDI, when provided, has been toggled compared to the value of the previous received transmission corresponding to this TB; or - if the HARQ process is equal to the broadcast process and if this is the first received transmission for the TB according to the system information schedule indicated by RRC; or - if the HARQ process is associated with a transmission indicated with a M-RNTI for FeMBMS PMCH (i.e. 5G Broadcast) and Time Interleaving is configured for PMCH, and this is the first received transmission for the TB according to the MTCH schedule indicated by MSI or extended MSI; or - if this is the very first received transmission for this TB (i.e. there is no previous NDI for this TB): - consider this transmission to be a new transmission. - else: - consider this transmission to be a retransmission.

#### 5.3.2.2 HARQ process … - if the HARQ process is associated with a transmission indicated with a Temporary C-RNTI and the Contention Resolution is not yet successful (see clause 5.1.5); or - if the HARQ process is equal to the broadcast process; or - if the HARQ process is not associated with a transmission indicated with a PUR-RNTI and the timeAlignmentTimer, associated with the TAG containing the serving cell on which the HARQ feedback is to be transmitted, is stopped or expired; or - if the HARQ procees is associated with a transmission indicated with a M-RNTI for the FeMBMS PMCH (i.e. 5G Broadcast) and Time Interleaving is configured for PMCH: - do not indicate the generated positive or negative acknowledgement to the physical layer. - else if the HARQ feedback is disabled for the corresponding HARQ process: - if harq-FeedbackEnablingforSPSactive is configured and the transmission is the first SPS PDSCH transmission after SPS activation: - indicate the generated positive or negative acknowledgement for this TB to the physical layer. - else: - do not indicate the generated positive or negative acknowledgement to the physical layer. - else: - indicate the generated positive or negative acknowledgement for this TB to the physical layer.

Proposal 6: RAN2 to specify the HARQ handling for Rel-19 5G Broadcast with regard to differentiating new transmission and retransmission/repetition, allowing for soft combining and addressing HARQ feedback generation aspects.

### UE capability signalling

RAN1 Agreement [2]: Support at least the following values of time interleaver depth: - Values M={4,8,16,32} and N={1,2,4,8,16} are supported. - FFS: UE behaviour in case values of M and N exceed UE capabilities in terms of soft buffer size and maximum TBS supportable in a TTI - FFS: larger values, including M=64, N={32,64}.

With M=32 and N=16, an interleaving depth of 512 ms can be achieved. Further larger values e.g., M=64 and N=32, 64 are prohibitively large for a majority of UEs considering the memory requirements. Therefore, it is preferable to exclude M=64, N=32, 64 values. Decision lies with RAN1.

Further, we think UE should report to the network the maximum supported values for M and N, depending on its implementation. In our understanding, the maximum TB supported for broadcast is already related to that supported for unicast (a scaling based relationship applies). However, we understand it is preferable for the UE to provide maximum supported value of M and maximum supported value of N based on UE specific implementation. At present, RAN2 should wait for RAN1 LS for capability parameters and accordingly, RAN2 specify the UE capability information signalling for Rel-19 5G Broadcast.

Proposal 7: Wait for RAN1 LS for capability parameters and accordingly, RAN2 specify the UE capability information signalling for Rel-19 5G Broadcast.

# 3 Conclusion

Based on the discussion above, request RAN2 to consider the following proposals:

Proposal 1: Co-existence for legacy and R19 transmission and UEs is achieved with defining R19 PMCHs to cater to Time Interleaving based configurations, scheduling and transmissions in addition to legacy PMCHs.

Proposal 2: Wait for RAN1 decision on the configuration for the time interleaving transmission and accordingly, incorporate in to the configuration signaling.

Proposal 3: RAN2 to incorporate an extended set of configurable periodicities for MCH scheduling period for R19 PMCHs once RAN1 provides the relevant set of values.

Proposal 4: Wait for RAN1 to decide on UE behaviour in case of residual mismatch even after new periodicities are adopted, taking into account the amount of subframe available at the end of the periodicity and accordingly, if needed, RAN2 specify the MAC protocol.

Proposal 5: RAN2 to adopt and specify minimal multiplexing enhancements for MCH reception that are applied only for scheduling of R19 PMCHs based MTCHs and include:

- Not allowing multiplexing of two MTCHs in same subframe

- Not applying Time interleaving to subframe carrying MSI/eMSI/MCCH

- Not allowing multiplexing of MTCH with MSI/eMSI/MCCH in a sub-frame due to Time interleaving difference

- Inserting and/ interpreting padding to account for remaining portion of the subframe in above scenarios

Proposal 6: RAN2 to specify the HARQ handling for Rel-19 5G Broadcast with regard to differentiating new transmission and retransmission/repetition, allowing for soft combining and addressing HARQ feedback generation aspects.

Proposal 7: Wait for RAN1 LS for capability parameters and accordingly, RAN2 specify the UE capability information signalling for Rel-19 5G Broadcast.

# 4 References

[1] RP-250794, “Revised WID on LTE-based 5G Broadcast Phase 2”, 3GPP TSG RAN Meeting #107, Incheon, Korea, March 2025

[2] R1-2503143, “Final summary on Rel-19 LTE-based 5G Broadcast Phase 2”, 3GPP RAN1 Meeting#120bis, Wuhan, China, April 2025 (EBU)