3GPP TSG-RAN WG2 Meeting #130											R2-2503511

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.5

Source: 	Sharp

Title:  	Open Issues on RLC Enhancements

Document for: 	Discussion

# Introduction

In RAN2#129bis meeting, RAN2 made the following agreements:

Agreements on RLC enahncements - When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

This document discusses remaining open issues on RLC enhancements.

# Discussion

## Autonomous Retransmission

The first issue is whether the autonomous retransmission should increase RETX_COUNT. In RAN2#129, RAN2 agreed to allow at most one autonomous retransmission for an SDU. Usually, the maximum number of RLC retransmissions is set to a very high number, e.g. 16 or 32, since the maximum retransmission triggers a radio link failure (RLF) which should be declared only in an extreme radio condition. We see that one additional count does not have any side effect. Also, the current running CR assumes reusing legacy behavior that any type of retransmissions increases RETX_COUNT. In this context, we can rely on the existing retransmission procedure and do not see a big necessity of the exceptional case.

Autonomous retransmission may increase RETX_COUNT, same as legacy retransmission.

## Autonomous Polling

In RLC running CR [1], there is an editor’s note on excessive polling, as follows:

Editor’s Note: It is still open on how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple.

The current procedure captured in the running CR is that a poll is triggered whenever an SDU has remaining timer below the remaining time threshold. It may trigger an excessive polling. However, even if multiple triggered polls, only one poll per RLC entity is transmitted by a MAC PDU. There might not be a critical problem if only one autonomous polling is triggered per SDU.

If RAN2 needs to consider an enhancement to avoid excessive polling, we may consider using a different set of polling parameters to be used when the remaining time is below the threshold. The new set with smaller pollPDU and pollByte can accelerate poll triggering and avoid the excessive polling.

RAN2 to consider one of the following options:

- Option 1: Only a single autonomous retransmission will be triggered per RLC SDU. (No other enhancement)

- Option 2: Another set of polling parameters is used when the remaining time is below the threshold.

## Unnecessary Retransmission Avoidance

In RLC running CR [1], the UE behaviour at expiry of t-PollRetransmit is captured as follows:

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured); or - consider any RLC SDU which has not been positively acknowledged for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured). - include a poll in an AMD PDU as described in clause 5.3.3.2.

The main change is to exclude SDU which has been already stopped from the candidate of polling, when the transmission buffer and retransmission buffers are empty, or window stalling happens. It is beneficial to select the RLC SDU which has not been stopped for transmission/retransmission to carry the poll. But as described in the open issue RLC-11, there is a problematic case that there is no available SDU when all the SDUs stopped for transmission/retransmission are excluded, for instance:

- For the case that both the transmission buffer and the retransmission buffer are empty, all the RLC SDUs or segments are stopped for transmission and retransmission.

- For the case of RLC window stalling, all the RLC SDUs or segments in retransmission buffer are stopped for retransmission.

In these cases, the current text prohibits any poll transmission at t-PollRetransmit expiry. It induces a big problem in RLC window stalling that the RX cannot detect any out-of-sequence reception anymore. If the transmitter does not receive any status report, the transmit window cannot move forward. This means that the window stalling cannot be resolved at all. This is considered as a broken operation that RAN2 should fix. Thus, sending a poll should be allowed regardless of presence of unstopped SDUs.

A poll is always transmitted at the expiry of t-PollRetransmit for the following two cases:

- In both the transmission buffer and the retransmission buffer, the transmission and retransmission of all the RLC SDUs or RLC SDU segments are stopped.

- In the retransmission buffer, the transmission and retransmission of all the RLC SDUs or RLC SDU segments are stopped when no new RLC SDU or RLC SDU segment can be transmitted.

When there is no other RLC SDUs or RLC SDU segments whose transmission and retransmission are not stopped, we need to allow retransmission of a stopped SDU:

When there is no other RLC SDUs or RLC SDU segments whose transmission and retransmission are not stopped, one of the following RLC SDUs are considered for retransmission:

- The RLC SDU which has been stopped for transmission or retransmission and with the highest SN among the RLC SDUs submitted to lower layer; or

- Any RLC SDU which has been stopped for transmission or retransmission and which has not been positively acknowledged.

In RAN2#129bis meeting, RAN2 agreed that the expiration of t-RxDiscard triggers an RLC status report. The main reason was that the status report should let the TX know that there are abandoned SDUs at the RX to avoid the unnecessary retransmission. In this case, it is reasonable to set t-StatusProhibit to a very small value or zero.

In case of a very small value of t-StatusProhibit timer, there is an issue of duplicate retransmissions due to close SR triggerings. Since two SRs are triggered by t-reassembly and t-RxDiscard independently, both triggered SRs could be close to each other. The second status report tiggered by t-RxDiscard could arrive during ongoing HARQ retransmission based on status report triggered by t-reassembly. In Fig. 1, it is assumed that both status reports triggered by t-reassembly and t-RxDiscard request the RX to consider SN=5, 7, and 8 for retransmissions. But the first retransmission of SN=5, 7, and 8 can be ongoing in HARQ retransmission. The second retransmission is another unnecessary duplicate retransmission.

Fig. 1 Duplicate retransmissions

We see that this side-effect should be resolved. An easiest way could be that the status report triggered by t-RxDiscard does not trigger a retransmission.

Status report triggered by t-RxDiscard shall not trigger a retransmission.

# Conclusions

Based on the discussion above, RAN2 is requested to agree the following proposals:

- Autonomous retransmission may increase RETX_COUNT, same as legacy retransmission.

- RAN2 to consider one of the following options:

- Option 1: Only a single autonomous retransmission will be triggered per RLC SDU. (No other enhancement)

- Option 2: Another set of polling parameters is used when the remaining time is below the threshold.

- A poll is always transmitted at the expiry of t-PollRetransmit for the following two cases:

- In both the transmission buffer and the retransmission buffer, the transmission and retransmission of all the RLC SDUs or RLC SDU segments are stopped.

- In the retransmission buffer, the transmission and retransmission of all the RLC SDUs or RLC SDU segments are stopped when no new RLC SDU or RLC SDU segment can be transmitted.

- When there is no other RLC SDUs or RLC SDU segments whose transmission and retransmission are not stopped, one of the following RLC SDUs are considered for retransmission:

- The RLC SDU which has been stopped for transmission or retransmission and with the highest SN among the RLC SDUs submitted to lower layer; or

- Any RLC SDU which has been stopped for transmission or retransmission and which has not been positively acknowledged.

- Status report triggered by t-RxDiscard shall not trigger a retransmission.

# Reference

[1] R2-2503620	RLC running CR for R19 XR, vivo